| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0291 |
| --- | --- |

| Title: | CE2.h related: Crosscheck on Enhanced Disparity Vector Prediction (D0170) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | X. Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal D0170. The source code was implemented based on HTM-6.0. We compiled, inspected, and ran the code of these cases with CTC and all-intra configurations. The cross-check has been completed successfully and the results match those provided by proponents of JCT3V- D0170.

# Test description and crosscheck results

This document reports the cross-check results of D0170. The setting of the test case is set as follow:

#define     FW_DV_EST_ZJU       1

#define     ZJU_ADAP_DMV_POS   1

The encoder and decoder reconstructed YUV files generated from the above settings are matched. The experimental results of the rate-distortion are illustrated in Error: Reference source not found, which shows the test results of the CTC. These results match the BD-rate results provided by proponents of JCT3V- D0170.

Table 1: HTM6.0 anchor vs. proposed method in D0170 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.3% | -0.1% | -0.07% | -0.05% | -0.10% | 100.3% | 105.9% | 103.0% |
| Kendo | 0.0% | -0.1% | -0.2% | -0.06% | -0.05% | -0.01% | 99.2% | 94.6% | 98.4% |
| Newspaper_CC | 0.0% | -0.3% | -0.2% | -0.08% | -0.06% | -0.11% | 99.7% | 98.9% | 93.5% |
| GT_Fly | 0.0% | -0.1% | -0.1% | -0.03% | -0.04% | -0.08% | 99.9% | 97.6% | 101.6% |
| Poznan_Hall2 | 0.0% | -0.2% | -0.4% | -0.16% | -0.13% | -0.19% | 99.9% | 100.1% | 100.7% |
| Poznan_Street | 0.0% | 0.1% | 0.1% | 0.03% | 0.01% | 0.00% | 123.0% | 102.1% | 116.2% |
| Undo_Dancer | 0.0% | -0.2% | -0.4% | -0.09% | -0.10% | -0.64% | 136.9% | 96.5% | 100.6% |
| 1024x768 | 0.0% | -0.2% | -0.1% | -0.07% | -0.05% | -0.07% | 99.7% | 99.8% | 98.3% |
| 1920x1088 | 0.0% | -0.1% | -0.2% | -0.06% | -0.07% | -0.23% | 115.0% | 99.1% | 104.8% |
| average | 0.0% | -0.1% | -0.2% | -0.07% | -0.06% | -0.16% | 108.4% | 99.4% | 102.0% |