| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0294 |
| --- | --- |

| Title: | 3D-CE7.a: Cross-check of skip flag coding in 3D-AVC (JCT3V-D0115) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy (Nokia) | Email: | dmytro.rusanovskyy@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document presents results of a crosscheck of JCT3V-D0115 proposal from Samsung. Cross checker confirmed that the software and the text provided by the proponent are consistent and simulation results produced by cross-checker are matched to those reported by proponents in D0115.

# Introduction

Proposal D0115 describes a new method of coding of skip flags in dependent views of texture and all views of depth implemented. It is proposed to use run-length coding for compact representation of mb_skip_flag series in dependent views and all depths of a 3D video sequence which is realized by introducing a new syntax element. The proposal is integrated in the 3D-ATM 7.0 reference test model and simulated under the JCT-3V CTC. Simulation results presented in D0115 shows that proposed technique provides about 1% of dBR reduction on average for coded MVD and synthesised views.

# Cross-check results

- Coding and view synthesis results produced with proponent’s software were found to be identical to results reported in D0115.

- All coded bitstreams were found to be decodable and no mismatch was observed between reconstructed and decoded YUV files.

- Execution time reported in D0115 has been confirmed for decoder with accuracy 7% on average. However, significant fluctuations are observed, probably due to the cluster shared use.

- Analysis of the proposed algorithm shows that computation complexity for the decoder is expected to be very close to the anchor complexity.

Table 1. Simulation results produced with D0115 in Nokia

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -1.94 | 0.06 | -6.62 | 0.36 | -2.59 | 0.08 | -2.65 | 0.08 |
| S02 | -1.20 | 0.04 | -4.12 | 0.18 | -1.55 | 0.05 | -1.57 | 0.05 |
| S03 | -0.18 | 0.01 | -1.44 | 0.11 | -0.30 | 0.01 | -0.33 | 0.01 |
| S04 | -0.19 | 0.01 | -1.12 | 0.06 | -0.41 | 0.01 | -0.42 | 0.01 |
| S05 | -0.10 | 0.01 | 2.06 | -0.10 | -0.26 | 0.01 | -0.23 | 0.01 |
| S06 | -0.50 | 0.02 | 0.26 | -0.01 | -0.72 | 0.03 | -0.76 | 0.03 |
| S08 | -0.54 | 0.02 | -1.28 | 0.06 | -0.73 | 0.03 | -0.67 | 0.02 |
| Average | -0.66 | 0.02 | -1.75 | 0.10 | -0.93 | 0.03 | -0.95 | 0.03 |

Table 2. Execution time estimates, D0115 vs. the anchor.

| Complexity estimate (ratio to anchor) |  |  |
| --- | --- | --- |
| Encoder Time, % | Decoder Time, % | Rendering Time, % |
| 153.54 | 106.04 | N/A |
| 145.84 | 85.73 | N/A |
| 160.79 | 91.67 | N/A |
| 161.03 | 74.75 | N/A |
| 169.45 | 104.68 | N/A |
| 214.46 | 141.12 | N/A |
| 167.35 | 144.15 | N/A |
| 167.49 (To be confirmed) | 106.88 | N/A |

# Reference

- JCT3V-C1100, “Common Test Conditions of 3DV Core Experiments”