| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0301 |
| --- | --- |

| Title: | AHG 10 Overview of Algorithmic Intrinsic Complexity Measures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gwo Giun Lee1 T. Ikai2 K. Rapaka3 D. Rusanovskyy4 | Tel: Email: | 1+886-6-2757575 ext 62448 clee@mail.ncku.edu.tw ikai.tomohiro@sharp.co.jp krapaka@qti.qualcomm.com dmytro.rusanovskyy@nokia.com |
| Source: | 1National Cheng Kung University 2SHARP Corporation 3Qualcomm Incorporated 4Nokia Corporation |  |  |

_____________________________

# Abstract

This document provides an informative overview on the intrinsic complexity metrics, in terms of the number of operations, data storage requirement, data transfer rate, and degree of parallelism. The complexity metrics can be used and are transparent to either software and hardware. Since these metrics are platform independent, they convey information on the intrinsic complexity of the algorithm which does not rely on any implementation consideration. Hence in the present context, algorithm refers to the computational model

# Intrinsic Complexity Metrics

The complexity metrics should be transparent to hardware and software implementations. In addition, the complexity metrics are also needed to be indicative of the software, hardware or system architecture requirements. In contrast to the above description, the complexity metrics should be known early in the design phase to avoid changes the design in the lower level or later stages. Hence, four complexity measures will be studied and described in complexity assessment.

## Number of operations

This metric calculates the number of each type of operations, including (I) addition/subtraction, (II) multiplication, (III) division, (IV) shift, and (V) logic operations which are the basic operators used commonly in signal processing algorithms.

Additions and subtractions are considered to be operators having equivalent complexities since they can be implemented using two's complement and the complexity in using a not operation and extra addition of one is almost negligible.  A similar argument extends to describing logical operators such as larger than (>), smaller than (<), and the equality (=) operators as being equivalent in complexity to addition and subtraction.  The threee previous logical operations can be performed by checking the sign bit after subtraction which again has negligible complexity.  Multipliers are more complicated than adders as additions are also needed in the multiplication process.  Shift operators are consider the least complex since they can be performed by moving the corresponding significant bits.

Two operand operations with constant input and variable input should be differentiated to provide better accuracy in the estimation of complexity. E.g. X + Y is more complex than X + 5 since Y is a variable where as the second operand 5 is a constant. Another example is that X×Y is more complex than X×5.

Furthermore, bit precision of each operands discussed above should also be taken into account, since it can significantly influence the complexity especially in many standards having 4:4:4 data format with more than 10 bits precision.

## Data storage requirement

Data storage requirement is transparent to hardware/software but depends on the processing order or dataflow of the algorithm. This can be obtained by analyzing the lifetime of the required input data. For this category, a good measurement is to estimate the number of data accesses from the potential data storage device.

It is important to distinguish between data storage which is algorithmic intrinsic and the memory configuration or requirement which is dependent upon implementation issues.  For example, different signal processing systems or platforms may use SDRAM from different companies with different specifications for the cross page reference.  Hence this may result in different design of clock speeds due to efficiency and real time concerns even for the same data size resulting in unfair complexity comparisons.  Hence it is significant to stay with algorithmic intrinsic properties and also not to be biased by different platforms having different physical constraints.

## Data transfer rate

The average data transfer rate needs to be distinguished from the instantaneous or peak data transfer rate. The average data transfer rate is algorithmic intrinsic and conveys information on the general data required to be transferred. This can be measured by the amount of data transferred in one second which is a very large length of time. On the other hand, the instantaneous or peak data transfer rate indicates the amount of data required to be transferred within an extremely short timing window.  Hence, the instantaneous or peak data transfer rate is an architecture or platform dependent metric which is affected by (I) types of data transaction, (II) memory hierarchy, (III) data alignment in memory, and (IV) datapath architecture. With the help of the instantaneous or peak data transfer rate, the designer is provided with the insight of clock speed and bit widths of the interconnect bus

## Degree of parallelism

Based on the data dependency, this metric on the degree of parallelism characterizes the potential for which the operations can be executed concurrently. The metric defined here is capable of quantifying the degree of parallelism based on the dataflow graph models [1] and also provides a systematic methodology in the assessment of the complexity of algorithms using spectral graph theory.

As an example, Fig. 1 illustrates a simple dataflow graph with three nodes.  In the dataflow graphs, nodes represent processing elements (PE) which could be simple operations such as addition, multiplication or combinations of these basic operations.  The non-directed edges represents the dependency between the data to be processed by the PEs at the corresponding data granularity.  For ease of discussion the data granularity in the current example is merely one data sample  The corresponding adjacency matrix is described in Eq. , with the definition of adjacency matrix being defined in Eq. .

Fig. 1 Dataflow graph

In Eq.  the Laplacian matrix is defined as L=D-A, with D being a diagonal matrix and each diagonal element representing the number of edges connected to that corresponding node. Thus, the Laplacian matrix of Fig. 1 is described in Eq. .

When the graph consists of only one connected component, the spectrum of the Laplacian matrix will reveal only one eigenvalue whose value is zero. In Fig. 2, a graph having several connected components, has the spectrum of the graph is the union of the spectra of its connected components. The number of connected components in the graph is equal to the number of eigenvalue with value being zero.

Fig. 2 The cases of dataflow representation

Eq.  describes the simple computational model or algorithm provided here as a simple example.  The corresponding dataflow graph is shown in Fig. 3. The Laplacian mrtrix can then be constructed as shown in Eq. .

Fig. 3 Dataflow graph

The spectrum of this Laplacian matrix is shown in Eq. . From the calculated eigenvalues, there are two eigenvalues whose values are zero. Hence there exists two connected components in the graph which indicated that potentially these two components can be executed in parallel either in software or hardware. Therefore, the degree of parallelism is quantified to be two.

Furthermore the eigenvector corresponding to zero eigenvalue can further provide information of the vertices belonging to the same connected component as can be seen in Eq.  and Fig. 3.

Fig. 4 Quantified independent component set (Homogenous components)

This approach provides a systematic way to quantify the parallelism of algorithms and the spectrum of Laplacian matrix of a graph is invariant irrespective to the ways or orders in which the vertices are labeled. This is illustrated in the example in Fig. 5. Different numbering order of the graph was shown to result in the same spectrum of the Laplacian matrix.

Fig. 5 Invariant properties of degree of parallelism quantification by spectrum of Laplacian matrix

Note:  It is important to note that:

- All four complexity measures are required during complexity assessment.

- The data granularity used in the dataflow model during complexity assessment will result in different implementations

# Reference

- Gwo Giun Lee, He-Yuan Lin, Chun-Fu Chen, Tsung-Yuan Huang, "Quantifying Intrinsic Parallelism Using Linear Algebra for Algorithm/Architecture Co-Exploration," IEEE Transactions on Parallel and Distributed Systems, Vol. 23, Iss. 5, pp. 944-957, May. 2012.

- Gwo Giun (Chris) Lee, Yen-Kuang Chen, Marco Mattavelli, and Euee S. Jang, "Algorithm/Architecture Co-Exploration of Visual Computing: Overview and Future Perspectives, " IEEE Trans. on Circuits and Systems for Video Technology. Vol. 19, Iss.11, pp. 1576-1587, Nov. 2009.

- Gwo Giun Lee, He-Yuan Lin, and Sun-Yuan Kung, "Algorithm/Architecture Co-exploration," in Multimedia Image and Video Processing, Second Edition, ed: CRC Press, 2012, pp. 573-608.

# Patent rights declaration(s)

NOTE – Activities in the JCT-3V and contributions to the JCT-3V are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-3V proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. Two examples of such an informal IPR notification statement for a contribution are provided below.)

NCKU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).