| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0304 |
| --- | --- |

| Title: | 3D-CE6.h related: Crosscheck results on signalling and DPCM coding of depth lookup table (JCT3V-D0150-v4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Li Ping Wu No. 50 Ruanjiandadao Road, ZTE R&D Center District II, Nanjing, China 210012 | Tel: Email: | +86-25-88014644 li.ming42@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

Method for signalling and DPCM coding of depth lookup table were proposed in JCT3V-D0150, which is a combination of slice-level signalling (JCT3V-D0147) and DPCM coding (JCT3V-D0142) of DLT. In this document, the crosscheck results of this method are presented, and no mismatches are identified. The cross-checkers also reviewed and studied the source codes from the proponents, and verified that the proposed approach was correctly implemented on HTM6.0 which matched the descriptions in JCT3V-D0150-v4. The process of populating the DLT data in I-slice to its following P-slices and B-slices are specified in the proposed text in JCT3V-D0147-v4.

# Introduction

Method for signalling and DPCM coding of depth lookup table were proposed in JCT3V-D0150. The proposed method signals depth lookup table (DLT) elements in slice segment headers of I-slices. The DLT elements are DPCM coded and used for the B-slices and/or P-slices following the I-slices. In this document, the implementation of the proposed method was crosschecked, and the test results matched the ones presented in JCT3V-D0150.

The source codes were reviewed and studied. The proposed approach was correctly implemented on HTM6.0 as described in JCT3V-D0150-v4. The process of populating the DLT data in I-slice to its following P-slices and B-slices are specified in the proposed text in JCT3V-D0147-v4.

# Results

The source code was compiled using Microsoft Visual Studio 2008 on a server with 64-bit Window Server 2003 system. The crosscheck simulations were carried out on servers with the same OS. There are large fluctuations in the values of the run-time, especially in the CTC cases in Table 1. No mismatches were identified in the bit-rate and PSNR values.

Table 1.  Results for 3-view case in common test conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.01% | 0.03% | 0.04% | 84.7% | 86.0% | 31.3% |
| Kendo | 0.0% | 0.0% | 0.0% | -0.01% | 0.03% | 0.05% | 97.1% | 82.8% | 34.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.02% | 106.4% | 81.3% | 33.9% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 99.5% | 78.0% | 44.3% |
| Poznan_Hall2 | 0.0% | -0.5% | -0.4% | -0.23% | -0.26% | -0.22% | 92.6% | 85.3% | 42.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 108.7% | 85.4% | 47.2% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 96.6% | 79.6% | 41.4% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.04% | 96.1% | 83.3% | 33.2% |
| 1920x1088 | 0.0% | -0.1% | -0.1% | -0.06% | -0.07% | -0.06% | 99.3% | 82.0% | 43.8% |
| average | 0.0% | -0.1% | -0.1% | -0.03% | -0.03% | -0.02% | 97.9% | 82.6% | 39.2% |

Table 2.  Results for 3-view case in all intra test conditions

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.21% | 0.21% | 103.0% | 101.1% | 108.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.09% | -0.10% | 109.2% | 98.4% | 91.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.17% | 0.17% | 110.2% | 110.3% | 103.2% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 96.2% | 99.4% | 96.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.19% | 0.14% | 103.3% | 92.9% | 109.2% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 96.4% | 96.9% | 100.4% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 103.5% | 102.0% | 102.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.16% | 0.09% | 107.5% | 103.2% | 100.8% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.05% | 0.04% | 99.9% | 97.8% | 101.9% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.00 | 0.00 | 103.1% | 100.1% | 101.4% |

# Conclusions

The obtained results matches the ones presented JCT3V-D0150 under CTC and all intra configurations for HTM6.0. The software was reviewed and identified as correct implementation. The process of populating the DLT data in I-slice to its following P-slices and B-slices are specified in the proposed text in JCT3V-D0147-v4.