| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0450_v5 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0309_v5 |

| Title: | Report of the joint BoG on high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA Jill Boyce 433 Hackensack Ave Hackensack, NJ 07726 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com jill@vidyo.com |
| Source: | BoG chairs |  |  |

_____________________________

# Abstract

This document provides a report of the JCT-VC and JCT-3V joint break-out group (BoG) on high-level syntax for 3DV and scalable extensions of HEVC.

Version 1 of this report includes reviewing notes for the BoG sessions on Sunday April 21, 2013. After the Sunday sessions, there were about 40 documents that have not been reviewed.

Version 2 of this report additionally includes reviewing notes for the BoG sessions on Monday April 22, 2013. After the Monday session, there were 30 documents that have not been reviewed.

Version 3 of this report additionally includes reviewing notes for the BoG sessions on Tuesday April 23, 2013. After the Tuesday sessions, there were 7 documents that have not been reviewed (6 on parallel processing 1 on HRD), in addition to 20 revisits, and there were 16 items the BoG decided to recommend for adoption and 2 editorial items the BoG decided to recommend for editors’ consideration.

Version 4 of this report additionally includes reviewing notes for the BoG session on Wednesday April 24. Two more documents were added into the list. Only one document remained to be reviewed, and 3 additional revisits were added, no new adoption recommendations were made.

Version 5 of this report additionally includes reviewing notes for the BoG session on Thursday April 25. For those proposals that were submitted to both JCT-VC and JCT-3V, each of the JCT-3V document numbers is also mentioned under the notes of the corresponding JCT-VC document.

In the future, it would be much easier to map the JCT-VC document numbers and the corresponding JCT-3V document numbers if proponents can include, the abstract, the corresponding document number submitted to the other JCT. For example, "JCT3V-D1000 contains the same proposal" in the abstract of the corresponding JCTVC document, and vice versa.

Totally there are 58 documents listed below, 9 of which have been submitted only to JCT-3V. These 9 documents are highlighted in turquoise.

# High-level syntax issues of joint interest with JCT-3V (3)

JCTVC-M0045 High-level syntax modifications for HEVC extensions [J. Boyce, D. Hong, W. Jang (Vidyo)]

This document contains the same proposal as in JCT3V-D0059.

Several high level syntax design philosophy and/or syntax changes common to extensions to HEVC, including SHVC, MV-HEVC and 3D-HEVC, are proposed, with respect to the SHVC Test Model JCTVC-L1007, SHVC Working Draft JCTVC-L1008, MV-HEVC PDAM JCT3V-C1004, and 3D-HEVC Test Model JCT3V-C1005.

The proposals are:

- Allow different frame rate for different layers

Agreed for SHVC. The proposal seems to be SHVC only. Text related to AU boundary detection and related to AUD should be clarified, e.g. when a version 1 decoder is fed with a multi-layer bitstream with different picture rates for different layers.

- Define an access unit as containing pictures from multiple layers, as in SVC and MVC

Overlaps with JCTVC-M0201, JCTVC-M0204, and JCTVC-M0264. See notes elsewhere.

- Conditionally include VPS extension syntax elements based on scalability_mask[ i ]

JCTVC-M0268 covers the same particular topic on target output layer set signalling.

At the moment we don't have any syntax element in the VPS extension that would be present for one extension not the other. No action at this time of point. However, when in the future the above situation occurs, this could be considered.

- Add extension_type_flag[ i ] syntax elements in SPS extension and use for conditional syntax elements

This would work only when all the extensions are finalized at the same time.

It is considered desirable to avoid parsing of SEs in SPS extension that is not needed for a particular extension, e.g. inter_view_mv_vert_constraint_flag, by an SHVC decoder. Currently, it is not clear whether there will be and, if yes, what SEs will be included in SPS extension. This should be kept in mind. No action at this meeting.

- Conditionally include syntax elements in slice segment header based on ViewId[ nuh_layer_id ], DepthFlag[ nuh_layer_id ], DependencyId[ nuh_layer_id ]

The generic mechanism seems desirable. Currently, it is not clear what exact SEs should be conditioned that way. This should be kept in mind. No action at this meeting.

- Profile constraints for Stereo main and proposed Scalable main profiles to disallow scalability combinations

Agreed in principle on not to support mixed scalability types for now. Concrete language to be worked out by the editors.

Also proposed to include a bare minimum definition of the Scalable Main profile into the SHVC WD. Agreed.

JCTVC-M0167 Parameter set and other related issues [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)] [late]

This document contains the same proposal as in JCT3V-D0240.

For HEVC extensions, e.g. MV-HEVC, SHVC, and 3D-HEVC, the parameter set properties and cross-layer alignments are proposed, with the new definitions, group of layer and independent group of layer. A group of layer is simply a group of layer within a bitstream. A complete group of layers can be defined as a new term, namely independent group of layer, where any layer in an independent group of layer has a dependency from another layer in the same independent group of layer, while a layer in an independent group of layer has no dependency from any layer out of the independent group of layer. The parameter sharing or prediction across layers may be supported in both MV-HEVC and SHVC. However, the parameter sharing or prediction across independent group of layers are not desirable, because any parsing or decoding dependency across independent group of layers may interfere a bitstream extraction and a separated management of simulcast streams. Moreover, IRAP or POC alignments within an independent group of layer will give more flexibility than alignment for a whole layers. The text on the activation of VPS will be also proposed to be aligned with the proposed definitions.

- Disallow cross-layer parameter set sharing in certain scenarios. No benefit. No action.

- Require cross-layer IDR alignment for an independent set of layers. No action.

- No requirement of cross-layer alignment of IRAP pictures across independent sets of layers. No action with respect to IDR and CRA. For the BLA aspect, revisit at a larger group together with other related proposals in JCTVC-M0206, JCTVC-M0207, and JCTVC-M0266.

- On POC alignment: Resolved elsewhere.

- On VPS activation: VPS is an all-layer thing - using of more than one VPS for one CVS is directly against the design intent. No action.

JCTVC-M0208 AHG9: Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)]

This document contains the same proposal as in JCT3V-D0081.

This document proposes comments on SHVC Draft Text 1 and MV-HEVC Draft Text 3.  The following are proposed:

- A restriction on inter-layer reference picture set RefPicSetInterLayer of a random access decodable leading (RADL) picture is proposed.

- A restriction on the IDR and BLA NAL unit types for pictures with the same PicrOrderCntVal value is relaxed to apply to individual groups of layers considering the layer dependency.

- A change in the restriction on the NumPocTotalCurr value of CRA pictures is proposed.

- Proposal#1: The BoG agreed that it is desirable to allow for an AU to contain both RASL and RADL pictures, and when this occurs, it is restricted that a RADL picture shall not use a RASL picture for inter-layer prediction. How exactly this restriction is realized depends on the inter-layer prediction signalling mechanism, for which there are a number of related proposals.

Revisit the exact text once JCTVC-M0458 is concluded.

- Proposal#2: Same as item 3 in JCTVC-M0167. No action with respect to IDR.

For the BLA aspect, revisit at a larger group, together with other related proposals in JCTVC-M0206, JCTVC-M0207, and JCTVC-M0266.

- Proposal#3: It was agreed to clarify in the WD that the value of NumPocTotalCurr shall be equal to 0 for a BLA or CRA picture if nuh_layer_id is equal to 0.

# Random access and layer switching structures (2)

JCTVC-M0170 Temporal layer management for HEVC extension [B. Choi, Y. Cho, M. W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

This document contains the same proposal as in JCT3V-D0243.

- The nuh_layer_id values of TSA/STSA NAL units can be greater than 0.

Already supported.

- TSA/STSA NAL units need not be aligned across layers.

Currently it is this way. JCTVC-M0266 includes a proposal to require cross-layer alignment of TSA/STSA pictures.

- On allowing different TemporalId values for VCL NAL units within one AU

Revisited later on with a diagram showing the possibility of having different TemporalId values of VCL NAL units within one AU.

Whether to allow for the highest layer ID to change across AUs within a CVS? In SVC, this is disallowed. See notes under JCTVC-M0040.

No action.

- On Pictures with a smaller TemporalId in enhancement layers cannot refer to pictures with a greater TemporalId in referenced layers.

This is already the case, e.g. required by the sub-bitstream extraction process.

- Allowing different picture rates for different layers.

Currently allowed. See notes elsewhere (under JCTVC-M0045).

- A flag signalled in VPS is proposed, which indicates whether temporal rates and TSA/STSA are synchronized across layers.

Revisit at a larger group, together with related proposals, e.g. in JCTVC-M0266.

JCTVC-M0199 On flexible stream switching [P. Lopez, P. Bordes, F. Hiron (Technicolor)]

This contribution proposes to extend SHVC to support enhancement of already decoded pictures in the base layer DPB in order to enable stream-switching at any points (not only CRA) and hence claims to increase the stream-switching flexibility. It is proposed sequence level flags for SHVC to indicate the stream switching operation and describe the decoding process when stream switching occurs. Experiment results are provided which compare the performance of stream switching with and without the proposed method.

The proposed mechanism breaks the constraint that all pictures having the same output time shall be continuous in decoding order. Also this requires allowing for the highest layer ID to change across AUs within a CVS. No action.

# (6.4.4) Parameter sets (8)

## Parameter set prediction (5)

Chaired by J. Boyce

Overview table presented by T. Rusert.

Really inheritance, not prediction since no differences signaled.

Table shows groups of parameters which can be inherited separately in various contributions.

Motivation for parameter set prediction is bitrate savings in the SPS, achievable when multiple layers share parameters.  .Another possible benefit is session setup delay reduction, because of smaller parameter sets.

Groups of parameters:

- Representative format (picture size, etc.) likely to be identical for SNR scalability, view scalability.

- Scaling list – likely to be identical in layers, since selection based on temporal motion which is common,

- Other coding tools – usually for enabling/disabling coding tools, not a lot of bits

- RPS – likely to be identical in layers but might be different if frame rate varies between layers

Current design allows multiple layers to refer to same SPS, could typically be used for multi-view and SNR scalability, especially since profile_tier_level not sent in SPS for enhancement layers.

For now camera parameters for 3D are in the SPS extension, which would make it more difficult for layers to refer to same SPS.

Discussion about how parameter set inheritance impacts decoding process.  Some of the methods affect the parameter set activation process.

Major approaches are VPS to SPS prediction and SPS to SPS prediction.  Both approaches have some pros and cons.  A participant expressed interest in having a greater justification of bit savings provided or other benefits to justify adoption of either approach.

Revisited when some scaling list bit estimates were made available.

Some bit estimate data provided by O. Nakagawa, will be attached in a revision of JCTVC-M0211.  Worst case scaling list matrix is 15kbits per picture.  Some typical cases from earlier core experiment (CE4) in JCTVC-H0230 are closer to 1kbit per picture. 2-16 kbps is typical when not updated for each picture.

Scaling list signaling would likely benefit from some type of inter-layer prediction. Further study encouraged.

JCTVC-M0211 AHG9: On Scaling List Signalling for SHVC [S. Liu, X. Zhang, S. Lei (MediaTek), K. Sato (Sony)]

Chaired by J. Boyce.

This contribution proposes the syntax and methods for signaling scaling list data in the enhancement layer for SHVC. Due to the similarity between an enhancement layer and its temporal collocated base layer, it is intuitive to derive the enhancement layer scaling list data from the base (or reference) layer scaling list data. Hence, this contribution proposes a few solutions for achieving this purpose, including (1) modifying the semantics of existing syntax elements “sps_scaling_list_data_present_flag” and “pps_scaling_list_data_present_flag”; (2) introducing new syntax elements “sps_base_pred_scaling_list_flag” and ““pps_base_pred_scaling_list_flag” to existing SPS and PPS syntax; (3) modifying the semantics of existing syntax element “scaling_list_pred_matrix_id_delta”; and (4) introducing new syntax element “base_pred_scaling_list_flag” to scaling list data syntax. Besides, inter-layer scaling list prediction for the size 16x16 and 32x32 is not allowed if the reference layer is the base layer and the value of avc_base_layer_flag in vps_extension() is 1. Furthermore, it is proposed to apply Inter scaling list data on blocks that are inter-layer texture predicted.

Proposes 4 options.  In all options, default scaling list for enhancement layer can be replaced by the inherited scaling list from the base layer.

In option 1, changes semantics of scaling list, no syntax change.  Can inherit default scaling list from a reference layer.   Could inherit scaling list from the AVC base layer for 4x4 and 8x8 blocks.

In option 2, add syntax element in SPS and PPS to control inheritance.

In option 3, change semantics in scaling list, so default scaling list is inferred from reference layer

In option 4, add syntax elements in scaling list per matrix, to allow individual matrix inheritance..

No indication of which reference layer used.

How are these activated?  Could be similar to in JCTVC-M0134.  This proposal has more granularity of inheritance than JCTVC-M0134.

8000 bits required to store the default list, after decoding.

Motivation is to save bits in the SPS and PPS.  It would be useful to know how many bits in the bitstream are used for typical scaling list.

A participant expressed concern about inheritance from an AVC base layer in this manner, which goes beyond current design’s use of AVC.

JCTVC-M0268 AHG9: VPS and SPS designs in HEVC 3DV and scalable extensions [Y.-K. Wang, Y. Chen, A. K. Ramasubramonian (Qualcomm)]

Chaired by J. Boyce.

JCT3V-D0047 contains basically the same content as this document.

This document proposes some VPS and SPS changes that apply to both SHVC and the 3DV family of HEVC extensions. Proposed changes include some changes for signalling of profile, tier and level information for layer sets, signalling of output layer sets, signalling of layer dependency, some changes for inclusion of more information in VPS for session negotiation, and some changes for efficient signalling of SPS parameters with individual control of different categories.

In the first proposal, avoid need to do entropy decoding to access profile_tier_level( ) in VPS extension. Also link profile_tier_level( ) to output layer set.   Defer consideration of this part of the proposal.

Proposal on efficient signalling of SPS parameters, proposes to move many parameters currently in the SPS to the VPS.   In the VPS, for each category of parameters, signal a number of parameters sent in that category.  Also in the VPS, for each layer signal which of the number of parameters is used.

Motivation for including information in the VPS rather than using inter-SPS prediction is design philosophy related, to avoid predicting at the same level. Only have one active VPS, don’t need a special activation process.

Including information in the VPS requires that it be included in a base layer.  After sub-bitstream extraction, base layer could be rather large since it contains information for all layers.

Some parameters are duplicated in the base layer SPS and also in the VPS, so can’t save bits present in the SPS base layer.

For session negotiation, it is expected to use the VPS and VPS extension.  In this proposal, the VPS would contain some information not needed for session negotiation, making it larger than necessary.

Changing parameters for one layer present in the VPS requires a new VPS for this proposal.  However, VPS to SPS prediction could address this.

Suggests moving profile tier level signaling to earlier in the VPS extension to avoid need to do entropy decoding, and to use only fixed length coding in profile/tier/level signaling.  Also adds another level of indirection for profile/tier/level signaling, first make a list of profile/tier/levels and then send an index to it for each layer set.

An alternate suggestion was made to avoid the additional level of indirection, but to just modify the existing design to avoid entropy decoding and add a fixed length coded max loop index.

A suggestion was made to move the direct dependency flag syntax section to directly follow the dimension_id syntax (ahead of profile/tier/level) signaling. Agreed.

Proposal to change the syntax element output_layer_set_idx[ i ] to output_layer_set_idx_minus1[ i ] due to the following reasons:

The syntax element layer_id_included_flag[ ][ ] used for output_layer_set_idx[ i ] equal to 0 is for layer set 0, while for layer set 0 layer_id_included_flag[ ][ ] is not defined in HEVC version 1.

It is clear that the target output layer for layer set 0 is layer 0 (the base layer) anyway.

Agreed.

Currently at most one output layer set can be signalled for each layer set. In multiview scenarios, the case is often that one layer set is associated with multiple sets of different target output layers. Thus, the syntax element output_layer_flag[ lsIdx ][ j ] is changed to output_layer_flag[ i ][ j ], and related semantics are changed accordingly.

In current design, profile/tier/level is associated with layer set.  It is proposed to associate it with output layer set instead.

A use case was mentioned where even in the spatial scalability case where some devices will decode the base layer and other devices will decode the base and enhancement layers, it might be useful to indicate more than one output layer set, so that HRD parameters can be sent for each output set.

During revisit, a revised version was discussed.

In the revision, a default_one_target_output_layer_flag specifies what the default mechanism is, either for use in SHVC to output only the top layer, and for MV-HEVC to output all layers.  Additional output layer sets beyond the default may be signaled.

Agreed to recommend adopting section 3 of the v2 document.

Design principles for output layer set design:

- Avoid entropy decoding

- Make typical cases for both SHVC and MVHEVC as simple as possible

- Consider if extra level of indirection of profile/tier/level signaling necessary.

Proposed to signal representation format (picture size, colour format, bit depth ) in VPS.

Could have an SPS that could be used for both the base layer and the enhancement layer, and the enhancement layer decoder would ignore the SPS extension.  An enhancement layer layer decoder would ignore the representation format info present in the SPS, and instead use the info in the VPS.

An implication is that if the representation format changed for a single layer, a VPS would have to be reactivated, which means that an IRAP is required for all layers.   Consider if there is a way to avoid this while allowing sharing of SPS by multiple layers.

For further study.

JCTVC-M0214 and JCTVC-M0210 are related.

Don’t signal sps_max_sub_layers_minus1 and sps_temporal_id_nesting_flag when nuh_layer_id_> 0.  Agreed.

It is considered desirable for DPB related parameters to be present in the VPS extension.  Further study to determine best design after DPB design is more mature.

JCTVC-M0134 AHG9: Inter-layer SPS prediction for HEVC extensions [T. Rusert (Ericsson)]

Chaired by J. Boyce.

The contribution is a follow-up from JCTVC-L0137 with minor modifications considering agreed SPS/VPS designs according to JCTVC-L1003_v34 and JCTVC-L1008.

The contribution addresses a redundancy problem that is asserted to be present when two or more different SPSs are present for different layers in multi-layer HEVC extensions. Since many SPS syntax elements may have identical values across SPSs, those values would be duplicated, thus causing bit overhead for parameter set transmission and storage.

First, criteria for the design of VPS and SPS for HEVC extensions are suggested. Specifically, it is suggested that (1) the VPS should (per its primary design purpose) include parameters that are useful for bitstream extraction and/or capability exchange, (2) parameters that are not required for bitstream extraction and/or capability exchange should not be present in the VPS, (3) sequence-level parameters that may change in some layers while not affecting the general layer structure should not be present in the VPS. For the design of the SPS with nuh_layer_id great than 0, it is suggested that (4) parameters present in the VPS should not be duplicated in the SPS (except for those parameters that are required for base layer decoding), and (5) sequence-level parameters that are not present in the VPS should be included in the SPS.

Then, following the design criteria, it is proposed for HEVC extensions to introduce inter-layer SPS prediction to reduce SPS-related redundancy. This implies extending the definition of SPS in the extension specifications. In the proposed SPS prediction scheme, SPS parameters are inherited at the time of SPS activation. The reference SPS for prediction is determined as the active SPS of the reference layer, which is signaled in the referencing SPS. The parameters to be inherited are indicated by flags.

Reference layer refers to the active SPS associated with the referred to layer_id.  Can only inherit from an active SPS.

For common conditions, bit counts for each category:

- representation format (e.g. picture size): 52 bits

- RPS: 140 bits

- VUI: 1 bit

- coding tool related parameters: 29

- Total: 222 bits

JCTVC-M0140 AHG9: Inter-layer RPS Prediction [S. Lu, K. Sato (Sony)]

Chaired by J. Boyce

HEVC employs RPS (Reference Picture Set) to manage DPB (decoded picture buffer).

In many cases, scalable layers are coded using the same GOP structure, and inter-layer RPS prediction is proposed by JCTVC-L0231.

The contribution asserts that the JCTVC-L0231 has the following shortcomings:

- Only in slice header –In Version 1 RPS is transmitted both in SPS & slice header

- One flag for both Short-Term and Long-Term Ref Frames –In Ref_idx Framework base layer images are marked as long term ref which means base layer and enhancement layer cannot have same long term ref set.

- used_by_curr_pic_flag should not be predicted –Enhancement layer may have less numbers of DPBs although GOP structure is same as Base layer

- In AVC-baselayer case prediction is not possible but not considered

The contribution proposes syntax to address the above issues.

SPS inheritance and slice header inheritance of RPS.  No indication of which reference layer is referred to.

The contribution uses the same syntax element name in SPS and slice header, which is undesirable.

In proposal 2, only a portion of the RPS is inherited.

In proposal 4, separate flags for copy of short term and long term RPS.

JCTVC-M0165 On parameter set prediction [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

This document contains the same proposal as in JCT3V-D0239.

Chaired by J. Boyce

In order to remove duplicated syntax signalling between sequence parameter sets (SPSs) in multi-layered extension of HEVC, three approaches are proposed; SPS-to-SPS prediction approach, VPS-to-SPS prediction approach, and a hybrid design enabling both approaches operationally. The proposed solution for SPS-to-SPS prediction is aligned to JCTVC-L0137 with minor changes, and the proposed solution for VPS-to-SPS prediction signals the common parts of SPS in a VPS once and each SPS refers to the VPS for parameter prediction. The contribution asserts that containing common syntaxes of SPSs of multi-layers in VPS extension is beneficial to inform the structure of coded video sequences for session negotiation and fast player and decoder set-up, as well as parameter set compression.

3 options proposed; VPS to SPS prediction, SPS to SPS prediction, and hybrid.  3rd option (hybrid) is preferred by proponent.

SPS to SPS prediction includes syntax change to VPS extension.  Proposes new syntax element in VPS, but doesn’t provide text in which VPS syntax element is used.  Hybrid approach also requires change to VPS extension, and not clear how those syntax elements are used.

Syntax elements with names ending in _idx might more appropriately end in _idc.

## Other parameter set proposals (3)

JCTVC-M0153 Extension parameter set [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

This document contains the same proposal as in JCT3V-D0052.

This document introduces an extension parameter set (EPS) that carries new syntax elements in the unified extension design.

In the version 1 HEVC specification, three parameter sets are defined to carry coding parameters at video, sequence, and picture levels. With the standardization activity of unified HEVC extensions, we need a new parameter set that carry parameters related to extension coding tools.

It was commented that camera parameters for 3D-HEVC can change from picture to picture. The size of the parameter, according to the proposal here, seems to be a few bytes.

It was commented that this seems belonging to the slice header. It was commented that currently the parameter is present in both SPS and slice header.

A new parameter set, like APS (e.g. as proposed in JCTVC-M0179), can potential save bits.

For further study.

Revisit at a larger group to discuss at a larger group in the future how to coordinate between a high-level syntax activity and a coding tool activity.

JCTVC-M0163 On video parameter set extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

This document contains the same proposal as in JCT3V-D0237.

In order to improve the syntaxes related to mapping nuh_layer_id with splitting_flag in video parameter set. Several changes are proposed:

- Signalling the maximum number of layers for each scalability dimension.

The information can be derived based on existing signalling in the VPS. No action.

- Mandatory signalling layer_id_in_nuh[ i ], when splitting_flag is equal to 1.

It was commented that this increases complexity as an additional mode is added, though it potentially saves a few bits.

B. Choi and R. Skupin will work offline on a syntax design for the third item above.

JCT3V-D0311 contains a revision of JCTVC-M0163

Contains 2 proposals, recommend 2nd one.

This proposal is aimed at bitrate savings in the VPS. Replaces a semantic constraint with an inference.

In the existing design, a decoder could ignore the value of splitting flag.  With this proposal, the flag has normative changes.

Some editorial improvement would be needed to handle setting default values for scalability type dimensions that are not present.

Agreed to recommend adopting proposal 2.  Note that the editorial improvement noted above should be addressed in a revision.

- Replacing dimension_id[ i ][ j ] with split_dimension_id[ i ][ j ], which specifies the j-th value of the identifier of the i-th scalability dimension type, when splitting_flag is equal to 1. Or skipping the signal of dimension_id[ i ][ j ].

There might be cases where the splitting_flag is equal to 1 but still full-sized layer ID is signalled. This will reduce flexibility a bit. No action.

- No signalling of the last dimension_id_len_minus1[ i ], when splitting_flag is equal to 1.

Agreed.

JCTVC-M0214 AHG9/ AHG14: On Color Gamut Scalable Video Coding [S. Deshpande, L. Kerofsky, A. Segall (Sharp)]

The upcoming deployment of UHDTV devices and content will use a different color gamut than legacy devices.  Specifically, HD uses the ITU-R BT.709 recommendation, while UHDTV will use the ITU-R BT.2020 recommendation.  A key difference between these systems is that the color gamut of UHDTV is significantly larger than HD.  It is asserted that this will provide a more “life like” viewing experience, which is consistent with other UHDTV characteristics, such as high resolution.  A motivation for the proposed support for color gamut scalability is this difference between color gamuts of base and enhancement layers when complying with the ITU-R standards.

This document proposes:

1. A new bit depth scaling process for reference pictures to support color gamut scalable coding.

2. It additionally proposes signaling color gamut and bit depth information regarding each layer in VPS extension to support session negotiation allowing end devices to select layers to decode based on their bit depth and color support capability.

Proposal 1 was not discussed in this BoG.

Proposal 2 overlaps with JCTVC-M0268. See notes under JCTVC-M0268. For further study.

# Signalling for inter-layer processing (10)

JCTVC-M0203 AHG 9: Signalling Inter-layer Prediction Indication [Hendry (LG), M. M. Hannuksela, K. Ugur (Nokia), D.-K. Kwon (TI)]

Study on the trade-off between coding efficiency and implementation cost of enabling / disabling inter-layer prediction based on temporal sub-layer has been reported in JCTVC-L0278. Furthermore, the initial design for signalling method to support controlling the use of inter-layer prediction based on temporal sub-layer at sequence level has been provided in JCTVC-L0449. This contribution re-proposes the signalling of inter-layer prediction at sequence level as described in JCTVC-L0449 and also provides use-cases describing the asserted benefits of such signalling.

It is claimed that computational complexity can be saved. It is also claimed that potential DPB size saving is possible. However, encoders can control the encoding anyway to achieve such savings without the proposed change.

The indication can be useful for streaming servers, middle boxes or decoders to avoid sending or decoding of non-required lower-layer pictures. However, if the indication is not used in any normative decoding process, then it could also be signalled in an SEI message. Is there any advantage of signalling the indication in the VPS compared to in an SEI message?

JCTVC-M0209 proposes a method to utilize the information in reference picture marking.

Agreed with a change of "<=" to "<" in the loop of the added syntax.

JCTVC-M0209 AHG9: On Design for Signalling Inter-layer Prediction [S. Deshpande (Sharp)]

JCT3V-D0082 contains the same proposal.

A “strawman” design for signalling of inter-layer prediction indication was discussed during 12th JCTV-VC meeting. The syntax and semantics for this design considered for signaling inter-layer prediction indication was described in JCTVC-L0449. This document proposes following additions to the design considered for signaling inter-layer prediction.

- A change to the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed.

Case 1 of JCTVC-M0162 is the same proposal. It is claimed that this enable marking of certain picture as "unused for reference" sooner, and potentially save some DPB memory. Agreed.

- A change to the decoding of inter-layer reference picture set and reference picture list construction is proposed.

Agreed. An editorial improvement is needed regarding the deviation of a variable NumInterLayerRpsPics that is currently used in the decoding process.

JCTVC-M0162 Unused reference picture management [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0236 contains the same proposal.

A reference picture marking process is optimized. A flag InterlayerPredictionFlag is introduced, which whether the current layer is used as reference picture for inter-layer prediction of other layers. The value of InterlayerPredictionFlag can be inferred from the syntax element max_sublayer_for_ilp_plus1[ i ] in JCTVC-L0449 or slice_reserved_flag[ 0 ] in JCTVC-L0152. When InterlayerPredictionFlag is equal to 0, a decoded picture with TemporalId equal to HighestTid is immediately marked as “unused for reference” after decoding.

Case 1: Same proposal as proposal 1 in JCTVC-M0209. See notes under JCTVC-M0209.

Case 2: A picture that has nuh_layer_id greater than 0 and discardable_flag (see notes under JCTVC-M0152) equal to 1 is marked as "unused for reference" after its decoding. Agreed.

Case 3: A combination of cases 1 and 2. Resolved by resolving cases 1 and 2.

JCT3V-D0074 MV-HEVC: On Marking for sub-layer non-reference pictures [X. Yang, P. Yue, Y. Zhang (Huawei)]

The conditions that marking process for sub-layer non-reference pictures applied only for pictures with nuh_layer_id greater than 0 restricts the effect of DPB space saving, this contribution proposes also to apply the marking process for base-layer pictures.

The proposed method was discussed at the previous meeting and it as concluded not beneficial.

No action.

JCTVC-M0152 Slice level inter-layer prediction signalling [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

This document proposes to use one reserved flag in the slice header to indicate that the picture is not used for inter-layer prediction.

Suggestion (based on discussions of JCTVC-M0152 and JCTVC-M0129): Use one reserved flag in the slice header (discardable_flag), when equal to 1, it indicates that the picture is not used for inter-layer prediction and not used for inter prediction. Agreed.

JCTVC-M0129 AHG 9: Inter-layer Prediction Indication at Picture Level [Hendry, B. Jeon (LG), D.-K. Kwon, M. Budagavi (TI)]

This contribution proposes two flags called ilp_flag and ilp_ref_flag and their use during decoding process. The proposed flags indicate whether or not the current picture is needed for inter-layer prediction and inter prediction, respectively. It is assessed that the two flags may help to lighten decoding process by allowing decoder to recognize pictures that can be skip from decoding process.

In the first revision, description of why two flags instead of one flag to indicate inter-layer prediction at picture level is needed and also more details on proposed item related to reference picture list management based on the proposed two flags is described.

See notes under JCTVC-M0152.

JCTVC-M0205 MV-HEVC/SHVC HLS: On inter-layer sample and syntax prediction indications [M. M. Hannuksela, K. Ugur (Nokia)]

JCT3V-D0042 contains the same proposal.

This contribution proposes:

- For MV-HEVC and SHVC (both reference-index-based and textureRL-based), the VPS extension syntax is proposed to be extended to include an indication whether a reference layer is used for sample prediction, syntax prediction, or both.

Why not in VPS, not in SPS? VPS looks better as the additional bits are not many and currently layer dependency information is there already.

ue(v) coding should not be used, unless another loop is used for the new information.

The method can also work for MV-HEVC and 3D-HEVC.

During discussion, it was considered useful for IntraBL based SHVC approach to signal detailed types of inter-layer dependency.

For RefIdx, whether signalling of detailed types of inter-layer dependency depends on whether current SPS flag on TMVP has any effect on inter-layer motion prediction.

See notes under JCTVC-M0046.

- For MV-HEVC and reference-index-based SHVC, the variables and decoding processes related to the inter-layer reference picture set and inter-layer reference pictures in reference picture lists are proposed to apply only to those layers that are indicated to be references for sample prediction in the VPS extension.

If to do this, do it only for reference picture list, not for reference picture set. Revisit.

- For MV-HEVC and reference-index-based SHVC, it is proposed to use one slice_reserved_flag (named alt_collocated_indication_flag) to specify whether a reference index (to a reference picture list) or an inter-layer reference index (to a list of reference pictures that are indicated to be references for motion prediction in the VPS extension) is present in case both are allowed by VPS and SPS level signalling. When alt_collocated_indication_flag is equal to 1, the inter-layer reference index is present in the slice segment header.

If to do this, it seems desirable to restrict that all slices of a picture shall point to the same collocated picture. This feature seems needed for proposal#4 in JCTVC-M0046. Revisit.

- It is proposed to remove sps_inter_layer_mfm_enable_flag from the SPS extension syntax specified in the SHVC test model. For specifying the decoding processes, alt_collocated_indication_flag is proposed to be used instead of sps_inter_layer_mfm_enable_flag.

Depending on dealing of propoal#1 in the document. Revisit after proposal#1 is concluded.

JCTVC-M0151 AHG9: Inter-layer Prediction Signalling [K. Sato(Sony), T.-D. Chuang(MediaTek)]

In SHVC draft text (JCTVC-L1008), dependency between layers at sequence-level is specified in vps_extension(). Inter-layer prediction signalling at picture-level would be necessary for the purpose of providing trade-off between coding efficiency and complexity or error resiliency. For example, a certain encoder implementation will be based on multi-loop scalability while other will be based on single-loop scalability, depending on memory bandwidth capabilities.

It is proposed by JCTVC-K0175, K0264, L0258, L0278 and L0449 that inter-layer prediction signalling be specified in terms of temporal hierarchy. Single-loop scalability as proposed by JCTVC-L0111 and L0154 can be regarded as special case of this signalling. Additionally it is proposed by JCTVC-L0258 that inter-layer prediction signaling be specified at the slice header.

In addition, it is proposed by JCTVC-L0071 and L0278 that inter-layer texture prediction and inter-layer syntax prediction separately. This can also be applied for SVC-style single-loop scalability as proposed by JCTVC-L0154. Motion vector information is the only syntax element for inter-layer prediction in current SHM (JCTVC-L1008). However prediction of other syntax elements like intra mode (JCTVC-L1101), residue (JCTVC-L1103), or slice syntax (JCTVC-L0231) is now being studied. Single-loop scalability as proposed by JCTVC-L0154 also contains such inter-layer syntax prediction. Therefore separation of indications of inter-layer texture prediction and inter-layer syntax prediction will be useful.

This document contains proposed syntax for inter-layer texture and syntax prediction signalling.

The proposals overlap with proposal#3 and proposal#4 in JCTVC-M0046. See notes under JCTVC-M0046.

JCTVC-M0046 High-level syntax modifications for SHVC [J. Boyce, D. Hong, W. Jang (Vidyo)]

- Add syntax element in slice header to signal inter-layer texture and motion prediction from different direct dependent reference layers, for TextureRL (IntraBL) mode only

Discussion: For SHVC, should it be allowed for one picture to use multiple pictures for inter-layer prediction? This was further discussed during the review of JCTVC-M0269.

Revisited later together with another proposal related to picture-level signalling of inter-layer prediction reference picture(s) in JCTVC-M0269.

Revisit of this proposal piece in the BoG was chaired by S. Deshpande, together with JCTVC-M0269.

inter_layer_pred_layer_idc syntax element which is conditionally sent only for spatial/ SNR scalability case specifies that the reference layer picture, used for inter layer texture prediction or inter layer motion prediction belongs to a particular reference layer.

As proposed in the document the syntax is proposed for TextureRL case only although it was asserted that this can be applicable also for RefIdx approach

Discussed together with JCTVC-M0269. See more notes under JCTVC-M0269.

- Add syntax elements in the slice segment header for enhancement layer slice skipping

Chaired by S. Deshpande.

The proposal in JCTVC-M0277 is closely related.

A syntax element is proposed but decoding process is not proposed.

The two syntax elements are conditionally present and only in nuh_layer_id > 0.

Additionally based on the value of one syntax element (scalable_slice_skip_flag), slice_segment_data() may not be sent.

A participant commented that this is similar to “dummy slice” in SVC which is useful and that participant expressed support for this feature in general.

It was commented that L0055 showed some results regarding slice skipping.

A participant commented that the slice skipping flag is useful for rate control purposes.

It was discussed if such a signaling should be done on slice level or on CU level. Benefits of signaling on slice level were described.

It was asked if decoding process needs to be changed for this flag. Some change to the decoding process will be needed.

A participant asked what is the benefit compared to current coding method. It was answered that this could avoid some decoding. It was commented that this feature was present in SVC.

A use case regarding benefit of this was described for real implementation scenarios and there was support expressed for such a use case. Some work would be necessary regarding decoding of a skipped slice.

For further study.

- Add syntax elements in the SPS extension to enable individual inter-layer prediction coding tools

Would only one flag for ILP of texture be good enough for both RefIdx and IntraBL approaches? The separation is mainly for editorial purpose (in case one of the two approaches would be removed finally, it would be easy to remove). See notes below on general discussion of signalling of different types of inter-layer dependency.

- Add syntax elements in the slice segment header for adaptive and default inter-layer texture prediction and enabling motion prediction, and the signalling is used the RPLC process.

The adaptive signalling for texture is similar as in SVC.

What is the reference layer for motion? That is in included in proposal 1.

For the RefIdx approach, a mechanism is needed to signal the collocated picture, which is not addressed in this document. Proposal#3 in JCTVC-M0205 seems working for this purpose.

General discussion of signalling of different types of inter-layer dependency (motion/syntax and texture/sample), based on related proposals in JCTVC-M0205, JCTVC-M0151, and JCTVC-M0046

During discussion, it was considered useful for IntraBL based SHVC approach to signal detailed types of inter-layer dependency.

For RefIdx, whether signalling of detailed types of inter-layer dependency depends on whether current SPS flag on TMVP has any effect on inter-layer motion prediction. Careful checking of the semantics in HEVC version 1 is needed, particularly for the potential high-level syntax only SHVC profile to work properly.

Questions:

- Whether to indicate the use of different tools for each reference layer?

- Whether to have such signalling in the slice level?

- Whether to put the signalling in the VPS or in the SPS?

Why in VPS, not in SPS? VPS looks better as the additional bits are not many and currently layer dependency information is there already.

If signalled in the VPS, ue(v) coding should not be used, unless another loop is used for the new information, such that the existing layer dependency information in the VPS (after adoption of the related change in JCTVC-M0269) can be accessed with entropy decoding of any syntax elements in the VPS NAL unit.

The method can also work for MV-HEVC and 3D-HEVC.

SPS seems to be the traditional place to put coding tool enabling tools.

Putting in the SPS may increase the need of the number of SPSs needed.

In multiview+depth scenarios, ILP of motion may only be used between depth and texture, while between depth and depth or texture and texture, both sample and motion prediction are used.

If ILP of sample between depth and texture is not used anyway, it is possible to derive the information based on scalability types.

Revisited after offline work aiming a harmonized design, JCTVC-M0457v2, among the proponents. See notes under JCTVC-M0457.

JCTVC-M0457 MV-HEVC/SHVC HLS: On inter-layer sample and syntax prediction indications (combining aspects of JCTVC-M0205 and JCTVC-M0046) [M. M. Hannuksela (Nokia), J. Boyce (Vidyo)]

JCT3V-D0313 contains the same proposal.

This contribution integrates JCTVC-M0205 and JCTVC-M0046 into one proposal.

The v2 revision tries to better enable future harmonization with other contributions.

Proposed pieces:

- Sequence-level signalling is in VPS (not in SPS)

  - A separate loop than the layer dependency signalling, 2 types of ILP (sample and motion) for both SHVC approaches.

  - If in the future only one of the two SHVC approaches is supported, then a change can be done to support only that approach.

  - Supports a hypothetical scenario of having both SHVC approaches supported even within a coded video sequence, but cannot be combined within a layer in a CVS.

  - Four variables that are currently used in the SHVC test model text are derived based on the syntax elements.

- Slice-level signalling

  - First part is the same as in proposal#3 of JCTVC-M0205.

  - Second part: Two flags for ILP of sample enabled and ILP of sample only. And another flag for ILP of motion.

- Derivation of TMVP collocated picture (same as in proposal#3 of JCTVC-M0205)

- Derivation of RPS and RPL, to include only the lower-layer pictures that are used for ILP of sample. It was agreed that, if to do this, only do it for RPL, not for RPS.

- Remove the _mfm_ flag in the SHVC test model (same as in proposal#4 of JCTVC-M0205).

- For the TextureRL based approach, use the slice level flag to derive a flag for syntax controlling in the CU syntax.

Sequence-level signalling in the VPS seems to be more flexible than in the SPS. Also, it would avoid increasing the number of SPSs.

Agreed.

# (6.4.6) Reference picture signalling and management (10)

JCTVC-M0081 AHG9: On initialization process of reference picture lists for HEVC extensions [O. Nakagami, T. Suzuki (Sony)]

JCT3V-D0057 contains the same proposal.

This contribution is a follow-up of JCT3V-C0079.

This contribution proposes to modify the initialization process. ILRP is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter in creating the temporal list. The motivation is to reduce the RPLM signalling for common coding structures.

Two options are studied in two contexts, scalable and multi-view extensions. Option1 modifies the initialization prosess in List0. Option2 does in both List0 and List1. The proposed options were tested on SHM1.0 (refidx approach) and HTM6.0.

Under SHVC common test conditions, it is reported that the number of RPLM signalling is reduced from 14 to 7 in option1 and to 8 in option2, and that the BD-rate difference is 0.0% in all conditions for both two options. Under 3DV test conditions, the number of RPLM signalling is reduced from 8 to 5 in option1, but increased to 13 in option2 with the BD-rate difference -0.1% and 0.1%, respectively.

As conclusion, it is recommended to modify the process as option1. It is believed that the change is beneficial for the common reference picture structure (e.g. N15M3) in the extensions.

Discussion:

It is possible to increase the number of active index.

The document included a report of BD rate reduction of about 0.1%.

One slice per picture is used for the results.

See more notes on rreference picture list initialization general discussion under JCT3V-D0220.

Further study encouraged.

JCTVC-M0104 AHG 9: On order of inter-layer reference pictures in reference picture lists [Hendry, B. Jeon (LG)]

Chaired by J. Boyce

This contribution proposes to modify the order of inter layer reference pictures in RefPicListTempX in descending order of their layer id, that is, inter layer reference picture from higher layer shall be located in lower index in RefPicListTempX. It is argued, inter-layer reference picture from nearer layer may be used more often compared to inter-layer reference picture from farther layer. Thus, for initial reference picture list, inter-layer reference pictures should be ordered in descending order of their layer id so that inter-layer reference picture from nearer layer shall occupy lower index in RefPicListTempX.

Only the list construction process is affected. No syntax changes.

See more notes on rreference picture list initialization general discussion under JCT3V-D0220.

For further study.

JCTVC-M0105 AHG 9: On position of inter-layer reference pictures in reference picture lists [Hendry, B. Jeon (LG)]

Chaired by J. Boyce

This contribution proposes to allow inter-layer reference pictures to be inserted into RefPicListTempX in front of reference pictures for inter prediction depending on the value of TemporalId of current picture. It is asserted that the proposed idea would work well to support the observation reported in JCTVC-L0278 that when time distance between the current and the reference pictures is far, which is the case when current picture is in lower temporal sub-layer, inter-layer prediction may be more preferred than inter prediction. Thus, having inter-layer preference pictures located in earlier position in RefPicListTempX would be more advantageous since they can be referred with smaller index value.

Is a coding efficiency improvement proposal, but doesn't have any experimental results.

Introduces a new syntax element in the VPS to indicate at which temporal layer the default ordering changes. Doesn’t provide information with examples of RPLM commands saved.

See more notes on rreference picture list initialization general discussion under JCT3V-D0220.

For further study.

JCTVC-M0148 Signalling inter-layer reference picture positions [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

Chaired by J. Boyce

This document proposes new syntax elements in a slice header extension that signals positions of inter-layer reference pictures.

RPLM provides this flexibility, but proposal asserts to save bits for sending reference picture list.

No experimental results or analysis of reduction of RPLM.

See more notes on rreference picture list initialization general discussion under JCT3V-D0220.

For further study.

JCTVC-M0150 Inter-layer reference picture set initialization [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

Chaired by J. Boyce

JCT3V-D0053 contains the same proposal.

This document proposes to have inter-layer reference picture lists be initialized in the order of closeness to the current picture.

RPS is affected.

JCTVC-M0104 proposes something similar.

A participant noted that in SHVC the quality of the higher layer is generally better, while in MV-HEVC the base view will likely have the highest quality.

See more notes on rreference picture list initialization general discussion under JCT3V-D0220.

For further study.

JCT3V-D0198 AHG7: Inter-layer reference pictures in reference picture list initialization [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang, L. Zhang(Qualcomm)]

Chaired by J. Boyce

This document presents a reference picture list initialization when inter-view reference pictures are involved. In many typical cases, by signaling the start position of the inter-view reference pictures in a reference picture list, syntax and decoding process for reference picture list modification are not needed.

In revision 1 of this document, bit count results are presented for two sequences with three views for the proposed method.

Affects list construction and not RPS.

The location of the proposed syntax is in the slice header extension, but it was questioned if it needs to use this location rather than being condition on nuh_layer_id > 0, since it isn’t needed for the base layer.

See more notes on rreference picture list initialization general discussion under JCT3V-D0220.

For further study.

JCT3V-D0220 AHG7: MV-HEVC inter-view reference picture list construction based on ViewId [T. Rusert, A. Norkin (Ericsson)]

Chaired by J. Boyce

This contribution addresses handling of inter-view reference pictures in the initial reference picture list construction in MV-HEVC and 3D-HEVC. It is proposed to sort inter-view reference pictures based on their ViewId. Two reference picture sets are created, which are inserted at some predefined positions making it possible to minimize use of RPLM commands.

Aimed only at MV-HEVC and 3D-HEVC, and not relevant to SHVC.

Revisit (for the second aspect that has not yet reviewed).

Reference picture list initialization general discussion

Chaired by J. Boyce

JCTVC-M0104, JCTVC-M0105, JCTVC-M0148, JCTVC-M0150, JCT3V-D0198, JCT3V-D0220

Questions:

Should the RPS or just the RPL initialization be modified?

Some participants expressed preference to only modify RPL initialization. Others are not too concerned about potentially modifying RPS.

Should RPL initialization depend upon TemporalId?

For SHVC, this depends on using the RefIdx approach

Desirable to have same design for 3D and SHVC families

Desirable to avoid a lot of special cases, prefer a clean design

Side activity led by A. Norkin to draft a BoG output document with recommended test conditions for input documents to next meeting related to this topic. Revisit for review of the BoG output document.

JCTVC-M0161 Reference picture marking process for HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0235 contains the same proposal.

In the current MV-HEVC and SHVC specification, the reference picture marking process has several drawbacks:

- Redundant marking process: The inter-layer reference pictures are marked as “used for long-term reference” and “used for short-term reference” iteratively as many as the number of layers that have the dependency from the layer of the inter-layer reference picture.

- Confliction with multiple layer decoding: When multiple layers, which may be decoded in parallel, access the same inter-layer reference picture at the same time, the reference picture marking processes can be mixed up.

- No explicit marking for inter-layer reference pictures.

In order to simplify and improve the reference picture marking processing for inter-layer prediction, three alternative reference picture marking processes are proposed. The proposed solutions reportedly remove the redundant marking process while one of them provides an explicit distinction from other decoded pictures in a decoder picture buffer, which are not used for inter-layer prediction.

Proposal 1: No action.

Proposal 2: Editorial change. Recommend for editors' consideration.

Proposal 3: Editorial change. Recommend for editors' consideration.

JCTVC-M0269 AHG9: Signalling and derivation of inter-layer RPS for HEVC 3DV and scalable extensions [Y.-K. Wang, J. Chen, K. Rapaka, Y. Chen, X. Li, M. Karczewicz (Qualcomm)]

Chaired by S. Deshpande.

JCT3V-D0048 contains the same proposal.

This contribution proposes a change to the signalling and derivation of inter-layer RPS for the following purposes:

- The inter-layer RPS subset can be used to detect loss of pictures used for inter-layer picture reference.

- In SHVC it can be restricted that at most one reference layer picture is used in inter layer prediction for coding a picture so that up to one picture resampling process is needed to code a picture. In this case, when pictures from multiple directly dependent layers are present, and the one of the highest layer is used for inter prediction, the information is not explicitly signaled in slice header.

It was discussed whether this is applicable to both RefIdx and Texture RL approach. The conclusion was that this is applicable to both.

It was asked if it makes sense to limit inter-layer prediction for particular picture to use only at most one inter-layer reference picture (for SHVC only). It was agreed that this restriction should only apply to cases where filtering is needed for each lower layer reference picture, e.g. spatial scalability.

A participant expressed opinion that a unified syntax is desired for both SHVC and MV-HEVC.

The VPS related signaling part is only for bit savings.

An asserted benefit of adding new syntax was to detect loss of pictures used for inter-layer reference. A participant commented that without the new syntax this could be detected later during slice parsing. Proponent asserted that this would require parsing all slices before such a detection could be made otherwise.

It was asked if error resilience/ loss detection aspect needs to be signalled in normative part. Proponent asserted that this will be needed for normative inter-layer reference picture set derivation process using this information.

A participant commented that the early loss detection is useful feature. It was commented by another participant that loss detection is a nice feature but the additional signalling cost needed to be considered.

A participant commented that a VPS level flag could signal different picture rates or different number of layers in an AU and that could be used for loss detection. It was commented that such a flag may not support the case when middle boxes could individually discard pictures,

The proposed syntax supports cases where pictures from only up to 1 or  more than 1 reference layers can be used for inter-layer reference prediction.

Proposal 1 in JCTVC-M0046 has similar goal of signaling on a picture basis which reference layer is used for inter-layer prediction. One difference is in JCTVC-M0046 only one inter-layer picture can be chosen for prediction.

Questions related to JCTVC-M0269, and proposal#1 of JCTVC-M0046:

- Usefulness of signaling for all HEVC extensions (including Refidx and Texture RL of SHVC):

A participant commented that for Texture RL a slice level signaling such as in these proposals is useful.

- Is sequence level signaling which can be over-ridden in slice level needed or is only signaling at slice level for ILP ref layer indication enough

- Do we need to support that more than one reference layer could be used for inter-layer reference for SHVC can be signaled at picture level.

Revisited after offline activity regarding above questions if a common agreement is reached among proponents, in JCTVC-M0458.

See notes under JCTVC-M0458.

JCTVC-M0458 MV-HEVC/SHVC HLS: On signalling and derivation of inter-layer RPS (combining aspects of JCTVC-M0046 and JCTVC-M0269) [Y.-K. Wang, J. Chen, K. Rapaka, Y. Chen, X. Li, M. Karczewicz (Qualcomm), J. Boyce, D. Hong, W. Jang (Vidyo)]

Chaired by M. M. Hannuksela

This contribution integrates proposal#1 in JCTVC-M00046 and the proposal in JCTVC-M0269 into one proposal.

The contribution includes the following aspects:

- max_one_active_ref_layer_flag in VPS extension

- syntax elements in the slice segment header to indicate which inter-layer reference pictures (out of those allowed by the layer dependency information provided in the VPS extension) may be used as reference for the current picture. The variable NumActiveRefLayerPics is derived accordingly.

- Reference picture list and inter-layer reference picture set related changes where only those pictures indicated to be used as inter-layer reference pictures are included (already included in JCTVC-M0269)

Aspect 3 has a relation to JCTVC-M0457.

It was noted that the contribution impacts reference picture list initialization. The order of inter_layer_idc[ i ] syntax element enables different orders of inserting inter-layer reference pictures into the initial reference picture lists.

Agreed to recommend adoption of all aspects but have a semantic constraint that inter_layer_idc[ i ] shall be increasing. Editorial notes that further improvements related to aspect 3 are encouraged. Also agreed to let the editors to combine text of JCTVC-M0457 and JCTVC-M0458.

# (6.4.8) Inter-layer slice header prediction (3)

JCTVC-M0218 Inter-layer slice header syntax elements prediction in SHVC and MV-HEVC [D.-K. Kwon, M. Budagavi (TI)]

In many scalable and multi-view video applications, it is reported that all layers are coded using the same GOP structure and that it is common for all layer pictures in the same temporal position to have the same temporal reference pictures and reference picture lists. Moreover, when the pictures at the same temporal position are coded using the identical reference pictures, their weighted prediction parameters (WPPs) are reported to be likely identical to each other.  Based on this observation, it is proposed to signal in slice header whether reference picture lists and WPP are predicted from those of scalable inter-layer reference picture or inter-view reference picture.

The variables interDLlayerPredFlag and interVLayerPredFlag used in the syntax are not defined.

Two flags added into the slice header for prediction of POC LSB, RPS and RPL syntax elements in the slice header from the reference layer (assuming one reference layer is used).

Two more flags added for controlling the prediction of weighted prediction parameters.

No bits saving results are available.

See notes on overall discussion on slice header prediction under JCT3V-D0197.

JCTVC-M0171 Slice header design for HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0244 contains the same proposal.

In terms of header compression, it is considerable to reduce the bits of slice headers, because the sum of bits of all slice headers in multi-layered bitstreams, occupy a sizable portion of the total bits, especially in a low bit rate. In this proposal, to enable the inter-layer prediction of slice header syntaxes, allowing a dependent slice segment at the first of slices in enhancement layers is proposed. If the first slice segment is a dependent slice segment in an enhancement layer, the value of each slice segment header syntax element that is not present is inferred to be equal to the value of the first slice segment header syntax element in the first slice header in a base layer in the same access unit.

Enabling prediction of slice header syntax elements after the slice_segment_address syntax element for the first slice of a picture with nuh_layer_id greater than 0.

For a dependent slice segment, other slice header parameters would still be inherited from the previous independent slice segment.

No bits saving results are available.

JCT3V-D0197 AHG7: Slice header prediction for MV-HEVC [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang(Qualcomm)]

This document proposes a method to predict certain slice header parameters of view components from their reference view components. The prediction of different sets of slice header parameters can be enabled or disabled by means of flags.

The first slice header of a reference view component of the current view component in the same AU with the highest nuh_layer_id value is chosen for predicting the slice header syntax elements in the non-base views. The syntax elements of the slice header that can be predicted are categorized into four types:

- Common information – these include syntax elements that do not change for all the slices in all the views; e.g., POC LSB and reference picture set information

- Reference picture list related information – these include the syntax elements that are related to the reference picture lists, including list sizes and list indices

- Prediction weight table information – these include the syntax elements that are used to signal the parameters for weighted prediction

- Deblocking parameters information – these include syntax elements that are related to signaling of the deblocking parameters

It was claimed that the method also apply to SHVC.

The source for prediction is the first slice of the picture in the highest directly dependent layer.

The mechanism does not apply to dependent slice segments.

No bits saving results are available.

Overall discussion on slice header prediction

Chaired by J. Boyce.

Do we see any design philosophy issue to enable slice header prediction that creates cross-slice dependeny? In 3D-AVC slice header prediction is supported. Syntax elements may be inherited from the previous reference layer or the base layer. Four categories: generic, RPL, WP weights, and ref picture marking.

How many bits saving are we talking about? Currently not available.

It was remarked that the RPS and POC LSB information might be very few if ST-RPS is inherited from the SPS. Otherwise, it can be many bits.

List modifications of a base layer would usually be different than those for enhancement layers. Among enhancement layers, they tend to be the same.

Weighted prediction parameters: Could be helpful for illumination compensation in 3D/multiview scenarios. For SHVC RefIdx based approach, signalling of additional weights might be needed for ILP. For SHVC IntraBL based approach, the parameters are likely to be the same among layers.

Deblocking parameters: There are usually only few bits. Deblocking for depth is usually turned off (e.g. in the common conditions).

If to do anything, it is necessary to provide separate control of different categories of parameters? More categories need more bits for the controlling flags.

It was commented that currently there is no design that explicitly addresses the cases with multiview plus depth, e.g. to allow flexibly choosing the prediction source slice from a reference layer, and this should be addressed.

For further study, taking into account the above comments, particularly the ones on bits saving results.

# Parallel processing (8)

General discussions related to inter-layer parallelism decoding delay indication

- VUI vs SEI?

- SEI can change mid-sequence while VUI applies to the entire sequence.

- SEI requires more bits than VUI

- additional bits in VUI only required when constraint flag set

- v1 min spatial segmentation is in the VUI

- SEI message information is available before decoding of enhancement layer slice is started

- Additional sample-level offset?

- CTB only, or also per tile and per slice signaling?

  - If tiles are used, is an additional parallelism indicator really needed?

- Separate horizontal and vertical dimensions or total CTBs?

Side activity led by R. Skupin to make a design with the following characteristics:

– signal in VUI

– maximize coding efficiency, especially in the case when feature is not used

- separate horizontal and vertical dimensions

- only signal 2nd dimension if needed

- only signal type if needed

- consider whether or not to signal parameters per direct reference layer

- cannot change VUI definition for base layer for backwards compatibility

Revisit after side activity.

JCTVC-M0200 Layer decoding delay indication for SHVC and 3D/MV-HEVC [R. Skupin, K. Suehring, Y. Sanchez, T. Schierl (Fraunhofer HHI)]

The contribution proposes to add syntax elements that allow signalling the minimum required decoding delay between the current and all reference layers. The signalling can be in units of slices, tiles, substreams or CTUs.

Adding two syntax elements as bitstream restriction parameters in VUI.

JCT3V-D0199 and JCT3V-D0064 are related. The current MV-HEVC WD includes an SPS parameter that is related.

Proposes a VUI restriction, rather than an SEI message as in JCT3V-D0064 and JCT3V-D0199.

Can signal delay in CTBs, tiles, or slices.

Could reorder syntax elements and conditionally include the second only if necessary, with a non-zero value for the delay.

See notes under General discussions related to inter-layer parallelism decoding delay indication.

JCT3V-D0064 AHG13: Parallel decoding SEI message [T. Ikai, Y. Yamamoto (Sharp)]

At the last meeting, the normative disparity vector constraints are incorporated in Stereo profile of MV-HEVC to enable parallel decoding. The utilization of constraints is explicitly signalled in SPS. The flag guarantees disparity vector restriction between the target view and all reference views. Although this general constraint flag is thought to be effective in practical usage, there are other cases that more specific constraint signalling is needed where the disparity vectors related to some reference views are restricted and the disparity vectors related to other reference views are not restricted.  This contribution proposes a parallel decoding SEI message which can deliver such specific constraint.

Imposes a vertical restriction, and only sends a horizontal restriction if a vertical restriction is sent.

Proposed syntax has not been updated to match current specification text.

Syntax includes per sample offset information using a flag indicating either 0 or 4 sample offset.  The value of providing this additional granularity is questionable.

See notes under General discussions related to inter-layer parallelism decoding delay indication.

JCT3V-D0199 AHG7: Parallel decoding SEI message for MV-HEVC [Y. Chen, A. K. Ramasubramonian, Y.-K. Wang, S. Seregin, L. Zhang(Qualcomm)]

A parallel decoding SEI message for MV-HEVC similar to the parallel decoding SEI message in MVC is proposed with three modifications: 1) the delay required for parallel decoding is signalled in unit of coding tree units (CTU); 2) the horizontal CTU delay is signalled together with CTU row delay to avoid large increased delay caused by the size of a CTU; 3) the delay is signalled for each reference view of a current view.

In the first revision of this document, it is proposed to use the collocated CTU in the reference view components to determine the unreferenced region, after discussion with experts working in this area.

Proposed syntax has not been updated to match current specification text.

In the proposed syntax, this SEI message cannot be used with certain tile configurations.

See notes under General discussions related to inter-layer parallelism decoding delay indication.

JCTVC-M0204 Ultra-low delay multi-layer decoding by interleaving of decoding units in SHVC and 3D/MV-HEVC [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (Fraunhofer HHI)]

(Presented initially in joint discussion with JCT-3V Sat 20th pm, but not closed in that discussion.)

The contribution proposes to allow a bitstream layout that enables multi-layer ultra-low delay decoding operation by parallel decoding of sub-pictures. Decoding units belonging to different layers/views would be allowed to be interleaved within an access unit. The order is constrained in a way that all necessary reference layer/view data shall be available before sending an enhancement layer decoding unit. In addition this contribution proposes a clarification on the definition of access units and coded pictures in multi-layer coding environments.

Revisit.

General discussion about inter-layer tile parallelization

Both slice based and tile based upsampling restrictions are useful, but can they both be allowed in the same picture? If yes, but the number of boundaries should be bounded.  The usefulness of allowing both was questioned.

Semantic constraints vs normative decoder processes.

Question: Is an additional constraint on temporal MV prediction needed in order to enable independent decoding of tiles for SHVC?  The bottom right vector from the base layer could never be selected by the encoder.

BoG suggests to revisit after side activity led by K. Suehring to draft a document aligned with JCTVC-M0035 containing the following:

- VUI flag to indicate strict tile alignment semantic constraint

- SPS extension flag to indicate upsampling filter restriction normative decoding process (SHVC) only

- VUI or SPS flag to indicate temporal MV Prediction restriction for MV-HEVC and SHVC, and for MV-HEVC also constrains the inter-layer motion vector

JCTVC-M0198 Independent tile upsampling for SHVC [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (Fraunhofer HHI)]

This contribution proposes a constraint for the spatial scalable upsampling filter at the border of tiles in reference layers. A syntax element is proposed to enable the constraint. If the syntax element is enabled, no pels shall be used for upsampling that lie outside the tile that the collocated reference layer pel lies in.

Proposes a flag in the PPS extension.  It is noted that the existing v1 flag for disabling deblocking filter at tile boundaries is also in the PPS.  Proposed feature is more useful if the deblocking filter across tiles is also disabled in the reference layer.

Experimental results were provided indicating that the influence on coding efficiency is 0.0 – 0.1%, smaller than the impact of turning off the deblocking filter at loop boundaries.

Does not provide text of decoding process.

It is noted that JCTVC-M0035 provides similar functionality across slice boundaries, and does have text that can be adapted for tiles.  During the track review of JCTVC-M0035 an AHG was proposed.

See notes under General discussion about inter-layer tile parallelization.

JCTVC-M0334 Parallel Processing Indications for Tiles in HEVC Extensions [K. Rapaka, W. Pu, X. Li, J. Chen, Y.-K. Wang, M. Karczewicz (Qualcomm)]

HEVC supports tile based coding to enable parallel processing. In this contribution, some problems are discussed related to parallel processing of tiles across layers and methods are proposed to address the problems and to enable more friendly parallel processing of tiles across layers.

In the first option, proposes a flag in the VUI to impose an encoder constraint on use of inter-layer prediction on tile boundaries. Requires tile alignment between layers.  Doesn’t require any normative decoder process changes. Could alternatively be placed in an SEI message.Proposed semantic language isn’t very precise.  No software implementation or experimental results.

In the second option, proposes flag in the SPS extension to require padding at tile boundaries instead of using samples from neighboring tiles.  Requires that tiles be aligned across layers, which is also a constraint imposed by setting the flag.

Does not provide text of decoding process.

JCTVC-M0035 is related.

See notes under General discussion about inter-layer tile parallelization.

JCT3V-D0051 AHG 13: Restriction of inter-layer prediction on tile boundaries [G. Tech, Y. Sanchez, K. Suehring, R. Skupin, T. Schierl (HHI)]

This contribution proposes to restrict inter-layer prediction for a particular tile in a dependent layer to a particular tile in a reference layer. The constraint enables coding of tiles of dependent layer, before the complete reference layer has been coded and is therefore in particular suitable for low-delay applications. To enable the tile constraint a flag in the SPS is proposed. For MV-HEVC bitstream restrictions apply when the constraint is enabled. For 3D-HEVC the decoding process is modified when the constraint is enabled.

Version 2 of the document has been revised. A missing constraint related to TMVP has been added.

Only applies to MV, not SHVC.  Only applies to inter-layer prediction.

Restricts the use of even small sub-pel motion vectors of the PUs along the boundary of the tile.

Requires decoding process changes.

No experimental results that quantify the coding efficiency costs of using the proposed method. Costs would be higher if tiles are not aligned.

Imposes a semantic constraint on the bitstream, but is still proposed to be included in the SPS, because a related 3D-HEVC use would require normative decoder change.

Question about if this constraint should be combined with a tile alignment constraint.

Support was expressed for this capability to enable parallel tile decoding for MV-HEVC, but it was questioned whether a flag should be included in the VUI or the SPS, which is impacted by the decision in 3D-HEVC.

Further study to determine if method could also apply to spatial/SNR scalability.

See notes under General discussion about inter-layer tile parallelization.

JCTVC-M0202 Indication of tile boundary alignment [K. Suehring, R. Skupin, T. Schierl (Fraunhofer HHI)]

The contribution proposes to add a flag to VUI of an enhancement layer that allows signalling that the tile boundaries of all base layers are aligned with tile boundaries of that enhancement layer. This flag can be used in a decoder as guide for a parallelization strategy that allows parallel decoding of base and enhancement layer tiles. This has previously been discussed in JCTVC-L0197. In combination with additional restrictions in inter-layer and inter-view prediction as presented in JCTVC-M0198 and JCT3V-Dxxxx a decoder can infer decoding independence between reference and enhancement layer tiles covering different regions based on this flag.

Add a flag in the VUI to impose a semantic constraint that every tile boundary in the reference layer has a corresponding tile boundary in the enhancement layer.  Is not a strict alignment between the tiles across the layers, since the enhancement layer may have more tiles than the base layer.

If there is a strict alignment between tiles across layers it could enable CTB-level synchronized decoding across layers within a tile and reduce delay.  This is more useful if the resolution is the same between the layers.

An alternative would be to include two flags, with an additional flag indicating the strict alignment.

Parallelization based upon tiles might provide a higher degree of parallelization if a higher number of tiles is allowed in the enhancement layer.

See notes under General discussion about inter-layer tile parallelization.

# Special functionalities (4)

JCTVC-M0040 AHG9: Using SHVC for adaptive resolution change and efficient trick mode [K. Ugur, H. Roodaki, M. M. Hannuksela (Nokia)]

SHVC inherently supports resolution change and inter-layer prediction; therefore it can effectively be used for traditional single-layer use-cases, where resolution change could be desired. One such use-case is Adaptive Resolution Change (ARC), which refers to dynamically changing the resolution within the video sequence without using IDR pictures. Another use-case is application of SHVC for trick mode so that downsampled pictures are used for trick mode compared to full resolution pictures to achieve reduced bitrate for trick mode. This contribution proposes high-level syntax for SHVC, so that these use-cases can be more efficiently supported

Revision 1 provides an updated specification text where among other things the definition of access unit as agreed by JCT-VC on 20th April 2013 was taken into use.

This requires allowing changing of the top layer ID across AUs within one CVS.

Trick mode: It was claimed that HEVC improves inter coding better than intra coding, which might be a problem for trick mode. The base layer pictures are coded as intra to enable random access as well. This is currently already supported.

The indication in the bitstream does not seem to be useful for the trick mode use case, but seems to be useful for the ARC use case. Such indication in a system, e.g. file format, is certainly useful for the trick mode use case.

The indication does not have any impact on the normative decoding process. However, putting it in the VPS could be useful for session negotiation purpose and so on, and it is just a flag.

Revisit at a larger group, particular on the question on whether to allow changing of the top layer ID across AUs within one CVS, which is not allowed in SVC. Same question was briefly discussed also during the review of JCTVC-M0107.

JCT3V-D0211 MV-HEVC/SHVC HLS: Extending the supported number of layers [K. Suehring, G. Tech, R. Skupin, T. Schierl(HHI)]

This contribution proposes an extension mechanism for layer identifiers to increase the number of supported layers in MV-HEVC and 3D-HEVC, which currently is 64. The range of nuh_layer_id is extended by an additional syntax element within the NAL units. The concept of so-called layer clusters allows using the existing extraction processes to select groups of related layers. As second part of the proposal an extension of the layer dependency signalling applicable to layer id extension is proposed.

Camera arrays with more than 64 cameras were shown in the presentation.

nuh_layer_id in the slice header becomes MSB of layer ID. Three-bit layer_id_ext_len is signalled in the VPS. layer_id_ext ue(v) coded for SPS/PPS/SEI NAL units and u(v) coded in the slice header for VCL NAL units using the up-to-7 reserved bits early in the slice header.

Also proposed a so-called cluster based layer dependency signalling, which is claimed to be fully compatible with the existing dependency signalling.

The proposal looks very interesting to the BoG. Some design aspects, such as backward compatibity etc., are pretty clean. It was commented that most likely something like this would be included in the final standard.

The cluster based layer dependency signalling is not easy to understand, participants mentioned more time was needed to digest the design. A check is needed to see whether this should also be done for other non-VCL NAL unit types.

Further study encouraged.

JCTVC-M0164 Layer identifier extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCT3V-D0238 contains the same proposal.

To support more than 64 layers, conforming to the current HEVC spec., we propose the layer identifier extension solution. In NAL unit headers, three bits are allocated for nuh_temporal_id_plus1. Forcing all NAL units of all layers into having the same TemporalId in an access unit, the nuh_temporal_id_plus1of non-base layers can be interpreted as extra bits of nuh_layer_id to represent additional layers. The TemporalID of layers with non-zero nuh_layer_id is inferred from the TemporalID of the base layer. Approximately 500 layers can be represented by the proposed solution.

TemporalId is inferred from the base layer. It was commented that this would disallow middle boxes to decide whether to pass NAL units based on TemporalId without storing states and trying to figure out which NAL units belong to the corresponding base layer picture.

The proposal cannot be applied when different layers have different picture rate.

No action.

JCTVC-M0277 A Use Case and High Level Syntax of SHVC: Scalability Based Region of Interest [C. Auyeung, J. Xu, O. Nakagami, A. Tabatabai (Sony)]

This contribution presents a use case and high-level syntax of SHVC for region of interest (ROI) scalability where higher quality/resolution through scalability is needed on a part of a picture but not the whole picture. Application of ROI scalability includes traffic monitoring and security monitoring.

Non-significant slice segment: similar to "skipped slice" or "dummy slice". A new NAL unit type is proposed for such slice segments.

Non-significant tiles: No normative change is proposed for this concept.

Proposal#2 in JCTVC-M0046 is closely related.

For further study.

# Sub-bitstream extraction (2)

JCTVC-M0131 AHG 9: Sub-bitstream extraction for pictures not needed for inter-layer prediction [Hendry, B. Jeon (LG)]

In the 12th JCTVC meeting, ideas to signal indication of inter-layer prediction at both sequence and picture level have been proposed. Such indication will enable identification of pictures that can be removed from the input bitstream. This contribution proposes sub-bitstream extraction process that consider removal based on whether or not a picture in is needed by other picture for inter-layer prediction.

For further study.

JCTVC-M0272 AHG9: Multi-mode bitstream extraction in HEVC 3DV and scalable extensions [Y.-K. Wang, Y. Chen, A. K. Ramasubramonian (Qualcomm)]

Document JCT3V-D0050 contains basically the same content.

This contribution discusses different trade-offs in the design of the sub-bitstream extraction process, targeting best sub-bitstream self-completeness (such that each sub-bitstream further extracted from a sub-stream, recursively, can be a conforming bitstream) or the optimal size of a sub-bitstream (such that some or all pictures that are not needed for correct decoding of the target output layers do not have to be included in the extracted sub-bitstream) and proposes a multi-mode bitstream extraction process in order to allow for choosing the best bitstream extraction mode depending on application scenarios.

For further study.

# Hypothetical reference decoder (HRD) and DPB management (5)

JCTVC-M0169 Decoded picture buffer for HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

Initial discussion chaired by Y.-K. Wang, and later (concluding) discussion chaired by J. Boyce.

JCT3V-D0242 contains the same proposal.

In this proposal, to support both the synchronized management of decoded picture buffers (DPBs) in all layers and the independent management of each DPB to minimize the DPB size, two flags are signalled in the VPS. A dbp_synchronize_all_layers_flag indicates whether the DPBs of all layers are synchronized with a base layer, while dbp_synchronize_flag indicates whether a DPB of a layer is synchronized with a base layer. When a DPB of a layer is synchronized, signaling the syntaxes related to RPS and sub-layer ordering in SPS or slice header can be skipped.

Some syntax changes are proposed to VPS as well as slice header. The slice header change is to ensure all layers to have the same RPS for all pictures in an AU such that reference picture marking are synchronized.

Which DPB operations (storing, marking, output, removal) are included? Marking, output (applicable only when there are multiple target output layers), and removal are included.

Supporting both methods in the design would be difficult to implement and difficult to specify.

For further study.

JCTVC-M0270 AHG9: On DPB operations in HEVC 3DV and scalable extensions [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

Chaired by J. Boyce.

Document JCT3V-D0049 contains basically the same content.

This contribution discusses DPB management operations in HEVC 3DV and scalable extensions, including decoded picture storage, marking, output and removal. With the assumption of multi-loop decoding in SHVC and the need for the support of spatial scalability, DPB management involving decoded pictures of different sizes becomes necessary. Currently, the mechanism for DPB management and the definition of DPB size (which is important for level definitions) in SHVC for spatial scalability are not clearly specified.

Two fundamentally different approaches are possible. One is AU-level (with the assumption that AU is defined to consist of all pictures pertaining to a particular output time), the other is picture-level. AU-level DPB management is asserted to be simple from design, implementation and editing points of view, while picture-level DPB management can be more optimal in memory requirement, at least for some non-typical scenarios.

Both approaches are summarized with some preliminary text changes provided. It is suggested that the group discusses these topics and tries to make a preliminary conclusion on which direction to go. In case no essential conclusion can be made at this point of time, it is suggested that some straw-man designs for both directions are made as some break-out group activity and the straw-man designs are documented in some kind of live document similarly as being done for VPS designs earlier.

Information document. Can influence level definition.

A participant noted that there some experience in defining a DPB for option 2 in MVC+D, which counts MBs, not pictures or layer components. A participant noted that it might be interesting to measure in terms of Min CTU size.   JCTVC-K0225 proposed DPB extension to handle different bit depths and picture resolutions.

A solution that combines some aspects of options 1 and 2 may be interesting.

A participant noted that backwards compatibility for DPB handling of the base layer should be maintained.

Picture based approach is more memory efficient for the variable frame rate per layer use case.  Picture base approach generally allows for more optimal DPB memory size. Memory optimization is seen as important design criteria, with higher priority than simpler specification.

For further study.

JCTVC-M0130 AHG 9: Signalling required DPB size for in layer set [Hendry, B. Jeon (LG), D.-K. Kwon, M. Budagavi (TI)]

Initial discussion chaired by Y.-K. Wang, and later (concluding) discussion chaired by M. M. Hannuksela.

This contribution is the result of further study recommended for previous contribution JCTVC-L0262. This contribution proposes to signal the required DPB size for each layer in layer set. It is assessed that the information may help decoder to understand the memory requirement for selecting the layer set it will be operating on.

This proposal assumes HRD operation for each layer (including a layer that depends on other layers).

For further study.

JCTVC-M0210 AHG9: On Signalling DPB Parameters and DPB Operation [S. Deshpande (Sharp)]

Chaired by M. M. Hannuksela.

JCT3V-D0083 contains the same proposal.

This document proposes signaling vps_max_dec_pic_buffering_minus1, vps_max_num_reorder_pics, vps_max_latency_increase_plus1 parameters for sub-layers for the CVS for operation points in VPS extension for SHVC and MV-HEVC. In a variant proposal these parameters could be signaled in VPS extension for the output layer sets instead of for operation points. The document also proposes the operation and bumping process for output order DPB of SHVC and MV-HEVC to use the above parameters from VPS extension if they are signaled for the selected operation point (or output layer set in the variant proposal). Otherwise the SPS level parameters are used.

Some participants expressed support of having VPS indication(s) for concluding the total DPB size per an output layer set, which could be useful e.g. in session negotiation.

For further study.

JCTVC-M0254 Consideration of buffer management issues and layer management in HEVC scalability [Sam Narasimhan, Ajay Luthra (Motorola)]

Chaired by J. Boyce

The buffer management in AVC based scalability (SVC and MVC) required an extension to system STD buffer model and introduced an additional layer of complexity to re-purposing and re-distribution equipment. The extensions required management of both the base layer buffer and buffer with base and enhancement layer/layers at the same time in both transmission and decoding equipment. This contribution suggests some options to reduce this complexity in HEVC based scalability.

AVC based scalability did not provide explicit capability to combine the scalability between SVC and MVC and this is being considered in HEVC. In addition, it did not provide the ability for layer management (such as add or remove scalable layers from pre-compressed content) for re-purposing to receivers with varying capabilities. This contribution suggests some options to support the capability for layer management.

Level indication has relatively high granularity.  Sending additional per layer bitrate and buffer parameter information is requested to provide more accurate information than provided by the level indication.  Desirable for this information to be optionally available.

The layers present SEI message might address some of the desired features for layer management.

# SEI messages (3)

JCTVC-M0043 Layers present SEI message [J. Boyce, D. Hong, W. Jang (Vidyo)]

A layers present SEI message is proposed for SHVC and MV-HEVC.  Its function is similar to the layers not present SEI message in SVC, but with modified syntax.  Its purpose is to enable a middle box to communicate to a decoder when layers are removed during sub-bitstream extraction.  The proposed SEI message uses only fixed length coding, which is convenient for use by cascaded middle boxes.   For MV-HEVC, layer set present flag variable values may be derived using the syntax in the proposed SEI message, providing similar functionality to MVC’s operation point not present SEI message.

Agreed with the following change: the persistence scope of the SEI message should be further restricted to be within a CVS.

JCTVC-M0212 AHG9: On Output Layer Sets Change Signalling [S. Deshpande (Sharp)]

JCT3V-D0084 contains the same proposal.

SHVC Draft Text 1 and MV-HEVC Draft Text 3 supports signaling layer dependency information and information about output layer sets using video parameter set extension. A layer dependency change can also be signaled using layer dependency change SEI message in SHVC and MV-HEVC. It is asserted that as the layer dependency changes the output layer sets could change. Currently no mechanism exists to allow signaling a change in the output layer sets without sending a new video parameter set. This document proposes output layer sets change signaling. Additionally this document also defines derivation of target output layers and target decoded layers based on the output layer sets.

On the SEI message part, some participants expressed that there is no valid use case. No other participant other than the proponent thought that there is a valid use case. No action on this aspect.

On the other part, it was commented that after the DPB behavior is clearer, such a derivation process of the layer set from the list of target output layers might be needed, as the inputs to the bitstream conformance test may include a parameter that indicates the list of target output layers.

For further study.

JCT3V-D0218 3D reference displays information SEI message for MV-HEVC and 3D-HEVC [A. Norkin (Ericsson)]

This contribution proposes to add an SEI message to add a 3D reference displays information SEI message to the MV-HEVC and 3D-HEVC draft specification. The SEI message is the same as in the MVC extension for inclusion of depth maps with the exception that the ViewId of two views are used instead of explicitly specifying the reference baseline.

It was commented that four syntax elements represent floating point coding of data, which does not seem nice, but such coding was the case already included in MVC, and these same syntax elements were also coded this way in MVC+D.

Agreed.