| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0310 |
| --- | --- |

| Title: | Report of the BoG on DLT Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | BoG Report |  |  |
| Author(s) or Contact(s): | Fabian Jäger | Tel: Email: |  |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This document provides a report of the Break-out Group on DLT Signaling in 3D-HEVC.

In the current 3D-HEVC design the DLT is coded in the VPS (SPS in the software) for each view independently. There are multiple proposals recommending to move the DLT signaling to the slice header and/or to introduce inter-view or even temporal update mechanisms to the DLT signaling.

JCT3V-D0147 3D-CE6.h: Results on Signalling of Depth Lookup Table [I. Lim, H. Wey, J. Y. Lee, B. Choi, C. Kim]

In this proposal, we moved VPS signalling to intra slice-layer signalling in Depth Lookup Table in 3DV-HTM. The current DLT is sent in the Video Parameter Set (VPS) but slice layer signalling is more appropriate. Because, the current DLT in VPS is decided through the initial analysis and the DLT is applied to the whole sequence. The initial analysis is not good solution to DLT construction in the varying scene and can cause coding loss in the various test sequences. In this proposal, we make and send new DLT either at every intra slice.

The DLT intra slice-layer signalling was implemented and the test results in CTC show 0.0% (video PSNR / video bitrate), 0.0% (video PSNR / total bitrate), 0.0% (synth PSNR / total bitrate) BD-rate. The test results in all intra condition show 0.00% (video PSNR / video bitrate), 0.29% (video PSNR / total bitrate), 0.25% (synth PSNR / total bitrate) BD-rate when compared with 3DV-HTM ver. 6.0 [2]. The coding loss in all intra conditions is negligibly small compared to DLT’s own coding gain.

Finally, it is more appropriate for the intra slice-layer signalling to be adopted into 3DV-HTM.

D0147 proposes to code the DLT in the slice header of intra slices instead of signaling it in the SPS for each view. The DLT for other slices is copied/referenced from the intra slice DLT. Functionality of DLT is reduced as different views/slices use the same DLT entries.

JCT3V-D0153 3D-CE6.h related: A flexible DLT signaling method in 3D-HEVC [K. Zhang, Y.-W. Huang, J. An, J.-L. Lin, S. Lei (MediaTek)]

This contribution proposes a flexible depth lookup table (DLT) signaling method in 3D-HEVC. First, it is allowed to signal DLTs in SPS. Second, a use_sps_dlt_flag is signaled in the slice header to indicate whether to use a DLT coded in SPS. If the use_sps_dlt_flag is set to 1, a sps_dlt_idx is signaled to indicate which DLT in SPS is selected when there are more than one DLT coded in SPS; otherwise, a new DLT or a DLT from a reference picture can be selected as follows. For an I slice, a new DLT is coded in the slice header. For a B or P slice, a use_ref_pic_dlt_flag is signaled. If the use_ref_pic_dlt_flag is set to 1, a dlt_ref_idx is signaled to explicit indicate the reference picture DLT; otherwise, a new DLT is coded. The proposed method can accommodate encoders that can analyze sequences in advance. It can also allow quick adaptation to scene changes and avoid introducing much side information when there is no scene change. It is reported that the proposed method results in similar coding efficiency as the anchor when there is no scene change.

D0153 proposes to indicate the reference DLT by a reference index that is used for the current slice. If no reference DLT is used, a new DLT is coded in the slice header. DLT can alternatively be coded in the VPS. Introduces a dependency to the reference DLT. A comment was made that signaling multiple DLTs in the VPS might be too much data for a VPS.

JCT3V-D0054 CE6.H related: Results on Updating Mechanism for Coding of Depth Lookup Table (Delta-DLT) [F. Jäger (RWTH Aachen University)]

Negligible coding loss/gain for CTC and all-intra coding configuration.

Benefit was questioned by one of the eperts. The proponent claims the flexibility in varying depth values in the inter-view and the temporal direction.

In the proposal the reference idx is not signaled as the reference DLT is stored until the next RAP. It is commented that for sequences with strongly changing depth values, a resending oft he whole DLT might be simpler and similarly efficient as the fine-grained slice-level update oft he DLT.

JCT3V-D0124 3D-CE6.h related: SDC with DLT in intra slices [J. Y. Lee, H.-C. Wey, C. Kim (Samsung)]

Simplified depth coding (SDC) with depth lookup table (DLT) was integrated on the current 3D-HTM (JCT3V-B0036). SDC predicts a depth block with DMM1, DC, and Planar modes of a 2Nx2N partition. When the block is coded with SDC, it signals an index with DLT instead of a residual value. DLT is a mapping table from a residual value to an index, and generated once based on the first several depth frames. DLT is transmitted to a decoder for the inverse mapping from an index to a value in a sequence parameter set (SPS) level. At the last meeting, DLT signalling in intra slices instead of SPS and its simplification was proposed (JCT3V-C0093 and JCT3V-C0142). In addition, it was proposed that DLT is generated in every intra slice to avoid the performance degradation by scene changes in the middle of a sequence. In this proposal, SDC with DLT is only used in intra slices. If DLT is generated per every intra slice, only using DLT in intra slices is more reasonable. Experimental result shows that the proposed method has no coding loss.

D0124 restricts the usage of DLT and SDC to intra slices. The coding efficiency is increased by 0.3% for CTC and decreased by 0.5% for AI.

The proposal claims that disabling SDC/DLT completely yields performance gain in CTC (0.28% on synthesized views) and losses for All-Intra (1.23% on coded and 0.51% on synthesized views). It may be that the relation of SDC with VSP might result in these inconclusive results. Needs to be cross checked and analyzed further (e.g. as part of the CE).

JCT3V-D0142 3D-CE6.h: Improved DLT coding method [J. Heo, S. Yea (LG)]

This contribution presents an improvement of depth lookup table (DLT) used in simplified depth coding (SDC) for depth map coding. In the current SDC, DLT was employed to encode the residual index and all valid depth values in DLT are encoded with Exp-Golomb code at the sequence parameter set (SPS) level. However since each value is coded in itself without considering the correlation between sequential values, the number of bits used to code a valid depth value in DLT is redundant. Therefore, this contribution proposes an improved DLT coding method. Under common test condition this method provides 0.0% BD-rate gain with no impact on the encoder/decoder complexity.

The proposal changes the way DLT entries are coded. Coding gain is negligible.

JCT3V-D0182 CE6.h related: On Signaling of DLT for depth coding [X. Zhao, L. Zhang, Y. Chen, M. Karczewicz(Qualcomm)]

This contribution proposes a more efficient method to signal depth look-up tables (DLTs) used for depth map intra coding in 3D-HEVC. In the proposed method, both prediction of the values of one DLT table of one depth view and prediction of DLT tables of multiple depth views are enabled to avoid largly increased number of bits requried in Video Parameter Set (VPS). With proposed methods, the total number of DLT bits is reduced to 12% of the original size without any impact on the coded PSNR values.

D0182 proposed to signal the DLT in the VPS and allow inter-view update of the base view’s DLT. The update is signaled by delta values for the dependent views. The coding of DLT values is also modified as in D0142. Fixed-length coding is used for all syntax elements. Performance gain is negligible.

A comment was made that SPS or slice header signaling is preferred. Another expert comments that VPS signaling is acceptable as there is per-view identification, too. VPS signaling may be beneficial to reduce redundancies, which occur with SPS signaling of this kind of data.

JCT3V-D0172 3D-CE6.h related: Differential coding method for DLT in 3D-HEVC [M. Li, P. Wu, H. Li, G. Shang, Y. Xie (ZTE)]

In HTM6.0, depth loop-up table (DLT) is signalled in the corresponding sequence parameter set (SPS) for depth components in each view. Statistical results show that DLT information takes about 66% of the total coding bits of SPS. To reduce the coding bits of DLT, differential coding is proposed to perform lossless compression on DLT elements. Experimental results demonstrate that compared with HTM6.0 anchors, an average of about 75% DLT bits are saved with slight BD-BitRate reduction.

Signaling of DLT is moved from SPS (anchor) to VPS in the proposed method. A more efficient differential coding method of DLT values is also introduced.

One expert comments that a simplified coding method for DLT values is preferable. Differential coding methods show best performance among the different approaches. A slice-level updating may not be necessary.

Another expert raises concerns about D0182 to be too complicated and to sequence-optimized. Inter-view prediction of DLT values needs to be in VPS, as proposed in D0182.

Complexity for coding DLT values is not critical, especially not relative to overall complexity. Moreover, the differential coding method is straight-forward and switchable.