| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0311 |
| --- | --- |

| Title: | On Scalable Identifier Derivation |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |
| Author(s) or Contact(s): | Robert Skupin, Byeongdoo Choi | Tel: Email: | +49 30 31428773 robert.skupin@hhi.fraunhofer.de b.d.choi@samsung.com |  |  |
| Source: | Fraunhofer HHI, Samsung Electronics Co., Ltd. |  |  |  |  |

_____________________________

# Abstract

This contribution proposes to condition the presents of one or several syntax elements in the VPS extension syntax on the syntax element splitting_flag and optionally to add a derivation process for scalable identifier variables to reduce the number of bits for scalability signalling. Proposed specification text changes and achieved bit savings are reported.

# Introduction

During the 12th JCT-VC Meeting, the syntax element splitting_flag [1] was adopted to be included in the VPS extension syntax of the 3D-HEVC [4], MV-HEVC [3] and SHVC [2] extensions of HEVC. In the extensions the splitting_flag indicates that an alternative derivation of the scalable identifiers, e.g. ViewID, is possible through a masked bitcopy of the bits of nuh_layer_id as opposed to the derivation process described in the semantics of dimension_id.

However, the current specification requires the related syntax elements (vps_nuh_layer_id_present_flag, layer_id_in_nuh[ i ], dimension_id[ i ][ j ]) to be signalled, even when the alternative derivation through a masked bitcopy is possible without the listed redundant syntax elements when splitting_flag is equal to 1.

Conditioning the presents of syntax elements vps_nuh_layer_id_present_flag, layer_id_in_nuh[ i ] and  dimension_id[ i ][ j ] on the value of the syntax element splitting_flag and adding an derivation process as opposed to the current semantic constraint is argued to be a negligible complexity increase in relation to the reported bit savings in Section 3 and a cleaner design.

Revision 1 of this document includes an additional proposal and results in which only presence of the syntax element dimension_id[ i ][ j ] is conditioned on the value of the syntax element splitting_flag and semantic changes to infer dimension_id[ i ][ j ] when not present.

Revision 2 of this document adds inference of dimension_id[ i ][ j ] for nuh_layer_id that are not present in the bitstream to proposal 2 as suggested during BoG review.

# Proposal 1

This document proposes to add a derivation process to the semantics of the syntax element splitting_flag and add a condition to the VPS extension syntax, exemplarily taken from [3]. A corresponding note is deleted and the semantics of dimension_id are adjusted. Deletions are marked in red and additions are highlighted in yellow.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| splitting_flag | u(1) |
| … |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| if (!splitting_flag) { |  |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

The variable dimBitOffset[ 0 ] is set equal to 0 and for j in the range of 1 to NumScalabilityTypes, inclusive, dimBitOffset[ j ] is derived as follows.

dimBitOffset[ j ]=dimIdx=0j−1dimension_id_len_minus1[ dimIdx ]+1	(F-1)

splitting_flag equal to 1 indicates that the bits of the nuh_layer_id syntax element in the NAL unit header are split into n segments with a length, in bits, according to the values of the dimension_id_len_minus1[ i ] syntax element and that the n segments are associated with the n scalability dimensions indicated in scalability_mask_flag[ i ]. When splitting_flag is equal to 1, the value of the j-th segment of the nuh_layer_id of i-th layer shall be equal to the value of dimension_id[ i ][ j ]. splitting_flag equal to 0 does not indicate the above constraint.

the variable ViewId[ i ] specifying the view identifier of the layer with nuh_layer_id equal to i is derived as follows:

ViewId[ i ] =  ( i  & ( (1 << dimBitOffset[1 ] ) − 1) ) >> dimBitOffset[ 0]

NOTE 1 – When splitting_flag is equal to 1, i.e. the restriction reported in the semantics of the dimension_id[ i ][ j ] syntax element is obeyed, scalable identifiers can be derived from the nuh_layer_id syntax element in the NAL unit header by a bit masked copy as an alternative to the derivation as reported in the semantics of the dimension_id[ i ][ j ] syntax element. The respective bit mask for the i-th scalable dimension is defined by the value of the dimension_id_len_minus1[ i ] syntax element and dimBitOffset[ i ] as specified in the semantics of dimension_id_len_minus1[ j ].

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits. When splitting_flag is equal to 0, 1, it is a requirement of bitstream conformance that dimension_id[ i ][ j ] shall be equal to ( ( layer_id_in_nuh[ i ] & ( (1 << dimBitOffset[ j + 1 ] ) − 1) ) >> dimBitOffset[ j ] ).

tThe variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer and the variable ViewId[ layer_id_in_nuh[ i ] ] specifying the view identifier of the i-th layer are derived as follows:

for (i = 0; i <= vps_max_layers_minus1; i++) {

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  ! =  0 )  &&  scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

}

# Results for Proposal 1

The proposal allows for bits savings as reported in the following table in the extensions of HEVC compared to the current scalability signalling.

| Video setup | vps_nuh_layer_id_present_flag = 0 | vps_nuh_layer_id_present_flag = 1 |
| --- | --- | --- |
| 1 spatial enhancement layer | 2 bits | 8 bits |
| 3 views with depth | 19 bits | 55 bits |
| 3 views with depth and 1 spatial enhancement layer | 37 bits | 109 bits |

# Proposal 2

This proposal conditions only the presents of the syntax element dimension_id[ i ][ j ] on the value of the syntax element splitting_flag. Proposal 2 yields lower bit savings compared to Proposal 1 but does not require a second derivation process. It is proposed to infer the value of dimension_id[ i ][ j ] when it is not present. Deletions are marked in red and additions are highlighted in yellow.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| splitting_flag | u(1) |
| … |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if (!splitting_flag) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| … |  |
| } |  |

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits. When not present, the value of When splitting_flag is equal to 1, it is a requirement of bitstream conformance that dimension_id[ i ][ j ] is inferred to shall be equal to ( ( layer_id_in_nuh[ i ] & ( (1 << dimBitOffset[ j + 1 ] ) − 1) ) >> dimBitOffset[ j ] ) for each value of i in the range of 1 to vps_max_layers_minus1, inclusive. Otherwise, dimension_id[ i ][ j ] is inferred to be equal to 0.

# Results for Proposal 2

Proposal 2 allows for bits savings as reported in the following table in the extensions of HEVC compared to the current scalability signalling in the VPS extension syntax.

| Video setup | Bit savings compared to current VPS extension syntax |
| --- | --- |
| 1 spatial enhancement layer | 1 bits |
| 3 views with depth | 18 bits |
| 3 views with depth and 1 spatial enhancement layer | 36 bits |

# References

- R. Skupin, V. George, K. Suehring, T. Schierl (Fraunhofer HHI), “On VPS extension”, JCTVC-L0200, Geneva, CH, Jan. 2013

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela	, “SHVC Working Draft 1”, JCTVC-L1008, Geneva, CH, Jan. 2013

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 3”, JCT3V-C1004, Geneva, CH, Jan. 2013

- G. Tech, K. Wegner, Y. Chen, S.Yea, “3D-HEVC Test Model 3“, JCT3V-C1005, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).