| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0312 |
| --- | --- |

| Title: | Report of the BoG on block size for the second disparity compensation in BVSP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | BoG Report |  |  |
| Author(s) or Contact(s): | Gwo Giun Lee | Tel: Email: | +886-6-2757575 ext 62448 clee@mail.ncku.edu.tw |
| Source: | Nation Cheng Kung University |  |  |

_____________________________

# Abstract

This document provides a report of the Break-out Group meeting on block size for the second disparity compensation in BVSP in 3D-HEVC which was held in Songdo Convensia, Incheon Korea on April 22, 2013 from 4:00Pm ~ 6:00PM.  The goal was to perform complexity assessment of the proposed technologies in JCT3V-D0089, JCT3V-D0179, JCT3V-D0213, JCT3V-D0165 with various block partitioning algorithms based on the number of operations, data transfer rate, and data storage requirements.

# Participants

| Participant | Contact | Email |
| --- | --- | --- |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp |
| Mitsubishi Electric | Dong Tian | tian@merl.com |
| Qualcomm | Li Zhang Ying Chen | lizhang@qti.qualcomm.com cheny@qti.qualcomm.com |
| NCKU | Chun-Fu Chen | n28991146@mail.ncku.edu.tw |

# Description of Each Proposed Technology

JCT3V-D0089 3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Complexity Analysis [C.-F. Chen, G. G. Lee, B.-S. Li(NCKU)]

This proposal proposes the techniques which would adaptively partition the depth block to derive the disparity vectors. This adaptively partition scheme is based on the gradients of depth block, the supported block sizes include 4x8, 8x4, and 8x8 block sizes. Four different settings on the calculation of gradients are tested. Detailed complexity analysis based on the number of the logical operations, data storage requirement, and data transfer rate are also provided.

Method of D0089 subdivides into 8x8, then an analysis of the corresponding depth block (based on gradient) is made to determine whether it is further subdivided into 8x4/4x8 or left as 8x8. Then, the maximum value of depth is used for compensation in the block

JCT3V-D0292 Cross check of 3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Complexity Analysis (JCT3V-D0089) [C.-H. Hung, C.-C. Lin(ITRI)] [late]

JCT3V-D0213 3D-CE1.h: Adaptive block size on VSP [S. Shimizu, S. Sugimoto, H. Kimata (NTT)]

This contribution proposes to use either 4x8 or 8x4 block size in VSP. The block size is adaptively decided by analyzing four corner samples of virtual depth map generated by the first DCP. This contribution also asserted that the current VSP implementation call the interpolation function (core MC/DC function) for each pixel by clipping the disparity vector for each pixel although disparity vector is derived for each 4x4 block. This means that pixel-based MC/DC is performed since block based clipping provides difference results.

Method of D0213 analyses four corner samples of the 8x8 subblock, and then decides whether it is split horizontally or vertically (it is always split) – remaining procedure stays

JCT3V-D0219 CE1.h: Crosscheck on NTT proposal JCT3V-D0213 [D. Tian (MERL)] [late]

JCT3V-D0179 CE1.h related: Low-complexity BVSP for 3D-HEVC [L. He, L. Zhang, Y. Chen(Qualcomm)]

This contribution proposes that the size of motion compensation unit in VSP is set based on the size of the CU so that when the size of the CU becomes larger, less blocks in an inter-view reference picture need to be accessed.

Method of D0179 further subdivides the PU where VSP is signaled into N/2xN/2 blocks (e.g. 2NxN is subdivided into 8 blocks – asymmetric partitioning currently not in HTM), then proceeds as usual, minimum VSP block size still 4x4.

JCT3V-D0251 CE1.h related: cross-check of Low-complexity BVSP (JCT3V-D0179) [T. Ikai (Sharp)] [late]

JCT3V-D0165 3D-CE1.h: VSP Complexity Analysis and Constraint VSP (CVSP) [F. Zou, D. Tian, A. Vetro, H. Sun (MERL)]

This contribution proposes pixel-based disparity compensation with some constraints for VSP. One constraint is zero vertical disparity, and the other constraint is to limit variation of horizontal disparity vector within one block. These constraints reduce the number of fetched pixels for VSP compensation to maintain comparable complexity with traditional block-based MC. More detailed analysis based on a practical hardware implementation.

Method of D0165 uses the depth values of the corresponding area in the depth map to apply pixel-based compensation, with some additional constraints.

JCT3V-D0225 3D-CE1.h: VSP Complexity Analysis and Constraint VSP (JCT3V-D0165) [S. Shimizu, S. Sugimoto (NTT)] [late]

JCT3V-D0293 3D-CE1.h related: Cross-Check of the results on the Constraint VSP in JCT3V-D0165 [Y.-W. Chen, J.-L. Lin (MediaTek)] [late]

## Coding performance

| D number | Min. size | Max. size | How to decide | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D0089 | 4x8, 8x4 | 8x8 | Gradient of virtual depth map | 0.01% | -0.04% | 101.3% | N |
|  |  |  |  | 0.04% | -0.01% | 100.2% | Y (D0292) |
|  |  |  |  | 0.04% | 0.00% | 100.4% | N |
|  |  |  |  | 0.05% | 0.03% | 100.2% | Y (D0292) |
| D0179 | 4x4 | N/2 x N/2 | CU size | 0.11% | 0.04% | 99.0% | Y (D0251) |
| D0213 | 4x8, 8x4 | 4x8, 8x4 | Corner samples of virtual depth map | 0.08% | 0.02% | 97.2% | Y (D0219) |
| D0165 | 1x1 | 1x1 | n/a | -0.22% | -0.21% | 107.6% | Y (D0225) |
|  |  |  |  | -0.20% | -0.19% | 103.2% |  |

# Summary of Complexity Assessment in BoG Meeting

## Complexity assessment

JCT3V-D0089, D0179, and D0213: analysis is made under one PU partition. Three complexity metrics are analyzed, including number of operations, data storage, and data transfer rate.

Table 1 Overall summary of worst case

|  | # of operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Add/Sub/Com (ratio to anchor) | Constant Mul. (ratio to anchor) | Data storage (ratio to anchor) | Data storage (Bit difference to anchor) | Data transfer rate (ratio to anchor) | # of times of storage accessing (ratio to anchor) |
| JCT3V-D0089 | 103.95% | 100.00% | 101.07% | 704 | 100.00% | 50.00% |
| JCT3V-D0179 | 100.00% | 100.00% | 103.93% | 2592 | 71.16% | 6.25% |
| JCT3V-D0213 | 101.15% | 100.00% | 100.53% | 352 | 100.00% | 50.00% |

Note:

The worst case is the number of the maximum requirement of the three complexity metrics among the possible subblocks at various PU size

JCT3V-D0165: analysis is made under a 64x64 block with all 8x4 PB partitions

| Complexity metrics | MC in HEVC | 1x1 VSP | 4x4 VSP MC | 1x1 CVSP |
| --- | --- | --- | --- | --- |
| Off-chip memory data rate | 100% | 155% | 53% | 99% |
| Number of data access | 100% | 300% | 72% | 36% |
| Off-chip random access | Yes | Yes | Yes | Yes |
| Off-clip line memory loading | Yes | No | Yes | Yes |
| Off-chip irregular addressing | No | Yes | No | No |
| Off-chip overall | Anchor | Much more complex | A bit more | Much less |
| Encoder impacts | Motion estimation | No ME | No ME | No ME |
| Chip changes | Anchor | Yes | Yes | Yes |
| Bitrate saving | 0.0% | 1.0% | 0.8% | 1.0% |

In the worst case scenario tabulated in Table 1, the overhead in the number of operations for JCT3D-D0089 is due to the estimation of the relatively more adaptive mechanism in block partitioning as compared to JCT3V-D0213.  JCT3V-D0179 performs a fixed block partitioning of N/2 and hence does not introduce overhead when compared to the anchor.  JCT3V-D0179 has the lowest data transfer rate and number of data storage transfers but however has the largest requirement for data storage since due to their algorithm, data is fetch and stored.  This of course reduces both the number data access and transfer rates.

The constraint VSP algorithm in JCT3V-D165 has revealed lower complexity in both the data transfer rate and number of data accesses as compared to both the anchor and their original VSP algorithm

The details of calculations are tabulated in subsequent tables.

### Number of operations

| JCT3V- D0089 | Possible sizes of subblocks number of operations |  |  |  |  |  |  |  |  |  | Anchor |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16x16 |  | 8x8 |  | 8x4 |  | 4x8 |  | 4x4 |  | 4x4 |  |
| PU_size | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub/ Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul |
| 64x64 | N/A | N/A | 40449 | 32768 | 40449 | 32768 | 40449 | 32768 | N/A | N/A | 38913 | 32768 |
| 64x32 | N/A | N/A | 20225 | 16384 | 20225 | 16384 | 20225 | 16384 | N/A | N/A | 19457 | 16384 |
| 32x64 | N/A | N/A | 20225 | 16384 | 20225 | 16384 | 20225 | 16384 | N/A | N/A | 19457 | 16384 |
| 32x32 | N/A | N/A | 10113 | 8192 | 10113 | 8192 | 10113 | 8192 | N/A | N/A | 9729 | 8192 |
| 32x16 | N/A | N/A | 5057 | 4096 | 5057 | 4096 | 5057 | 4096 | N/A | N/A | 4865 | 4096 |
| 16x32 | N/A | N/A | 5057 | 4096 | 5057 | 4096 | 5057 | 4096 | N/A | N/A | 4865 | 4096 |
| 16x16 | N/A | N/A | 2529 | 2048 | 2529 | 2048 | 2529 | 2048 | N/A | N/A | 2433 | 2048 |
| 16x8 | N/A | N/A | 1265 | 1024 | 1265 | 1024 | 1265 | 1024 | N/A | N/A | 1217 | 1024 |
| 8x16 | N/A | N/A | 1265 | 1024 | 1265 | 1024 | 1265 | 1024 | N/A | N/A | 1217 | 1024 |
| 8x8 | N/A | N/A | 633 | 512 | 633 | 512 | 633 | 512 | N/A | N/A | 609 | 512 |
| 8x4 | N/A | N/A | 305 | 256 | 305 | 256 | N/A | N/A | N/A | N/A | 305 | 256 |
| 4x8 | N/A | N/A | 305 | 256 | N/A | N/A | 305 | 256 | N/A | N/A | 305 | 256 |

| JCT3V- D0089 | Possible sizes of subblocks number of operations (ratio to anchor) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16x16 |  | 8x8 |  | 8x4 |  | 4x8 |  | 4x4 |  |
| PU_size | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub/ Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul |
| 64x64 | N/A | N/A | 103.95% | 100.00% | 103.95% | 100.00% | 103.95% | 100.00% | N/A | N/A |
| 64x32 | N/A | N/A | 103.95% | 100.00% | 103.95% | 100.00% | 103.95% | 100.00% | N/A | N/A |
| 32x64 | N/A | N/A | 103.95% | 100.00% | 103.95% | 100.00% | 103.95% | 100.00% | N/A | N/A |
| 32x32 | N/A | N/A | 103.95% | 100.00% | 103.95% | 100.00% | 103.95% | 100.00% | N/A | N/A |
| 32x16 | N/A | N/A | 103.95% | 100.00% | 103.95% | 100.00% | 103.95% | 100.00% | N/A | N/A |
| 16x32 | N/A | N/A | 103.95% | 100.00% | 103.95% | 100.00% | 103.95% | 100.00% | N/A | N/A |
| 16x16 | N/A | N/A | 103.95% | 100.00% | 103.95% | 100.00% | 103.95% | 100.00% | N/A | N/A |
| 16x8 | N/A | N/A | 103.94% | 100.00% | 103.94% | 100.00% | 103.94% | 100.00% | N/A | N/A |
| 8x16 | N/A | N/A | 103.94% | 100.00% | 103.94% | 100.00% | 103.94% | 100.00% | N/A | N/A |
| 8x8 | N/A | N/A | 103.94% | 100.00% | 103.94% | 100.00% | 103.94% | 100.00% | N/A | N/A |
| 8x4 | N/A | N/A | 100.00% | 100.00% | 100.00% | 100.00% | N/A | N/A | N/A | N/A |
| 4x8 | N/A | N/A | 100.00% | 100.00% | N/A | N/A | 100.00% | 100.00% | N/A | N/A |

| JCT3V- D0179 | Possible sizes of subblocks number of operations |  |  |  |  |  |  |  |  |  | Anchor |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16x16 |  | 8x8 |  | 8x4 |  | 4x8 |  | 4x4 |  | 4x4 |  |
| PU_size | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub/ Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul |
| 64x64 | 38913 | 32768 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 38913 | 32768 |
| 64x32 | 19457 | 16384 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 19457 | 16384 |
| 32x64 | 19457 | 16384 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 19457 | 16384 |
| 32x32 | N/A | N/A | 9729 | 8192 | N/A | N/A | N/A | N/A | N/A | N/A | 9729 | 8192 |
| 32x16 | N/A | N/A | 4865 | 4096 | N/A | N/A | N/A | N/A | N/A | N/A | 4865 | 4096 |
| 16x32 | N/A | N/A | 4865 | 4096 | N/A | N/A | N/A | N/A | N/A | N/A | 4865 | 4096 |
| 16x16 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 2433 | 2048 | 2433 | 2048 |
| 16x8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 1217 | 1024 | 1217 | 1024 |
| 8x16 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 1217 | 1024 | 1217 | 1024 |
| 8x8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 609 | 512 | 609 | 512 |
| 8x4 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 305 | 256 | 305 | 256 |
| 4x8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 305 | 256 | 305 | 256 |

| JCT3V- D0179 | Possible sizes of subblocks number of operations (ratio to anchor) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16x16 |  | 8x8 |  | 8x4 |  | 4x8 |  | 4x4 |  |
| PU_size | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub/ Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul |
| 64x64 | 100.00% | 100.00% | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 64x32 | 100.00% | 100.00% | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 32x64 | 100.00% | 100.00% | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 32x32 | N/A | N/A | 100.00% | 100.00% | N/A | N/A | N/A | N/A | N/A | N/A |
| 32x16 | N/A | N/A | 100.00% | 100.00% | N/A | N/A | N/A | N/A | N/A | N/A |
| 16x32 | N/A | N/A | 100.00% | 100.00% | N/A | N/A | N/A | N/A | N/A | N/A |
| 16x16 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 100.00% | 100.00% |
| 16x8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 100.00% | 100.00% |
| 8x16 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 100.00% | 100.00% |
| 8x8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 100.00% | 100.00% |
| 8x4 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 100.00% | 100.00% |
| 4x8 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | 100.00% | 100.00% |

| JCT3V- D0213 | Possible sizes of subblocks number of operations |  |  |  |  |  |  |  |  |  | Anchor |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16x16 |  | 8x8 |  | 8x4 |  | 4x8 |  | 4x4 |  | 4x4 |  |
| PU_size | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub/ Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul |
| 64x64 | N/A | N/A | N/A | N/A | 39361 | 32768 | 39361 | 32768 | N/A | N/A | 38913 | 32768 |
| 64x32 | N/A | N/A | N/A | N/A | 19681 | 16384 | 19681 | 16384 | N/A | N/A | 19457 | 16384 |
| 32x64 | N/A | N/A | N/A | N/A | 19681 | 16384 | 19681 | 16384 | N/A | N/A | 19457 | 16384 |
| 32x32 | N/A | N/A | N/A | N/A | 9841 | 8192 | 9841 | 8192 | N/A | N/A | 9729 | 8192 |
| 32x16 | N/A | N/A | N/A | N/A | 4921 | 4096 | 4921 | 4096 | N/A | N/A | 4865 | 4096 |
| 16x32 | N/A | N/A | N/A | N/A | 4921 | 4096 | 4921 | 4096 | N/A | N/A | 4865 | 4096 |
| 16x16 | N/A | N/A | N/A | N/A | 2461 | 2048 | 2461 | 2048 | N/A | N/A | 2433 | 2048 |
| 16x8 | N/A | N/A | N/A | N/A | 1231 | 1024 | 1231 | 1024 | N/A | N/A | 1217 | 1024 |
| 8x16 | N/A | N/A | N/A | N/A | 1231 | 1024 | 1231 | 1024 | N/A | N/A | 1217 | 1024 |
| 8x8 | N/A | N/A | N/A | N/A | 616 | 512 | 616 | 512 | N/A | N/A | 609 | 512 |
| 8x4 | N/A | N/A | N/A | N/A | 305 | 256 | N/A | N/A | N/A | N/A | 305 | 256 |
| 4x8 | N/A | N/A | N/A | N/A | N/A | N/A | 305 | 256 | N/A | N/A | 305 | 256 |

| JCT3V- D0213 | Possible sizes of subblocks number of operations (ratio to anchor) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16x16 |  | 8x8 |  | 8x4 |  | 4x8 |  | 4x4 |  |
| PU_size | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub/ Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul | Add/Sub /Comp | Cons_Mul |
| 64x64 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 64x32 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 32x64 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 32x32 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 32x16 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 16x32 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 16x16 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 16x8 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 8x16 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 8x8 | N/A | N/A | N/A | N/A | 101.15% | 100.00% | 101.15% | 100.00% | N/A | N/A |
| 8x4 | N/A | N/A | N/A | N/A | 100.00% | 100.00% | N/A | N/A | N/A | N/A |
| 4x8 | N/A | N/A | N/A | N/A | N/A | N/A | 100.00% | 100.00% | N/A | N/A |

### Data storage

| JCT3V-D0089 | Possible sizes of subblocks data storage (bits) |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | N/A | 66604 | 66124 | 66348 | N/A | 65900 |
| 64x32 | N/A | 33836 | 33356 | 33580 | N/A | 33132 |
| 32x64 | N/A | 33836 | 33356 | 33580 | N/A | 33132 |
| 32x32 | N/A | 17452 | 16972 | 17196 | N/A | 16748 |
| 32x16 | N/A | 9260 | 8780 | 9004 | N/A | 8556 |
| 16x32 | N/A | 9260 | 8780 | 9004 | N/A | 8556 |
| 16x16 | N/A | 5164 | 4684 | 4908 | N/A | 4460 |
| 16x8 | N/A | 3116 | 2636 | 2860 | N/A | 2412 |
| 8x16 | N/A | 3116 | 2636 | 2860 | N/A | 2412 |
| 8x8 | N/A | 2092 | 1612 | 1836 | N/A | 1388 |
| 8x4 | N/A | 1100 | 1100 | N/A | N/A | 876 |
| 4x8 | N/A | 1324 | N/A | 1324 | N/A | 876 |

| JCT3V-D0089 | Possible sizes of subblocks data storage (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | N/A | 101.07% | 100.34% | 100.68% | N/A |
| 64x32 | N/A | 102.12% | 100.68% | 101.35% | N/A |
| 32x64 | N/A | 102.12% | 100.68% | 101.35% | N/A |
| 32x32 | N/A | 104.20% | 101.34% | 102.67% | N/A |
| 32x16 | N/A | 108.23% | 102.62% | 105.24% | N/A |
| 16x32 | N/A | 108.23% | 102.62% | 105.24% | N/A |
| 16x16 | N/A | 115.78% | 105.02% | 110.04% | N/A |
| 16x8 | N/A | 129.19% | 109.29% | 118.57% | N/A |
| 8x16 | N/A | 129.19% | 109.29% | 118.57% | N/A |
| 8x8 | N/A | 150.72% | 116.14% | 132.28% | N/A |
| 8x4 | N/A | 125.57% | 125.57% | N/A | N/A |
| 4x8 | N/A | 151.14% | N/A | 151.14% | N/A |

| JCT3V-D0089 | Possible sizes of subblocks Difference bits (compared to anchor) based on this PU |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | N/A | 704 | 224 | 448 | N/A |
| 64x32 | N/A | 704 | 224 | 448 | N/A |
| 32x64 | N/A | 704 | 224 | 448 | N/A |
| 32x32 | N/A | 704 | 224 | 448 | N/A |
| 32x16 | N/A | 704 | 224 | 448 | N/A |
| 16x32 | N/A | 704 | 224 | 448 | N/A |
| 16x16 | N/A | 704 | 224 | 448 | N/A |
| 16x8 | N/A | 704 | 224 | 448 | N/A |
| 8x16 | N/A | 704 | 224 | 448 | N/A |
| 8x8 | N/A | 704 | 224 | 448 | N/A |
| 8x4 | N/A | 224 | 224 | N/A | N/A |
| 4x8 | N/A | 448 | N/A | 448 | N/A |

| JCT3V- D0179 | Possible sizes of subblocks data storage (bits) |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | 68492 | N/A | N/A | N/A | N/A | 65900 |
| 64x32 | 35724 | N/A | N/A | N/A | N/A | 33132 |
| 32x64 | 35724 | N/A | N/A | N/A | N/A | 33132 |
| 32x32 | N/A | 17356 | N/A | N/A | N/A | 16748 |
| 32x16 | N/A | 9164 | N/A | N/A | N/A | 8556 |
| 16x32 | N/A | 9164 | N/A | N/A | N/A | 8556 |
| 16x16 | N/A | N/A | N/A | N/A | 4460 | 4460 |
| 16x8 | N/A | N/A | N/A | N/A | 2412 | 2412 |
| 8x16 | N/A | N/A | N/A | N/A | 2412 | 2412 |
| 8x8 | N/A | N/A | N/A | N/A | 1388 | 1388 |
| 8x4 | N/A | N/A | N/A | N/A | 876 | 876 |
| 4x8 | N/A | N/A | N/A | N/A | 876 | 876 |

| JCT3V- D0179 | Possible sizes of subblocks data storage (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | 103.93% | N/A | N/A | N/A | N/A |
| 64x32 | 107.82% | N/A | N/A | N/A | N/A |
| 32x64 | 107.82% | N/A | N/A | N/A | N/A |
| 32x32 | N/A | 103.63% | N/A | N/A | N/A |
| 32x16 | N/A | 107.11% | N/A | N/A | N/A |
| 16x32 | N/A | 107.11% | N/A | N/A | N/A |
| 16x16 | N/A | N/A | N/A | N/A | 100.00% |
| 16x8 | N/A | N/A | N/A | N/A | 100.00% |
| 8x16 | N/A | N/A | N/A | N/A | 100.00% |
| 8x8 | N/A | N/A | N/A | N/A | 100.00% |
| 8x4 | N/A | N/A | N/A | N/A | 100.00% |
| 4x8 | N/A | N/A | N/A | N/A | 100.00% |

| JCT3V-D0179 | Possible sizes of subblocks Difference bits (compared to anchor) based on this PU |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | 2592 | N/A | N/A | N/A | N/A |
| 64x32 | 2592 | N/A | N/A | N/A | N/A |
| 32x64 | 2592 | N/A | N/A | N/A | N/A |
| 32x32 | N/A | 608 | N/A | N/A | N/A |
| 32x16 | N/A | 608 | N/A | N/A | N/A |
| 16x32 | N/A | 608 | N/A | N/A | N/A |
| 16x16 | N/A | N/A | N/A | N/A | 0 |
| 16x8 | N/A | N/A | N/A | N/A | 0 |
| 8x16 | N/A | N/A | N/A | N/A | 0 |
| 8x8 | N/A | N/A | N/A | N/A | 0 |
| 8x4 | N/A | N/A | N/A | N/A | 0 |
| 4x8 | N/A | N/A | N/A | N/A | 0 |

| JCT3V- D0213 | Possible sizes of subblocks data storage (bits) |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | N/A | N/A | 66028 | 66252 | N/A | 65900 |
| 64x32 | N/A | N/A | 33260 | 33484 | N/A | 33132 |
| 32x64 | N/A | N/A | 33260 | 33484 | N/A | 33132 |
| 32x32 | N/A | N/A | 16876 | 17100 | N/A | 16748 |
| 32x16 | N/A | N/A | 8684 | 8908 | N/A | 8556 |
| 16x32 | N/A | N/A | 8684 | 8908 | N/A | 8556 |
| 16x16 | N/A | N/A | 4588 | 4812 | N/A | 4460 |
| 16x8 | N/A | N/A | 2540 | 2764 | N/A | 2412 |
| 8x16 | N/A | N/A | 2540 | 2764 | N/A | 2412 |
| 8x8 | N/A | N/A | 1516 | 1740 | N/A | 1388 |
| 8x4 | N/A | N/A | 1004 | N/A | N/A | 876 |
| 4x8 | N/A | N/A | N/A | 1228 | N/A | 876 |

| JCT3V- D0213 | Possible sizes of subblocks data storage (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | N/A | N/A | 100.19% | 100.53% | N/A |
| 64x32 | N/A | N/A | 100.39% | 101.06% | N/A |
| 32x64 | N/A | N/A | 100.39% | 101.06% | N/A |
| 32x32 | N/A | N/A | 100.76% | 102.10% | N/A |
| 32x16 | N/A | N/A | 101.50% | 104.11% | N/A |
| 16x32 | N/A | N/A | 101.50% | 104.11% | N/A |
| 16x16 | N/A | N/A | 102.87% | 107.89% | N/A |
| 16x8 | N/A | N/A | 105.31% | 114.59% | N/A |
| 8x16 | N/A | N/A | 105.31% | 114.59% | N/A |
| 8x8 | N/A | N/A | 109.22% | 125.36% | N/A |
| 8x4 | N/A | N/A | 114.61% | N/A | N/A |
| 4x8 | N/A | N/A | N/A | 140.18% | N/A |

| JCT3V-D0213 | Possible sizes of subblocks Difference bits (compared to anchor) based on this PU |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | N/A | N/A | 128 | 352 | N/A |
| 64x32 | N/A | N/A | 128 | 352 | N/A |
| 32x64 | N/A | N/A | 128 | 352 | N/A |
| 32x32 | N/A | N/A | 128 | 352 | N/A |
| 32x16 | N/A | N/A | 128 | 352 | N/A |
| 16x32 | N/A | N/A | 128 | 352 | N/A |
| 16x16 | N/A | N/A | 128 | 352 | N/A |
| 16x8 | N/A | N/A | 128 | 352 | N/A |
| 8x16 | N/A | N/A | 128 | 352 | N/A |
| 8x8 | N/A | N/A | 128 | 352 | N/A |
| 8x4 | N/A | N/A | 128 | N/A | N/A |
| 4x8 | N/A | N/A | N/A | 352 | N/A |

### Data transfer rate

| JCT3V- D0089 | Possible sizes of subblocks data transfer rate (bits/PU) |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | N/A | 172044 | 172044 | 213004 | N/A | 213004 |
| 64x32 | N/A | 86028 | 86028 | 106508 | N/A | 106508 |
| 32x64 | N/A | 86028 | 86028 | 106508 | N/A | 106508 |
| 32x32 | N/A | 43020 | 43020 | 53260 | N/A | 53260 |
| 32x16 | N/A | 21516 | 21516 | 26636 | N/A | 26636 |
| 16x32 | N/A | 21516 | 21516 | 26636 | N/A | 26636 |
| 16x16 | N/A | 10764 | 10764 | 13324 | N/A | 13324 |
| 16x8 | N/A | 5388 | 5388 | 6668 | N/A | 6668 |
| 8x16 | N/A | 5388 | 5388 | 6668 | N/A | 6668 |
| 8x8 | N/A | 2700 | 2700 | 3340 | N/A | 3340 |
| 8x4 | N/A | 1356 | 1356 | N/A | N/A | 1676 |
| 4x8 | N/A | 1676 | N/A | 1676 | N/A | 1676 |

| JCT3V- D0089 | Possible sizes of subblocks data transfer rate (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | N/A | 80.77% | 80.77% | 100.00% | N/A |
| 64x32 | N/A | 80.77% | 80.77% | 100.00% | N/A |
| 32x64 | N/A | 80.77% | 80.77% | 100.00% | N/A |
| 32x32 | N/A | 80.77% | 80.77% | 100.00% | N/A |
| 32x16 | N/A | 80.78% | 80.78% | 100.00% | N/A |
| 16x32 | N/A | 80.78% | 80.78% | 100.00% | N/A |
| 16x16 | N/A | 80.79% | 80.79% | 100.00% | N/A |
| 16x8 | N/A | 80.80% | 80.80% | 100.00% | N/A |
| 8x16 | N/A | 80.80% | 80.80% | 100.00% | N/A |
| 8x8 | N/A | 80.84% | 80.84% | 100.00% | N/A |
| 8x4 | N/A | 80.91% | 80.91% | N/A | N/A |
| 4x8 | N/A | 100.00% | N/A | 100.00% | N/A |

| JCT3V- D0089 | Possible sizes of subblocks # of times of storage access based on subblocks tabulated |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | N/A | 192 | 384 | 384 | N/A | 768 |
| 64x32 | N/A | 96 | 192 | 192 | N/A | 384 |
| 32x64 | N/A | 96 | 192 | 192 | N/A | 384 |
| 32x32 | N/A | 48 | 96 | 96 | N/A | 192 |
| 32x16 | N/A | 24 | 48 | 48 | N/A | 96 |
| 16x32 | N/A | 24 | 48 | 48 | N/A | 96 |
| 16x16 | N/A | 12 | 24 | 24 | N/A | 48 |
| 16x8 | N/A | 6 | 12 | 12 | N/A | 24 |
| 8x16 | N/A | 6 | 12 | 12 | N/A | 24 |
| 8x8 | N/A | 3 | 6 | 6 | N/A | 12 |
| 8x4 | N/A | N/A | 3 | N/A | N/A | 6 |
| 4x8 | N/A | N/A | N/A | 3 | N/A | 6 |

| JCT3V- D0089 | Possible sizes of subblocks # of times of storage access based on subblocks tabulated (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 64x32 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 32x64 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 32x32 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 32x16 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 16x32 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 16x16 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 16x8 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 8x16 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 8x8 | N/A | 25.00% | 50.00% | 50.00% | N/A |
| 8x4 | N/A | N/A | 50.00% | N/A | N/A |
| 4x8 | N/A | N/A | N/A | 50.00% | N/A |

| JCT3V- D0179 | Possible sizes of subblocks data transfer rate (bits/PU) |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | 151564 | N/A | N/A | N/A | N/A | 213004 |
| 64x32 | 75788 | N/A | N/A | N/A | N/A | 106508 |
| 32x64 | 75788 | N/A | N/A | N/A | N/A | 106508 |
| 32x32 | N/A | 43020 | N/A | N/A | N/A | 53260 |
| 32x16 | N/A | 21516 | N/A | N/A | N/A | 26636 |
| 16x32 | N/A | 21516 | N/A | N/A | N/A | 26636 |
| 16x16 | N/A | N/A | N/A | N/A | 13324 | 13324 |
| 16x8 | N/A | N/A | N/A | N/A | 6668 | 6668 |
| 8x16 | N/A | N/A | N/A | N/A | 6668 | 6668 |
| 8x8 | N/A | N/A | N/A | N/A | 3340 | 3340 |
| 8x4 | N/A | N/A | N/A | N/A | 1676 | 1676 |
| 4x8 | N/A | N/A | N/A | N/A | 1676 | 1676 |

| JCT3V- D0179 | Possible sizes of subblocks data transfer rate (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | 71.16% | N/A | N/A | N/A | N/A |
| 64x32 | 71.16% | N/A | N/A | N/A | N/A |
| 32x64 | 71.16% | N/A | N/A | N/A | N/A |
| 32x32 | N/A | 80.77% | N/A | N/A | N/A |
| 32x16 | N/A | 80.78% | N/A | N/A | N/A |
| 16x32 | N/A | 80.78% | N/A | N/A | N/A |
| 16x16 | N/A | N/A | N/A | N/A | 100.00% |
| 16x8 | N/A | N/A | N/A | N/A | 100.00% |
| 8x16 | N/A | N/A | N/A | N/A | 100.00% |
| 8x8 | N/A | N/A | N/A | N/A | 100.00% |
| 8x4 | N/A | N/A | N/A | N/A | 100.00% |
| 4x8 | N/A | N/A | N/A | N/A | 100.00% |

| JCT3V- D0179 | Possible sizes of subblocks # of times of storage access based on subblocks tabulated |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | 48 | N/A | N/A | N/A | N/A | 768 |
| 64x32 | 24 | N/A | N/A | N/A | N/A | 384 |
| 32x64 | 24 | N/A | N/A | N/A | N/A | 384 |
| 32x32 | N/A | 48 | N/A | N/A | N/A | 192 |
| 32x16 | N/A | 24 | N/A | N/A | N/A | 96 |
| 16x32 | N/A | 24 | N/A | N/A | N/A | 96 |
| 16x16 | N/A | N/A | N/A | N/A | 48 | 48 |
| 16x8 | N/A | N/A | N/A | N/A | 24 | 24 |
| 8x16 | N/A | N/A | N/A | N/A | 24 | 24 |
| 8x8 | N/A | N/A | N/A | N/A | 12 | 12 |
| 8x4 | N/A | N/A | N/A | N/A | 6 | 6 |
| 4x8 | N/A | N/A | N/A | N/A | 6 | 6 |

| JCT3V- D0179 | Possible sizes of subblocks # of times of storage access based on subblocks tabulated (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | 6.25% | N/A | N/A | N/A | N/A |
| 64x32 | 6.25% | N/A | N/A | N/A | N/A |
| 32x64 | 6.25% | N/A | N/A | N/A | N/A |
| 32x32 | N/A | 25.00% | N/A | N/A | N/A |
| 32x16 | N/A | 25.00% | N/A | N/A | N/A |
| 16x32 | N/A | 25.00% | N/A | N/A | N/A |
| 16x16 | N/A | N/A | N/A | N/A | 100.00% |
| 16x8 | N/A | N/A | N/A | N/A | 100.00% |
| 8x16 | N/A | N/A | N/A | N/A | 100.00% |
| 8x8 | N/A | N/A | N/A | N/A | 100.00% |
| 8x4 | N/A | N/A | N/A | N/A | 100.00% |
| 4x8 | N/A | N/A | N/A | N/A | 100.00% |

| JCT3V- D0213 | Possible sizes of subblocks data transfer rate (bits/PU) |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | N/A | N/A | 172044 | 213004 | N/A | 213004 |
| 64x32 | N/A | N/A | 86028 | 106508 | N/A | 106508 |
| 32x64 | N/A | N/A | 86028 | 106508 | N/A | 106508 |
| 32x32 | N/A | N/A | 43020 | 53260 | N/A | 53260 |
| 32x16 | N/A | N/A | 21516 | 26636 | N/A | 26636 |
| 16x32 | N/A | N/A | 21516 | 26636 | N/A | 26636 |
| 16x16 | N/A | N/A | 10764 | 13324 | N/A | 13324 |
| 16x8 | N/A | N/A | 5388 | 6668 | N/A | 6668 |
| 8x16 | N/A | N/A | 5388 | 6668 | N/A | 6668 |
| 8x8 | N/A | N/A | 2700 | 3340 | N/A | 3340 |
| 8x4 | N/A | N/A | 1356 | N/A | N/A | 1676 |
| 4x8 | N/A | N/A | N/A | 1676 | N/A | 1676 |

| JCT3V- D0213 | Possible sizes of subblocks data transfer rate (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | N/A | N/A | 80.77% | 100.00% | N/A |
| 64x32 | N/A | N/A | 80.77% | 100.00% | N/A |
| 32x64 | N/A | N/A | 80.77% | 100.00% | N/A |
| 32x32 | N/A | N/A | 80.77% | 100.00% | N/A |
| 32x16 | N/A | N/A | 80.78% | 100.00% | N/A |
| 16x32 | N/A | N/A | 80.78% | 100.00% | N/A |
| 16x16 | N/A | N/A | 80.79% | 100.00% | N/A |
| 16x8 | N/A | N/A | 80.80% | 100.00% | N/A |
| 8x16 | N/A | N/A | 80.80% | 100.00% | N/A |
| 8x8 | N/A | N/A | 80.84% | 100.00% | N/A |
| 8x4 | N/A | N/A | 80.91% | N/A | N/A |
| 4x8 | N/A | N/A | N/A | 100.00% | N/A |

| JCT3V- D0213 | Possible sizes of subblocks # of times of storage access based on subblocks tabulated |  |  |  |  | Anchor |
| --- | --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 | 4x4 |
| 64x64 | N/A | N/A | 384 | 384 | N/A | 768 |
| 64x32 | N/A | N/A | 192 | 192 | N/A | 384 |
| 32x64 | N/A | N/A | 192 | 192 | N/A | 384 |
| 32x32 | N/A | N/A | 96 | 96 | N/A | 192 |
| 32x16 | N/A | N/A | 48 | 48 | N/A | 96 |
| 16x32 | N/A | N/A | 48 | 48 | N/A | 96 |
| 16x16 | N/A | N/A | 24 | 24 | N/A | 48 |
| 16x8 | N/A | N/A | 12 | 12 | N/A | 24 |
| 8x16 | N/A | N/A | 12 | 12 | N/A | 24 |
| 8x8 | N/A | N/A | 6 | 6 | N/A | 12 |
| 8x4 | N/A | N/A | 3 | N/A | N/A | 6 |
| 4x8 | N/A | N/A | N/A | 3 | N/A | 6 |

| JCT3V- D0213 | Possible sizes of subblocks # of times of storage access based on subblocks tabulated (ratio to anchor) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PU_size | 16x16 | 8x8 | 8x4 | 4x8 | 4x4 |
| 64x64 | N/A | N/A | 50.00% | 50.00% | N/A |
| 64x32 | N/A | N/A | 50.00% | 50.00% | N/A |
| 32x64 | N/A | N/A | 50.00% | 50.00% | N/A |
| 32x32 | N/A | N/A | 50.00% | 50.00% | N/A |
| 32x16 | N/A | N/A | 50.00% | 50.00% | N/A |
| 16x32 | N/A | N/A | 50.00% | 50.00% | N/A |
| 16x16 | N/A | N/A | 50.00% | 50.00% | N/A |
| 16x8 | N/A | N/A | 50.00% | 50.00% | N/A |
| 8x16 | N/A | N/A | 50.00% | 50.00% | N/A |
| 8x8 | N/A | N/A | 50.00% | 50.00% | N/A |
| 8x4 | N/A | N/A | 50.00% | N/A | N/A |
| 4x8 | N/A | N/A | N/A | 50.00% | N/A |