| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D1005-v4 |
| --- | --- |

| Title: | 3D-HEVC Test Model 4 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-3V |  |  |
| Purpose: | Draft of 3D-HEVC Test Model Description |  |  |
| Author(s) or Contact(s): | Gerhard Tech Fraunhofer HHI Krzysztof Wegner Poznan University of Technology Ying Chen Qualcomm Incorporated Sehoon Yea LG Electronics | Email: Email: Email: Email: | gerhard.tech@hhi.fraunhofer.de kwegner@multimedia.edu.pl cheny@qti.qualcomm.com sehoon.yea@lge.com |
| Source: | Editor |  |  |

_____________________________

Abstract

Draft 4 of 3D-HEVC Test Model Description

Ed. Notes (WD4) (changes compare to JCT3V-C1005):

----------- Release v4 -----------

Accepted all change marks.

----------- Release v3 -----------

(GT-C02) Corrections, editorial improvements.

(KM-C01) Corrections, editorial improvements.

----------- Release v2 -----------

----------- Release v1 -----------

(GT-C01) Corrections, editorial improvements

(VSP) Incorporated missing draft text from last meeting, including JCT3V-D0105and 3DN-04/JCT3V-D0092

(Removal of parts that are not in SW)

(3DN-18/JCT3V-D0032/JCT3V-D0141/JCT3V-D0034) SDC Residual CABAC contexts.

(3DN-17/JCT3V-D0035) DLT for DMM deltaDC coding

(3DN-16/JCT3V-D0195) Unification of new intra modes in 3D-HEVC

(3DN-14/JCT3V-D0183) Simplified DC predictor for depth intra modes

(3DN-13/JCT3V-D0110) Sample-based simplified depth coding.

(3DN-12/JCT3V-D0060) Removal of parsing dependency for illumination compensation

(3DN-11/JCT3V-D0122) AMVP candidate list construction

(3DN-09/JCT3V-D0177) Advanced residual prediction for multiview coding

(3DN-08/JCT3V-D0138) Simplified DV derivation for DoNBDV and BVSP

(3DN-07/JCT3V-D0112) Default disparity vector derivation

(3DN-05/JCT3V-D0191) Clean-ups for BVSP in 3D-HEVC.

(3DN-03/JCT3V-D0181) CE2.h related: CU-based Disparity Vector Derivation

(3DN-01/JCT3V-D0156) HLS for stereo compatibility.

Ed. Notes (WD3) (changes compare to JCT3V-B1005):

(3DC-GT2) Editorial improvements, small corrections.

----------- Release d0 -----------

Split of Test Model text and specification text

(3DE-05) Alignment with MV-HEVC draft 3.

(3DE-01) Reordered sub-clauses related to disparity estimation and additional motion candidates.

(3DN-20) Alignment of JCT3V-C0152 + JCT3V-C0137.

(3DN-07/JCT3V-C0137) Texture motion vector candidate for depth.

(3DN-07/JCT3V-C0137) Removal of MPI.

(3DN-19) Camera parameters

(3Dn−03) Wedgelet pattern generation process.

(3Dn-01) Incorporated missing intra-predicted wedgelet partition mode

(3DN-08/JCT3V-C0138) Removal of parsing dependency for inter-view residual prediction.

(3DN-18/JCT3V-C0160) QTL disabled for RAP.

(3DN-17/JCT3V-C0154) Reference sub-sampling for SDC and DMM.

(3Dc-03) Fix SDC

(3DN-16/JCT3V-C0096) Removal of DMM 2 from SDC.

(3DN-15/JCT3V-C0034) Delta DC processing for DMMs.

(3DN-14/JCT3V-C0044) Signalling of wedgeIdx for DMM3.

(3DN-02/JCT3V-C0152) View synthesis prediction (without disparity derivation part).

(3DN-03/JCT3V-C0112) Restricted search of max disparity.

(3DN-01,02/JCT3V-C0131,JCT3V-C0152) Disparity derivation from depth maps.

(3Dc-02) Incorporated missing conditions of long/short-term pictures in AMVP (related to JCT3V-B0046).

(3DN-13/JCT3V-C0116) Inter-view vector scaling for AMVP.

(3DE-03) Incorporated derivation process for AMVP from base spec.

(3DN-12/JCT3V-C0115) Signalling of inter-view motion vector scaling.

(3DE-02) Incorporated TMVP text from base spec.

(3DN-09/JCT3V-C0047) Alternative reference index for TMVP.

(3DN-10/JCT3V-C0051) Unification of inter-view candidate derivation.

(3DE-01) Revised text related to residual prediction.

(3DN-06/JCT3V-C0129) Vertical component in residual prediction.

(3DN-05/JCT3V-C0097/JCT3V-C0141) Temporal blocks first in DV derivation.

(3DC-GT1) Editorial improvements, small corrections.

(3DN-04/JCT3V-C0135) Restriction on the temporal blocks for memory bandwidth reduction in DV derivation.

(3Dn-02) Full sample MV accuracy for depth.

(3DN-11/JCT3V-C0046) Extension of illumination compensation to depth.

(3Dc-01) Fix Illumination compensation (including ic_flag for skip).

Ed. Notes (WD2) (changes compared to JCT3V-A1005)

Accepted changes and marked delta to base spec

(3DC-GT2) Editorial improvements, small corrections

(3DC-CY) Editorial improvements, small corrections

(MVS-02/JCT3V-B0046) Treatment of inter-view pictures as long term- reference pictures

(3DE-11) Revised text related to 3Dn-01

(3Dn-01/m23639) Results on motion parameter prediction

(3DE-12) Revised text related residual prediction

(3DE-10) Revised text Related to Illumination compensation.

(3DN-01/JCT3V-B0045) Illumination compensation for inter-view prediction.

(3Dn-02/m24766) Restricted Inter-View Residual Prediction

(3DE-09) Revised text related to depth intra: Edge Intra

(3DE-09) Revised text related to depth intra: SDC

(3DE-09) Revised text related to depth intra: DMMs

(3DO-01/JCT3V-B0131) Depth distortion metric with a weighted depth fidelity term

(3DN-12/JCT3V-B0036) Simplified Depth Coding with an optional Depth LUT

(3DN-13/JCT3V-B0039) Simplified Wedgelet search for DMM modes 1 and 3

(3DN-03/JCT3V-B0083) Unconstrained motion parameter inheritance

(3DE-08) Incorporated context tables for SDC

(3DE-07) Improved MPI text.

(3DN-02/JCT3V-B0068) Incorporated Depth Quadtree Prediction.

(3DE-06) Incorporated parsing process, including tables for DMMs.

(3DE-05) Added missing initialization of invalid motion/disparity parameters

(3DC-03) Added missing pruning of collocated merge candidate due to number of total candidates.

(3DE-04) Moved pruning of spatial merge candidate B2 due to number of total candidates.

(3DE-03) Moved derivation of disparity one level higher in process hierarchy.

(3DE-02) Inserted "Derivation process for motion vector components and reference indices" from base spec

(3DC-02) Fixed storage of IvpMvFlagLX and IvpMvDisp.

(3DN-09-10-11/JCT3V-B0048,B0069,B0086) Modification inter-view merge candidates

(3DC-01) Fixed derivation of inter-view merge candidates.

(3DE-01) Revised derivation of disparity from temporal candidates

(3DN-04/JCT3V-B0047) Improvements for disparity vector derivation)

(3DN-08/JCT3V-B0136) Support of parallel merge in disparity vector derivation

(3DN-05/JCT3V-B0135) Modified disparity vector derivation process for memory reduction

(3DN-04/JCT3V-B0111) Decoupling inter-view candidate for AMVP

(3DN-07/JCT3V-B0096) Removal of dependency between multiple PUs in a CU for DV-derivation

(3DC-GT) Small corrections, editorial improvements

Ed. Notes (WD1) ( changes compare to N12744)

(3D08/JCT3V-A0126) (T,N) Simplified disparity derivation

(3D16) Moved 3D-tool related flags from SPS to VPS, removal camera parameters

(3D09/JCT3V-A0049) (N) Inter-view motion prediction modification

(3D13/JCT3V-A0119) (T) VSO depth fidelity

(3D07/JCT3V-A0070) (T,N) Region boundary chain coding for depth maps

(3D06/JCT3V-A0087) (T) RDO selection between Non-Zero Residual and All-Zero Residual Intra

(3D12) (T) Depth Quadtree Prediction

(3D15) (N) Fix references

(3D11) (T,N) Improvement of text of already adopted tools

(3D10/JCT3V-A0097) (T;N) Disparity vector generation

(3D02) (N) Removed MV-Part and update to Annex F

(3D03) (T) Labelling of tools not in CTC/Software. Removal?

(3D05/JCT3V-A0093) (T) VSO early skip

(3D04/JCT3V-A0033) (T) VSO model based estimation

(3D14) (N) Update of low level specification to match HEVC text specification 8(d7)

(3D01) (N): Removed HEVC text specification

Contents

Abstract	2

Contents	5

List of Figures	6

List of Tables	7

1.	Data Format and System Description	8

2.	Coding Algorithm	9

2.1	Coding of the Independent View	11

2.2	Coding of Dependent Views	11

2.2.1	Disparity-compensated prediction	11

2.2.1.1	Reference list construction and modification	12

2.2.2	Inter-view motion prediction	13

2.2.2.1	Derivation of Disparity Vectors	14

2.2.2.2	Usage of Inter-View Motion Parameter Prediction	20

2.2.2.3	Derivation of co-located motion vector candidate	21

2.2.3	Inter-view residual prediction	22

2.2.4	Illumination compensation (IC)	24

2.2.5	View synthesis prediction (VSP)	24

2.3	Coding of Depth Maps	26

2.3.1	Modified motion compensation and motion vector coding	26

2.3.2	Disabling of in-loop filtering	26

2.3.3	Depth modelling modes	26

2.3.3.1	Mode 1: Explicit Wedgelet Signalization	27

2.3.3.2	Mode 2: Intra-predicted Wedgelet Partitions	28

2.3.3.3	Mode 3: Restricted signalling and inter-component prediction of Wedgelet partitions	29

2.3.3.4	Mode 4: Inter-component prediction of Contour partitions	29

2.3.3.5	Constant partition value coding	29

2.3.3.6	Co-located Texture Luma Block intra direction constraint	31

2.3.3.7	Coarse wedgelet search with refinement step	32

2.3.3.8	Mode selection	32

2.3.3.9	Signalling of mode parameters in the bitstream	33

2.3.4	Region boundary chain coding	33

2.3.4.1	Signalling in the bitstream	34

2.3.5	Simplified depth coding	35

2.3.5.1	Depth Lookup Table	35

2.3.6	Unified signalling of depth intra modes	36

2.3.7	Motion parameter inheritance	36

2.3.8	Depth Quadtree Prediction	37

2.4	Encoder Control	38

2.4.1	View Synthesis Optimization	38

2.4.1.1	Synthesized View Distortion Change (SVDC)	39

2.4.1.2	Model based synthesized view distortion estimation without rendering	43

2.4.1.3	Depth fidelity term	43

2.4.1.4	Integration of distortion metrics in the Encoder Control	43

2.4.1.5	Adaptation of the Lagrange Multiplier	43

2.4.2	Zero residual coding for depth intra CUs	44

2.4.3	Optional Encoder Control using a depth quadtree limitation	44

2.4.4	Optional Encoder Control for Renderable Regions in Dependent Views [Not in CTC]	44

3.	View Synthesis Algorithms	45

3.1	Fast 1-D View Synthesis (VSRS 1D Fast Mode)	45

3.1.1	Upsampling of input video pictures	46

3.1.2	Warping, interpolation and hole filling	46

3.1.3	Reliability map creation	47

3.1.4	Similarity enhancement	47

3.1.5	Combination	47

3.1.6	Chroma decimation	47

3.2	VSRS (alternative view synthesis algorithm) [Not in CTC]	48

3.2.1	General mode	48

3.2.2	1-d mode	50

4.	Software	53

4.1	Software repository	53

4.2	Build System	53

4.3	Software Structure	53

List of Figures

Figure 1: Overview of the system structure and the data format for the transmission of 3D video.	8

Figure 2: Access unit structure and coding order of view components.	9

Figure 3: Basic codec structure with inter-component prediction (red arrows).	11

Figure 4: Disparity-compensated prediction as an alternative to motion-compensated prediction.	12

Figure 5: Update of decoded picture buffer.	12

Figure 6: Construction of reference lists (L0, L1).	13

Figure 7: Modification of reference lists (L0, L1).	13

Figure 8: Basic principle of deriving motion parameters for a block in a current picture based on motion parameters in an already coded reference view and an estimate of the depth map for the current picture.	14

Figure 9: Mapping of a depth map into another view: (left) original depth map; (middle) converted depth map after displacing the original samples; (right) final converted depth map after filling of holes.	15

Figure 10: Generation of an initial depth map estimate after coding the first dependent view of a random access unit.	15

Figure 11: Derivation of a depth map estimate for the current picture using motion parameters of an already coded view of the same access unit.	16

Figure 12: Update of depth map estimate for a dependent view based on coded motion and disparity vectors.	17

Figure 13: Location of spatial neighbour blocks	17

Figure 14: Location of temporal neighbour blocks in temporal candidate pictures	17

Figure 15: The BR temporal block below the lower CTU row is not considered [ Ed. (GT): Replace "Current PU" with "Current CU" in the figure ]	18

Figure 16: The inter-view predicted motion vector of a MCP coded block.	19

Figure 17: Example: For the derivation of the disparity from DV-MCP neighbouring blocks of the current CU, above block B2, B1 and B0 are not used, since they are not located within the current CTU. [ Ed. (GT): Replace "Current PU" with "Current CU" in the figure ]	19

Figure 18: Retrieval of the virtual depth block	20

Figure 19: Inter-view motion vector scaling in TMVP	22

Figure 20: Prediction structure of the proposed ARP in 3D-HEVC.	23

Figure 21:  Relationship among current block, reference block and motion compensated block	23

Figure 22 Illustration of the VSP scheme with the neighboring block disparity vector	25

Figure 23: Wedgelet partition of a block: continuous (left) and discrete signal space (middle) with corresponding partition pattern (right).	27

Figure 24: Contour partition of a block: continuous (left) and discrete signal space (middle) with corresponding partition pattern (right).	27

Figure 25: Intra prediction of Wedgelet partition (blue) for the scenarios that the above reference block is either of type Wedgelet partition (left)or regular intra direction(right).	28

Figure 26: Prediction of Wedgelet (blue) and Contour (green) partition information from texture luma reference.	29

Figure 27: CPVs of block partitions: CPV prediction from adjacent samples of neighbouring blocks (left) and cross section of block (right), showing relation between different CPV types.	29

Figure 28: Selection of reference samples for difference partition pattern cases.	30

Figure 29: Optimized search strategy for non-quantized partition offset values.	31

Figure 30: Coarse subset of Wedgelet separation line start/end positions (left) and Wedgelet node with refinements (right).	32

Figure 31: Reconstructed edge using the relationship between previous and current directions	34

Figure 32: Example of region boundary chain coding	34

Figure 33: The derivation of corresponding texture block	37

Figure 34: Texture partitions and corresponding possible depth partitions	38

Figure 35: Definition of the SVDC related to the distorted depth data of the block B depicted by the hatched area in the bottom branch; VS denotes the view synthesis step and SSD stands for sum of squared differences.	39

Figure 36: Example for the dependencies between input, intermediate and output signals of the rendering or error calculation step.	41

Figure 37: Example of a CTB QT partitioning for the texture (left), allowed collocated depth CTB QT partitioning (centre), and disallowed collocated depth CTB QT partitioning (right).	44

Figure 38: Rendering from a left camera position to a right camera position using depth maps.	44

Figure 39: Distortion calculation on different tree depths. Renderable samples (gray shaded) are not taken into account.	45

Figure 40: Processing steps of the view synthesis approach.	46

Figure 41: Flow diagram for VSRS general mode.	49

Figure 42: Inpainting: “damaged” image, mask, and result after inpainting.	50

Figure 43: Flow diagram for VSRS 1D mode.	51

List of Tables

Table 1: The availability of the co-located motion vector	21

Table 2: Conditions for the availiability of the ic_flag	24

Table 3: Split flags and partition sizes of depth depending on split flags and partition sizes of texture	38

# Data Format and System Description

3D video is represented using the Multiview Video plus Depth (MVD) format, in which a small number of captured views as well as associated depth maps are coded and the resulting bitstream packets are multiplexed into a 3D video bitstream. After decoding the video and depth data, additional intermediate views suitable for displaying the 3D content on an auto-stereoscopic display can be synthesized using depth-image-based rendering (DIBR) techniques. For the purpose of view synthesis, camera parameters are additionally included in the bitstream. The bitstream packets include header information, which signal, in connection with transmitted parameter sets, a view identifier and an indication whether the packet contains video or depth data. Sub-bitstreams containing only some of the coded components can be extracted by discarding bitstream packets that contain non-required data. One of the views, which is also referred to as the base view or the independent view, is coded independently of the other views and the depth data using a conventional HEVC video coder. The sub-bitstream containing the independent view can be decoded by an unmodified HEVC video decoder and displayed on a conventional 2D display. Optionally, the encoder can be configured in a way that a sub-bitstream representing two views without depth data can be extracted and independently decoded for displaying the 3D video on a conventional stereo display. The codec can also be used for coding multiview video signals without depth data. In that case alternative methods such as Image Domain Warping (IDW) may be used to generate a multiview signal. And, when using depth data, it can be configured in a way that the video pictures can be decoded independently of the depth data.

Figure 1: Overview of the system structure and the data format for the transmission of 3D video.

The basic concept of the system and data format is illustrated in Figure 1. In general the input signal for the encoder consists of multiple views, associated depth maps, and corresponding camera parameters. However, as described above, the codec can also be operated without depth data. The input component signals are coded using a 3D video encoder, which represents an extension of HEVC. At this, the base view is coded using an unmodified HEVC encoder. The 3D video encoder generates a bitstream, which represents the input videos and depth data in a coded format. If the bitstream is decoded using a 3D video decoder, the input videos, the associated depth data, and camera parameters are reconstructed with the given fidelity. For displaying the 3D video on an autostereoscopic display, additional intermediate views are generated by a DIBR algorithm using the reconstructed views and depth data. If the 3D video decoder is connected to a conventional stereo display instead of to an autostereoscopic display, the view synthesizer can also generate a pair of stereo views, in case such a pair is not actually present in the bitstream. At this, it is possible to adjust the rendered stereo views to the stereo geometry to provide suitable viewing conditions. One of the decoded views or an intermediate view at an arbitrary virtual camera position can also be used for displaying a single view on a conventional 2D display.

The 3D video bitstream is constructed in a way that the sub-bitstream representing the coded representation of the base view can be extracted by simple means. The bitstream packets representing the base view can be identified by inspecting transmitted parameter sets and the packet headers. The sub-bitstream for the base view can be extracted by discarding all packets that contain depth data or data for the dependent views and, then, the extracted sub-bitstream can be directly decoded with an unmodified HEVC decoder and displayed on a conventional 2D video display.

The encoder can also be configured in a way that the sub-bitstream containing only two stereo views can be extracted and directly decoded using a stereo decoder. The encoder can also be configured in a way that the views can be generally decoded independently of the depth data. It is also possible to synthesize intermediate view using only the stereo sequences as input of the view synthesis.

A detailed description of the coding scheme is given in sec. 2. Depth-image-based rendering algorithms are described in sec. 3.

# Coding Algorithm

In the following, the coding algorithm based on the MVD format, in which each video picture is associated with a depth map, is described. The coding algorithm can also be used for a multiview format without depth maps. The video pictures and, when present, the depth maps are coded access unit by access unit, as it is illustrated in Figure 2. An access unit includes all video pictures and depth maps that correspond to the same time instant. Non-VCL NAL units containing camera parameters may be additionally associated with an access unit. It should be noted that the coding order of access units doesn't need to be identical to the capture or display order. In general, the reconstructed data of already coded access units can be used for an efficient coding of the current access unit. Random access is enabled by so-called random access units or instantaneous decoding refresh (IDR) access units, in which the video pictures and depth maps are coded without referring to previously coded access units. Furthermore, an access unit doesn't reference any access unit that precedes the previous random access unit in coding order. If the picture of the base view in an access unit is an IDR picture, the access unit is an IDR access unit.

Figure 2: Access unit structure and coding order of view components.

The video pictures and depth maps corresponding to a particular camera position are indicated by a view order index (viewIdx). All video pictures and depth maps that belong to the same camera position are associated with the same value of viewIdx. The view order indices  are used for specifying the coding order inside the access units and detecting missing views in error-prone environments. Inside an access unit, the video picture and, when present, the associated depth map with viewIdx equal to 0 are coded first, followed by the video picture and depth map with viewIdx equal to 1, etc. A video picture and depth map with a particular value of viewIdx are transmitted after all video pictures and depth maps with smaller values of viewIdx. For the independent view, the video picture is always coded before the associated depth map. For dependent views, the video picture maybe coded before or after the associated depth map (i.e., the depth map with the same value of viewIdx). It should be noted that the value of viewIdx doesn't necessarily represent the arrangement of the cameras in the camera array. For ordering the reconstructed video pictures and depth map after decoding, each value of viewIdx is associated with another identifier called view identifier (viewId). The view identifier is a signed integer values, which specifies the ordering of the coded views from left to right. If a view A has a smaller value of viewId than a view B, the camera for view A is located left to the camera of view B. In addition, camera parameters required for converting depth values into disparity vectors are included in the bitstream. For the considered linear setup, the corresponding conversion parameters consist of a scale factor and an offset. The vertical component of a disparity vector is always equal to 0. The horizontal component is derived according to

dv = ( s * v + o ) >> n,

where v is the depth sample value, s is the transmitted scale factor, o is the transmitted offset, and n is a shift parameter that depends on the required accuracy of the disparity vectors.

All of the video and depth sequences are associated with a video parameter set. Each video sequence and depth sequence is associated with a separate sequence parameter set and a separate picture parameter set. The picture parameter set syntax, the NAL unit header syntax, and the slice header syntax for the coded slices haven't been modified for including a mechanism by which the content of a coded slice NAL units can be associated with a component signal.

The video parameter set provides the following information about all component sequences:

- the view order index (indicates the coding order of a view);

- the depth flag (indicates whether video data or depth data are present);

- the view identifier (indicates the location of the view relative to other coded views);

For the base view, view order index and depth flag are not present and inferred to be equal to 0.

The sequence parameter set for all component sequences except the base view has been extended.  These sequence parameter sets contain the following additional parameters:

- an indicator specifying whether camera parameters are present in the sequence parameter set or in the slice headers;

- when camera parameters are present in an sequence parameter set, for each viewIdx value smaller than the current view order index, a scale and an offset specifying the conversion of a depth sample of the current view to a horizontal disparity between the current view and the view with viewIdx;

- when camera parameters are present in an sequence parameter set, for each viewIdx value smaller than the current view order index, a scale and an offset specifying the conversion of a depth sample of the view with viewIdx to a horizontal disparity between the current view and the view with viewIdx;

The sequence parameter set for the base view doesn't contain the additional parameters. The sequence parameter sets for dependent views include a flag, which specifies whether the camera parameters are constant for a coded video sequence or whether they can change on a picture by picture basis. If this flag indicates that the camera parameters are constant for a coded video sequence, the camera parameters (i.e., the scale and offset values described above) are present in the sequence parameter set. Otherwise, the camera parameters are not present in the sequence parameter set, but instead the camera parameters are coded in the slice headers that reference the corresponding sequence parameter set.

[Ed. (GT): In the HTM-7.0 and below the meaning of view identifier and view order index are swapped. ]

Figure 3: Basic codec structure with inter-component prediction (red arrows).

The basic structure of the 3D video codec is shown in the block diagram of Figure 3. In principle, each component signal is coded using an HEVC-based codec. The resulting bitstream packets, or more accurately, the resulting Network Abstraction Layer (NAL) units, are multiplexed to form the 3D video bitstream. The base or independent view is coded using an unmodified HEVC codec. Given the 3D video bitstream, the NAL units containing data for the base layer can be identified by parsing the parameter sets and NAL unit header of coded slice NAL units (up to the picture parameter set identifier). Based on these data, the sub-bitstream for the base view can be extracted and directly coded using a conventional HEVC decoder.

For coding the dependent views and the depth data, modified HEVC codecs are used, which are extended by including additional coding tools and inter-component prediction techniques that employ already coded data inside the same access unit as indicated by the red arrows in Figure 3. For enabling an optional discarding of depth data from the bitstream, e.g., for supporting the decoding of a stereo video suitable for conventional stereo displays, the inter-component prediction can be configured in a way that video pictures can be decoded independently of the depth data. A detailed description of the added coding tools is given in the following subsections.

## Coding of the Independent View

The independent view, which is also referred to as the base view, is coded using an unmodified HEVC codec.

## Coding of Dependent Views

For the dependent views, the same concepts and coding tools are used as for the independent view. However, additional tools have been integrated into the HEVC codec, which employ already coded data in other views for efficiently representing a dependent view. The additionally integrated tools are described in the following.

### Disparity-compensated prediction

As a first coding tool for the dependent views, the well-known concept of disparity-compensated prediction (DCP), which is also used in MVC, has been added as an alternative to motion-compensated prediction (MCP). At this, MCP refers to an inter-picture prediction that uses already coded pictures of the same view, while DCP refers to an inter-picture prediction that uses already coded pictures of other views in the same access unit, as it is illustrated in Figure 4.

Figure 4: Disparity-compensated prediction as an alternative to motion-compensated prediction.

The macroblock syntax and decoding process haven't been changed for adding DCP, only the high-level syntax has been modified so that already coded video pictures of the same access unit can be inserted into the reference pictures lists (as described in 2.2.1.1). As illustrated in Figure 4, the transmitted reference picture index (R in the figure) signals whether an inter-coded blocks is predicted by MCP or DCP. The motion vector prediction is modified in a way that the motion vectors of motion-compensated blocks are predicted by only using the neighbouring blocks that also use temporal reference pictures, while the disparity vectors of disparity-compensated blocks are predicted by only using the neighbouring blocks that also use inter-view reference pictures.

#### Reference list construction and modification

For motion-/disparity-compensated prediction, the inter-view reference frames as well as the inter-frame reference frames are included in the reference lists (L0, L1). The reference lists are constructed as specified in the following steps.

- Update of decoded picture buffer (DPB)

- The inter-view reference frames, which are already coded frames in the same access unit, are added to the current DPB. The status of the inter-view reference frames are signalled as view dependency in the Video Parameter Set (VPS)

- The status of view components present in the DPB of a view is signalled by the Reference Picture Set (RPS).

Figure 5: Update of decoded picture buffer.

- Construction of reference lists (L0, L1)

- The inter-frame references are first added into the reference lists (L0, L1) in the same manner as the reference list construction in HEVC. After that, the inter-view references are added at the end of the inter-frame references. The inter-view references can be included in L0, L1, or both.

Figure 6: Construction of reference lists (L0, L1).

- Modification of reference lists (L0, L1)

- The constructed reference lists can be modified based on the reference picture list modification syntax table as defined in HEVC.

- One example of the final reference picture lists 0 after modifying the default list is shown in Figure 7.

Figure 7: Modification of reference lists (L0, L1).

### Inter-view motion prediction

The basic concept of the inter-view prediction of motion parameters is illustrated in Figure 8. For the following overview, it is assumed that an estimate of a pixel-wise depth map for the current picture is given. Below, it is described how such an estimate can be derived. For deriving candidate motion parameters for a current block in a dependent view, the maximum depth value d within the associated depth block is converted to a disparity vector. By adding the disparity vector to the sample location x, which is in the middle of the block, a reference sample location xR is obtained. The prediction block in the already coded picture in the reference view that covers the sample location xR is used as the reference block. If this reference block is coded using MCP, the associated motion parameters can be used as candidate motion parameters for the current block in the current view. The derived disparity vector can also be directly used as a candidate disparity vector for DCP.

Figure 8: Basic principle of deriving motion parameters for a block in a current picture based on motion parameters in an already coded reference view and an estimate of the depth map for the current picture.

#### Derivation of Disparity Vectors

The concept of inter-view motion prediction requires a disparity vector to locate a corresponding block of the current PU/CU in an already coded picture of the same time instance. Therefore the codec provides four possibilities to derive a disparity vector. [ Ed. (GT): When method 1 and method 2 are not migrated from HTM-7.0 to the updated version, they should be removed from the test model text. ]

In method 1 the disparity vector is derived from a depth map belonging to a view coded prior to the current view. Therefore the complete depth map is warped to the current view. In method 2 a complete low resolution depth map is estimated for the current picture without utilizing any coded depth maps. In method 3 coding of a depth map is not required as well. However, in contrast to method 2 an estimation of a complete depth map is not performed. The disparity is derived from spatial and temporal neighbouring blocks which are using inter-view prediction or from motion vectors which are obtained by inter-view prediction. Method 4 combines methods 1 and 3. In contrast to method 1, where forward warping is utilized, in method 4 a disparity vector is derived first as done in method 3. This disparity vector is then utilized to identify a depth block in an already coded depth view to perform backward warping.

Methods 1 and 4 require the transmission of depth data as part of the bitstream, and by using one of these methods a decoder must decode the depth maps of previously coded views for decoding dependent views. Methods 2 and 3 are also applicable if depth maps are not coded inside the bitstream, and when depth maps are coded, the decoding of the video pictures is still independent of the depth maps.

In the following, all four methods by which a suitable disparity vector for the current block can be derived based on already transmitted information are described. All methods have been integrated in the codec, and one of the methods can be chosen by configuring the encoder (or by macro switches) accordingly.

##### Method 1: Disparity estimate based on already coded depth map [Not in CTC]

Since the depth map for a reference view is coded before the current picture, the reconstructed depth map is mapped into the coordinate system of the current picture for obtaining a suitable depth map estimate for the current picture. In Figure 9, such a mapping is illustrated for a simple depth map, which consists of a square foreground object and background with constant depth. For each sample of the given depth map, the depth sample value is converted into a sample-accurate disparity vector. Then, each sample of the depth map is displaced by the disparity vector. If two or more samples are displaced to the same sample location, the sample value that represents the minimal distance from the camera (i.e., the sample with the larger value) is chosen. In general, the described mapping leads to sample locations in the target view to which no depth sample value is assigned (black area in the middle picture of Figure 9). These areas represent parts of the background that are uncovered due to the movement of the camera and can be filled using surrounding background sample values. Therefore, a hole filling algorithm, which processes the converted depth map line by line, is used. Each line segment that consists of successive sample location to which no value has been assigned is filled with the depth value of the two neighbouring samples that represents a larger distance to the camera (i.e., the smaller depth value).

Figure 9: Mapping of a depth map into another view: (left) original depth map; (middle) converted depth map after displacing the original samples; (right) final converted depth map after filling of holes.

The disparity vector used for inter-view motion or residual prediction of a block of the current picture is finally derived based on the maximum value within the associated depth block.

##### Method 2: Depth/Disparity estimate based on coded disparity and motion vectors [Not in CTC]

The above described method 1 is only applicable if depth maps are included in the bitstream, and by using this method, the video pictures (except the base view) cannot be decoded independently of the depth maps. In the following, a method for deriving depth map estimates that only uses data that are available in the coded representations of the video pictures is described. When using this method, one depth sample is derived for a 4x4 block of luma samples. Consequently, the estimated depth maps have 1/4-th of the horizontal and vertical resolution of the luma components. The disparity vector used for inter-view motion or residual prediction of a block of the current picture is finally derived based on the maximum value within the associated depth block.

In random access units, all blocks of the base view picture, are intra-coded. In the pictures of dependent views, most blocks are typically coded using DCP and the remaining blocks are intra-coded. When coding the first dependent view in a random access unit, no depth or disparity information is available. Hence, candidate disparity vectors are derived using a local neighbourhood, i.e., by conventional motion vector prediction. But after coding the first dependent view in a random access unit, the transmitted disparity vectors are used for deriving a depth map estimate, as it is illustrated in Figure 10. Therefore, the disparity vectors used for DCP are converted into depth values and all depth samples that correspond to a disparity-compensated block are set equal to the derived depth value. The depth samples of intra-coded blocks are derived based on the depth samples of neighbouring blocks; the used algorithm is similar to spatial intra prediction. If more than two views are coded, the obtained depth map is mapped into other views using the method described above and used as depth map estimate for deriving candidate disparity vectors. During this mapping, the calculation of the disparity vectors takes into account that the estimated depth maps have 1/4-th of the horizontal and vertical resolution of the luma components.

Figure 10: Generation of an initial depth map estimate after coding the first dependent view of a random access unit.

The depth map estimate for the picture of the first dependent view in a random access unit is used for deriving a depth map for the next picture of the first dependent view. The basic principle of the algorithm is illustrated in Figure 11. After coding the picture of the first dependent view in a random access unit, the derived depth map is mapped into the base view and stored together with the reconstructed picture. The next picture of the base view is typically inter-coded. For each block that is coded using MCP, the associated motion parameters are applied to the depth map estimate. A corresponding block of depth map samples is obtained by MCP with the same motion parameters as for the associated texture block; instead of a reconstructed video picture the associated depth map estimate is used as reference picture. The block of depth samples that is associated with a block of luma samples has 1/4-th of the horizontal and vertical resolution of the luma block. In order to simplify the motion compensation and avoid the generation of new depth map values, the MCP for depth block doesn't involve any interpolation. The motion vectors are rounded to depth-sample-precision (1/4-th of the luma sample precision) before they are used. The depth map samples of intra-coded blocks are again determined on the basis of neighbouring depth map samples. Finally, the depth map estimate for the first dependent view, which is used for the inter-view prediction of motion parameters, is derived by mapping the obtained depth map estimate for the base view into the first dependent view.

Figure 11: Derivation of a depth map estimate for the current picture using motion parameters of an already coded view of the same access unit.

After coding the second picture of the first dependent view, the estimate of the depth map is updated based on actually coded motion and disparity parameters, as it is illustrated in Figure 12. For blocks that are coded using DCP, the depth map samples are obtained by converting the disparity vector into a depth value. The depth map samples for blocks that are coded using MCP are obtained by MCP of the previously estimated depth maps, similar as for the base view.

In an optional configuration, new depth values are determined by adding a depth correction. The depth correction is derived by converting the difference between the motion vectors for the current block and the corresponding reference block of the base view into a depth difference. The depth values for intra-coded blocks are again determined by a spatial prediction. The updated depth map is mapped into the base view and stored together with the reconstructed picture. It is also used for deriving a depth map estimate for other views in the same access unit.

Figure 12: Update of depth map estimate for a dependent view based on coded motion and disparity vectors.

For all following pictures, the described process is repeated. After coding the base view picture, a depth map estimate for the base view picture is determined by MCP using the transmitted motion parameters. This estimate is mapped into the second view and used for the inter-view prediction of motion parameters. After coding the picture of the second view, the depth map estimate is updated using the actually used coding parameters. At the next random access unit, the inter-view motion parameter prediction is not used, and after decoding the first dependent view of the random access unit, the depth map is re-initialized as described above.

##### Method 3: Disparity vector from neighbouring blocks (NBDV)

For each CU a disparity vector is derived from a motion vector of a spatial or temporal DCP neighbouring block of the CU or from a disparity vector associated with an MCP neighbouring block of the CU. Once a disparity motion vector is found, the whole disparity vector derivation process terminates.

First temporal DCP neighbouring blocks are evaluated as specified in section 2.2.2.1.3.2, followed by a check of the spatial DCP neighbours, as specified in section 2.2.2.1.3.1. Finally, MCP coded neighbour blocks are searched as described in section 2.2.2.1.3.3.

When no disparity motion vector is found from the neighbouring blocks, a zero disparity vector is used.

| Figure 13: Location of spatial neighbour blocks | Figure 14: Location of temporal neighbour blocks in temporal candidate pictures |
| --- | --- |

###### Spatial Neighbouring Blocks

Five spatial neighbouring blocks are used for the disparity vector derivation. They are the below-left, left, above-right, above and above-left blocks of current prediction unit (PU), denoted by A0, A1, B0, B1 or B2, as defined in Figure 13.

One constraint in the block search is to only consider the blocks that are also utilized in the merge scheme in the HEVC base specification, when the DV is derived for the derivation of an inter-view merge candidate.

The checking order of the five spatial neighbouring blocks is: A1, B1, B0, A0 and B2.

###### Temporal Neighbouring Blocks

Up to two reference pictures from the current view are treated as candidate pictures for temporal neighbours. The first candidate picture is the co-located picture as used for Temporal Motion Vector Prediction (TMVP) in HEVC without low delay check. The co-located picture is indicated in a slice header. The second picture is derived in the reference picture lists with the ascending order of reference picture indices, and added into the candidate list, as follows:

- A random access point (RAP) is searched in the reference picture lists. If found, the RAP is placed into the candidate list for the second picture and the derivation process is completed. In the case that the RAP is not available for the current picture, step (2) applies.

- A picture with the lowest temporalID (TID) is selected and placed into the candidate list of the temporal pictures as the second entry.

- If multiple pictures with the same lowest TID exist, a picture of a smaller POC difference with the current picture is chosen.

As shown in the above description, the second temporal candidate picture is chosen in a way that disparity motion vectors can have more chance to be present in the picture. The derivation process of the second candidate picture can be done on the slice level and be invoked only once per slice.

For each candidate picture up to two temporal neighbouring blocks BR and Centre as depicted in Figure 14 are searched in that order (i.e. BR, Centre). The BR block is not considered when it is located below the lower CTU row of the current CTU, as depicted in Figure 15.

.

Figure 15: The BR temporal block below the lower CTU row is not considered
[ Ed. (GT): Replace "Current PU" with "Current CU" in the figure ]

###### Disparity derivation from MCP coded neighbour blocks

In addition to the DCP coded blocks, blocks coded by motion compensated prediction (MCP) are also used for the disparity derivation process. When a neighbouring block is an MCP coded block and its motion is predicted by inter-view motion prediction, as shown in Figure 16, the disparity vector used for the inter-view motion prediction represents a motion correspondence between the current and the inter-view reference picture. This type of motion vector is referred to as inter-view predicted motion vector (IvpMv) and the blocks are referred to as DV-MCP blocks in the following. The motion correspondence is used for the disparity derivation process in the following way:

V0 (independent view)

(dependent view)

Figure 16: The inter-view predicted motion vector of a MCP coded block.

To indicate whether a blocks is a DV-MCP block and to save the disparity vector used for the inter-view motion prediction, two variables are used:

- IvpMvFlag

- IvpMvDisparityX.

The block whose motion vector is inter-view predicted is identified when the 0th motion parameter candidate of MERG/SKIP mode is selected. In this case, the IvpMvFlag and IvpMvDisparityX corresponding to the location of the current PU and the horizontal component of the disparity vector used for the inter-view motion prediction are both set to 1.

The disparity vector is derived from SKIP coded DV-MCP blocks. When a block is coded by skip mode, neither mvd (motion vector difference) data nor residual data is signalled. This implies that the disparity vector used for SKIP-coded DV-MCP blocks better describes the motion correspondence than the disparity vector used for DV-MCP blocks that are not SKIP-coded.

If a DCP coded block is not found in the spatial and temporal neighbouring blocks, the disparity derivation process scans the spatial neighbouring blocks for DV-MCP compensation in following order: A0, A1, B0, B1, B2. If a neighbouring block is a SKIP coded DV-MCP block, then the value of IvpMvDisparityX from the neighbouring block is returned as the derived disparity. The vertical component of the disparity vector is set equal to zero.

To reduce the amount of memory required for the derivation of the disparity from DV-MCP blocks, blocks B0, B1 and B2 are only utilized when they are located in the current CTU. An example for this can be seen in Figure 17. Here only spatial neighbouring blocks A1 and A0 are utilized.

Figure 17: Example: For the derivation of the disparity from DV-MCP neighbouring blocks of the current CU, above block B2, B1 and B0 are not used, since they are not located within the current CTU.
[ Ed. (GT): Replace "Current PU" with "Current CU" in the figure ]

##### Method 4: Disparity vector derivation from a depth map of a different view component

While coding the texture of a dependent view, the decoded depth of the base view is already available. So the disparity derivation needed for the coding of the texture of the dependent view might be improved by utilizing the depth map of the base view. A disparity vector (which might be a better estimate than a disparity vector derived with method 3) can be extracted by the following steps:

- A disparity vector is derived, using method 3.

- The disparity vector is used to locate the corresponding block in the coded depth of the base view.

- The depth in the corresponding block in the base depth is assumed to be the "virtual depth block" of the current block in the dependent view.

- The maximum depth value of the four edge samples of the virtual depth block is retrieved.

- The maximum depth value is converted to disparity

An example is depicted in Figure 18. The coded depth map in view 0 is denoted as Coded D0. The texture to be coded is T1. For the current block (CB) a depth block in the coded D0 is derived using a disparity vector, as described in method 3.

Figure 18: Retrieval of the virtual depth block

To enable coding of texture without depth maps, disparity derivation from depth maps can be disabled by a VPS flag.

#### Usage of Inter-View Motion Parameter Prediction

Inter-view motion vector prediction is applied for the merge mode (and skip mode). In the merge mode of HEVC (as well as in the skip mode, which represents the merge mode without coding a residual signal), the same motion parameters (number of hypotheses, reference pictures, and motion vectors) as for a neighbouring block are used. If a block is coded in merge mode, a candidate list of motion parameters is derived, which includes the motion parameters of spatially neighbouring blocks as well as motion parameters that are calculated based on the motion parameters of the co-located block in a temporal reference picture. The chosen motion parameters are signalled by transmitting an index into the candidate list.

The candidate list of motion parameters is extended by a motion parameter candidate for MCP (IvMC) that is obtained using inter-view motion prediction. Moreover a motion parameter candidate for DCP constructed from the derived disparity (IvDC) is added. The derivation of both additional candidates is described in the following.

For the derivation of the IvMC and the IvDC candidate, a corresponding block in a view component at the same time instant as the current view component is utilized. The corresponding block is determined by shifting the position of the current block using the disparity vector derived as described in section 2.2.2.1.

If the corresponding block is coded using MCP it is tested for each motion hypothesis of the current block, whether a motion vector and a reference index can be derived for the IvMC candidate. This is the case if a picture is included in the reference picture list belonging to the current slice and motion hypothesis with a picture order count equal to the picture order count of a reference picture of the corresponding block. When such a picture is found the reference index of this picture in the reference picture list belonging to the current slice and motion hypothesis and the motion vector of the hypothesis of the corresponding block are used to derive the IvMC candidate.

To derive the IvDC candidate it is tested for each motion hypothesis of the current block, if the corresponding block is located in an inter-view reference picture that is included in the reference picture list belonging to the current slice and motion hypothesis. If such a reference picture is found, the IvDC candidate is constructed by using the derived disparity as motion vector and the reference index of the found reference picture in the reference picture list of the current slice.

In the case that all motion parameter candidates are available, the position of the IvMC candidate is the first and the position of the IvDC candidate is the fifth in the merge candidate list, At second, third and fourth position spatial candidates are inserted. Moreover pruning of the list is carried out if the IvMC candidate or the IvDC candidate are equal to one of the motion parameter candidates of the spatially neighbouring blocks A1 and B1.

#### Derivation of co-located motion vector candidate

The availability of the co-located vector is specified in Table 1. For AMVP the co-located motion vector is available for motion vector prediction if the current PU utilized the same kind of prediction (inter prediction or inter-view prediction) as the co-located PU. Otherwise, the co-located motion vector is not available.

For Merge, the temporal motion vector prediction (TMVP) candidate might still be available; i.e. when the target reference index specifies a reference picture in the same view, while the motion vector of the co-located prediction unit (PU) is related to an inter-view reference picture or vice versa. Therefore an alternative target reference index is derived as described in section 2.2.2.3.1.

In case that both, the current PU and the co-located PU, utilize inter-view prediction, inter-view motion vectors are scaled for Merge as specified in section . 2.2.2.3.2. For AMVP no inter-view motion vector scaling is applied.

Table 1: The availability of the co-located motion vector

| Availability of co-located vector |  | Prediction type of current PU | Prediction type of co-located PU |
| --- | --- | --- | --- |
| Merge | AMVP |  |  |
| Available | Available | temporal | temporal |
| Potentially available | Not available | temporal | inter-view |
| Potentially available | Not available | inter-view | temporal |
| Available | Available | inter-view | inter-view |

##### Derivation of the alternative reference index for merge

An alternative reference index is derived for Merge, in case that the reference picture with the current target reference index is a different kind of reference picture as the reference picture of the co-located PU.

When the reference picture with the current target reference index is an inter-view reference picture, but the reference picture of the co-located PU is a temporal reference picture, the current target reference index is modified to be the first reference index in the reference picture list of the current block, that specifies an inter-view reference picture.

When the reference picture with the current target reference index is a temporal reference picture, but the reference picture of the co-located PU is an inter-view reference picture, the current target reference index is modified to be the first reference index in the reference picture list of the current block, that specifies a temporal reference picture.

##### Scaling of inter-view motion vectors

The scaling function for inter-view motion vectors is the same as that in temporal MV scaling, but the scaling factors are derived differently. In case of temporal MV scaling the scaling factor “tb” is the difference between POC of coding block and coding reference block and “td” is the difference between POC of co-located block and this reference block. In case of inter-view scaling, the scaling factors “tb” and “td” are calculated with the difference between view order indices of each block instead of POCs, Hence:

DistScaleFactor = Clip3( –1024, 1023, ( tb * tx + 32 ) >> 6 )
tx = ( 16384 + Abs( td / 2 ) ) / td

where td and tb are derived as:

td = Clip3( –128, 127, ColViewOrderIdx – ColRefViewOrderIdx )
tb = Clip3( –128, 127, CurrViewOrderIdx – CurrRefViewOrderIdx )

The variables in the above equations are specified as follows:

CurrViewOrderIdx :ViewOrderIdx of current picture

ColViewOrderIdx :ViewOrderIdx of co-located picture

CurrRefViewOrderIdx :ViewOrderIdx of the picture that is referenced by the current picture

ColRefViewOrderIdx :ViewOrderIdx of the picture that is referenced by the co-located picture

An example for the scaling of inter-view motion vectors is depicted in Figure 19. The coding block refers to the reference picture of V0 and the neighbouring block refers to the picture of V2. The predictive vector from neighbouring block is scaled since the difference of view index (V1-V0) between coding block and coding reference block is not equal to that (V1-V2) between neighbouring block and this reference block.

Figure 19: Inter-view motion vector scaling in TMVP

[Ed. (GT): In the normative annex the meaning of view identifier and view order index has been swapped. ViewOrderIdx here and in software describes a coding order independent value, that might be chosen to represent spatial positions. ]

### Inter-view residual prediction

Figure 20 illustrates the prediction structure of residual prediction method. Dc represents the current block in the current view (view 1), Bc, and Dr denote the representation of Dc in the reference view (view 0) at time Tj and Dc’s temporal prediction from the same view (view 1) at time Ti. VD denotes the motion from Dc to Dr. Since Dc and Bc are actually projections of the same object in two different views, these two blocks should share the same motion information. Therefore, Bc’s temporal prediction Br in view 0 at time Ti can be located from Bc by applying the motion information of VD. The residual of Bc with motion information of VD is then multiplied by a weighting factor and used as the predictor for current residual.

Figure 20: Prediction structure of the proposed ARP in 3D-HEVC.

Figure 21:  Relationship among current block, reference block and motion compensated block

Decoding process of ARP

The process of ARP at the decoder side can be described as follows:

- Obtain a disparity vector as specified in section 2.2.2.1, pointing to a target reference view. Then, in the picture of the reference view within the same access unit, the corresponding block is located by the disparity vector.

- Re-use the motion information of the current block to derive the motion information for the reference block. Apply motion compensation for the corresponding block based on the same motion vector of current block and derived reference picture in the reference view for the reference block, to derive a residue block. The relationship between the current block, corresponding block and motion compensated block is shown in Figure 21. The reference picture in the reference view (V0) which has the same POC (Picture Order Count) value as the reference picture of the current view (Vm) is selected as the reference picture of the corresponding block.

- Apply the weighting factor to the residue block to get a weighted residue block and add the values of the weighted residue block to the predicted samples.

Weighting factor

Three weighting factors are used in residual prediction: 0, 0.5 and 1. The one leading to minimal rate-distortion cost for the current CU is selected as the final weighting factor and the corresponding weighting index (0, 1 and 2 which correspond to weighting factor 0, 1, and 0.5, respectively) is transmitted in the bitstream at the CU level. All PU predictions in one CU share the same weighting factor. When the weighting factor is equal to 0, residual prediction is not used for the current CU.

Reference picture selection via motion vector scaling

The motion vectors of the current PU are scaled towards a fixed picture before performing motion compensation, when the weighting factor is unequal to 0. The fixed picture is defined as the first reference picture of each reference picture list.

Interpolation filter

A bi-linear interpolation filter is used regardless whether the block is in base view or non-base view when residual prediction is applied.

### Illumination compensation (IC)

A linear illumination compensation model is utilized to adapt luminance and chrominance of inter-view predicted blocks to the illumination of the current view. The parameters of the linear model are estimated for each CU using reconstructed neighbouring samples of the current block and of the reference block used for prediction.

[Ed. (GT). Some more information specifying how the linear parameters are computed, which samples are used for computation and how the computation is realized using the LUT would be helpful here.]

The rate distortion optimized (RDO) selection, whether to enable illumination compensation, is not carried out for all inter modes. Instead of this, first a RDO selection between IC on and IC off is performed for the Merge mode with 2Nx2N partitioning. When IC is chosen in this test, IC is switched on when testing the following inter modes.

The illumination compensation algorithm is also applied to depth data.

Signalling of illumination compensation

Whether illumination compensation is used is signalled by the ic_flag on coding unit level. In Skip / Merge mode, ic_flag is conditionally sent, depending on merge_idx and the slice segment header flag slice_ic_skip_mergeidx0_flag. If ic_flag is not sent in Merge mode, ic_flag is set to 0. The detailed conditions are shown in Table 1.

Table 2: Conditions for the availiability of the ic_flag

| Mode | Condition | Note |
| --- | --- | --- |
| Skip / Merge (merge_flag[ x0 ][ y0 ]) | !(merge_idx[ x0 ][ y0 ] = = 0 && slice_ic_skip_mergeidx0_flag) | When inter-view candidate is used (merge_idx == 0) and encoder send a new flag slice_ic_skip_merge_flag, then ic_flag is not sent (inferred to 0) (independent of ref_idx_lX) |
| AMVP (!merge_flag[ x0 ][ y0 ]) | When the CU has an inter-view reference picture. | conditionally sent depending on ref_idx_lX |

When merge_idx is 0, mostly temporal inter-view motion prediction is used, while inter-view prediction is rarely used. To avoid a signalling overhead of the ic_flag, illumination compensation is not available in case of inter-view prediction.

In some pictures where the inter-view prediction is mostly used, the above assumption does not hold. So the merge_idx base skipping of the ic_flag is only applied under the condition that (POC % IntraPeriod) is not 0. This POC based condition is decided by the encoder, which sends a slice header flag slice_ic_skip_mergeidx0_flag. This allows encoders to control the condition depending on a particular coding structure or sequence.

Furthermore, illumination compensation is only applied for 2Nx2N case.

### View synthesis prediction (VSP)

View synthesis prediction (VSP) provides a predictor for each PU using depth information to reduce inter-view redundancy. In VSP, a neighbouring block disparity vector (DVx, DVy) is derived as shown in Step 1 of Figure 22, as described in section 2.2.2.1.3. With the derived disparity vector (DVx, DVy), a depth block (x+DVx, y+DVy) is fetched from a reference view depth image and used as an estimator for the depth information of the current PU, as shown in Step 2 of Figure 22. From the estimated depth block, disparity vectors are derived accordingly at the sub-block level with the input camera parameters. Given the derived disparity vectors, a backward warping is invoked to find corresponding reference pixels from the reference view and generate  the VSP predictor for the current PU, as shown in Step 3 of Figure 22.

Figure 22 Illustration of the VSP scheme with the neighboring block disparity vector

Signalling

To signal the usage of VSP for a PU block, a VSP candidate is inserted into the merge candidate list with the derived neighboring block disparity vector and the associated reference view index as described above. It is noted that while a new VSP candidate is added only once to avoid redundancy in the merge list, multiple VSP candidates may be inherited from spatial neighboring blocks. When a spatial neighboring block is coded in a VSP mode, the associated disparity vector and reference view index from the neighboring block is used to define a VSP candidate for the current PU.

To enable coding of texture without depth maps, VSP can be disabled by a VPS flag.

P- and B prediction

VSP can either be uni-direction prediction or bi-direction prediction. The prediction direction depends on the slice type and the availability of reference views in the reference picture lists, which is elaborated as follows.

- If the current slice is a P slice, VSP is performed in the manner of uni-direction prediction. One predictor is generated from the reference picture list, which has the interview reference picture identified by the disparity vector defined in the VSP candidate.

- If the current slice is a B slice, VSP may be performed in either uni-direction or bi-direction prediction. And the prediction direction depends on the availability of reference views in the reference picture lists.

  - If each reference picture list has a different interview reference picture, VSP is performed in the bi-direction manner. Specifically, a depth block is fetched using the derived neighboring block disparity vector from the depth image of the associated reference view. The depth values are then converted to different disparity vectors relative to the different reference views in the two reference picture lists and two compensation predictors are generated. Finally, the two predictors are combined into a single predictor in a weighted process.

  - Otherwise, VSP is achieved in the uni-direction manner. The predictor is generated from the reference picture list, which has the interview reference picture as identified by the disparity vector defined in the VSP candidate.

Handling of VSP coded block

After the Merge candidate list is constructed the reference index is replaced by an inter-view reference picture index. This way, BVSP coded blocks can be considered as normal inter predicted blocks.

In section 2.2.2.1.3 a methods is described that provides a disparity vector when a neighbouring block is coded in inter-view prediction mode called NBDV. As the VSP mode can be assumed as a special inter-view prediction method, NBDV also provides a disparity vector if a neighbouring block is coded in a VSP mode. The returned disparity vector is derived from the maximum disparity of the subsampled depth values in the associated depth block which were used for VSP compensation.

## Coding of Depth Maps

For the coding of depth maps, basically the same concepts of intra-prediction, motion-compensated prediction, disparity-compensated prediction, and transform coding as for the coding of the video pictures are used. However, some tools have been modified for depth maps, other tools have been generally disabled, and additional tools have been added.

As a first difference to the coding of video pictures, the inter-view motion, residual prediction and view synthesis prediction as described in sections  2.2.2. 2.2.3 and 2.2.5 , respectively, are not used for depth coding. Instead, motion parameters are derived based on coded data in the associated video pictures as will be described in sec. 2.3.4 below. The other differences are described in the following subsections.

### Modified motion compensation and motion vector coding

In contrast to natural video, depth maps are characterized by sharp edges and large regions with nearly constant values. The eight-tap interpolation filters that are used for motion-compensated interpolation in HEVC, can produce ringing artefacts at sharp edges in depth maps, which are visible as disturbing components in synthesized intermediate views. For avoiding this issue and for decreasing the encoder and decoder complexity, the motion-compensated prediction (MCP) as well as the disparity-compensated prediction (DCP) has been modified in a way that no interpolation is used. That means, for depth maps, the inter-picture prediction is always performed with full-sample accuracy. For the actual MCP or DCP, a block of samples in the reference picture is directly used as prediction signal without interpolating any intermediate samples. In order to avoid the transmission of motion and disparity vectors with an unnecessary accuracy, full-sample accurate motion and disparity vectors are used for coding depth maps. The transmitted motion vector differences are coded using full-sample instead of quarter-sample precision.

### Disabling of in-loop filtering

The in-loop filters in the HEVC design have been particularly designed for the coding of natural video. For the coding of depth maps, these filters are less useful. In order to decrease the encoder and decoder complexity, the in-loop filters have been disabled for depth coding. This includes the following filters:

- the de-blocking filter;

- the adaptive loop filter (Wiener  filter);

- the sample-adaptive loop filter.

### Depth modelling modes

Depth maps are mainly characterized by sharp edges (which represent object borders) and large areas of nearly constant or slowly varying sample values (which represent object areas). While the HEVC intra prediction and transform coding is well-suited for nearly constant regions, it can result in significant coding artefacts at sharp edges, which are visible in synthesized intermediate views. For a better representation of edges in depth maps, four new intra prediction modes for depth coding are added. In all four modes, a depth block is approximated by a model that partitions the area of the block into two non-rectangular regions, where each region is represented by a constant value. The information required for such a model consists of two elements, namely the partition information, specifying the region each sample belongs to, and the region value information, specifying a constant value for the samples of the corresponding region. Such a region value is referred to as constant partition value (CPV) in the following. Two different partition types are used, namely Wedgelets and Contours, which differ in the way the segmentation of the depth block is derived. The depth modelling modes are integrated as an alternative to the conventional intra prediction modes specified in HEVC. Similar as for the intra prediction modes, a residual representing the difference between the approximation and the original depth signal can be transmitted via transform coding. In the following, the approximation of depth blocks using the four new depth modelling modes is described in more detail.

It is differentiated between Wedgelet and Contour partitioning. For a Wedgelet partition, the two regions are defined to be separated by a straight line, as illustrated in Figure 23, in which the two regions are labelled with P1 and P2. The separation line is determined by the start point S and the end point P, both located on different borders of the block. For the continuous signal space (see Figure 23, left), the separation line can be described by the equation of a straight line. The middle image of Figure 23 illustrates the partitioning for the discrete sample space. Here, the block consists of an array of samples with size uB×vB and the start and end points correspond to border samples. Although the separation line can be described by a line equation as well, the definition of regions P1 and P2 is different here, as only complete samples can be assigned as part of either of the two regions. For employing Wedgelet block partitions in the coding process, the partition information is stored in the form of partition patterns. Such a pattern consists of an array of size uB×vB and each element contains the binary information whether the corresponding sample belongs to region P1or P2. The regions P1and P2 are represented by black and white samples in Figure 23 (right), respectively.

Figure 23: Wedgelet partition of a block: continuous (left) and discrete signal space (middle) with corresponding partition pattern (right).

Unlike for Wedgelets, the separation line between the two regions of a Contour partition of a block cannot be easily described by a geometrical function. As illustrated in Figure 24, the two regions P1 and P2 can be arbitrary shaped and even consist of multiple parts. Apart from that the properties of Contour and Wedgelet partitions are very similar. For employing Contour partitions in the coding process, the partition pattern (see example in Figure 24, right) is derived individually for each block from the signal of a reference block. Due to the lack of a functional description of the region separation line, no pattern lookup lists and consequently no search of the best matching partition are used for Contour partitions.

Figure 24: Contour partition of a block: continuous (left) and discrete signal space (middle) with corresponding partition pattern (right).

Apart from the partition information, either in form of a Wedgelet or a Contour partition, the second information required for modelling the signal of a depth block is the CPV of each of the two regions. For a given partition the best approximation is consequently achieved by using the mean value of the original depth signal of the corresponding region as the CPV.

Four depth-modelling modes, which mainly differ in the way the partitioning is derived and transmitted, have been added:

- Mode 1: Explicit Wedgelet signalling;

- Mode 2: Intra-predicted Wedgelet partitioning;

- Mode 3: Restricted signalling and inter-component prediction of Wedgelet partitions;

- Mode 4: Inter-component-predicted Contour partitioning.

These depth-modelling modes as well as the signalling of the modes and the constant partition values are described in the following four subsections.

#### Mode 1: Explicit Wedgelet Signalization

The basic principle of this mode is to find the best matching Wedgelet partition at the encoder and transmit the partition information in the bitstream. At the decoder the signal of the block is reconstructed using the transmitted partition information.

The Wedgelet partition information for this mode is not predicted. At the encoder, a search over a set of Wedgelet partitions is carried out using the original depth signal of the current block as a reference. During this search, the Wedgelet partition that yields the minimum distortion between the original signal and the Wedgelet approximation is selected. The resulting prediction signal is then evaluated using the conventional mode decision process.

A fast search of the best matching partition is essential for employing Wedgelet models in the depth coding process. This fast search algorithm is further described in section 2.3.3.7.

#### Mode 2: Intra-predicted Wedgelet Partitions

The basic principle of this mode is to predict the Wedgelet partition from data of previously coded blocks in the same picture, i.e. by intra-picture prediction. For a better approximation, the predicted partition is refined by varying the line end position. Only the offset to the line end position is transmitted in the bitstream and at the decoder the signal of the block is reconstructed using the partition information that results from combining the predicted partition and the transmitted offset.

Figure 25: Intra prediction of Wedgelet partition (blue) for the scenarios that the above reference block is either of type Wedgelet partition (left)or regular intra direction(right).

The prediction process of this mode derives the line start position and the gradient from the information of previously coded blocks, i.e. the neighbour blocks left and above of the current block. Note that for some blocks one or both of the neighbouring blocks are not available. In such a case the processing for this mode is carried out with setting the missing information to meaningful default values. As illustrated in Figure 25 two main prediction methods have to be distinguished: The first method covers the case when one of the two neighbouring reference blocks is of type Wedgelet, shown in the example in Figure 25, left. The second method covers the case when the two neighbouring reference blocks are not of type Wedgelet, but of type intra direction, which is the default intra coding type, shown in the example in Figure 25, right.

If the reference block is of type Wedgelet, the prediction process works as follows: The principle of this method is to continue the reference Wedgelet into the current block, which is only possible if the continuation of the separation line of the reference Wedgelet actually intersects the current block. Therefore, it is first checked whether it is possible to continue the reference Wedgelet. In case the check is positive, the start position Spand the end position Ep are predicted by calculating the intersection points of the continued line with block border samples.

If the reference block is of type intra direction, the prediction process works as follows: First, the gradient is derived from the intra prediction direction. As the intra direction is only provided in the form of an abstract index, a mapping or conversion function is defined that associates each intra prediction mode with a gradient. Second, the start position Sp is derived from information that is also available at the decoder, namely the adjacent samples of the left and above neighbouring block, by selecting the sample position with the maximum slope. Finally, the end position Ep is calculated from the start point and the gradient.

The line end position offset for refining the Wedgelet partition is not predicted, but searched within the estimation process at the encoder. For the search, candidate partitions are generated from the predicted Wedgelet partition and an offset value for the line end position Eoff, as illustrated in Figure 25. By iterating over a range of offset values and comparing the distortion of the different resulting Wedgelet partitions, the offset value of the best matching Wedgelet partition is determined using a distortion measure.

#### Mode 3: Restricted signalling and inter-component prediction of Wedgelet partitions

The basic principle of this mode is to signal a Wedgelet partition as index to a restricted set of Wedgelet partitions. To restrict the set two methods can be applied. The first method predicts probable Wedgelet partitions from a texture reference block, namely the co-located block of the associated video picture. This type of prediction is referred to as inter-component prediction. Unlike temporal or inter-view prediction, no motion or disparity compensation is used, as the texture reference picture shows the scene at the same time and from the same perspective.

Figure 26: Prediction of Wedgelet (blue) and Contour (green) partition information from texture luma reference.

The prediction of a Wedgelet partition pattern from the texture reference is illustrated in the top row of Figure 26. It can be observed that the luma intra direction correlates with the edge direction in the depth data, therefore the set of possible Wedgelet partitions can be restricted utilizing this information as described in section 2.3.3.6.

The second method can be used when the co-located texture block is not intra coded. In this case the possible Wedgelet partitions are restricted to a coarse subset of partitions utilized in Mode 1. The search for the Wedgelet partition is carried out as specified in section 2.3.3.7, however the refinement step as described there is not applied for Mode 3..

#### Mode 4: Inter-component prediction of Contour partitions

The basic principle of this mode is to predict a Contour partition from a texture reference block by inter-component prediction. Like for the inter-component prediction of a Wedgelet partition pattern, the reconstructed luminance signal of the co-located block of the associated video picture is used as a reference, as illustrated in the bottom row of Figure 26. In contrast to Wedgelet partitions, the prediction of a Contour partition is realized by a thresholding method. Here, the mean value of the texture reference block is set as the threshold and depending on whether the value of a sample is above or below the sample position is marked as part of region P1 or P2 in the resulting Contour partition pattern.

#### Constant partition value coding

The method for CPV coding is the same for all four modes introduced above, as it does not distinguish between partition types, but rather assumes that a partition pattern is given for the current depth block. As illustrated in Figure 27, three types of CPVs are differentiated: original, predicted, and delta CPVs.

Figure 27: CPVs of block partitions: CPV prediction from adjacent samples of neighbouring blocks (left) and cross section of block (right), showing relation between different CPV types.

The cross section of the block in Figure 27, right, schematically shows that the original CPVs are calculated as the mean value of the signal covered by the corresponding region. Although these values lead to the best approximation for the given partition, they are not available at the decoder as they require the original signal.

Therefore prediction of CPVs is introduced. These predicted CPVs are derived from information that is also available at the decoder, namely adjacent samples of the neighbouring left and top block As shown in Figure 28, two samples of the top reference sample row or the left reference sample column may be chosen to generate a DC predictor for different partition patterns. Therefore, the DC predictor is calculated from the two samples as follows:

Assume the partition value of the left-top sample (c0,0) is X, where X= 0 or 1, given the partition pattern bPatternx,y, where x = 0..N − 1, y = 0..N−1, the predicted DC values, i.e., DC0 and DC1, are derived as follows:

Set bT = (bPattern0,0 != bPatternN-1,0)? 1 : 0;

Set bL = (bPattern0,0 != bPattern0,N-1)? 1 : 0

If bT equals bL

- DCX = (p-1,0 + p0,-1) >> 1

- DC1-X = bL ? (p-1,N-1 + pN-1,-1) >> 1 : 2BitDepth-1

Otherwise

- DCX = bL ? p(N-1)>>1,-1 : p-1, (N-1)>>1

- DC1-X = bL ? p-1,N-1 : pN-1,-1

(a) Case 1 				(b) Case 2

(c) Case 3 				(d) Case 4

Figure 28: Selection of reference samples for difference partition pattern cases.

Depending on the similarity between the original signal of the block and adjacent samples, the predicted and original CPVs may differ significantly. This difference is referred to as delta CPVs. By calculating the delta CPVs at the encoder and transmitting them in the bit stream, it is possible to reconstruct the CPVs at the decoder.

Depending on the Intra Mode of the current prediction unit, delta CPVs can be determined and signalled in two different ways. Quantized signalling is used for region boundary chain coding as described in section 2.3.4. Signalling using a DLT is applied for the DMM modes as described in section 2.3.5.

Quantized signalling

Although the distortion of the reconstructed signal is considerably reduced by the delta CPVs, the benefit of this approach is delimited by the additional bit rate required for transmitting the delta CPVs. Therefore, a linear quantization is introduced for the delta CPVs. This method is also used in transform coding and the step size of the quantization is set as a function of the QP. The delta CPVs are linearly quantized at the encoder and de-quantized before reconstruction at the decoder.

Search of optimal DC values

In case the distortion is not measured for the original depth, but for synthesized views, the delta CPV derivation process is extended by a minimum distortion search, which iterates over all possible delta CPV combinations for the two partitions. For the sake of efficient processing and signalling the range of tested values is limited. The search results in the combination of the two delta CPVs that causes the minimum distortion in synthesized views..

The optimized search strategy consists of a coarse search and a refinement step as follows: Initially the distortion of the partition values that are calculated as the mean value of the original sample values, that are covered by the corresponding region, is determined. For SDC as described in section 2.3.5 the offset between these values and the predicted partition values is simply transmitted without a VSO-based minimum distortion search. However, for DMM modes the search tests all combinations of offset values in a certain range around the predicted and original partition values. The limits of the search range start from an offset value of 0 and the upper limit is restricted by the actual range of depth values. First, a coarse search is carried out, testing offset values at intervals of 4. For each tested combination of offset values the distortion is compared to the initial distortion achieved with the original partition values. Only if at least one of the coarse offset combinations leads to a smaller distortion than the original partition values, the refinement step is carried out for the best coarse combination. The refinement step simply consists of testing all offsets in the range of [-3, 3] around the best coarse offset combination.

Figure 29: Optimized search strategy for non-quantized partition offset values.

#### Co-located Texture Luma Block intra direction constraint

For DMM mode 3, the intra direction of the co-located texture luma block can be utilized to constrain the set of possible Wedgelet partitions and the Wedgelet search at the encoder to most probable Wedgelet directions. Basically, the wedgelet search consists of two parts: first, for each intra direction, a restricted Wedgelet list is initialized with Wedgelet patterns sharing the similar directionality of the intra direction, second, during the Wedgelet search, only the Wedgelet patterns included in the Wedgelet list for the intra direction of co-located texture luma block are considered. For transmission, the found Wedgelet partition is signalled as an index to the restricted Wedgelet list.

Wedgelet list generation for each intra direction. To generate the Wedgelet list, each Wedgelet pattern index idxW is first mapped to an intra direction widx by applying the following steps:

- For each intra direction i from 2 to 34, Di is calculated as Di = | Vi (Xs-Xe) - Hi (Ye-Ys) |, where Hi and Vi are obtained from Table 1, (Xs, Ys) and (Xe, Ye) indicate the start and end point position of the Wedgelet pattern, respectively.

- The Wedgelet pattern index idxW is mapped to the intra direction widx which minimizes D among all intra directions.

Table 1: Values of H and V for angular intra directions

| intra direction | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| V | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | -2 | -5 | -9 | -13 | -17 | -21 | -26 | -32 |
| intra direction | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| H | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |  |
| V | -32 | -32 | -32 | -32 | -32 | -32 | -32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |  |

Then for each intra direction pidx a Wedgelet list is constructed that includes all the Wedgelet patterns with mapped intra direction widx satisfying | widx - pidx | ≤ 1.

Wedgelet search. The intra direction of co-located texture luma block is first derived as pidx, then only the Wedgelet patterns in the Wedgelet list of intra direction pidx is used in the Wedgelet search function as described in section 2.3.3.7, thus large portion of Wedgelet searches are skipped.

#### Coarse wedgelet search with refinement step

The coarse search with refinement step is used at the encoder side for DMM mode 1 and DMM mode 3. The search method basically consists of three steps: first, the wedgelet pattern list is initialized. In the second step a coarse subset of the Wedgelet pattern list is searched for the minimum distortion partition. The resulting Wedgelet partition is refined in the third step.

Wedgelet pattern list initialization. The Wedgelet pattern lists are generated during encoder and decoder initialization.

For this purpose, the patterns for all possible combinations of start and end point positions are generated and stored in a lookup table for each block size prior to the coding process. The Wedgelet pattern list contains only unique patterns. The resolution for the start and end positions used for generating the Wedgelet patterns depends on the block size. For 16x16 and 32x32 blocks, the possible start and end positions are restricted to locations with an accuracy of 2 samples. For 8x8 blocks, full-sample accuracy is used, and for 4x4 blocks, half-sample accuracy is used

A coarse subset of Wedgelet patterns are marked by a flag during the list generation process. The subset contains the Wedgelets for every second start and end position (see Fig. 1, left). Furthermore, a Wedgelet node is derived for each Wedgelet of the coarse subset, containing up to eight references to refinement Wedgelets (see Fig. 1, right).

Figure 30: Coarse subset of Wedgelet separation line start/end positions (left) and Wedgelet node with refinements (right).

Search function. The following search algorithm is applied to find the Wedgelet within the pattern list, that provides the minimal distortion: In a first step the minimum distortion Wedgelet pattern is searched for the coarse subset by iterating over the Wedgelet node list and testing the referenced pattern. In a second step the Wedgelet partition found in the first step is refined. For this purpose the minimum distortion Wedgelet pattern is searched from the up to eight refinement patterns referenced in the corresponding node.

#### Mode selection

In the encoding process, for an intra-coded CU, one of the described depth modelling modes or one of the conventional intra prediction modes is selected. If a depth modelling mode is selected, the selected mode and the associated prediction data have to be signalled in the bitstream in addition to a syntax element that specifies the usage of a depth modelling mode. The following four depth modelling modes are defined:

- Wedgelet_ModelIntra: Intra modelling of Wedgelet block partition

- Wedgelet_PredIntra: Intra prediction of Wedgelet block partition

- Wedgelet_PredTexture: Inter-component prediction of Wedgelet block partition

- Contour_PredTexture: Inter-component prediction of Contour block partition

Each of the four modes can be applied with or without delta CPVs, resulting in eight different mode_IDs for signalling the decoder, which type of processing has to be applied for prediction and reconstruction of the block.

#### Signalling of mode parameters in the bitstream

For DMMs, the following parameters are signalled in the depth intra parameters structure as described in section 2.3.6.

Mode Wedgelet_ModelIntra: For this mode, the Wedgelet partition information is explicitly signalled in the bitstream by the index of the corresponding pattern in the Wedgelet pattern lookup list. The index is signalled with a fixed number of bins. The number of bins used for transmitting the index is given by the size of the list of possible Wedgelet patterns.

Mode Wedgelet_PredIntra: For this mode, only the refinement of the Wedgelet partition in terms of the line end position offset is signalled in the bitstream. A first bin indicates whether the offset is zero or not. If the offset is not zero, k+1 additional bins follow for signalling offset values in the range ±2k, where the first bin represents the sign and the remaining k bins the absolute value of the offset.k is set equal to 2.

Mode Wedgelet_PredTexture: For this mode, the Wedgelet partition information is explicitly signalled in the bitstream by the index of the corresponding pattern in a restricted Wedgelet pattern lookup list. The index is signalled with a fixed number of bins. The number of bins used for transmitting the index is given by the size of the list of possible Wedgelet patterns.

Mode Contour_PredTexture: For this mode, no additional signalling regarding the partition information is required.

Delta CPVs: In case the delta CPVs are transmitted (which is signalled by the transmitted mode ID), the two quantized values are signalled in the bitstream consecutively. For each CPV, a bin string consisting of the absolute value and the sign is transmitted. The sign is coded as a single bin, and the absolute value is coded using a truncated unary code (with 13 bins in the unary part and an exponential golomb code suffix).

### Region boundary chain coding

The region boundary chain coding mode partitions the block into two regions by signalling the region boundaries with chain codes. The region boundary chain coding consists of four steps.

Step 1: Find internal edges

The internal edges inside a depth-map block are calculated in the encoder. The step consists of several procedures.

- Calculate differences between vertically and horizontally adjacent pixels.

- Mark as edge candidates if the difference is greater than the threshold.

- Eliminate the edge candidates which have smaller differences than neighbouring edge candidates.

- Connect unlinked edges if necessary.

- Prune unconnected edges.

- Check whether the block consists of two regions exactly. Otherwise, the method will not be applied further.

Step 2: Code the edges using chain codes

The edges are encoded by using chain codes. First, an edge starts from the block boundaries. Then, the next edge is chosen as an edge connected to the current one. The final edge ends at the boundaries. The chain codes is constructed by defining seven traverse types (0, 45, -45, 90, -90, 135, and -135 degree) and their edge patterns. Figure 31 shows the patterns when the previous pixel is the left one (grey arrow). The patterns of other directions differ from the figure but can be constructed in the same manner.

Figure 31: Reconstructed edge using the relationship between previous and current directions

Step 3: Convert the chain codes into bitstream syntax

The region boundaries are expressed as a set of an edge starting point and the traverse codes.

Step 4: Calculate the predictors and fill them into the block

After the regional average values are chosen as the predictors for partitioned regions, the method for constant partition value (CPV) coding described in 2.3.3.5 can be applied to the mean value of the signal covered by the corresponding region.

#### Signalling in the bitstream

For the chain coding mode, the following parameters are signalled in the depth intra parameters structure as described in section 2.3.6:

Two syntax elements (edge_start_left_flag and edge_start_position) are used to determine the starting point. The traverse codes are signalled with num_edge_codes_minus1 and edge_code as described in Figure 32. The edge_code is constructed to have a different code word length according to the frequency of traverse types.

In case the CPVs are transmitted (which is signalled by edge_dc_flag ), two quantized values are signalled in the bitstream consecutively. For each CPV, a bin string consisting of the absolute value and the sign is transmitted. The sign is coded as a single bin, and the absolute value is coded using a truncated unary code (with 13 bins in the unary part and an exponential golomb code suffix).

Figure 32: Example of region boundary chain coding

### Simplified depth coding

The Simplified Depth Coding (SDC) approach is an alternative intra coding mode. Whether SDC is used is signalled in the depth intra parameters structure as described in section 2.3.6 at PU level. For SDC the depth block is intra predicted by a conventional intra mode or depth modelling mode 1. The partition size of CU containing a SDC coded PU is always 2Nx2N. Residual data is not coded as quantized transform coefficients but one or two constant residual values are signalled.

In summary the following information are signalled for SDC-coded blocks:

- The type of segmentation/prediction of the current block. Possible values are

  - DC (1 segment)

  - DMM Mode 1 – Explicit Wedgelets (2 segments)

  - Planar (1 segment)

- For the DMM mode, additional prediction information is coded, as described in  section 2.3.3

- For each resulting segment, a residual value (in the pixel domain) is signalled in the bitstream

Before coding, the residual values are mapped to values, which are present in the original, uncompressed depth map by using a Depth Lookup Table (DLT). Consequently, residual values can be coded by signalling only the index into this lookup table, which reduces the bit depth of residual magnitudes. This mapping table is transmitted to the decoder for the inverse lookup from index to valid depth value.

The advantage of using this lookup table is the reduced bit depth of the residual index for sequences with reduced depth value range (e.g. all estimated depth maps where not all depth values are present).

At the encoder side, the SDC process utilizes the mean of the original depth value (dorig) and the predicting depth value (dpred) which is the mean of the initial prediction samples of current depth block generated by intra prediction with mode DC, DMM mode 1, or Planar mode. A depth lookup table (DLT) is used to map the dorig and dpred to index values. The residual index to be transmitted to the decoder is given by

iresi=Idorig-Idpred,

where I. denotes the depth lookup table. At the decoder side, the reconstructed mean depth value (dorig) is derived as

dorig=I-1Idpred+iresi,

where I-1. denotes the inverse depth lookup table. The mean residual signal is then derived as

dresi=dorig-dpred.

The reconstructed value Px,y is derived by adding the residual signal on each prediction sample Px,y:

Px,y=Px,y+dresi.

The computed residual index iresi is then coded with a significance flag, a sign flag and the magnitude of the residual index.

A concatenation of unary binarization and FL binarization is used for the magnitude of the residual. For residual index binarization, the concatenate binarization is constructed by the prefix part of the unary code and the suffix part of the FL code. Only one kind of context model for residual index coding is applied to the prefix part. Since the suffix part of the concatenate binarization uses the fixed length code and the occurrence probability of the suffix part is lower, context modelling are not used.

All above parameters described above are signalled in the depth intra parameters structure as described in section 2.3.6.

#### Depth Lookup Table

The Depth Lookup Table supports depth maps, which don’t utilize the full range of 28 values. Here, only a small amount of different depth levels is used, e.g. due to coarse quantization. In the encoder, a dynamic depth lookup-table is constructed by analysing a certain number of frames (e.g. one intra period) of the input sequence. This depth lookup-table is used during the coding process to reduce the effective signal bit-depth of the residual signal.

##### Construction of the depth lookup table

In encoder reads a pre-defined number of frames from the input video sequence to be coded and scans all samples for available depth map values. During this process a mapping table is generated that maps depth values to valid depth values based on the original uncompressed depth map.

The Depth Lookup Table D(.), the index Lookup Table I(.), the Depth Mapping Table M(.) and the number of valid depth values dvalid are derived by the following algorithm, that analyses the depth map Dt

- Initialization

- boolean vector Bd=FALSE for all depth values d

- index counter i=0

- Process each pixel position p in Dt for multiple time instances t:

- Set BDtp=TRUE to mark valid depth values

- Count number of TRUE values in B(d)  dvalid

- For each d with Bd==TRUE:

- Set Di=d

- Set Md=d

- Set Id=i

- i=i+1

- For each d with Bd==FALSE:

- Find d=arg mind-d and Bd==TRUE

- Set Md=d

- Set Id=I(d)

### Unified signalling of depth intra modes

The depth intra modes, including depth modelling modes, simplified depth coding, and the chain code mode are signalled by one syntax element: depth_intra_mode. Based on the value of this syntax element, other additional information is signalled (as. e.g. wedgelet parameters and delta DCs as described above). Therefore a new syntax structure (depth_intra_parameters) is introduced that has minimum interaction with the HEVC syntax elements from the base specification.

The values of depth_intra_mode are shown in table Table 1

Table 1: Depth intra modes

| depth_intra_mode | DepthIntraMode | Section |
| --- | --- | --- |
| 0 | INTRA_DEP_SDC_PLANAR | 2.3.5 |
| 1 | INTRA_DEP_NONE | normal HEVC intra modes |
| 2 | INTRA_DEP_SDC_DMM_WFULL | 2.3.5 |
| 3 | INTRA_DEP_DMM_WFULL | 2.3.3.1 |
| 4 | INTRA_DEP_DMM_CPREDTEX | 2.3.3.4 |
| 5 | INTRA_DEP_DMM_WPREDTEX | 2.3.3.3 |
| 6 | INTRA_DEP_SDC_DC | 2.3.5 |
| 7 | INTRA_DEP_DMM_WPREDDIR | 2.3.3.2 |
| 8 | INTRA_DEP_CHAIN | 2.3.4 |

### Motion parameter inheritance

The basic idea behind the motion parameter inheritance (MPI) mode is that the motion characteristics of the video signal and its associated depth map should be similar, since they are both projections of the same scenery from the same viewpoint at the same time instant. Therefore, in order to enable efficient encoding of the depth map data, a texture candidate for the merge mode in depth coding that allows the inheritance of motion parameters from the texture signal has been introduced. The derivation of the texture candidate for depth is depicted in figure Figure 33. The motion parameters of the corresponding texture block are added as candidate to the merge list of the PU in the depth picture.

Figure 33: The derivation of corresponding texture block

Since the motion vectors of the video signal have quarter-sample accuracy, whereas for the depth map signal only full-sample accuracy is used, in the inheritance process the motion vectors are quantized to their nearest full-sample position.

[Ed. (GT): This is currently not reflected in draft, but might be added:

Since, when using MPI, not only the partitioning and the motion vectors, but also the reference picture indices are inherited from the video signal, it has to be ensured, that the depth maps that correspond to the video reference pictures are also available in the reference picture buffer for the depth map signal. ]

### Depth Quadtree Prediction

Depth quadtree prediction performs a prediction of the depth quadtree from the texture quadtree. It is applied in inter slices that do not belong to random access pictures. The partitioning of the depth is limited to the same level as the partitioning of the texture. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture. Moreover, when the texture is split in 2NxN (or Nx2N), partitioning to 2NxN, Nx2N, or NxN is not performed for depth. The possible partitioning is depicted in Figure 34.. Corresponding split flags and partition sizes for depth depending on the split flags and partition sizes of texture are summarized in Table 3.

Figure 34: Texture partitions and corresponding possible depth partitions

Table 3: Split flags and partition sizes of depth depending on split flags and partition sizes of texture

| Texture SplitFlag | Texture PartSize | Depth SplitFlag | Depth PartSize | Residual Depth SplitFlag | Residual Depth PartSize |
| --- | --- | --- | --- | --- | --- |
| 1 | - | 1 | - | 1 | - |
| 1 | - | 0 | 0, 1, 2 or 3 | 0 | 0, 1, 2 or 3 resp. |
| 0 | 0,1, 2 or 3 | 0 | 0 | - | - |
| - | 0, 1 or 2 | - | 0 | - | - |
| - | 3 | - | 0, 1, 2 or 3 | - | 0,1, 2 or 3 resp. |
| SplitFlag: 0 = no split, 1 = split; PartSize: 0 = 2Nx2N, 1 = Nx2N, 2 = 2NxN, 3 = NxN. |  |  |  |  |  |

## Encoder Control

For mode decision and motion estimation, a Lagrangian technique by which a cost measure D+λ⋅R is determined for each candidate mode or parameter, and the mode or parameter with the smallest cost measure is selected.Dis the distortion that is obtained by coding the considered block in a particular mode or with a particular parameter, R is the number of bits that are required for representing a block in a given mode or that are required for coding a given parameter, and λ is the Lagrangian multiplier that is derived based on the used quantization parameter. As measure for the distortion, the sum of squared differences (SSD) or the sum of absolute differences (SAD) between the original and the reconstructed sample values is used (for the coding of depth maps this measure was modified as described below).

For the coding of depth maps, basically the same decision process is used. However, the distortion measure has been replaced with a measure that considers the distortion in synthesized intermediate views. This technique is described in the following subsection.

### View Synthesis Optimization

The geometry information given by depth data is exploited only indirectly in the rendering process. Hence, the lossy coding of depth data causes distortions in the synthesized intermediate views. The depth map itself is not visible for a viewer. The efficiency of depth coding is improved by considering this property. As a consequence, the distortion measure for the mode decision process for depth maps is modified in a way that a weighted average of the synthesized view distortion and the depth map distortion. To obtain a measure of the synthesized view distortion, two different metrics  are applied in RDO.

The first metric, discussed in section 2.4.1.1, is the synthesized view distortion change (SVDC). The computation of the SVDC requires the usage of rendering functionalities in the encoding process. Since computational complexity is a critical factor in distortion calculation, a method, which is also referred to as renderer model, has been utilized that allows minimal re-rendering of parts of the synthesized view that are affected by a depth distortion. For this, a special renderer is included in the encoder, which supports the basic functionalities, shared by most rendering approaches, like sub-pixel accurate warping, hole filling and view blending.

The second metric, presented in section 2.4.1.2, is a model based synthesized view distortion estimation without rendering. Basic idea of this metric is to derive an estimate for the synthesized view distortion by weighting the depth distortion with a factor derived from the absolute value of the derivation of texture view in horizontal direction.

The integration of both metric in the encoder control is presented in sections 2.4.1.4 and 2.4.1.5.

#### Synthesized View Distortion Change (SVDC)

##### Definition of the SVDC

Since the encoding algorithm operates block-based, the mapping of depth distortion to the synthesized view distortion must be block-based as well. Moreover, the sum of partial distortions (of sub-blocks) must be equal to the overall distortion of a block in order to enable an independent distortion calculation for all partitions of a subdivided block, as hierarchical block structures are used in HEVC.

A relationship between a depth map sDand a synthesized texture sT'is created by the used view synthesis approach. However, disocclusions and occlusions prevent a bijective mapping of the distorted areas in depth maps to distorted areas in the synthesized views. For example, areas in the synthesized view, which depend on depth data of a considered block, can become visible due to the distortions in other depth blocks; or vice versa, the distortion of a depth block has no effect on the synthesized view, since the block is occluded there. Hence, an exact mapping between the distortion of a block of the depth data and an associated distortion in the synthesized view is not possible considering only the depth data within a currently processed block.

For resolving this issue, the change of the overall distortion in a synthesized view depending on the change of the depth data within a block B is determined, while simultaneously also considering depth data outside the block B. For this purpose, the synthesized view distortion change (SVDC) is defined as distortion difference ΔDbetween two synthesized textures sT'and sT,

|  | ΔD=D- D= x,y∈IsTx,y-s'T,Refx,y2-x,y∈IsT'(x,y)-s'T,Refx,y2 | (1) |
| --- | --- | --- |

s'T,Refdenotes a reference texture rendered from original video and depth data. Irepresents the set of all samples in the synthesized view. To illustrate how the textures sT'and sT' are obtained, the SVDC definition from eq. (1) is also depicted in Figure 35. sT'denotes a texture rendered from a depth map sD consisting of encoded depth data in already encoded blocks and original depth data in the other blocks. The current block B, for which the distortion has to be computed, contains original depth data as well. For the synthesis of the texture sT' a depth mapsDis used that differs from the depth map sDin that it contains the distorted depth data also for the current block B.

Figure 35: Definition of the SVDC related to the distorted depth data of the block B depicted by the hatched area in the bottom branch; VS denotes the view synthesis step and SSD stands for sum of squared differences.

The SVDC definition above is motivated by three reasons. First, an exact distortion measure is provided, therefore the overall distortion of the synthesized view and thereby disocclusions and occlusions are considered. Second, the measure is related to a block and third partial distortions are additive. For the latter two reasons, the change of the synthesized view distortion caused by a change of a depth block is employed instead of the total synthesized view distortion itself.

Figure 35 shows the SVDC definition for the extrapolation of virtual views from one input view only. However, the encoder side view synthesis algorithm supports also the interpolation of the texture sT' from a left and a right view. Hence, rendering requires a left sD,l and a right sD,r depth map. To extend the SVDC computation to this two view case, the original depth map of the second view can be used when encoding the first depth map. Subsequently the first already encoded depth map can be utilized for the SVDC computation when encoding the second depth map.

##### Efficient Computation of the SVDC

A straightforward approach to compute the SVDC would be the direct implementation of eq. (1). However, this would require the complete rendering of the synthesized textures sT' and sT' and a rendering of a whole view is computational too complex to be feasible in a rate-distortion optimization process. To overcome this problem, a method which enables a fast computation of the SVDC and is integrated in the encoder.

Renderer Model

The renderer model provides three basic functionalities to the encoder: Initialization, partial re-rendering, and SVDC calculation.

- The initialization of the renderer model is carried out before the encoding of a depth map is started. In the initialization process, the complete synthesized view is rendered using the original input depth maps and the input textures. The input depth maps are stored as the renderer models depth states sD,l and sD,r and the rendered view as the synthesized view state sT'. Intermediate variables used in the rendering process are also stored to enable a fast re-rendering.

- Partial re-rendering is carried out to update the renderer model when the encoding of a block B is finished and the final depth data for the block is known. For this purpose, the reconstructed depth data and the position of block B are signalled to the renderer model. The renderer model changes the block in the depth state sD,l or sD,rfrom original to coded data and re-renders only local parts of the synthesized view state sT' and the intermediate variables that are affected by the change of the depth data. Thus, the renderer model is transferred to a state that is required to compute the SVDC for blocks of the depth data encoded subsequently.

- For the computation of the SVDC, the position and the depth data of a block B to be tested are provided to the renderer model. The renderer model then computes the SVDC as defined in eq. (1). Here, re-rendering followed by the computation of the sum of squared distortions SSD is carried out. However, instead of considering all positions x,y∈I again only positions affected by the depth change are considered. Note that the re-rendering carried out here does not modify any state variables of the renderer model. Hence, the SVDC can be computed for multiple depth candidates successively without the need to re-render with original data in blockB.

Re-Rendering and Error Calculation Algorithm

The main objective of the algorithm is a computational low complex distortion calculation or state transition, hence a low complex re-rendering of the parts of the synthesized view that are affected by a depth change in one of the input depth maps.

Conventional view synthesis consists of multiple steps such as warping of the input samples, interpolation at sub pixel positions, blending with a second view obtained similarly, and hole filling. Typically these steps are executed as independent algorithms that are applied successively using the results of the previous step. To enable fast re-rendering of only parts of the synthesized view, all steps are combined in single algorithm that can be applied pixel-wise to the input depth map. This allows a region-wise processing of the depth map, and thus an update of related regions in the synthesized view.

This process is illustrated in Figure 36 for an example for rendering from a left view to the right. Rendering is applied row wise, hence all depicted signals represent one row of input, intermediate, or output data. The single signals are from bottom to top: the left input texture sT,l, a shifting chart, the texture synthesized from left s'T,lthe texture synthesized from right s'T,r, the blended textures'T, and the reference textures'T,Ref. The arrows denote the relationship between the single samples or sample positions of the signals. Dots shown in the shifting chart represent samples from the input view. Their horizontal position is equal to their position x'in the synthesized view. The vertical position shows their disparities. Since the depth is monotonically decreasing with increasing disparity, the top-most samples in the chart are the samples closest to the camera. Hence, it can be seen from the shifting chart which samples are occluded in the synthesized view.

Figure 36: Example for the dependencies between input, intermediate and output signals of the rendering or error calculation step.

While a conventional view synthesis approach would carry out the single steps depicted from bottom to top for all samples in the intervals (a) to (g), the method supports an interval-wise processing. Hence, all steps are first conducted for interval (a) before continuing with interval (b). Re-rendering and error calculation are carried out by iterating only once over the input depth samples. If only the view synthesis distortion is calculated there is no need to store intermediate results in the state of the renderer model.

The boundaries of an interval in the output view are defined by the warped positions x’s and x’eof two neighboring input view samples at positions xsand xe. For warping, disparities are computed from the depth map as described in the beginning of sec. 2. Subsequently to the calculation of the interval boundaries, processing continues with interpolation, disocclusion handling, or occlusion handling:

- Interpolation is carried out in non-occluded ranges that are not disoccluded, as for example in the intervals (a, c, d, g, h). The accuracy of the warping is higher than the accuracy given by the sampling rate of synthesized view; hence an interpolation at the full sample position x’FPlocated between the interval boundaries x’sand x’eis carried out. For this, samples from an up-sampled version of the input texture sT,l are mapped to the interpolation positions x’FPin the synthesized view s'T,l. The position xin the up-sampled view is derived from the distance of the interpolation position to the interval boundaries:

|  | x=4⋅xFP'- xs'xe'-xs'+xs | (2) |
| --- | --- | --- |

The up-sampled view sT,lis created in the initialization step by interpolating the input texture with quarter-sample accuracy using the FIR-filters specified for motion-compensated interpolation in HEVC.

- Disocclusions: If the width of the warped interval x’e– x’sis greater than two times the width of the sampling distance, as for example for interval (b), a disocclusion is assumed in the synthesized view. Instead of interpolation, hole filling is carried. For this purpose, the samples in the interval are set equal to the value of the sample belonging to the right interval boundary sT,l(xe) (which belongs to the background). If the leftmost full sample position within the interval is close to the left interval border, it is assumed that it belongs to the foreground and it is set equal to the value of the left interval boundary sT,l(xs). Note, that the positions of disoccluded and filled samples are stored as additional information in the a filling map s'F,l.

- Occlusions: Whether an interval is entirely occluded in the synthesized view, as for example interval (f), is determined by detecting if the interval boundaries are reversed (x’e< x’s), hence no complex z-buffering is required. To derive whether other samples left to interval (f) are occluded, the rendering process stores the position of the foreground edge. This stored position is then be utilized when processing the next intervals, for example interval (e), to determine which parts of theses intervals are occluded. If re-rendering does not start at the right image border, the position of the last foreground edge is recovered by carrying out a search to the right of the changed depth samples.

Sample values derived from interpolation or hole filling sT,l', are instantly combined with the texture sample values from a second view sT,r' synthesized the same way and stored as intermediate variable in the renderer model. The result is the sample value that is used in the final synthesized view sT'.

The rendering model supports two different configurations. In the first configuration, a rendering process is considered that renders intermediate views using both surrounding actually coded views. The second configuration considers rendering processes by which an intermediate view is rendered mainly from one coded view; the other coded view is only used for rendering areas that are not present in the preferred coded view.

In the first configuration of the renderer model, the blending process is similar to that implemented in the VSRS software. Note that, although not depicted in Figure 36, a depth map sD,l' is rendered from sD,l, when rendering sT,l', using full sample accuracy. This depth map is used in the blending step. The decision how blending is carried out depends on the filling of sF,l'or sF,r' and the rendered depth maps sD,l'and sD,r'. While sF,l'and sD,l'have been obtained in the rendering process carried out before, sF,r' and sD,r' are stored as intermediate variables in the renderer model. The rules for determining the blended sample value s’T(x,y) from sT,l'(x,y) and sT,r'x,y are specified in the following:

- If the position (x,y) is disoccluded (as indicated by the filling map) in only one view, the sample value from the other view is used.

- Otherwise, if the position (x,y) is disoccluded in both views, the backmost sample value is used.

- Otherwise, if the depth difference retrieved from sD,l'(x,y) and sD,r'(x,y) is greater than a threshold, the front sample is used.

- Otherwise, a weighted average of sT,l'(x,y) and sT,r'x,y, with a higher weight for the view that is closer to the virtual view position, is used.

For the second configuration of the renderer model, the intermediate view is mainly rendered from one view and only holes are filled from the other view. If assuming thatsT,l' is the main view ,the rules to determine the sample value s’T(x,y) from sT,l'(x,y) and sT,r'x,y are specified in the following:

- If sF,l'x,yindicates that there is no disocclusion at sT,l'x,y, the sample valuesT,l'x,y is used.

- Otherwise, if sF,r'x,y indicates that there is a disocclusion at sT,r'x,y, the sample value sT,l'x,y is used.

- Otherwise, the average of sT,r'x,yand sT,l'x,y is used.

If only partial re-rendering is carried out, the result sT' and all intermediate results are stored after the combination step and the processing of the interval is stopped. Otherwise, if the SVDC is determined, the distortion of the calculated value sT' is computed by comparing it to the referencesT,Ref' in the next step.

To obtain the synthesized view distortion change the single intervals are rendered from right to left and the related distortions are summed up continuously. Moreover, and that is actually not depicted in Figure 36, the old per sample distortions of samples in the changed intervals are subtracted.

The renderer model only re-renders those parts of the synthesized view that are affected by the considered depth change. It has to be considered that in some cases not only the intervals related to the changed depth values must be re-rendered, but also some neighbouring intervals. A reason is that neighbouring intervals that are occluded before a depth change can become visible after the depth change. The algorithm detects such cases and continues rendering, until all change samples in the synthesized view are updated. The detection is carried out while warping by also considering the old shifted sample positions as they had been prior to the depth change and storing the left-most old position.

Chroma channels of the synthesized view are rendered together with the luma channel and are stored in the same resolution as luma. For this, up sampled versions of the chroma channels are created in the initialization step, which are later used for interpolation as described above. The sampling rate is increased by a factor of eight in horizontal direction and a factor of two in vertical direction using the interpolation FIR-filters that are specified for motion-compensated prediction in HEVC. However, the total distortion is obtained by a weighted sum of luma SVDC and chroma SVDC with a weight of 1 for luma and a weight of ¼ for each of the two chroma channels.

Early skip of SVDC computation

To increase the processing speed of the VSO algorithm the SVDC calculations is skipped for lines of a block for that the distortion of the disparity vector is zero. This means that if distorted depth and original depth are mapped to the same disparity vector for all pixels in a line of the depth block the SVDC calculation is not carried out and the distortion is assumed to be zero. .

#### Model based synthesized view distortion estimation without rendering

The distortion of depth maps does not linearly affect the synthesis distortion, and the impact of depth map distortions varies according to the corresponding texture information. For example, the same depth distortions on textured and textureless regions lead to different synthesis distortions.

In a conventional video coding system, one commonly used distortion function is the sum of squared differences (SSD), which is defined between original and encoded depth block as

|  | Ddepth=x,y∈BsDx,y-sDx,y2, | (3) |
| --- | --- | --- |

where sDx,y and sDx,y indicate the original and reconstructed depth map, respectively, and (x, y) the pixel position in a (macro-) block B. However, the conventional SSD metric is not an good estimate of the synthesized view distortion. Instead, the following view synthesis distortion (VSD) metric provides an better estimate by that weighting the depth distortion Ddepth with the sum of absolute horizontal texture gradients:

|  | VSD=x,y∈B12⋅α⋅sDx,y-sDx,y⋅sT(x,y)-sT(x-1,y)+sT(x,y)-sT(x+1,y)2 | (4) |
| --- | --- | --- |

sT indicates the reconstructed texture, and α is proportional coefficient determined by

|  | α=f⋅L255⋅1Znear-1Zfar | (5) |
| --- | --- | --- |

with f denoting the focal length, L denoting the baseline between the current and the rendered view, Znear and Zfar representing the values of the nearest and farthest depth of the scene, respectively.

#### Depth fidelity term

When encoding depth data, using the synthesized view distortion change or estimate only, the depth fidelity is strongly distorted. In order to preserve the depth fidelity the distortion measure used in RDO is computed by an weighted average of the synthesized view distortion or the estimated synthesized view distortion and the depth distortion. The distortion D used in RDO for depth maps is given by

|  | D= wsynthDsynth+wdepthDdepth | (6) |
| --- | --- | --- |

with Dsynth denoting the synthesized view distortion change or estimate, Ddepth denoting the distortion of the depth map itself (i.e. SAD or SSD), and wsynth and wdepth denoting the weights for the two distortion terms.

#### Integration of distortion metrics in the Encoder Control

To enable rate-distortion optimization using the SVDC, the described renderer model is integrated in the encoding process for depth data. For this, the conventional distortion computation is replaced with computation of the weighted average of depth distortion and SVDC in distortion computation steps related to the mode decision, coding unit (CU) partitioning, motion parameter inheritance and merging. Note that for updating the renderer model used for the SVDC calculation re-rendering is carried out when a final decision on the coding mode is taken by the encoder control.

In order to reduce the computational complexity the weighted average of SVDC and depth distortion in not used for all encoding decisions. A weighted average of VSD and depth distortion is used for intra-mode pre-selection and residual quadtree partitioning. For motion estimation and rate-distortion optimized quantization the conventional SSD of depth data is used.

#### Adaptation of the Lagrange Multiplier

The usage of the synthesized view distortion in the rate-distortion decisions requires the adaptation of the Lagrange multiplier  to obtain optimized coding results. This adaptation is carried out by adjusting the Lagrange multiplier using an additional scaling factor ls depending on the QP of the coded video. The factor enables an adjustment of video/depth rate allocation. As alternative a constant factor can be used.

The computation of rate-distortion cost  has been modified to

|  | J=D+ls⋅λ⋅R=D+λl⋅R | (7) |
| --- | --- | --- |

with D denoting the weighted average of depth and synthesized view distortion, lsdenoting a scaling factor, and R denoting the rate for the current coding mode.

### Zero residual coding for depth intra CUs

In the rate distortion optimized coding of inter blocks a decision between coding with and without residual data is carried out. For depth coding this principle is extended to intra coded blocks that are not part of slices using intra prediction only. Therefore, the residual is set to zero by the encoder. No additional signalling is used.

### Optional Encoder Control using a depth quadtree limitation

In the encoding process a given CTB is split into smaller CUs, based on RD optimized decisions. A corresponding quadtree (QT) is obtained for the texture, and another one for the depth. This tool prevents the encoder from making full investigation of every possible QT configuration for the depth.

The tool forces the encoder to limit the partitioning of the depth at the same level as the partitioning of the texture. For a given CTU, the quadtree of the depth is linked to the collocated CTB quadtree in the texture, so that a given CU of the depth cannot be split more than its collocated CU in the texture.

This encoder restriction results in encoder runtime saving for the depth.

Figure 37: Example of a CTB QT partitioning for the texture (left), allowed collocated depth CTB QT partitioning (centre), and disallowed collocated depth CTB QT partitioning (right).

Figure 37 illustrates this principle. On the left a CTB QT partitioning for the texture is represented. In the centre, the collocated CTB in the depth is represented. This QT partitioning is allowed because it is, CB by CB, coarser than the corresponding texture CB. On the right, another example of possible collocated CTB in the depth is represented. This QT partitioning is disallowed because one CB is more partitioned than the texture (red lines).

### Optional Encoder Control for Renderable Regions in Dependent Views [Not in CTC]

As an optional encoding technique, a mechanism is integrated by which regions in dependent views are identified that can be rendered based on the transmitted independent view and the associated depth maps. These regions are encoded by employing a modified cost measure, which mainly considers the required bit rates. After decoding, the renderable regions can be identified in the same way as in the encoder and replaced by rendered versions.

Figure 38: Rendering from a left camera position to a right camera position using depth maps.

The encoder identifies regions in the current frame that can be rendered from frames of the same time instance in a reference view based on the reconstructed depth maps of the reference view (see Figure 38). During the encoding process, the encoder checks for every CU, if all samples within that CU can be rendered. If all samples can be rendered, no residual is transmitted for this CU. Here, for inter prediction the no_residual_data_flag for the CU is set equal to 1 or for intra-prediction the coded block flag of the TUs within the CU is set equal to 0. It should be noted that no syntax change is applied; only the encoder decision is modified.

Due to the quadtree structure in HEVC, the rate-distortion (RD-)costs are compared between different granularities of possible block subdivisions for the R-D optimization. Rendering artefacts have a different impact on the subjective image/video quality perception than coding artefacts and cannot be compared using conventional measurements, such as MSE or PSNR. Samples in renderable regions are not taken into account for calculating the distortion term in the R-D optimized encoder decisions. In Figure 39, the right image shows a block subdivision that is one level deeper than the ones in the left image. The grey area labels the samples that can be rendered and that are therefore not considered in the calculation of the distortion. Thus, for example, the upper left block in the right image is not considered at all. Hence, the costs being compared are D0+λR0(left block subdivision) against D1+λ R1+ D2+λ+D3+λ R3+D4+λ R4 (right block subdivision), where the distortions are only calculated based on the white shaded samples. E.g., the distortion of block B1is D1=0. By this modification, blocks for which a subblock can be rendered are not automatically split, but also the entire block may be coded using a conventional coding mode if this improves the overall coding efficiency.

Figure 39: Distortion calculation on different tree depths. Renderable samples (gray shaded) are not taken into account.

For renderable blocks, the Lagrange multiplier λ is scaled by a factors>1 and the calculation of the R-D costs is changed from C=D+λR to C=D+sλR.

# View Synthesis Algorithms

In the following, two view synthesis algorithms are described. Sec. 3.1 describes the fast 1-dimensional view synthesis algorithm that is part of the HEVC-based 3DV software. It is also referred to as "VSRS 1D fast mode". In sec. 3.2, an alternative view synthesis algorithm is described. This algorithm is also referred to as "VSRS" and was developed during the 3DV exploration experiments.

## Fast 1-D View Synthesis (VSRS 1D Fast Mode)

An overview of the view synthesis method is depicted in Figure 40. The method supports the interpolation of a synthesized view form a left sT,l and right sT,r texture with corresponding depth maps sD,l and sD,r. For this, two texture sT,l' and sT,r' are extrapolated from the left and the right view at the position of the virtual view. Subsequently, the similarity of sT,l' and sT,r' is enhanced before combining them to synthesized output view sT'. The single processing steps are discussed in the following. Without the loss of generality steps carried out independently for both, the left and the right view, are discussed for the left view only.

Figure 40: Processing steps of the view synthesis approach.

Similarly as the renderer model used in the encoder control (cp. sec. 2.4.1), the view synthesis algorithm supports two configurations. In the first configuration, which is referred to as interpolative rendering, an intermediate view is synthesized using both surrounding coded views. In the second configuration, which is referred to as non-interpolative rendering, an intermediate view is rendered mainly from one coded view; the other coded view is only used for rendering areas that are not present in the preferred coded view.

### Upsampling of input video pictures

The luma channel of input texture sT,l is upsampled by a factor of four in horizontal direction. Chroma channels are upsampled by a factor of eight in horizontal direction and two in vertical direction. For upsampling, the FIR filters specified in HEVC for the purpose of motion-compensated interpolation are used. The resulting upsampled texture is denoted as sT,l.

### Warping, interpolation and hole filling

Warping, interpolation and hole filling are carried out in a combined step. For warping disparities are computed as described in the beginning of sec. 2. Warping, interpolation and hole filling is carried out line wise and within a line interval wise. Processing direction is from left to right. An interval in the output view is defined by the warped positions x’s and x’eof two neighboring input view samples at positions xsand xe. Subsequently to the calculation of the interval boundaries, processing continues depending on the width of the interval.

- Interpolation is applied if the width of the warped interval x’e– x’s is less than or equal to two times the sampling distance. An interpolation at the full sample position x’FPlocated between the interval boundaries x’sand x’eis carried out. For this, samples from the up-sampled version of the input texture sT,l are mapped to the interpolation positions x’FPin the synthesized view s'T,l. The position xin the up-sampled view is derived from the distance of the interpolation position to the interval boundaries:

|  | x=4⋅xFP'- xs'xe'-xs'+xs | (8) |
| --- | --- | --- |

- Disocclusions: If the width of the warped interval x’e– x’sis greater than two times the width of the sampling distance a disocclusion is assumed in the synthesized view. Instead of interpolation hole filling is carried. For this purpose samples in the interval are set to the value of sample belonging to the right interval boundary sT,l(xe) (which belongs to the background). If the leftmost full sample position within the interval is close to the left interval border it is assumed that it belongs to the foreground and it is set to the value of the left interval boundary sT,l(xs). Disoccluded and filled sample position are stored in the filling map s'F,l.

- Occlusions: If the boundaries of an interval are reversed (x’e< x’s) the interval is occluded in the synthesized view. Rendering at a full sample position close to xe' might be carried out, if the next interval is not occluded and xe' belongs to a foreground object. Moreover, the algorithm uses the property that occluded background intervals are automatically overwritten by foreground objects in the synthesized views'T,l, due to the processing direction from left to right.

Chroma channels of the synthesized view are rendered together with luma channel and stored in the same resolution as luma. Moreover, if interpolative rendering is used, also a depth map sD,l' is extrapolated with full sample accuracy from the input depth map sD,l within the steps described above.

### Reliability map creation

In this step the filling map sF,l' is converted to the reliability map sR,l'. If interpolative rendering is used, positions marked as disocclusions in sF,l' are mapped to a reliability of 0. In areas located right to a disocclusion with a width of six samples the reliability is linearly increased from 0 to 255 from left to right in horizontal direction. All other samples are assigned with a reliability of 255. If non-interpolative rendering is used, positions marked as disocclusions in sF,l' are mapped to a reliability of 0. All other samples are assigned with a reliability of 255.

### Similarity enhancement

In this step the histogram of s'T,l is adapted to the histogram of s'T,r. For this purpose a look up table (LUT) realizing a function f is created, that is subsequently applied to map the samples of s'T,l to adapt their values.

The function f and the corresponding LUT are obtained by approximately solving

|  | hfsT,l'= hsT,r' | (9) |
| --- | --- | --- |

whereh[…] denotes the histogram only regarding samples at positions (x,y)with reliabilities sR,l'x,yand sR,r'x,yof 255. Chroma channels are treated in the same way.

### Combination

s'T,lands'T,r are combined to obtain the synthesized output view in this step.

In the interpolative rendering mode is used, the decision how blending is carried out depends on the reliability maps sR,l'or sR,r' and the rendered depth maps sD,l'and sD,r'. The rules for determining the blended sample value s’T(x,y) from sT,l'(x,y) and sT,r'x,y are given in the following:

- If position (x,y) is disoccluded (reliability of 0) in only one view, the sample value from the other view is used.

- Otherwise, if position (x,y) is disoccluded in both views, the backmost sample value is used.

- Otherwise, if the depth difference retrieved form sD,l'(x,y) and sD,r'(x,y) is above a threshold, the front sample is used.

- Otherwise, if one sample is not reliable with a value of 255, a weighted average with the given reliabilities as weights is used.

- Otherwise, a weighted average of sT,l'(x,y) and sT,r'x,ywith a higher weight for the view that is closer to the virtual view position is used.

If the non-interpolative rendering mode is used, the intermediate view is mainly rendered from one view are utilized and only holes are filled from the other view. Assuming sT,l' is the main view,the rules for determining the sample value s’T(x,y) from sT,l'(x,y) and sT,r'x,y are given in the following:

- If sR,l'x,yis equal to 255 or sR,r'x,y is equal 0, the sample valuesT,l'x,y is used.

- Otherwise, if sR,l'x,y is equal to 0, the sample value sT,r'x,y is used.

- Otherwise, a weighted average with the given reliabilities as weights is used.

### Chroma decimation

To convert the 4:4:4 YUV representation obtained by rendering to the required 4:2:0 output, chroma channels are decimated by a factor of two in horizontal and vertical direction using the FIR filter (1;2;1).

## VSRS (alternative view synthesis algorithm) [Not in CTC]

The VSRS algorithm was developed during the MPEG 3DV Exploration Experiments. VSRS takes two reference views and two depth maps as input to generate a synthesized virtual view. The intrinsic and extrinsic camera parameters are required and 1D parallel and non-parallel camera setups are supported.

The software has two main modes referred to as “General mode” and “1D mode”. The reference views are reprojected to the target viewpoint using pixel-by-pixel mapping based on 3D warping in “General mode”, or horizontal pixel shifting in “1D mode”.

### General mode

In the general mode, virtual views are generated by a technique referred to as “3D warping”. This process involves two steps. At first the original view (reference view) is projected into 3D world space using the corresponding reference depth map. Then the 3D space points are projected into the image plane of the “virtual” view. For this, the intrinsic camera parameters A, and extrinsic camera parameters E=[R|t] are required. The intrinsic matrix A, transforms the 3D camera coordinates to its 2D image coordinates. The extrinsic matrix E=[R|t] transforms the world coordinates to camera coordinates, which is composed of rotation matrix R and translation vector t. The two-step warping can be formulated in two equations as in eq. (10) and (12). First a pixel (ur, vr) in the reference view is warped to the world coordinates (Xw, Yw, Zw), using the depth of the reference view:

(10)

where subscript r indicates the reference view and zr is the depth value in the reference view at location (ur, vr) calculated from

(11)

where v is an 8-bit intensity of the depth map value. It is noted that the values z, Znear, and Zfar are assumed to be either all positive or all negative values.

Then the 3D point is mapped to the virtual view:

(12)

where subscript v refers to the virtual view.

The general mode is based on a "reverse warping" algorithm. Instead of forward warping the left and right reference views to the virtual location, the left and right depth maps are warped to the virtual view location. Then after filtering, these depth maps are used to warp the reference views to the virtual view. This results in a higher rendering quality of the final synthesized view. Figure 41 depicts the flow diagram of the general mode.

Figure 41: Flow diagram for VSRS general mode.

The steps of VSRS general mode are briefly described below:

- First, the two depth maps are mapped to the target viewpoint. E.g. the left reference depth is warped to the virtual view location using eq. (10) and (12). If multiple pixels warp to the same location in the virtual view, then the pixel closest to the camera wins, so foreground pixels will occlude background pixels. The right depth map is also warped in a similar way. We denote these warped depth maps as DL’ and DR’, respectively.

- The mapped depth maps DL’ and DR’ may contain small holes. Small holes which are caused by rounding to integer coordinates are filled by a series of median filtering. Furthermore, binary masks for each side are maintained to indicate larger holes, for example caused by occlusions that remain after filtering. During the following steps, these binary masks are used and updated if necessary (for example during hole filling in step ).

- Next, the left and right texture reference views are mapped to the target viewpoint using the filtered depth map DL’ and DR’. So two texture images at the target viewpoint are obtained, one generated from the left reference view and the other from the right reference view. We denote them here as VL’ and VR’, respectively. Note that DL’ is used to warp the left reference, and DR’ is used to warp the right reference.

- Hole areas in the mapped texture images VL’ and VR’, which are caused by occlusion, are filled by pixels from the other mapped texture image. So holes in VL’ are filled from non-hole areas in VR’ and vice versa.

- Next, these two virtual images are blended. The general mode has two modes of blending: Blending-on and Blending-off. The Blending-on mode is a weighted blending based on the baseline distance. So pixels from the reference camera which is closer to the virtual view are assigned a higher weight, based on the baseline ratio. In Blending-off mode, all pixels visible in the closer reference view are copied to the virtual view, and only hole areas are filled from the farther reference view. During this step, the binary masks are merged to form one mask indicating remaining holes which are inpainted in the next step.

- Any remaining holes after blending are filled by an inpainting algorithm using the binary mask. Inpainting algorithms can be used to reconstruct damaged portions of images. Generally a mask is used to indicate which image regions need to be inpainted. Next, colour information is propagated inward from the region boundaries, i.e., the known image information is used to fill in the missing areas. An inpainting example is show in Figure 42.

Additionally, VSRS contains a Boundary Noise Removal algorithm. In this mode, the binary maps indicating holes caused by occlusion, are used to identify object boundaries. After identifying the background side of the holes based on the depth, the holes are expanded into the background. Then these areas in VL’ and VR’ are filled from the opposite reference view. This reduces noise around object boundaries, where foreground pixels are falsely projected into background objects due to depth errors.

Figure 42: Inpainting: “damaged” image, mask, and result after inpainting.

### 1-d mode

VSRS provides a second synthesis mode other than the general "3D warping" as described above: 1D mode. This mode is implemented with assumptions that the optical axes of camera are in parallel and the views are rectified such that no vertical disparities exist. Under the assumption of 1D mode, formulations can be simpler than in the general case:

- The rotation matrix for every camera is identical to each other.

- The translation vectors of all cameras share the same translation in Y and Z directions, that is, Ty and Tz are constant for every view.

- As a consequence

- Views are corrected (distortion and vertical disparity are null), so vertical position of intersection of optical axis in sensors is constant

So the matrix has the following form , where  and denote the horizontal and vertical focal length in pixels;and  the position of intersection with the optical axis in image ( is constant among cameras).

Then eq. (12) given for the general case can be simplified as,

and	(13)

The equation above is used to “warp” pixels from real views to the virtual one.

Figure 43 depicts the flow diagram of the VSRS 1D-mode.

Figure 43: Flow diagram for VSRS 1D mode.

The algorithm proceeds as follows:

- In a preliminary phase,

  - The chroma components are upsampled to 4:4:4 format (for implementation simplicity).

  - For suppressing transient depth errors, the depth maps can be temporally filtered according to the variations of the colour information if the TemporalImprovementOption is chosen.

  - The colour video may be further upsampled, if sub-pixel precision is specified in the configuration file, for example, half-pixel or quarter-pixel.

- During the warping process, the reference views and the depth maps are mapped to the target viewpoint using eq. (13), which is a 1D shifting on the samples. For each reference view, a binary mask is maintained indicating whether a pixel in the targeted map is filled or not (hole pixel). The warping procedure is also controlled by the splatting switch in configuration file. When splatting is selected, each pixel in the reference view may be mapped to two sample locations. Besides, two enhancement processing on warping (corresponding to CleanNoiseOption and WarpEnhancementOption) suppress some synthesis artefacts due to the texture-depth misalignment at object boundaries (which causes foreground pixels scattered to the background) and wrongly categorized holes in the foreground (which makes background pixels appear in the foreground). Warping of the unreliable pixels (which probably yield artefacts) is forbidden accordingly.

- Two warped images from left and right reference views are obtained from last step, which are then merged to a single image. This operation is also applied on warped depth maps and filling masks. In case of conflicts (two pixels present for the same target position), the MergingOption specified by the user is applied in the following way.

  - Z-buffer only: Take the pixel closest to camera always.

  - Averaging only: Mix colours using weights in reverse proportional to the distance of the virtual camera from the left and right reference views

  - Adaptive merging: Use either the proximity criterion (o) if depth level difference is greater than a threshold or, (o) if depth levels are too similar, uses the weighting method.

- Hole areas in the warped images are filled by propagating the background pixels into the hole along the horizontal row.

- Final view image is downsampled to original size if necessary and transformed to 4:2:0 format for output purposes.

Additionally, VSRS 1D mode can use the boundary noise removal algorithm already described as final processing step in the section dedicated to the general mode.

# Software

## Software repository

The source code for the software will be available in the MPEG SVN repository. An initial version of the software is available in the following SVN repository.

https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/

For tool integration a branch for a company can be obtained by contacting:

gerhard.tech@hhi.fraunhofer.de,

kwegner@multimedia.edu.pl

## Build System

The software can be built under linux using make. For Windows, solutions for different versions of Microsoft Visual Studio are provided.

## Software Structure

The 3D-HEVC Test Model Software includes several applications and libraries for encoding, decoding and view synthesis:

- Applications:

  - TAppEncoder, executable for bit stream generation

  - TAppDecoder, executable for reconstruction.

  - TAppRenderer, executable view synthesis

  - TAppExtractor, executable for bitstream extraction

- Libraries:

  - TAppCommon, library for handling encoder, decoder and renderer options and camera parameters

  - TLibEncoder, encoding functionalities

  - TLibDecoder, decoding functionalities

  - TLibRenderer, renderer functionalities

  - TLibExtractor, bitstream extraction functionalities

  - TLibCommon, common functionalities

  - TLibVideoIO, video input/output functionalities

