| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D1101 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1) on View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Shinya Shimizu | Email: | shimizu.shinya@lab.ntt.co.jp |
| Source: | JCT-3V CE1 Coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 1 (CE1) of the development of 3D extensions for HEVC. This Core Experiment investigates coding methods based on view synthesis prediction, which uses texture and depth information to construct a synthesized prediction signal for coding the dependent views.

# Participants

| Participant | Contact | Email | Proponent |
| --- | --- | --- | --- |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | P |
| Mitsubishi Electric | Dong Tian | tian@merl.com | P |
| MediaTek | Yu-Lin Chang Yi-Wen Chen Yu-Wen Huang | yulin.chang@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com |  |
| Qualcomm | Xin Zhao Jewon Kang Li Zhang | xinzhao@qti.qualcomm.com jewonk@qti.qualcomm.com lizhang@qti.qualcomm.com | P |
| Nokia | Dmytro Rusanovskyy | Dmytro.Rusanovskyy@nokia.com |  |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | P |
| LG Electronics | Sehoon Yea | sehoon.yea@lge.com |  |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com |  |
| ETRI | Gun Bang | gbang@etri.re.kr |  |
| KyungHee University | Kyung Yong Kim | kimky@khu.ac.kr |  |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp |  |
| KDDI | Kei Kawamura | kei@kddilabs.jp |  |
| GIST | Yunseok Song Yo-Sung Ho | ysong@gist.ac.kr hoyo@gist.ac.kr |  |
| HHI | Heribert Brust | heribert.brust@hhi.fraunhofer.de |  |
| NCKU | Chun-Fu Chen | n28991146@mail.ncku.edu.tw | P |

(P = proponent, C = cross checker)

# Tools under Investigation

The tools to be investigated in this Core Experiment use the decoded texture and depth view components of the already coded views to compute a prediction signal for the texture view components of the coding target views. This process is done by means of disparity compensation, where the disparity values are derived from the decoded depth information.

## CE1.h: View Synthesis Prediction for 3D-HTM

A block-based VSP using a backward warping was adopted into 3D-HTM at Geneva meeting, which is done with a two-step disparity compensation. In the first step, a virtual depth map is fetched for a CU by calling the NBDV function without the depth-based refinement. In the second step, VSP compensation is performed on a 4x4 block basis and prediction samples are generated. For VSP compensation, a disparity vector is derived from the derived virtual depth map, where a maximum depth values among those of 4 corners samples is selected. VSP is signaled by the merge mode.

The tools to be investigated can be categorized into the following 2 categories. All separable tools should be evaluated separately according to the following categorization. Reports on combined tools are also encouraged.

- Block size for the second disparity compensation

  - Adaptive 16x16, 8x8, or 4x4 based on CU size (JCT3V-D0179)

  - Adaptive 8x8, 8x4, or 4x8 based on gradient of virtual depth map (JCT3V-D0089)

  - Adaptive 8x4 or 4x8 based on 4 corner depth samples (JCT3V-D0213)

  - Fixed 1x1 with a constraint disparity range (JCT3V-D0165)

- Forward block-based VSP (JCT3V-D0168)

### JCT3V-D0179: CE1.h related: Low-complexity BVSP for 3D-HEVC

This contribution proposes that the size of motion compensation unit in VSP is set based on the size of the CU so that when the size of the CU becomes larger, less blocks in an inter-view reference picture need to be accessed.

- Pros. Complexity reduction on large CU, Cons. 4x4 disparity compensation is still required.

### JCT3V-D0089: CE1.h: 3D-HEVC: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Complexity Analysis

This proposal proposes the techniques which would adaptively partition the depth block to derive the disparity vectors. This adaptively partition scheme is based on the gradients of depth block, the supported block sizes include 4x8, 8x4, and 8x8 block sizes. Four different settings on the calculation of gradients are tested. Detailed complexity analysis based on the number of the logical operations, data storage requirement, and data transfer rate are also provided.

- Pros. No 4x4 disparity compensation, Cons. Slight increase of memory and complexity.

### JCT3V-D0213: 3D-CE1.h: Adaptive block size on VSP

This contribution proposes to use either 4x8 or 8x4 block size in VSP. The block size is adaptively decided by analyzing four corner samples of virtual depth map generated by the first DCP.

- Pros. No 4x4 disparity compensation, Cons. Slight increase of complexity.

### JCT3V-D0165: 3D-CE1.h: VSP Complexity Analysis and Constraint VSP (CVSP)

This contribution proposes pixel-based disparity compensation with some constraints for VSP. One constraint is zero vertical disparity, and the other constraint is to limit variation of horizontal disparity vector within one block. These constraints reduce the number of fetched pixels for VSP compensation to maintain comparable complexity with traditional block-based MC.

- Pros. Coding performance and less off-clip memory access compared to the traditional MC, Cons. Possible increase of on-clip memory access, which has not been thoroughly studied.

### JCT3V-D0168: CE1.h related: Forward Block-based View Synthesis Prediction

This contribution describes the forward block-based view synthesis prediction (FVSP) design for 3D-HEVC. FVSP estimates a reference block in the base view by only using the base view depth map. Then pixels in the reference block are warped to a target block in current coding view to be the prediction block of current coding block.

- Pros. Coding performance, Cons. Possible increase of memory access and complexity, which has not been studied. Thorough complexity analysis (in terms of number of computation, memory access etc.) compared to current BVSP approach and traditional MC is required.

# Core Experiment Conditions

## Software

Experiments will use the HTM 7.0 software, as recommended in JCT3V-D1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences

Test sequences will be used according to common test conditions (CTC) [1].

## Coding Conditions

The experiments are conducted under CTC [1].

## Evaluation Criteria

Evaluation will be done according to CTC [1].

  - Coding Performance Measurements: PSNR values shall be computed for the decoded texture views relative to original texture views and for the synthesized views relative to the synthesized views based on uncompressed texture and depth. The 4-point BD-rate according to common test conditions is used to report the overall simulation results.

  - Complexity measurements: For the complexity measurement, the reference software and the reference software with the proposed method implemented will be executed on the same machine with the same configuration and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.

Thorough complexity analysis (in terms of number of computation, memory access, memory usage, etc) compared to the HTM v7.0 (and traditional MC) shall also be reported. EXCEL sheet will be provided.

## Timeline

2013/05/03		Finalize CE description
2013/07/12		Make source code and simulation results available for all CE1 participants

2013/07/20		Register documents for the JCT-3V meeting

2013/07/20		Upload simulation and cross check results to the JCT-3V document repository

2013/07/27		Start the 5th JCT-3V meeting

# Reference

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG JCT3V-D1100, Incheon, KR, 2013.