| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D1102 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): Disparity Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA Sehoon Yea | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com +82-10-3968-0602 sehoon.yea@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment (CE) 2 on disparity vector derivation to be performed for the 5th JCT-3V meeting.

# Introduction

The goal of this CE is to further investigate the methods for disparity vector derivation in both ATM and HTM at the 4th JCT-3V meeting. Tools under test will be evaluated according to their impact on compression efficiency and implementation complexity.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| ETRI | Gun Bang | gbang@etri.re.kr | C |
| KHU | Kyung Yong Kim | kimky@khu.ac.kr | C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com | P |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | P |
| LG | Sehoon Yea | sehoon.yea@lge.com | P/C |
| Qualcomm | Jewon Kang Li Zhang | jewonk@qti.qualcomm.com lizhang@qualcomm.com | P/C |
| INRIA | Laurent Guillo | Laurent.guillo@inria.fr |  |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp |  |
| MERL | Dong Tian | tian@merl.com |  |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | C |

# Tools under test

## CE2.a: Disparity Vector Derivation in ATM

### JCT3V-D0108: Simplified disparity vector derivation for inter-view MVP

In current 3D-AVC, the disparity vector used for the inter-view MVP derivation in Skip mode and Direct mode is derived from the motion vectors of the neighboring blocks and from the disparity vector converted from the maximum of four corner depth samples in the associated depth block. This contribution proposes to simplify the disparity vector derivation by directly deriving the disparity from the maximum of four corner depth samples. With this simplification, the complexity of the inter-view candidate derivation and memory access for the motion data of the neighboring blocks are reduced.

## CE2.h: Disparity Vector Derivation in HTM

### JCT3V-D0194: Derived disparity vector for 3D-HEVC

In 3D-HEVC, the NBDV process checks the neighboring blocks for not only disparity motion vectors but also implicit disparity vectors (IDVs), generated and stored during the NBDV processes for the neighboring blocks. It is considered that the IDVs may slightly increase the storage. Therefore, in this proposal, IDV is simplified by using the so-called derived disparity vector (DDV).

### JCT3V-D0113: Simplification of disparity vector derivation

A disparity vector is searched by only using the coded depth map of the reference view, but not searching spatial or temporal neighbouring blocks. The proposed method is comprised of two steps. Firstly, the first disparity vector is derived from the corresponding depth block pointed at by the initial disparity vector corresponding to the depth value of (1 << (bit depth – 1)). And then, the final disparity vector is derived from the corresponding depth block pointed at by the first disparity vector.

The proposal suggests to remove the NBDV process while relying on multiple iterations of DV refinement via the DoNBDV step.

# Mandates

Mandates for the CE are as follows:

- To study the simplification of disparity vector dirivation in 3D-AVC, as described in JCT3V-D0108.

- To study the simplifications of disparity vector derivation process as proposed in JCT3V-D0194 and JCT3V-D0113 in 3D-HEVC.

- To study the coding efficiency impact of disparity vector derivation proposals.

# Software, Configuration and Evaluation

## Software

Experiments in CE2.a will use the ATM 8.0 software and experiments in CE2.h will use the HTM 7.0 software, as recommended in JCT3V-D1100.

Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT3V-D1100. Moreover, proponents and cross checkers are required to provide simulation results for the random access configuration as specified in JCT3V-D1100.

## Evaluation of CE Results

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment.

- Coding performance measurements: Measure impact on bitrate/PSNR. PSNR shall be calculated for the decoded texture views, relative to original texture views and for the synthesized views relative to uncompressed synthesized views. Use 4-point BD-PSNR and BD-Rate according to common conditions. The anchors will be generated according to common test conditions. Results on view synthesis prediction off case should also be provided for proposals on 3D-HEVC.

- Complexity measurements: Measure impact on encoding and decoding runtimes for all test cases relative to the unmodified test model software.

# Timelines

2013/05/03		Finalize CE description

2013/05/17		Release 3D-ATM 8.0

2013/05/??		Release 3D-HTM 7.0

2013/07/14		Make source code, simulation results, and draft text available for ALL CE2 participants, not only for the cross-checks.

2013/07/19		Register documents for the 5th JCT-3V meeting

2013/07/19		Upload simulation and cross check results to the JCT-3V document repository

2013/07/19		Upload contributions to JCT-3V 5th meeting.