| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D1103 |
| --- | --- |

| Title: | Description of Core Experiment 3 (CE3) on Inter-view/motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Sehoon Yea | Tel: Email: Tel: Email: | +886-3-5670766 ext. 25314 yulin.chang@mediatek.com +82-10-3968-0602 sehoon.yea@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment 3 on motion/mode parameter prediction to be performed for the 4th JCT-3V meeting.

# Introduction

The goal of this Core Experiment (CE) is to investigate the methods for motion/mode parameter prediction proposed at the 4th JCT-3V meeting. Tools under test will be evaluated according to their impact on compression efficiency and implementation complexity

# Participants

## CE3

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang Jewon Kang | lizhang@qualcomm.com jewonk@qti.qualcomm.com | P/C |
| INRIA | Thomas Guionnet Laurent Guillo Christine Guillemot | Thomas.Guionnet@inria.fr Laurent.Guillo@inria.fr Christine.Guillemot@inria.fr | P/C |
| LG | Taesup Kim Jin Heo Moonmo Koo Sehoon Yea | taesup.kim@lge.com jin78.heo@lge.com moonmo.koo@lge.com sehoon.yea@lge.com | P/C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yuwen.huang@mediatek.com | P/C |
| Sharp | Tadashi Uchiumi Tomohiro Ikai | uchiumi.tadashi@sharp.co.jp ikai.tomohiro@sharp.co.jp | C |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | C |
| Orange Labs | J. Jung | joelb.jung@orange.com | C |
| MERL | Dong Tian | tian@merl.com | C |
| Hisilicon | Yongbing Lin | yblin@huawei.com | C |
| Panasonic | Steffen Kamp | Steffen.Kamp@eu.panasonic.com | - |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | C |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | - |

# Tools under Test

## CE3

### Merge List Construction

JCT3V-D0109 3D-CE5.h related: Additional inter-view merging candidate [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

In this contribution, an additional inter-view merging candidate is proposed to be added in the merging candidate list for the texture coding in a dependent view. For the pruning process, the additional inter-view candidate is only compared with the original inter-view candidate for redundancy removal.

JCT3V-D0117 3D-CE1.h related: Simplification of VSP insertion process [J. Y. Lee, M. W. Park, B. Choi, C. Kim (Samsung)]

View synthesis prediction (VSP) was integrated on the current 3D-HTM (JCT3V-C0152). VSP is used as a new candidate in Skip and Merge modes and inserted right before a temporal candidate. However, if any of the adjacent blocks is coded with VSP, it is inserted as the candidate for the block. In the insertion process for a candidate from a potentially VSP block, the current process investigates all the neighboring blocks whether they are coded with VSP or not. In this contribution, three methods are introduced for simplification of the current VSP insertion process. Especially, the proposed VSP insertion process only based on the left and above blocks, such as the VSP skip flag signalling in 3D-AVC.

JCT3V-D0178 CE5.h: Merge candidates derivation from vector shifting [V. Thirumalai, L. Zhang, Y. Chen, M. Karczewicz (Qualcomm), T. Guionnet, C. Guillemot, L. Guillo (INRIA)]

As a harmonized follow-up proposal of JCT3V-C0045 and JCT3V-C0148, it is proposed to add two more candidates. Each candidate is derived with a horizontally shifted disparity vector being used for inter-view motion prediction and if it doesn’t provide an available candidate, then it is derived from a horizontally shifted spatial neighbouring candidate, if it contains a disparity motion vector.

 The coding performance and complexity such as additional computations and memory access of JCT3V-D0109, JCT3V-D0117, and JCT3V-D0178 will be tested and studied

 The interaction between the VSP merge candidate in JCT3V-C0152 and these two proposals will be studied. Additional data with disabling VSP will be provided.

### Interview SAO

JCT3V-D0091 CE5.h related : Inter-view SAO process in 3DV coding [T. Kim, J. Heo, S. Yea (LG)]

In this contribution, SAO process in the 3D HEVC (HTM 6.0) is migrated to that in the current HEVC (HM 9.1). The main modification is that SAO parameters are derived from LCU-based optimization (interleaving mode) rather than picture-based optimization (APS mode). Inter-view SAO process is proposed and modeled on the top of it to confirm the benefit of sharing base view’s SAO parameters on the dependent views through RD-optimization.

- Further investigate interview SAO as well as disabling SAO in dependent views from JCT3V-D0058 in CE.

# Mandates

## CE3

The mandates of this core experiment are as follows:

- To further investigate ways to improve merge list construction and the interaction between VSP merge candidate [JCT3V-D0109, JCT3V-D0117, JCT3V-D0178]

- To further investigate “interview SAO” as well as disabling SAO in dependent views [JCT3V-D0091]

# Software, Configuration and Evaluation

## Software

### CE3

Experiments in CE3 will use the 3D-HTM 7.0 software that is recommended in JCT3V-D1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT3V-D1100. Moreover, proponents and cross checkers are required to provide simulation results for the Random access configuration as specified in JCT3V-D1100.

## Evaluation of CE Results

- Measure impact on bitrate/PSNR. PSNR shall be calculated for the decoded texture views, relative to original texture views and for the synthesized views relative to uncompressed synthesized views. Use 4-point BD-PSNR and BD-Rate according to common conditions. The anchors will be generated according to common test conditions.

- Measure impact on encoding and decoding runtimes for all test cases relative to the unmodified test model software.

- Report comments of subjective visual quality according to common conditions.

# Timelines

## CE3

2013/05/03		Finalize CE description

2013/05/17		Release 3D-ATM 8.0

2013/05/??		Release 3D-HTM 7.0

2013/07/12		Make source code, simulation results, and draft text available for ALL CE3 participants, not only for the cross-checks.

2013/07/20		Register documents for the 5th JCT-3V meeting

2013/07/20		Upload simulation and cross check results to the JCT-3V document repository

2013/07/27		Start the 5th JCT-3V meeting

# References

- D. Rusanovksyy, A. Vetro, and K. Müller, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, JCT3V-D1100, Incheon, Apr., 2013.