| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D1105 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5) on Depth Bi-partition Modes |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Philipp Merkle | Email: | philipp.merkle@hhi.fraunhofer.de |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This document describes Core Experiment 5 (CE5) on depth bi-partition modes to be performed for the 5th JCT-3V meeting.

# Introduction

The goal of this Core Experiment (CE) is to investigate simplifications of the depth bi-partition modes in 3D-HEVC under sub-experiment CE5.H as proposed to the 4th JCT-3V meeting. Bi-partition modes are part of the depth intra coding tools DMM (depth modeling modes) and RBC (region boundary chain). The corresponding test model, including DMM and RBC, is described in JCT3V-D1005.

The objective of tools under test in CE5.H is evaluation of the performance and possible simplification of the existing DMM and RBC coding modes.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| HHI | P. Merkle | philipp.merkle@hhi.fraunhofer.de | X | X |
| LGE | J. Heo H. Liu | jin78.heo@lge.com hongbin.liu@lge.com | X | X |
| Samsung | J.-Y. Lee M. W. Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | X | X |
| GIST | Y. Song Y.-S. Ho | ysong@gist.ac.kr hoyo@gist.ac.kr | X | X |
| Qualcomm | X. Zhao | xinzhao@qti.qualcomm.com | X | X |
| SCU | Z. Gu | zgu@scu.edu |  | X |
| Hisilicon | X. Zheng | zhengxiaozhen@huawei.com |  | X |
| Zhejiang U. | P. Lu | yiqiu8932@gmail.com |  | X |
| MediaTek | Y.-L. Chang J.-L. Lin Y.-W. Huang | yulin.chang@mediatek.com jl.lin@mediatek.com yuwen.huang@mediatek.com |  | X |

# Tools Tests for Sub-experiment CE5.H

## Test 1: Performance of DMM and RBC depth intra modes

In this tool test the performance of the five depth bi-partition modes, referred to as DMM1~4 and RBC, is investigated.

Description:

Related proposals indicated that the coding performance differs for individual depth bi-partition modes. Therefore, this tool test investigates the performance of individual modes in a structured way, i.e. evaluating the impact of each single mode on RD performance and complexity.

Tests:

In this tool test the following configurations of DMM and RBC will be evaluated:

| Test | DMM1 | DMM2 | DMM3 | DMM4 | RBC | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1-A1 | off | on | on | on | on | disable mode via macro |
| 1-A2 | on | off | on | on | on | disable mode via macro |
| 1-A3 | on | on | off | on | on | disable mode via macro |
| 1-A4 | on | on | on | off | on | disable mode via macro |
| 1-B | on | on | on | on | off | disable mode via macro |
| 1-C | off | off | off | off | off | disable DMM via config parameter and RBC via macro |

Participants:

HHI (Test1-A,B,C)

## Test 2: Simplification of DMM and RBC depth intra modes

In this tool test simplifications to the five depth bi-partition modes of DMM and RBC are investigated according to proposals JCT3V-D0067, JCT3V-D0123, and JCT3V-D0184.

Description:

JCT3V-D0067: 	remove part of DMM3, that overlaps with DMM1, which is the case when the CTLB is not intra coded with mode 2~34.

JCT3V-D0184: 	modification of JCT3V-D0067 for DMM3, modification of signaling as intra mode plus Wedgelet sub-list index for DMM1 (including SDC_DMM1) and optionally remove DMM2.

JCT3V-D0123: 	remove inter-component dependency of DMM by disabling DMM3 and DMM4 (processing and signaling), or use only DMM1 for depth intra coding.

Tests:

In this tool test the following modifications to DMM and RBC will be evaluated:

| Test | DMM1 | DMM2 | DMM3 | DMM4 | RBC | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 2-A | on | on | D0067 | on | on |  |
| 2-B1 | D0184 | on | D0067 | on | on |  |
| 2-B2 | D0184 | off | D0067 | on | on |  |
| 2-C1 | on | on | off | off | on | adapted signaling |
| 2-C2 | on | off | off | off | on | adapted signaling |
| 2-D | on | off | off | off | off | adapted signaling |

Participants:

LG (Test2-A)
Qualcomm (Test2-B)
Samsung (Test2-C,D)

# Core Experiment Conditions

## Software

Sub-experiment CE5.H will use the 3D-HTM-7.0 software that is recommended in JCT3V-D1100. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

Participants of CE5.H are encouraged to provide additional results using HM-10 based 3D-HTM-X software.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT3V-C1100. Moreover, proponents and cross checkers are required to provide simulation results for the following two configurations:

- Random access configuration as specified in JCT3V-C1100.

- All-intra configuration (configuration files to be provided by CE coordinator).

## Evaluation of CE Results

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are considered to be used in this core experiment. A corresponding Excel sheet for reporting the simulation results will be provided by the CE coordinator.

Coding Performance Measurements: PSNR values shall be computed for the decoded texture views relative to original texture views and for the synthesized views relative to the synthesized views based on uncompressed texture and depth. The 4-point BD-rate according to common test conditions is used to report the results for random access as well as all-intra configuration.

Complexity measurements: For the complexity measurement, the reference software and the reference software with the proposed method implemented will be executed on the same machine with the same configuration and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.

## Timeline

2013/05/03		Final description of CE and proposals available
2013/05/24		Reference software 3D-HTM-7.0 available 
2013/07/10		Make source code, simulation results and draft text available for all proponents and cross-					checks
2013/07/19		Document registration and upload deadline (register documents and upload simulation 					and cross check results for CE and related proposals)
2013/07/26		CE summary report available