| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, Austria, July 27 – August 02, 2013 | Document: JCT3V-E0014 |
| --- | --- |

| Title: | JCT-3V AHG Report: Mixed Resolution Coding (AHG14) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Krzysztof Wegner (Poznan), Shinya Shimizu (NTT) | Email: | kwegner@multimedia.edu.pl shimizu.shinya@lab.ntt.co.jp |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-3V ad hoc group on Mixed Resolution Coding (AHG14) between the 4th JCT-3V meeting in Incheon (20–26 April, 2013) and 5th JCT-3V meeting in Vienna (July 27 – August 02, 2013).

# Mandates

•	Investigate possible benefits of mixed resolution coding in 3D-HEVC.

•	Identify commonality of reduced resolution depth coding in MVC plus depth, 3D-ATM and 3D-HTM.

•	Study approaches and design implications to support coding of mixed resolution data (texture and depth), including asymmetric texture coding.

•	Study implications of mixed resolution coding with regard to complexity and memory requirements of 3D-ATM and 3D-HTM.

The email reflector for AHG14 is jct-3v@lists.rwth-aachen.de.

# Related contributions

The following contributions are related to AHG14:

- JCT3V-E0269: Krzysztof Wegner, Olgierd Stankiewicz, Marek Domański, “Optimized QP/QD curve for 3D coding with half and full resolution depth maps”

# Report

During the Incheon JCT-3V meeting it has been noted, that comparisons of full resolution and half resolution depth maps coding require common reference in order to obtain meaningful results. At that time, the comparison has been performed under CTC conditions. Especially, quantization parameter for texture views and depth views were equal. Such condition is unfair when comparing coding with half and full-resolution of depth maps because codec don’t work at optimal operation point. Therefore optimized quantization parameters for both texture views and depth view that maximize coding performance of 3D-ATM for both full- and half-resolution depth maps coding separately have been provided.

In such a condition average bitrate bjontegard delta (BD-rate) between half- and full-resolution depth maps coding is -6.58% for HP and -7.93 for EHP on synthesized views.

Regarding the first mandate, mixed resolution coding in 3D-HEVC, there are many efforts as offline works although there is no input contribution at this meeting. Such offline works include migration of mixed resolution coding proposed in JCT3V-D0215 and JCT3V-D0216 to HTMv7.0r1. The simulation results are almost the same as those reported at the last meeting; quarter-resolution depth map coding shows BD rate increase of 3.1% with 30% decoding runtime reduction and quarter-resolution dependent view coding results in BD rate increase of 13.3% with 13% decoding runtime reduction.

It is also noted that the HTMv7.0r1-based implementation was provided to the proponents who required the software.

# Recommendations

The AHG on Mixed Resolution Coding recommends to:

- Review the related input contributions

- Revise Common Test Conditions for 3D-ATM and final test conditions for the 3D-AVC subjective test plan

- Re-establish the AHG for further study and experiments in this area