| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0021 |
| --- | --- |

| Title: | CE1: Summary report on View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This document is the summary report of Core Experiment 1 (CE1) on view synthesis prediction. This Core Experiment investigates various aspects of view synthesis prediction. Experiments are related to 3D-HTM. There are 3 CE1 proposal and 5 CE1 related proposals.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | P/C |
| MERL | Dong Tian | tian@merl.com |  |
| MediaTek | Yu-Lin Chang Yi-Wen Chen Yu-Wen Huang | yulin.chang@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com | P/C |
| Qualcomm | Xin Zhao Jewon Kang Li Zhang | xinzhao@qti.qualcomm.com jewonk@qti.qualcomm.com lizhang@qti.qualcomm.com |  |
| Nokia | Dmytro Rusanovskyy | Dmytro.Rusanovskyy@nokia.com |  |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | P/C |
| LG Electronics | Sehoon Yea | sehoon.yea@lge.com | P |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | P/C |
| ETRI | Gun Bang | gbang@etri.re.kr |  |
| KyungHee University | Kyung Yong Kim | kimky@khu.ac.kr |  |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp |  |
| KDDI | Kei Kawamura | kei@kddilabs.jp |  |
| GIST | Yunseok Song Yo-Sung Ho | ysong@gist.ac.kr hoyo@gist.ac.kr |  |
| HHI | Heribert Brust | heribert.brust@hhi.fraunhofer.de |  |
| NCKU | Chun-Fu Chen | n28991146@mail.ncku.edu.tw | P/C |

(P = proponent, C = cross checker)

# Tools under Investigation

The tools to be investigated in this Core Experiment use the decoded texture and depth view components of the already coded views to compute a prediction signal for the texture and depth view components of the coding target views. This process is done by means of disparity compensation, where the disparity values are derived from the decoded depth information.

## CE1.h: View Synthesis Prediction for 3D-HTM

A block-based VSP using a backward warping was adopted into 3D-HTM at Geneva meeting, which is done with a two-step disparity compensation. In the first step, a virtual depth map is fetched for a CU by calling the NBDV function without the depth-based refinement. In the second step, VSP compensation is performed on a 4x4 block basis and prediction samples are generated. For VSP compensation, a disparity vector is derived from the derived virtual depth map, where a maximum depth values among those of 4 corners samples is selected. VSP is signaled by the merge mode.

A total of 8 input contributions are submitted, among which there are 3 CE proposals and 5 CE-related proposals.

In the scope of this CE for 3D-HTM, the following three items are investigated.

- Block size for the second disparity compensation

  - JCT3V-E0128: 16x16, 16x8, 8x16, 8x8, 8x4, or 4x8 (using PU size, DLT flag, and virtual depth map)

  - JCT3V-E0207: 8x4 or 4x8 (using virtual depth map)

- Forward block-based VSP

  - JCT3V- E0205

The related proposals can be roughly classified into the following 2 categories.

- Disparity vector on fetching depth map

  - JCT3V-E0140: DoNBDV of current or neighbor CU

  - JCT3V-E0171: NBDV of the current CU (for inherited VSP)

- Simplification

  - JCT3V-E0171: Disallowing to inherit VSP modes from upper CTU row

  - JCT3V-E0208: Same disparity vector clipping with inter prediction

- Others (for coding efficiency improvement)

  - JCT3V-E0188: Ordering Constraint

  - JCT3V-E0246: VSP combined with motion compensated prediction

### List of input documents

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| NCKU, ASTRI | JCT3V-E0128 | CE1.h: Adaptive Virtual Depth Block Partition for View Synthesis Prediction and Complexity Analysis | Proposal |
| NCKU | JCT3V-E0129 | CE1.h: crosscheck on NTT's proposal (JCT3V-E0207) | Crosscheck |
| Samsung | JCT3V-E0140 | 3D-CE1.h related: Disparity vector used for BVSP | Proposal |
| Samsung | JCT3V-E0151 | 3D-CE1.h related: Cross check of Simplified view synthesis prediction (JCT3V-E00171) | Crosscheck |
| MediaTek, PKU, HIT | JCT3V-E0171 | 3D-CE1.h related : Simplified view synthesis prediction | Proposal |
| LG | JCT3V-E0188 | 3D-CE1.h Related: Ordering Constraint on View Synthesis Prediction | Proposal |
| Zhejiang University | JCT3V-E0205 | CE1.h: Forward Block-based View Synthesis Prediction | Proposal |
| NTT | JCT3V-E0207 | 3D-CE1.h: Adaptive block partitioning for VSP | Proposal |
| NTT | JCT3V-E0208 | 3D-CE1.h-related: Clipping operations in VSP | Proposal |
| NTT | JCT3V-E0215 | 3D-CE1.h: Cross-check on Adaptive Virtual Depth Block Partition for View Synthesis Prediction (JCT3V-E0128) | Crosscheck |
| NTT | JCT3V-E0216 | 3D-CE1: Cross-check on Forward Block-based View Synthesis Prediction (JCT3V-E0205) | Crosscheck |
| NTT | JCT3V-E0217 | 3D-CE1.h-related: Cross-check on Disparity vector used for BVSP (JCT3V-E0140) | Crosscheck |
| NTT | JCT3V-E0246 | 3D-CE1.h related: Improvement of View Synthesis Prediction | Proposal |
| MediaTek | JCT3V-E0275 | 3D-CE1: Cross-check on Clipping operations in VSP (JCT3V-E0208) | Crosscheck |
| Samsung | JCT3V-E0286 | 3D-CE1 related: Cross check of depth consistency check for view synthesis prediction (JCT3V-E0188) | Crosscheck |

### Summary

#### Test (a): block size for the second disparity compensation

| Doc. | Possible block size | How to decide | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | # of storage access (average) | # of operations (worst) | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E0128 | 16x16, 16x8, 8x16, 8x8, 8x4, 4x8 | PU size, DLT flag, Depth map | 0.05% | 0.03% | 35.99% | 99.47% | E0215 |
|  | 8x8, 8x4, 4x8 | depth map | 0.03% | 0.02% | 43.21% | 99.47% |  |
| E0207 | 8x4, 4x8 | depth map | 0.04% | 0.03% | 51.18% | 98.42% | E0129 |

#### Test (b): Forward block-based VSP

| Doc. | Constraint on disparity | BD-Rate (Video PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- |
| E0205 | N | -0.4% | -0.3% |  | 99.6% |  |
|  | Y | -0.2% | -0.2% |  | 98.2% | E0216 |

#### [Related] Category (i): Disparity vector on fetching depth map

|  | DV for non-inherited VSP | DV for inherited VSP | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HTM | NBDV of current CU | NBDV of neighbor |  |  |  |  |  |
| E0140 | DoNBDV of current CU | DoNBDV of neighbor | -0.07% | -0.10% | 99.9% | 97.0% | E0217 |
| E0171 | NBDV of current CU | NBDV of current CU | 0.05% | 0.03% | 99.2% | 96.9% | E0151 |

#### [Related] Category (ii): Simplification

|  | Proposal | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosschecked |
| --- | --- | --- | --- | --- | --- | --- |
| E0171 | Disallow inheriting VSP modes from upper CTU row | 0.04% | 0.00% | 99.5% | 97.9% | E0151 |
| E0208 | Bug fix of clipping on sub-PU level DCP | -0.01% | -0.01% | 99.0% | 99.5% | E0275 |
|  | Same vector clipping with inter predictions on fetching depth map | 0.01% | -0.02% | 99.3% | 100.7% |  |
|  | Both of above two | -0.01% | -0.02% | 99.1% | 99.6% |  |
|  | Combination with E0209 (Apply same clipping to DoNBDV) | 0.00% | -0.02% | 99.1% | 100.1% | E0206 E0280 |

#### [Related] Category (iii): Others (for coding efficiency improvement)

  - JCT3V-E0188: Ordering Constraint on View Synthesis Prediction

- Ordering constraint among disparity compensated sub-PUs

|  | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosscheck |
| --- | --- | --- | --- | --- | --- |
| Proposal | -0.08% | -0.09% | 100.1% | 100.4% | E0286 |

  - JCT3V- E0246: Improvement of View Synthesis Prediction

- Perform bi-prediction with motion compensated prediction with MVs coded at the corresponding block in the reference view

  - MVs are derived for each sub-PU by the same way with the inter-view motion prediction

|  | BD-Rate (Coded PSNR) | BD-Rate (Synthesis PSNR) | Encoder runtime | Decoder runtime | Crosscheck |
| --- | --- | --- | --- | --- | --- |
| Proposal | -0.06% | -0.01% | 101.8% | 99.7% | No |

# Recommendations

- To review the relevant input contributions

- ...