| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0023 |
| --- | --- |

| Title: | 3D-CE3: Summary report on inter-view/motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Sehoon Yea | Tel: Email: Tel: Email: | +886-3-5670766 ext. 25314 yulin.chang@mediatek.com +82-10-3968-0602 sehoon.yea@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

A summary of proposals and cross-check results of CE3 is reported.

# Introduction

A summary of proposals and cross-check results of CE3 is reported.

## Mandates

# Mandates

## CE3

The mandates of this core experiment are as follows:

- To further investigate ways to improve merge list construction and the interaction between VSP merge candidate [JCT3V-D0109, JCT3V-D0117, JCT3V-D0178]

- To further investigate “interview SAO” as well as disabling SAO in dependent views [JCT3V-D0091]

## Participants

Participants with proposals at this meeting in CE3 are as follows:

## CE3

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang Jewon Kang | lizhang@qualcomm.com jewonk@qti.qualcomm.com | P/C |
| INRIA | Thomas Guionnet Laurent Guillo Christine Guillemot | Thomas.Guionnet@inria.fr Laurent.Guillo@inria.fr Christine.Guillemot@inria.fr | P/C |
| LG | Taesup Kim Jin Heo Moonmo Koo Sehoon Yea | taesup.kim@lge.com jin78.heo@lge.com moonmo.koo@lge.com sehoon.yea@lge.com | P/C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yuwen.huang@mediatek.com | P/C |
| Sharp | Tadashi Uchiumi Tomohiro Ikai | uchiumi.tadashi@sharp.co.jp ikai.tomohiro@sharp.co.jp | C |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | C |
| Orange Labs | J. Jung | joelb.jung@orange.com | C |
| MERL | Dong Tian | tian@merl.com | C |
| Hisilicon | Yongbing Lin | yblin@huawei.com | C |
| Panasonic | Steffen Kamp | Steffen.Kamp@eu.panasonic.com | - |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | C |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | - |
| ETRI | Gun Bang | gbang@etri.re.kr | P |
| KHU | Young Su Heo Kyung Yong Kim | medist@khu.ac.kr kimky@khu.ac.kr | P |
| RWTH Aachen | Fabian Jäger | jaeger@ient.rwth-aachen.de | P |
| Huawei | Jacek Konieczny | jacek.konieczny@huawei.com | P |

# List of input documents

## CE3

A total of 38 input contributions were gathered in this category, among which were 20 proposals. Among the 20 proposals, three of the non-CE contributions relating to residual prediction were also collected for reference. The input contributions can be roughly categorized as follows:

- AMVP/Merge List Construction

  - Merge candidate derivation – E0126(Qualcomm, INRIA, and MediaTek, CE proposal), E0213(ETRI and KHU), E0145(Samsung), E0187(LGE)

  - AMVP candidate list construction – E0189(LGE)

- Residual Prediction

  - Advanced residual prediction(ARP) – E0044(Sharp), E0124(Qualcomm and PKU), E0144(Samsung), E0169(MediaTek), E0185(MediaTek)

  - Disparity vector for ARP – E0175(MediaTek)

- Interview motion prediction

  - Sub-PU level interview motion prediction – E0184(MediaTek)

  - Simplified interview merge candidate – E0186(MediaTek)

  - Disparity vector for interview merge candidate – E0214(KHU)

- Depth prediction

  - Disparity derived depth coding – E0174(MediaTek)

  - Texture merging candidate – E0182(MediaTek), E0183(MediaTek)

- Others

  - Depth-based block partitioning – E0118(RWTH Aachen and Huawei)

  - Motion data buffer reduction – E0170(MediaTek)

  - Faster encoder decision – E0173(MediaTek, PKU, and HIT)

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| Sharp | JCT3V-E0044 | ARP simplification | Proposal |
| MERL | JCT3V-E0073 | CE3.h: Crosscheck on MediaTek Proposal JCT3V-E0184 | Cross-Check |
| RWTH Aachen and Huawei | JCT3V-E0118 | CE3-related: Depth-based Block Partitioning | Proposal |
| Qualcomm and PKU | JCT3V-E0124 | Further improvements on advanced residual prediction | Proposal |
| Qualcomm, INRIA, and MediaTek | JCT3V-E0126 | CE3.h: Merge candidates derivation from vector shifting | Proposal |
| Samsung | JCT3V-E0144 | Simplification and improvement of ARP for 3D-HEVC | Proposal |
| Samsung | JCT3V-E0145 | 3D-CE3 related: VSP and temporal candidates in Merge list construction | Proposal |
| Samsung | JCT3V-E0149 | 3D-CE3.h related: Cross check of On Complexity reduction of bi-prediction for advanced residual prediction (JCT3V-E0169) | Cross-Check |
| MediaTek | JCT3V-E0169 | 3D-CE3 related: on complexity reduction of bi-prediction for advanced residual prediction | Proposal |
| MediaTek | JCT3V-E0170 | 3D-CE3 related: motion data buffer reduction for 3D-HEVC | Proposal |
| MediaTek, PKU, and HIT | JCT3V-E0173 | 3D-CE3.h related: Fast encoder decision for texture coding | Proposal |
| MediaTek | JCT3V-E0174 | 3D-CE3.h related: A disparity derived depth coding method in 3D-HEVC | Proposal |
| MediaTek | JCT3V-E0175 | 3D-CE3.h related: An adaptive disparity vector derivation method for ARP in 3D-HEVC | Proposal |
| MediaTek | JCT3V-E0182 | 3D-CE3.h related: A bug-fix for the texture merging candidate in 3D-HEVC | Proposal |
| MediaTek | JCT3V-E0183 | 3D-CE3.h related: An additional texture merging candidate in 3D-HEVC | Proposal |
| MediaTek | JCT3V-E0184 | 3D-CE3.h related: Sub-PU level inter-view motion prediction | Proposal |
| MediaTek | JCT3V-E0185 | 3D-CE3.h related: Advanced temporal residual prediction | Proposal |
| MediaTek | JCT3V-E0186 | 3D-CE3.h related: Simplified inter-view merge candidate by reducing checking of reference lists | Proposal |
| LGE | JCT3V-E0187 | 3D-CE3.h Related: Improvement on Merge Candidate List Construction | Proposal |
| LGE | JCT3V-E0189 | CE3.h related : AMVP candidate list construction for DCP blocks | Proposal |
| LGE | JCT3V-E0197 | 3D-CE3.h related: Cross-check on an adaptive disparity vector derivation method for ARP in 3D-HEVC (JCT3V-E0175) | Cross-Check |
| ETRI and KHU | JCT3V-E0213 | 3D-CE3.h related: Simple merge candidate list construction for 3DV | Proposal |
| KHU | JCT3V-E0214 | 3D-CE3.h related: CTU-aligned DV for a temporal inter-view motion vector candidate | Proposal |
| NTT | JCT3V-E0219 | 3D-CE3.h-related: Cross-check on AMVP candidate list construction for DCP blocks (JCT3V-E0189) | Cross-Check |
| MediaTek | JCT3V-E0228 | Crosscheck of JCT3V-E0044 on ARP simplification | Cross-Check |
| MediaTek | JCT3V-E0244 | Crosscheck of improvement on ARP proposed by Qualcomm | Cross-Check |
| Sharp | JCT3V-E0252 | CE3.h: cross-check of Merge candidates derivation from vector shifting (JCT3V-E0126) | Cross-Check |
| Sharp | JCT3V-E0253 | 3D-CE3.h related: cross-check of Simplified inter-view merge candidate by reducing checking of reference lists (JCT3V-E0186) | Cross-Check |
| NTT | JCT3V-E0259 | 3D-CE3.h related: Cross-check on motion data buffer reduction for 3D-HEVC (JCT3V-E0170) | Cross-Check |
| Gent University | JCT3V-E0261 | CE3-related: Cross check of Depth-based Block Partitioning (JCT3V-E0118) | Cross-Check |
| Qualcomm | JCT3V-E0271 | 3D-CE3.h related: Crosscheck on Advanced temporal residual prediction (JCT3V-E0185) | Cross-Check |
| MediaTek | JCT3V-E0274 | Cross-check on simplification and improvement of ARP for 3D-HEVC (JCT3V-E0144) | Cross-Check |
| LGE | JCT3V-E0278 | CE3.h related: Cross-check on simple merge candidate list construction for 3DV(JCT3V-E0213) | Cross-Check |
| Qualcomm | JCT3V-E0283 | CE3.h related: Crosscheck on Improvement on Merge Candidate List Construction (JCT3V-E0187) | Cross-Check |
| NTT | JCT3V-E0287 | 3D-CE3.h related: Cross-chekc on fast encoder decision for texture coding (JCT3V-E0173) | Cross-Check |
| HHI | JCT3V-E0290 | Cross check of Further improvements on advanced residual prediction (JCT3V-E0124) | Cross-Check |
| Qualcomm | JCT3V-E0291 | 3D-CE3.h related: Crosscheck on An additional texture merging candidate in 3D-HEVC (JCT3V-E0183) | Cross-Check |
| KWU | JCT3V-E0296 | 3D-CE3.h related: Cross-check result on the CTU-aligned DV for temporal inter-view motion vector candidate from KHU | Cross-Check |

# Summary of proposals & results

## CE3 results

### Merge List Construction

JCT3V-E0126 is the CE3 results for merge list construction.

- JCT3V-E0126 [Qualcomm, INRIA, and MediaTek] CE3.h: Merge candidates derivation from vector shifting (Cross-Checked by JCT3V-E0252 [Sharp])

This proposal is a result of harmonization of techniques in JCT3V-D0178 and JCT3V-D0109. In this joint contribution, it is proposed to add only one additional candidate in the merge candidate list. The additional candidate is firstly generated by shifting the disparity vector by considering the size of the current PU and utilizing the motion information of an identified block in the reference view, as it is done in JCT3V-D0109. Furthermore, similar to JCT3V-D0178, if the block doesn’t provide an available candidate, the candidate is derived by shifting horizontally a disparity motion vector. Compared to the current HTM design, the proposed method achieves a compression efficiency gain of 0.2% in terms of BD rate for both BVSP on and off cases, respectively.

#### Coding results of the CE contribution E0126

| E0126 | Video 1 | Video 2 | Video only | Synthesized only | Coded & Synthesized | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BVSP on | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | 100.3% | 102.6% |
| BVSP off | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | 102.5% | 100.8% |

## CE3 related

### AMVP/Merge Candiate List Construction

- JCT3V-E0213 [ETRI and KHU] 3D-CE3.h related: Simple merge candidate list construction for 3DV (Cross-Checked by JCT3V-E0278 [LGE])

This contribution proposes a simplified method for constructing MCL (Merge Candidate List) for dependent view or depth. The proposed method is a simple and low-complexity way to implement the MCL construction module for the 3D-HEVC by re-using the existing MCL construction module of the HEVC. Simulation results show that the proposed algorithm only brings marginally-small loss(0.19% and 0.2% for video and synthesized view over total bitrate) and on the coding performance while providing the MCL construction process for 3D-HEVC much simpler and also decreasing hardware implementation complexity significantly.

- JCT3V-E0145 [Samsung] 3D-CE3 related: VSP and temporal candidates in Merge list construction (Cross-Check by JCT3V-E0304 [MediaTek])

This proposal introduces three methods to guarantee the VSP and temporal candidates in the Merge mode when VSP is enabled. The first proposed method selects only five candidates among the inter-view motion vector, spatial, disparity, and VSP candidates and chooses one temporal candidate. In this case, the VSP candidate cannot be guaranteed but the temporal candidate is always added in the Merge mode. The second method chooses four candidates among the inter-view motion vector, spatial, and disparity candidates and then selects one VSP candidate and one temporal candidate. The third method increases the total number of the Merge candidates to seven. Then, both VSP and temporal candidates can be guaranteed in the Merge mode. The proposed methods achieve compression efficiency gain of 0.0%, 0.0%, 0.1% in terms of BD rate for video over total bitrate, and 0.0%, 0.1%, and 0.0% in terms of BD rate for synthesized view, respectively.

- JCT3V-E0187 [LGE] 3D-CE3.h Related: Improvement on Merge Candidate List Construction (Cross-Checked by JCT3V-E0283 [Qualcomm])

In this contribution, it is showed that the PU with VSP mode could use DoNBDV without any additional complexity. Moreover, the modified pruning process is proposed for the merge candidate list construction. The result is reported that bit-rate gains are respectively 0.25% and 0.37% for the dependent views 1, 2, and the overall total bit-rate gain is 0.1%.

- JCT3V-E0189 [LGE] CE3.h related : AMVP candidate list construction for DCP blocks (Cross-Checked by JCT3V-E0219 [NTT])

At the last meeting, a list construction process for AMVP candidates was changed identically as one for simulcast HEVC, which means the AMVP process does not consider any inter-view prediction benefit. However, the AMVP process can be improved when the reference picture is an inter-view reference picture. Therefore, the AMVP list construction process for DCP blocks is proposed. For the proposed method, the refined disparity of neighboring blocks, which has been used for merge process, is inserted into the AMVP candidate list. As a result, it achieved approximately 0.1% of coding gain from texture video.

### Residual Prediction

- JCT3V-E0044 [Sharp] ARP simplification (Cross-Checked by JCT3V-E0228 [MediaTek])

This proposal presents a simplification method for advanced residual prediction (ARP) aiming to remove line buffer in context derivation and to reduce the number of weight candidates. The proposal uses two weights instead of the current three weights and restricts ARP enabling mode to merge mode with merge_idx < 3 (disable for other merge block and AMVP block). The experimental result reportedly shows 0.0 %, 0.0% and 0.0% BD-rate change in texture, video and synth respectively and encoding time decreased by about 8 %

- JCT3V-E0124 [Qualcomm and PKU] Further improvements on advanced residual prediction (Cross-Checked by JCT3V-E0244 [MediaTek], JCT3V-E0247 [MERL], and JCT3V-E0290 [HHI])

In this proposal, several aspects are proposed to further improve the coding efficiency of ARP. Firstly, the ARP is extended for inter-view residue, when the current block utilizes inter-view prediction from an inter-view reference picture, in a similar way as current design. Secondly, a disparity motion vector associated with the reference block of current block, if available, is used during ARP, to replace the derived disparity vector. Thirdly, the signalling of Illumination Compensation (IC) flag and ARP weighting factors are jointly considered wherein the ic_flag is not signalled when the ARP weighting factor is unequal to 0. With these improvements, it is observed that there is a coding gain of 0.6% in terms of BD rate for texture views compared to HTM under the common test condition. Accordingly 0.5% BD rate reduction is achieved for the synthesized views.

- JCT3V-E0144 [Samsung] Simplification and improvement of ARP for 3D-HEVC (Cross-Checked by JCT3V-E0274 [MediaTek])

This contribution presents four methods for simplifying and improving the ARP process:

- Apply ARP only to merge mode.

- Not using ARP in case of enabling IC.

- Not using half weight for chroma.

- Allowing non-zero weight value for inter-view prediction block.

The proposed methods reportedly provide 0.3% bit-saving for dependent views (video 1 and video 2), and 0.1% bit-saving for both coded and synthesized views with 94.7% encoding time.

- JCT3V-E0169 [MediaTek] 3D-CE3 related: on complexity reduction of bi-prediction for advanced residual prediction (Cross-Checked by JCT3V-E0149 [Samsung])

In HTM-7.0r1, when the two pieces of motion information used for bi-prediction refer a same reference block, the encoder and the decoder non-normatively change the inter prediction direction from bi-prediction to uni-prediction to avoid the unnecessary computation caused by the redundant motion compensation process. However, with this non-normative change, HTM-7.0r1 could generate a bitstream which does not conform to the 3D-HEVC text due to the unequal operations between uni-prediction and bi-prediction when illumination compensation (IC) or advanced residual prediction (ARP) is enabled. To resolve this issue caused by IC, JCT3V-D0106 was proposed to normatively align the operations between uni-prediction and bi-prediction when IC is enabled. ARP was adopted into 3D-HEVC after JCT3V-D0106 was proposed.

In this contribution, it is reported that ARP also has the same issue as IC. Similar to the method that solved the IC issue, it is proposed to normatively align the operations between uni-prediction and bi-prediction for ARP. Experimental results reportedly show that the proposed method causes no coding efficiency loss while the unnecessary computation of bi-prediction can be removed when ARP is enabled. Additional experiments also show that the decoder could save 5.0% decoding time.

- JCT3V-E0185 [MediaTek] 3D-CE3.h related: Advanced temporal residual prediction (Cross-Checked by JCT3V-E0271 [Qualcomm])

This contribution proposes an advanced temporal residual prediction method to make the current advanced residual prediction method more complete. The experimental results show that the proposed method can achieve 0.8% BD-rate saving for video 1 and video 2, and 0.2% saving for synthesized views with no run time changes.

- JCT3V-E0175 [MediaTek] 3D-CE3.h related: An adaptive disparity vector derivation method for ARP in 3D-HEVC (Cross-Checked by JCT3V-E0197 [LGE])

This contribution presents an adaptive disparity vector derivation (ADVD) method for advanced residual prediction (ARP) in 3D-HEVC. In ADVD, three DV candidates are derived from neighbouring blocks. The encoder can find the best DV candidate used in ARP and signal the index to the decoder.  Experimental results show that the proposed method can achieve 0.33% BD-rate saving for the synthesized views in the common test condition.

### Interview motion prediction

- JCT3V-E0184 [MediaTek] 3D-CE3.h related: Sub-PU level inter-view motion prediction (Cross-Checked by JCT3V-E0073 [MERL])

This contribution proposes a sub-PU level inter-view motion prediction method used for the temporal inter-view merge candidate. The experimental results show that the proposed method can achieve 7% BD-rate saving for video 1 and video 2, and 1.6% BD-rate saving for synthesized views with modest complexity increase.

- JCT3V-E0186 [MediaTek] 3D-CE3.h related: Simplified inter-view merge candidate by reducing checking of reference lists (Cross-Checked by JCT3V-E0253 [Sharp])

This contribution proposes to simplify the temporal inter-view merge candidate by reducing the checking of the reference lists of the reference block. With the proposed method, the checking number in the worst case is reduced by half, and the experimental results show that there is almost no coding performance change (0.01% coding gain for synthesized views).

- JCT3V-E0214 [KHU] 3D-CE3.h related: CTU-aligned DV for a temporal inter-view motion vector candidate (Cross-Checked by JCT3V-E0296 [KWU])

The disparity vector (DV) is used for deriving candidate of motion parameters for a current block in a dependent view. The vertical component of the derived DV mostly closes to zero, since the current 3DV test sequences are rectified. However, the DV with very large vertical component may come up, since there is no restriction for the vertical motion search range for the inter-view prediction. It increases the required memory-access bandwidth on the decoder side. Therefore, in this contribution, the CTU-aligned DV for a temporal inter-view motion vector candidate is proposed to reduce the memory-access bandwidth. Simulation results show that the performance of the proposed method is slightly better(-0.01%) than that of the current HTM while the memory bandwidth to access the co-located motion data is far reduced.

### Depth prediction

- JCT3V-E0174 [MediaTek] 3D-CE3.h related: A disparity derived depth coding method in 3D-HEVC (Cross-Checked by JCT3V-E0200 [LGE])

This contribution presents a disparity derived depth (DDD) coding method in 3D-HEVC. When the collocated texture block of the current depth block is predicted by disparity-compensated prediction, a DDD candidate will be inserted into the merging candidate list right after the texture candidate. In a DDD candidate, prediction samples are derived from the disparity vector in the collocated texture block. Experimental results show that the proposed method can achieve 0.22% BD-rate saving for the synthesized views in the common test condition.

- JCT3V-E0182 [MediaTek] 3D-CE3.h related: A bug-fix for the texture merging candidate in 3D-HEVC (Cross-Checked by JCT3V-E0153 [Samsung])

This contribution presents a bug-fix for the texture merging candidate in 3D-HEVC. In the current 3D-HEVC, the reference indices and motion vectors of a depth block in texture merging mode are inherited directly from the collocated block in the texture picture. Although this design works well in the common test condition, it may not deal properly with the situation when reference lists of the texture component and the depth component are configured in different ways. To fix this bug, it is proposed to inherit the POCs and ViewIds of reference pictures instead of the reference indices. This bug-fix does not affect the coding performance under common test condition.

- JCT3V-E0183 [MediaTek] 3D-CE3.h related: An additional texture merging candidate in 3D-HEVC (Cross-Checked by JCT3V-E0291 [Qualcomm])

In 3D-HEVC depth map coding, a current texture merging candidate inherits motion parameters from the center position of the collocated texture prediction unit (PU). This contribution proposes an additional texture merging candidate, which inherits motion parameters from the right bottom position of the collocated texture PU. Experimental results show that the proposed method can achieve 0.1% BD-rate saving for the synthesized views in the common test condition.

### Others

- JCT3V-E0118 [RWTH Aachen and Huawei] CE3-related: Depth-based Block Partitioning (Cross-Check by JCT3V-E0261 [U. Gent])

In the proposed method, the dependent views' depth information is coded before the texture component to be able to utilize this coded depth data. Based on a binary segmentation mask computed from the reconstructed depth map, an arbitrarily shaped block partitioning for the collocated texture block is derived. Each of the two partitions (resembling foreground and background) is motion compensated and afterwards merged based on the depth-based segmentation mask.
Simulation results show the benefit of the proposed tools with 0.8% BD-rate savings on the video component (w.r.t. total bitrate) under flexible coding order.

- JCT3V-E0170 [MediaTek] 3D-CE3 related: motion data buffer reduction for 3D-HEVC (Cross-Checked by JCT3V-E0259 [NTT])

In the HEVC-based 3D video coding, HTM-7.0, it requires larger buffers and causes a bandwidth issue to store and access the motion data at full resolution when coding the pictures within an access unit (AU). In this contribution, two different schemes are proposed to store the motion data of each picture at lower resolution during the coding of the pictures within an AU. The first scheme is to compress the motion data into 1/4 resolution after encoding/decoding of each picture and then compress the motion data again into 1/16 resolution after encoding/decoding of all the pictures within the same AU. The second scheme is to directly compress the motion data into 1/16 resolution right after encoding/decoding of each picture. With the proposed two schemes, the motion data buffer and the memory access bandwidth for writing and reading motion data can both be reduced. The experimental results reportedly show that 0.0% and 0.4% BD-rate increases for overall results are observed for the first and second schemes, respectively.

The second scheme has been already integrated into SHM-2.0, and the first scheme is being investigated under CE in SHVC. It is recommended that unified scheme be adopted both with SHVC and 3DV.

- JCT3V-E0173 [MediaTek, PKU, and HIT] 3D-CE3.h related: Fast encoder decision for texture coding (Cross-Checked by JCT3V-E0287 [NTT])

This contribution proposes a fast encoding algorithm to encode the texture pictures of dependent views. The proposed fast algorithm uses two strategies to accelerate mode decision by exploiting the inter-view correlation. One is to make an early merge mode decision, and the other one brings an early CU splitting termination. Experimental results show that the proposed algorithm can achieve 47% encoding time reduction with overall 0.1% BD-rate reduction under the common test condition.

# Closing Remarks

It is recommended the CE be continued and investigate proposed designs in further details.