| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0024 |
| --- | --- |

| Title: | 3D-CE4 Summary Report on Illumination Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai | Email: | ikai.tomohiro@sharp.co.jp |
|  | Hongbin Liu | Email: | hongbin.liu@lge.com |
| Source: | CE Cordinators |  |  |

_____________________________

# Abstract

This document is the summary report of Core Experiment 4 (CE4) on Illumination Compensation. Experiments are related to 3D-HTM.

# Participants

| No. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Honbin Liu Jiwook Jung | LG | hongbin.liu@lge.com jiwook.jung@lge.com | P / C |
| 2 | Min Woo Park Jin Young Lee | Samsung | m.w.park@samsung.com jinyoung79.lee@samsung.com | P / C |
| 3 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp | P / C |
| 4 | Yi-Wen Chen Jian-Liang Lin Yu-Lin Chang Yu-Wen Huang | MediaTek | yiwen.chen@mediatek.com jl.lin@mediatek.com yulin.chang@mediatek.com yuwen.huang@mediatek.com | P / C |
| 5 | Xiaozhenzheng | Hisilicon | zhengxiaozhen@huawei.com | C |
| 6 | Jewon Kang | Qualcomm | jewonk@qti.qualcomm.com |  |

# CE and related contributions

|  | Category | # | Title | Author | Cross-check |
| --- | --- | --- | --- | --- | --- |
| CE | prediction | JCT3V-E0045 | CE4: Illumination compensation regression improvement and simplification | T. Ikai (Sharp) | JCT3V-E0047 |
| CE | prediction | JCT3V-E0191 | CE4.h: The results on illumination compensation using offset model | J. W. Jung, H. Liu, J. Jia, S. Yea (LGE) | JCT3V-E0195 |
|  | prediction | JCT3V-E0046 | CE4-related: Resampling in IC parameter derivation and 4x4 Chroma removal | T. Ikai (Sharp) | JCT3V-E0196 |
|  | prediction | JCT3V-E0143 | 3D-CE4.h related: Illumination compensation for BVSP and adaptive IC model | M. W. Park, J. Y. Lee, H.-C. Wey, C. Kim (Samsung) | JCT3V-E0250 |
| CE | bi-prediction | JCT3V-E0168 | 3D-CE4 results on complexity reduction of bi-prediction for illumination compensation | Y.-W. Chen, T.-D. Chuang, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek) | JCT3V-E0277 |
|  | IC flag | JCT3V-E0135 | CE4.h related: Coding of illumination compensation flag | J. Kang, Y. Chen, M. Karczewicz (Qualcomm) | JCT3V-E0203 |
|  | IC flag | JCT3V-E0192 | CE4.h related: Context model for illumination compensation flag | J. W. Jung, J. Heo, S. Yoo, S. Yea (LGE) | JCT3V-E0282 |

# Tools under investigation

## Test 1 Simplification and Robust parameter derivation

There are 2 input contributions in Test 1.

Test 1: Compensation and parameter derivation (CTC)

| No | Test | Anchor | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| 1.1 | E0045 (D0061) | HTM70 | Sharp | LG (E0047) |
| 1.3 | E0191 (D0096) | HTM70 | LG | Sharp (E0195) |
| CE related | E0046 | HTM70 | Sharp | LG (E0195) |
| CE related | E0143 | HTM70 | Samsung | Sharp (E0250) |

## Summary results

|  |  | Luma | Chroma | Depth | Note |
| --- | --- | --- | --- | --- | --- |
|  | HTM70 | Linear | Linear | Linear |  |
| CE | Test 1.1 E0045 (CE) | Linear | Linear | Linear | Improved linear model |
|  | Test 1.3 E0191 (CE) | Offset | Offset | Offset |  |
| CE related | E0046 | Linear | Linear | Linear | Test 1.1 plus chroma 4x4 off resampled |
|  | E0143 test2 | Linear | Offset | Linear |  |
|  | E0143 test3 | Linear | Linear | Offset |  |

Summary results of Test1

|  | video 0 | video 1 | video 2 | video bitrate | total bitrate | total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E0045 | 0.0% | -0.4% | -0.8% | -0.2% | -0.2% | -0.2% | 100.7% | 100.5% | 100.4% |
| E0191 | 0.0% | 0.2% | -0.3% | 0.0% | 0.0% | 0.0% | 100.3% | 97.4% | 105.3% |
| E0046 | 0.0% | -0.4% | -0.8% | -0.2% | -0.2% | -0.2% | 99.6% | 98.6% | 98.6% |
| E0143 test2 | 0.0% | -0.2% | -0.4% | -0.1% | -0.1% | -0.1% | 100.0% | 97.2% | 100.0% |
| E0143 test3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 97.7% | 99.8% |

Complexity assessment results (Luma + Chroma)

| HTM70 | Summary | Comparison | Add/Sub | Mul | ShiftC | ShiftA | Clip | Table | GetMSB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8x8CU | 36 | 266 | 172 | 0 | 123 | 99 | 3 | 9 |
|  | 16x16CU | 9 | 171 | 131 | 0 | 103 | 97 | 1 | 2 |
|  | 32x32CU | 2 | 131 | 113 | 0 | 98 | 96 | 0 | 1 |
|  | 64x64CU | 1 | 113 | 104 | 0 | 96 | 96 | 0 | 0 |
| Test1.1 | Summary | Comparison | Add/Sub | Mul | ShiftC | ShiftA | Clip | Table | GetMSB |
|  | 8x8CU | 24 | 257 | 175 | 105 | 21 | 99 | 3 | 3 |
|  | 16x16CU | 6 | 168 | 132 | 98 | 5 | 97 | 1 | 1 |
|  | 32x32CU | 2 | 130 | 113 | 97 | 1 | 96 | 0 | 0 |
|  | 64x64CU | 0 | 113 | 104 | 96 | 0 | 96 | 0 | 0 |
| Test1.3 | Summary | Comparison | Add/Sub | Mul | ShiftC | ShiftA | Clip | Table | GetMSB |
|  | 8x8CU | 18 | 169 | 0 | 0 | 6 | 96 | 0 | 0 |
|  | 16x16CU | 5 | 130 | 0 | 0 | 2 | 96 | 0 | 0 |
|  | 32x32CU | 1 | 113 | 0 | 0 | 0 | 96 | 0 | 0 |
|  | 64x64CU | 0 | 104 | 0 | 0 | 0 | 96 | 0 | 0 |
| E0046 | Summary | Comparison | Add/Sub | Mul | ShiftC | ShiftA | Clip | Table | GetMSB |
|  | 8x8CU | 8 | 107 | 85 | 67 | 7 | 65 | 1 | 1 |
|  | 16x16CU | 6 | 136 | 116 | 98 | 5 | 97 | 1 | 1 |
|  | 32x32CU | 2 | 114 | 105 | 97 | 1 | 96 | 0 | 0 |
|  | 64x64CU | 0 | 105 | 100 | 96 | 0 | 96 | 0 | 0 |
| E0143 | Summary | Comparison | Add/Sub | Mul | ShiftC | ShiftA | Clip | Table | GetMSB |
| test2 | 8x8CU | 24 | 212 | 100 | 0 | 77 | 97 | 1 | 3 |
|  | 16x16CU | 6 | 149 | 81 | 0 | 67 | 96 | 0 | 1 |
|  | 32x32CU | 2 | 121 | 72 | 0 | 65 | 96 | 0 | 0 |
|  | 64x64CU | 0 | 108 | 68 | 0 | 64 | 96 | 0 | 0 |

## Detail results

CE

Test 1.0 JCT3V-E0045 (CTC, Linear model)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.0% | -0.9% | -0.4% | -0.3% | -0.2% | 100.5% | 102.9% | 101.1% |
| Kendo | 0.0% | -0.7% | -1.7% | -0.5% | -0.4% | -0.3% | 100.4% | 100.0% | 101.0% |
| Newspaper_CC | 0.0% | -0.1% | -0.8% | -0.2% | -0.1% | -0.4% | 100.9% | 101.4% | 100.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 99.0% | 99.9% |
| Poznan_Hall2 | 0.0% | -1.0% | -1.7% | -0.6% | -0.5% | -0.3% | 100.8% | 99.9% | 99.8% |
| Poznan_Street | 0.0% | 0.0% | -0.6% | -0.1% | -0.1% | 0.0% | 100.4% | 100.1% | 100.2% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 101.3% | 100.4% | 99.8% |
| 1024x768 | 0.0% | -0.6% | -1.1% | -0.3% | -0.3% | -0.3% | 100.6% | 101.4% | 101.0% |
| 1920x1088 | 0.0% | -0.2% | -0.6% | -0.2% | -0.2% | -0.1% | 100.8% | 99.8% | 99.9% |
| average | 0.0% | -0.4% | -0.8% | -0.2% | -0.2% | -0.2% | 100.7% | 100.5% | 100.4% |

Test 1.3 JCT3V-E0191 (CTC, Offset model)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.4% | -0.3% | -0.1% | -0.1% | -0.1% | 100.1% | 97.5% | 103.0% |
| Kendo | 0.0% | 1.6% | 0.8% | 0.5% | 0.6% | 0.6% | 100.0% | 97.2% | 104.3% |
| Newspaper_CC | 0.0% | 0.3% | 0.1% | 0.1% | 0.2% | -0.2% | 100.7% | 97.2% | 102.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 95.7% | 106.6% |
| Poznan_Hall2 | 0.0% | -0.6% | -2.4% | -0.6% | -0.6% | -0.3% | 100.3% | 98.9% | 101.9% |
| Poznan_Street | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.3% | 95.8% | 108.6% |
| Undo_Dancer | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% | 100.3% | 99.5% | 109.6% |
| 1024x768 | 0.0% | 0.5% | 0.2% | 0.2% | 0.3% | 0.1% | 100.3% | 97.3% | 103.4% |
| 1920x1088 | 0.0% | -0.1% | -0.6% | -0.1% | -0.1% | 0.0% | 100.3% | 97.5% | 106.7% |
| average | 0.0% | 0.2% | -0.3% | 0.0% | 0.0% | 0.0% | 100.3% | 97.4% | 105.3% |

CE-related:

JCT3V-E0046 (CTC, Improved Linear model with resample? and 4x4 chroma off)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.9% | -0.8% | -0.3% | -0.3% | -0.3% | 100.5% | 100.1% | 99.6% |
| Kendo | 0.0% | -0.6% | -1.3% | -0.4% | -0.3% | -0.2% | 99.4% | 95.3% | 97.3% |
| Newspaper_CC | 0.0% | 0.0% | -0.7% | -0.1% | -0.1% | -0.4% | 100.6% | 101.6% | 100.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 96.7% | 97.9% |
| Poznan_Hall2 | 0.0% | -0.9% | -2.2% | -0.7% | -0.6% | -0.4% | 100.2% | 99.8% | 99.6% |
| Poznan_Street | 0.0% | -0.2% | -0.6% | -0.1% | -0.1% | -0.1% | 98.7% | 97.9% | 97.6% |
| Undo_Dancer | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.1% | 98.5% | 98.5% | 98.2% |
| 1024x768 | 0.0% | -0.5% | -0.9% | -0.3% | -0.2% | -0.3% | 100.2% | 99.0% | 99.0% |
| 1920x1088 | 0.0% | -0.3% | -0.7% | -0.2% | -0.2% | -0.1% | 99.2% | 98.2% | 98.3% |
| average | 0.0% | -0.4% | -0.8% | -0.2% | -0.2% | -0.2% | 99.6% | 98.6% | 98.6% |

JCT3V-E0143 (Test 3, Luma: HTM70 linear model, Chroma: offset model)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.5% | -0.5% | -0.2% | -0.2% | -0.2% | 99.4% | 89.9% | 100.0% |
| Kendo | 0.0% | -0.1% | -0.6% | -0.1% | -0.1% | 0.0% | 100.2% | 97.7% | 99.2% |
| Newspaper_CC | 0.0% | 0.1% | -0.5% | -0.1% | -0.1% | 0.0% | 100.3% | 95.7% | 99.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.5% | 96.8% | 101.7% |
| Poznan_Hall2 | 0.0% | -1.1% | -0.8% | -0.4% | -0.4% | -0.2% | 100.5% | 98.7% | 99.4% |
| Poznan_Street | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 99.3% | 99.0% | 100.2% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 99.8% | 102.7% | 99.7% |
| 1024x768 | 0.0% | -0.2% | -0.5% | -0.1% | -0.1% | -0.1% | 100.0% | 94.5% | 99.7% |
| 1920x1088 | 0.0% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 100.0% | 99.3% | 100.3% |
| average | 0.0% | -0.2% | -0.4% | -0.1% | -0.1% | -0.1% | 100.0% | 97.2% | 100.0% |

JCT3V-E0143 (Test 3, Luma/Chroma: HTM70 linear model and Depth offset model)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.0% | 89.4% | 98.1% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% | 99.3% | 96.7% | 99.1% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.2% | 99.9% | 94.5% | 100.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 97.8% | 100.8% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 99.7% | 99.2% | 100.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.3% | 97.7% | 99.6% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 108.4% | 99.8% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 99.4% | 93.5% | 99.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 100.7% | 100.3% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.5% | 97.7% | 99.8% |

## Test 2 Aligned operation for Bi-prediction

Test 2 consists of the following aspect:

- Align the operations between the uni-prediction and bi-prediction when IC is enabled.

In decoder side, the alignment enables uni-prediction compensation (identical motion compensation) if L0 motion parameter and L1 motion parameter is identical in IC case.

Note: Current HTM’s identical motion compensation is buggy because it doesn’t consider if IC on or off. But this bug doesn’t appear because encoder and decoder have the same bugs so there is no enc/dec mismatch.

In encoder side, the alignment is beneficial because L0 block and L1 block can be reused for Bi block in motion estimation stage.

Test 2: Bi-prediction operation (IBP)

| Test | Test | Anchor | Tester 1 | Tester 2 |
| --- | --- | --- | --- | --- |
| 2.0 | E0168 (D0106) | HTM70 | MediaTek | Hisilicon(E0xxx) |
| 2.1 | E0168+E0045 (D0106+D0061) | HTM70 | Sharp | MediaTek(E0277) |
| 2.3 | E0168+E0191 (D0106+D0096) | HTM70 | LG | MediaTek(E0277) |

Summary results of Test2

|  | video 0 | video 1 | video 2 | video bitrate | total bitrate | total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E0168 (CTC) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 101.3% | 99.9% |
| E0168 (IBP) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 101.9% | 100.2% | 101.0% |
| E0168+E0045(IBP) | 0.0% | -0.7% | -0.2% | -0.2% | -0.2% | -0.7% | 100.6% | 99.5% | 99.4% |
| E0168+E0191(IBP) | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.5% | 100.1% | 98.2% | 96.6% |

Test 2.0 JCT3V-E0168 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 100.5% | 102.4% | 101.3% |
| Kendo | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | 0.0% | 101.1% | 104.5% | 99.3% |
| Newspaper_CC | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 100.1% | 101.9% | 101.0% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.8% | 95.9% | 97.2% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 101.2% | 100.1% | 99.9% |
| Poznan_Street | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 100.7% | 99.0% | 100.8% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 105.0% | 99.7% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 102.9% | 100.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.7% | 100.0% | 99.4% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 101.3% | 99.9% |

Test 2.0 JCT3V-E0168 (IBP)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.1% | 0.3% | 0.1% | 0.1% | 0.0% | 101.8% | 99.1% | 99.8% |
| Kendo | 0.0% | 0.0% | 0.8% | 0.1% | 0.1% | 0.1% | 102.5% | 101.0% | 101.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.7% | 102.8% | 98.6% | 102.8% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.0% | 100.8% | 101.1% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 101.2% | 101.1% | 101.1% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 102.8% | 100.9% | 102.8% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 101.4% | 100.3% | 98.4% |
| 1024x768 | 0.0% | 0.0% | 0.4% | 0.1% | 0.1% | 0.3% | 102.4% | 99.6% | 101.3% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 101.6% | 100.8% | 100.9% |
| average | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 101.9% | 100.2% | 101.0% |

Test 2.1 JCT3V-E0168 + JCT3V-E0045(IBP)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -1.1% | -1.0% | -0.4% | -0.4% | -0.3% | 100.3% | 96.7% | 96.7% |
| Kendo | 0.0% | -1.2% | -0.3% | -0.4% | -0.2% | -0.1% | 100.7% | 100.7% | 100.2% |
| Newspaper_CC | 0.0% | -0.7% | 0.3% | -0.1% | -0.1% | -3.6% | 100.4% | 100.1% | 100.0% |
| GT_Fly | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 100.4% | 99.9% | 100.2% |
| Poznan_Hall2 | 0.0% | -1.9% | -0.2% | -0.5% | -0.5% | -0.4% | 100.5% | 100.3% | 99.9% |
| Poznan_Street | 0.0% | -0.3% | 0.1% | -0.1% | -0.1% | 0.0% | 100.6% | 98.5% | 99.5% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.6% | 100.8% | 100.3% | 99.5% |
| 1024x768 | 0.0% | -1.0% | -0.3% | -0.3% | -0.2% | -1.3% | 100.5% | 99.2% | 99.0% |
| 1920x1088 | 0.0% | -0.5% | 0.0% | -0.1% | -0.1% | -0.3% | 100.6% | 99.8% | 99.8% |
| average | 0.0% | -0.7% | -0.2% | -0.2% | -0.2% | -0.7% | 100.6% | 99.5% | 99.4% |

Test 2.3 JCT3V-E0168 + JCT3V-E0191 (IBP)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.5% | -1.0% | -0.3% | -0.2% | -0.3% | 100.5% | 111.7% | 98.8% |
| Kendo | 0.0% | 1.2% | -0.2% | 0.3% | 0.5% | 0.6% | 100.3% | 102.5% | 99.0% |
| Newspaper_CC | 0.0% | 0.1% | 0.6% | 0.1% | 0.2% | -3.3% | 99.8% | 91.4% | 98.9% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.3% | 97.3% | 95.4% |
| Poznan_Hall2 | 0.0% | -2.0% | 0.0% | -0.5% | -0.4% | -0.1% | 100.1% | 98.2% | 95.7% |
| Poznan_Street | 0.0% | 0.6% | 0.1% | 0.1% | 0.1% | 0.2% | 100.2% | 96.2% | 94.9% |
| Undo_Dancer | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | -0.8% | 99.9% | 90.2% | 93.3% |
| 1024x768 | 0.0% | 0.3% | -0.2% | 0.0% | 0.1% | -1.0% | 100.2% | 101.9% | 98.9% |
| 1920x1088 | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | -0.2% | 100.1% | 95.5% | 94.8% |
| average | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.5% | 100.1% | 98.2% | 96.6% |

# CE related contribution

The following items are directly related to CE test1.

- JCT3V-E0046

- JCT3V-E0143 test2 / test3

| prediction | JCT3V-E0046 | CE4-related: Resampling in IC parameter derivation and 4x4 Chroma removal | T. Ikai (Sharp) | JCT3V-E0196 |
| --- | --- | --- | --- | --- |
| prediction | JCT3V-E0143 | 3D-CE4.h related: Illumination compensation for BVSP and adaptive IC model | M. W. Park, J. Y. Lee, H.-C. Wey, C. Kim (Samsung) | JCT3V-E0250 |

JCT3V-E0143 test1 has another items (IC for BVSP block)

The following two contributions are related to illumination compensation flag.

| IC flag | JCT3V-E0135 | CE4.h related: Coding of illumination compensation flag | J. Kang, Y. Chen, M. Karczewicz (Qualcomm) | JCT3V-E0203 |
| --- | --- | --- | --- | --- |
| IC flag | JCT3V-E0192 | CE4.h related: Context model for illumination compensation flag | J. W. Jung, J. Heo, S. Yoo, S. Yea (LGE) | JCT3V-E0282 |

# Recommendations

- To review the relevant input contributions