| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0025 |
| --- | --- |

| Title: | CE5 Summary Report: Depth Bi-partition Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Philipp Merkle | Email: | philipp.merkle@hhi.fraunhofer.de |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This document summarizes the proposals and cross check results of CE5, including tools under test according to JCT3V-D1105 as well as related input contributions.

# Introduction

In this document a summary of proposals and cross check results for CE5 is reported. The goal of this Core Experiment (CE) is to investigate simplifications of the depth bi-partition modes in 3D-HEVC under sub-experiment CE5.H as proposed to the 4th JCT-3V meeting.

Bi-partition modes are part of the depth intra coding tools DMM (depth modeling modes) and RBC (region boundary chain). The corresponding test model, including DMM and RBC, is described in JCT3V-D1005. The objective of tools under test in CE5.H is evaluation of the performance and possible simplification of the existing DMM and RBC coding modes.

Participants in CE5 are as follows:

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| HHI | P. Merkle | philipp.merkle@hhi.fraunhofer.de | X | X |
| LGE | J. Heo H. Liu | jin78.heo@lge.com hongbin.liu@lge.com | X | X |
| Samsung | J.-Y. Lee M. W. Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | X | X |
| GIST | Y. Song Y.-S. Ho | ysong@gist.ac.kr hoyo@gist.ac.kr | X | X |
| Qualcomm | X. Zhao | xinzhao@qti.qualcomm.com |  | X |
| SCU | Z. Gu | zgu@scu.edu |  | X |
| Hisilicon | X. Zheng | zhengxiaozhen@huawei.com |  | X |
| Zhejiang U. | P. Lu | yiqiu8932@gmail.com |  | X |
| MediaTek | Y.-L. Chang J.-L. Lin Y.-W. Huang | yulin.chang@mediatek.com jl.lin@mediatek.com yuwen.huang@mediatek.com |  | X |

Tools tests in CE5.H are as follows:

| Test | DMM1 | DMM2 | DMM3 | DMM4 | RBC | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1-A1 | off | on | on | on | on | disable mode via macro |
| 1-A2 | on | off | on | on | on | disable mode via macro |
| 1-A3 | on | on | off | on | on | disable mode via macro |
| 1-A4 | on | on | on | off | on | disable mode via macro |
| 1-B1 | on | on | on | on | off | disable mode via macro |
| 1-B2 | off | off | off | off | on | disable DMM via config parameter |
| 1-C | off | off | off | off | off | disable DMM via config parameter and RBC via macro |
| 2-A | on | on | D0067 | on | on |  |
| 2-B1 | D0184 | on | D0067 | on | on | WITHDRAWN |
| 2-B2 | D0184 | off | D0067 | on | on | WITHDRAWN |
| 2-C1 | on | on | off | off | on | adapted signaling |
| 2-C2 | on | off | off | off | on | adapted signaling |
| 2-D | on | off | off | off | off | adapted signaling |

# List of input documents

## CE5.H proposals

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| HHI | JCT3V-E0120 | CE5.H: Results on DMM and RBC depth intra modes performance (Test1) | Proposal |
| Qualcomm | JCT3V-E0233 | 3D-CE5.h related: Cross check on results on DMM and RBC depth intra modes performance (JCT3V-E0120) | Cross check |
| Samsung | JCT3V-E0146 | 3D-CE5: DMM simplification and signalling | Proposal |
| LGE | JCT3V-E0201 | CE 5.h: Crosscheck on DMM simplification and signalling (JCT3V-E0146) | Cross check |
| PKU LGE | JCT3V-E0159 | CE 5.h: Results on Removal of Overlap between DMM1 and DMM3 | Proposal |
| Qualcomm | JCT3V-E0236 | 3D-CE5.h: Cross check on Results on Removal of Overlap between DMM1 and DMM3 (JCT3V-E0159) | Cross check |

## CE5.H related proposals

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| HHI | JCT3V-E0121 | CE5.H related: Results on DMM wedgelet prediction unification | Proposal |
| SCU | JCT3V-E0273 | CE5.H related: Crosscheck on HHI's Proposal JCT3V-E0121 | Cross check |
| MediaTek | JCT3V-E0179 | CE5.h related: a simplified DMM prediction method in 3D-HEVC | Proposal |
| Hisilicon | JCT3V-E0265 | CE5.h related: Crosscheck on simplified DMM prediction method (JCT3V-E0179) | Cross check |
| MediaTek | JCT3V-E0180 | CE5.h related: A simplified method on DMM mode coding in 3D-HEVC | Proposal |
| Hisilicon | JCT3V-E0266 | CE5.h related: Crosscheck on simplified method on DMM mode coding (JCT3V-E0180) | Cross check |
| LGE | JCT3V-E0193 | 3D-CE5.h Related: new intra coding structure for depth map | Proposal |
| Samsung | JCT3V-E0285 | 3D-CE5 related: Cross check of new intra coding structure for depth map (JCT3V-E0193) | Cross check |
| Zhejiang U. | JCT3V-E0204 | CE5 related: Results on Simplified Binarization for depth_intra_mode | Proposal |
|  |  |  | Cross check |
| Tsinghua Hisilicon | JCT3V-E0225 | CE5.h related: Simplification on DMM1 pattern generation mechanism | Proposal |
| Qualcomm | JCT3V-E0237 | CE5.h related: Crosscheck on simplification on DMM1 pattern generation mechanism (JCT3V-E0225) | Cross check |
| Hisilicon | JCT3V-E0226 | Modification of Binarization for Depth Intra Mode | Proposal |
| Qualcomm | JCT3V-E0292 | Crosscheck on modification of binarization for depth Intra mode (JCT3V-E0226) | Cross check |
| SCU Hisilicon | JCT3V-E0238 | 3D-CE5.h related: Fast Intra Prediction Mode Selection for Intra Depth Map Coding | Proposal |
| MediaTek | JCT3V-E0257 | 3D-CE5.h related: Cross-check on fast Intra prediction Mode selection for intra depth map coding by SCU/Hisilicon (JCT3V-E0238) | Cross check |
| SCU Hisilicon | JCT3V-E0242 | 3D-CE5.h related: On DMM simplification | Proposal |
| NTT | JCT3V-E0298 | 3D-CE5.h related: Cross-check on DMM simplification (JCT3V-E0242) | Cross check |

# Summary of proposals

All the proposals are related to the bi-partition modes in DMM and RBC.

## Test1: Performance of DMM and RBC depth intra modes

- JCT3V-E0120: investigating the performance of DMM and RBC depth intra modes in terms of RD and runtime performance for disabling individual modes or complete tool sets.

Test1 results for 3-view scenario under CTC: BD-rates and runtimes

| Test | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| 1-A1 | 0.0% | -0.1% | 0.4% | 97.1% | 100.4% | 99.7% |
| 1-A2 | 0.0% | 0.0% | 0.1% | 99.6% | 102.1% | 100.5% |
| 1-A3 | 0.0% | 0.0% | 0.1% | 99.1% | 102.2% | 100.2% |
| 1-A4 | 0.0% | 0.0% | 0.1% | 99.6% | 101.7% | 99.9% |
| 1-B1 | 0.0% | 0.0% | 0.2% | 99.8% | 100.4% | 100.1% |
| 1-B2 | 0.1% | -0.1% | 1.6% | 94.5% | 100.3% | 99.6% |
| 1-C | 0.1% | -0.2% | 2.7% | 94.4% | 98.0% | 99.9% |

Test1 results for 3-view scenario under all-intra configuration: BD-rates and runtimes

| Test | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| 1-A1 | 0.0% | -0.1% | 1.6% | 63.7% | 101.6% | 99.1% |
| 1-A2 | 0.0% | 0.0% | 0.0% | 94.7% | 100.2% | 99.2% |
| 1-A3 | 0.0% | 0.0% | 0.1% | 89.6% | 100.0% | 99.2% |
| 1-A4 | 0.0% | 0.0% | 0.1% | 94.5% | 99.3% | 99.0% |
| 1-B1 | 0.0% | 0.0% | 0.1% | 99.2% | 98.7% | 99.0% |
| 1-B2 | 0.0% | -0.1% | 4.9% | 41.4% | 100.6% | 99.6% |
| 1-C | 0.0% | -0.2% | 7.3% | 40.6% | 100.0% | 99.3% |

## Test2: Simplification of DMM and RBC depth intra modes

- JCT3V-E0159 (Test2-A): removing part of DMM3 that overlaps with DMM1, which is the case when the CTLB is not intra coded with mode 2~34.

- JCT3V-E0146 (Test2-C,D): testing subsets of modes with adapted signaling.

Test2 results for 3-view scenario under CTC: BD-rates and runtimes

| Test | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| 2-A | 0.0% | 0.0% | 0.0% | 99.3% | 99.7% | 44.9% |
| 2-C1 | 0.0% | 0.0% | 0.0% | 96.2% | 100.0% | 100.0% |
| 2-C2 | 0.0% | 0.0% | 0.1% | 94.5% | 99.9% | 100.0% |
| 2-D | 0.0% | 0.0% | 0.2% | 94.5% | 100.2% | 100.0% |

Test2 results for 3-view scenario under all-intra configuration: BD-rates and runtimes

| Test | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| 2-A | 0.0% | 0.0% | 0.0% | 94.7% | 100.0% | 88.2% |
| 2-C1 | 0.0% | 0.0% | 0.2% | 80.5% | 99.5% | 100.0% |
| 2-C2 | 0.0% | 0.0% | 0.2% | 76.2% | 99.5% | 100.0% |
| 2-D | 0.0% | 0.1% | 0.4% | 75.0% | 99.5% | 100.0% |

## Related proposals

- JCT3V-E0121: unifying DMM wedgelet prediction for inter-component and intra by removing DMM2 and replacing overlapping part of DMM3.

- JCT3V-E0179: simplifying and unifying segment DC value prediction for DMM.

- JCT3V-E0180: replace fixed-length code of DMM1~3 data by bypass mode, reducing number of contexts.

- JCT3V-E0193: replacing DMM, RBC, and SDC by simple SDC-like coding mode.

- JCT3V-E0204: using truncated unary code for syntax element depth_intra_mode.

- JCT3V-E0225: simplify DMM1 pattern generation mechanism with reduced number of patterns and changed binarization and signaling.

- JCT3V-E0226: changing binarization of depth intra modes for 8x8 to 32x32 and removing DMM3 and RBC for 4x4.

- JCT3V-E0238: fast encoder mode selection for DMM and RBC.

- JCT3V-E0242: replace division of DMM4 threshold calculation.

Results of related proposals for 3-view scenario under CTC: BD-rates and runtimes

| Proposal | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| JCT3V-E0121 | 0.0% | 0.0% | -0.0% | 99.7% | 99.3% | 99.7% |
| JCT3V-E0179 | 0.0% | 0.0% | -0.1% | 99.9% | 99.4% | 101.2% |
| JCT3V-E0180 | 0.0% | 0.0% | -0.0% | 100.0% | 96.2% | 100.0% |
| JCT3V-E0193 | 0.1% | -0.4% | 1.3% | 88.6% | 95.9% | 87.8% |
| JCT3V-E0204 | 0.0% | 0.0% | -0.1% | 99.1% | 100.0% | n/a |
| JCT3V-E0225 | 0.0% | -0.1% | -0.1% | 99.8% | 98.1% | 99.6% |
| JCT3V-E0226 | 0.0% | 0.0% | -0.1% | 99.8% | 99.8% | 99.7% |
| JCT3V-E0238 | 0.0% | 0.0% | 0.0% | 97.5% | 89.6% | 96.5% |
| JCT3V-E0242 | 0.0% | 0.0% | 0.0% | 100.7% | 101.8% | 101.6% |

Results of related proposals for 3-view scenario under all-intra configuration: BD-rates and runtimes

| Proposal | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| JCT3V-E0121 | 0.0% | 0.0% | 0.0% | 98.9% | 100.4% | 100.5% |
| JCT3V-E0179 | 0.0% | 0.0% | -0.1% | 99.5% | 97.7% | 98.3% |
| JCT3V-E0180 | 0.0% | 0.0% | -0.0% | 100.2% | 97.6% | 99.0% |
| JCT3V-E0193 | 0.0% | -0.3% | 4.0% | 37.1% | 104.2% | 105.2% |
| JCT3V-E0204 | 0.0% | 0.1% | 0.0% | 96.4% | 99.8% | n/a |
| JCT3V-E0225 | 0.0% | -0.1% | 0.0% | 90.3% | 99.3% | 99.3% |
| JCT3V-E0226 | 0.0% | 0.0% | 0.0% | 100.3% | 97.5% | 100.1% |
| JCT3V-E0238 | 0.0% | 0.0% | 0.0% | 73.5% | 94.2% | 89.6% |
| JCT3V-E0242 | 0.0% | 0.0% | 0.0% | 100.9% | 101.2% | 100.4% |

# Conclusion

Regarding CE5 tool tests, JCT3V-E0120 studied the performance of DMM and RBC in Test1 and JCT3V-E0159 and JCT3V-E0146 studied simplifications of DMM and RBC in Test2.

Regarding CE5 related contributions, the proposals can be categorized as follows:

Five contributions propose simplifications mainly related to the complexity of DMM prediction and processing. JCT3V-E0121, JCT3V-E0179, and JCT3V-E0242 target reducing the implementation and specification complexity. JCT3V-E0225 and JCT3V-E0238 target reducing the encoder processing complexity.

Three contributions (JCT3V-E0180, JCT3V-E0204, and JCT3V-E0226) propose improvements and simplifications mainly related to depth intra mode binarization.

Contribution JCT3V-E0193 proposes a general alternative for the depth intra coding structure.