| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0034 |
| --- | --- |

| Title: | AHG8: Revision of the Alternative Depth Info SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nikolce Stefanoski Aljosa Smolic Takanori Senoh Masayuki Tanimoto | Tel: Email: | stefanos@disneyresearch.com senoh@nict.go.jp tanimototentative3@yahoo.co.jp |
| Source: | Disney Research Zurich, National Institute of Information and Communications Technology, Nagoya Industry Science Research Institute |  |  |

_____________________________

# Abstract

At the last JCT-3V meeting, the Alternative Depth Info SEI message was adopted for inclusion in the specification of the 3D-HEVC Test Model. To address corresponding editorial comments available in the specification of the 3D-HEVC Test Model 4, we specify in this document the persistence scope of Alternative Depth Info SEI message and provide an update of the syntax and the semantics description text without changing the functionality of the SEI message.

# Introduction

At the last JCT-3V meeting, the Alternative Depth Info SEI message was adopted for inclusion in the specification of the 3D-HEVC Test Model. This SEI message signals to the receiver side that reconstructed depth samples describe an alternative depth format. To specify the persistence scope of this SEI message, we propose in this document to include a new syntax element called alternative_depth_info_cancel_flag to indicate if an SEI message cancels its persistence or if alternative depth information follows. In addition, we change the naming of two syntax elements from warp_map_width to warp_map_width_minus2 and warp_map_height to warp_map_height_minus2 to allow a better coding of this information since warp width and height are always 2 or larger. We also replace the former num_residual_depth_view syntax element with a residual_depth_flag which is used to derive the number of residual depth views. Finally, we update the semantics description according to the comments of the editor to clarify ambiguities and improve understandability.

# Revised syntax and semantics

  - SEI message syntax

    - Alternative depth information SEI message syntax

| alternative_depth_info ( payloadSize ) { | Descriptor |
| --- | --- |
| alternative_depth_info_cancel_flag | u(1) |
| if( alternative_depth_info_cancel_flag = = 0 ) { |  |
| depth_type | u(2) |
| if( depth_type = = 1 ) { |  |
| min_offset_x_int | se(v) |
| min_offset_x_frac | u(8) |
| max_offset_x_int | se(v) |
| max_offset_x_frac | u(8) |
| offset_y_present_flag | u(1) |
| if ( offset_y_present_flag ){ |  |
| min_offset_y_int | se(v) |
| min_offset_y_frac | u(8) |
| max_offset_y_int | se(v) |
| max_offset_y_frac | u(8) |
| } |  |
| warp_map_size_present_flag | u(1) |
| if( warp_map_size_present_flag ) { |  |
| warp_map_width_minus2 | ue(v) |
| warp_map_height_minus2 | ue(v) |
| } |  |
| } |  |
| if( depth_type = = 0 ) { |  |
| num_residual_texture_views_minus1 | ue(v) |
| residual_depth_flag | u(1) |
| } |  |
| } |  |
| } |  |

  - SEI message semantics

    - Alternative depth information SEI message semantics

[Ed. (GT): Persistence of the SEI is missing. ]

The alternative depth information SEI message indicates that decoded depth samples have to be interpreted as an alternative depth format. The persistence of this SEI message is specified with help of the alternative_depth_info_cancel_flag. To discriminate different alternative depth formats, a depth_type syntax element is used.

alternative_depth_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous alternative depth information SEI message in output order. alternative_depth_info_cancel_flag equal to 0 indicates that alternative depth information follows.

depth_type identifies an alternative depth type according to Table Error: Reference source not found-1. A value of depth_type is equal to 0 indicates that this SEI message signals Global View and Depth (GVD) information.  A value of depth_type is equal to 1 indicates that decoded depth samples can be used to derive a warp map and view synthesis can be performed by image-domain warping. Additional syntax elements contained in this SEI message are used to derive corresponding warp maps from decoded depth samples of each view.

Values of depth_type that are not listed in Table Error: Reference source not found-1 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore alternative depth information SEI messages that contain reserved values of depth_type.

Table Error: Reference source not found-1 – Interpretation of depth_type

| Value | Description |
| --- | --- |
| 0 | Global view and depth |
| 1 | Warp map |

min_offset_x_int, min_offset_x_frac specify the integer and the fractional part of the minimum offset for the horizontal direction of a warp map.

The variable minOffsetX is derived as follows:

  - minOffsetX = min_offset_x_int + min_offset_x_frac  256	(Error: Reference source not found-1)

max_offset_x_int, max_offset_x_frac specify the integer and the fractional part of the maximum offset for the horizontal direction of a warp map.

The variable maxOffsetX value is derived as follows:

  - maxOffsetX = max_offset_x_int + max_offset_x_frac  256

offset_y_present_flag equal to 1 specifies that min_offset_y_int, min_offset_y_frac, max_offset_y_int and max_offset_y_frac are present. offset_y_present_flag equal to 0 specifies that min_offset_y_int, min_offset_y_frac, max_offset_y_int and max_offset_y_frac are not present.

min_offset_y_int, min_offset_y_frac specify the integer and the fractional part of the minimum offset for the vertical direction of a warp map. When not present, min_offset_y_int and min_offset_y_frac are inferred to be equal to 0.

The variable  minOffsetY value is derived as follows:

  - minOffsetY = min_offset_y_int + min_offset_y_frac  256	(Error: Reference source not found-2)

max_offset_y_int, max_offset_y_frac specify the integer and the fractional part of the maximum offset for the vertical direction of a warp map. When not present, max_offset_y_int and max_offset_y_frac are inferred to be equal to 0.

The variable maxOffsetY value is derived as follows:

  - maxOffsetY = max_offset_y_int + max_offset_y_frac  256	(Error: Reference source not found-3)

warp_map_size_present_flag equal to 1 specifies that a new warp map size is present, which is valid for the current and all following warp maps in output orderuntil a new message with warp_map_size_present_flag  equal to 1 is received or alternative_depth_info_cancel_flag is equal to 1. warp_map_size_present_flag  equal to 0 specifies that the warp map size is not changed.

[Ed. (GT): This needs to be clarified together with the persistence scope of the SEI.]

warp_map_width_minus2 specifies the width of the warp map minus 2. The value of warp_map_width_minus2 shall be in the range of 0 to pic_width_in_luma_samples-2, inclusive.

[Ed. (GT): Is pic_width_in_luma_samples correct or should it be pic_width_in_luma_samples − 1? The same question applies for the other constraints.]

warp_map_height_minus2 specifies the height of the warp map minus 2. The value of warp_map_height_minus2 shall be in the range of 0 to ( pic_height_in_luma_samples  >>  offset_y_present_flag )-2, inclusive.

- The variables warp_map_width, warp_map_height, deltaX, deltaY, scaleX and scale Y are derived as specified in the following:

  - warp_map_width = warp_map_width_minus2 + 2

  - warp_map_height = warp_map_height_minus2 + 2

  - deltaX =  pic_width_in_luma_samples  ( warp_map_width − 1 )	(Error: Reference source not found-4)

  - deltaY =  pic_height_in_luma_samples  ( warp_map_height − 1 )	(Error: Reference source not found-5)

  - scaleX =  (maxOffsetX − minOffsetX) / ( ( 1  <<  BitDepthY ) − 1 )	(Error: Reference source not found-6)

  - scaleY =  (maxOffsetY − minOffsetY) / ( ( 1  <<  BitDepthY ) − 1 )	(Error: Reference source not found-7)

Let recSamples[ x ][ y ] correspond to the reconstructed sample array SL of a depth view component. The corresponding horizontal warp map component w[ x ][ y ][ 0 ] and the corresponding vertical warp map component w[ x ][ y ][ 1 ] for recSamples[ x ][ y ] are derived as specified in the following:

for( x = 0; x < warp_map_width ; x++ )
	 for( y = 0; y < warp_map_height; y++){
		w[ x ][ y ][ 0 ] = x * deltaX + minOffsetX + scaleX * recSamples[ x ][ y ]
		if( offset_y_present_flag ) 
			w[ x ][ y ][ 1 ] =	y * deltaY + minOffsetY + 
							scaleY * recSamples[ x ][ y + pic_height_in_luma_samples / 2 ]
		else
			w[ x ][ y ][ 1 ] =	y * deltaY
	}

[Ed. (GT): Below specifications are unclear to me, what means assigned. Forward warping? ]

A dedicated warp map w[x][y] is derived for each input view using reconstructed samples of its corresponding depth view component, i.e. each input view has an assigned warp map and vice versa. A warp map specifies a sparse set of positional correspondences. These correspondences identify semantically corresponding image locations between two views of the same time instance, i.e. the assigned input view and a neighbouring input view which is identified as follows.

When the warp map w[ x ][ y ] is assigned to the leftmost input view, then the warp map specifies for each sub-pel position (x*deltaX, y*deltaY ) in this input view, a corresponding sub-pel position (2 * w[ x ][ y ][ 0 ], 2 * w[ x ][ y ][ 1 ]) in the closest input view on the right.

When the warp map w[ x ][ y ] is assigned to an input view different to the leftmost input view, then the warp map specifies for each sub-pel position ( x * deltaX, y*deltaY ) in this input view, a corresponding sub-pel position ( 2 * w[ x ][ y ][ 0 ], 2 * w[ x ][ y ][ 1 ] ) in the closest input view on the left.

Note - A pixel dense set of positional correspondences can be derived by bilinear interpolation.

num_residual_texture_views_minus1 plus1 specifies the number of sub-residual texture views packed in the residual texture view. num_residual_texture_views_minus1 is in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that the number of packed depth views in a depth view component of non-base view is equal to the number of packed texture views in a texture view component. residual_depth_flag equal to 0 specifies there is only one depth view.

NOTE: When depth_type is equal to 0, tools for inter-view prediction and depth-texture interaction coding tools should be disabled.

The texture and depth view components of the non-base view contain up to four GVD’s sub-residual views, each represents a difference picture of non-base view and base view that is horizontally and vertically decimated in half leaving even samples relative to the corresponding texture or depth base view components.

depth_type equal to 0 indicates that N = num_residual_texture_views_minus1 + 2 input texture views and associated depth views (N=2, 3, 4 and up to 5) are converted into Global texture view (one base texture view and one residual texture view) and Global depth view (one base depth view or one base depth view plus one residual depth view).

Base texture view (B) is the input from central camera (camera ID=3) as shown in Figure H-2. In the case N=2, camera ID=2 and 3 are used. In the case N=3, camera ID=2, 3 and 4 are used. In the case N=4, camera ID=1, 2, 3 and 4 are used. In the case N=5, camera ID=1, 2, 3, 4 and 5 are used. The other N-1 input texture views are converted to N-1 half-size sub-residual texture views (R-x) and packed in the residual texture view from top-left (R-2), to bottom-left (R-4), top-right (R-1) and bottom-right (R-5) order. Their top-left co-ordinates in the residual view of width=W and height=H is shown in Table J-X. The residual texture view represents occluded area or out-of-frame area of the base texture view when it is projected to the N-1 input texture views by GVD process. The number of sub-residual texture views is signalled by num_residual_texture_views_minus1.

1     2     3     4     5

R-1   R-2    B    R-4   R-5

camera ID

GVD view

- Figure H-2 – Relation between camera ID and GVD texture views

- Table H-33 – Top-left corner co-ordinates of GVD sub-residual views packed in a Residual view of width=W and height=H.

| R-1 | (W/2, 0) |
| --- | --- |
| R-2 | (0, 0) |
| R-4 | (0, H/2) |
| R-5 | (W/2, H/2) |

Base depth view (B) is a sample-wise median between corresponding pixels of all input depth views and is associated with the base texture view (camera ID=3) as shown in Figure H-3. Residual depth view may exist or may not exist. residual_depth_flag equal to 1 specifies the existence of a residual depth view. The other N-1 input depth views are converted to N-1 half-size sub-residual depth views (R-x) by GVD process. The generation of sub-residual depth view is described in the following paragraph. They are packed in the residual depth view in the same positions as the residual texture view shown in Table H-33. Each sub-residual depth view associates with its sub-residual texture view. The generation of sub-residual texture view by GVD process is described in the following paragraph.

When residual_depth_flag = 0 and the number of input views is even (2 or 4), base depth view (B) is the center of input views (camera ID=2.5).

Since the input depth views are originally sub-sampled to the half-size of the input texture views, the sub-residual depth views are sub-sampled to the quarter-size of input texture views. The residual depth view represents occluded area or out-of-frame area of the base depth view when it is projected to the N-1 input depth views.

1     2     3     4     5

(R-1)  (R-2)  B   (R-4)  (R-5)

camera ID

GVD depth

1       2       3       4

2.5

B

camera ID

(even)

GVD depth

wo/res.

- Figure H-3 – Relation between camera ID and GVD depth views

Base depth view is generated by GVD process projecting N-input depth views to the base depth view (camera ID=3) and taking the pixel-wise median of them.

Residual depth view is generated by GVD process encoding and decoding the base depth view and then projecting it to the other input depth views (camera ID=1,2, 4 and 5), and then extracting the un-covered area by the projected base depth view to sub-residual depth views and then packing them in a residual depth view.

Residual texture view is generated by GVD process projecting the base texture view to the other input texture views (camera ID=1, 2, 4 and 5) by the decoded base depth view, and then extracting the un-covered area in the other input texture views (camera ID=1, 2, 4 and5) by the projected base texture view into sub-residual texture views and then packing them in a residual texture view in the case of residual_depth_flag = 0.

In the case of residual_depth_flag = 1, residual texture view is generated by GVD process encoding and decoding the residual depth view, and then projecting the base texture view to the other input texture views (camera ID=1, 2, 4 and 5) by the decoded base depth view and the decoded residual depth views, and then extracting the un-covered area in the other input texture views (camera ID=1, 2, 4 and5) by the projected base texture view into sub-residual texture views and then packing them in a residual texture view.

[Ed. (CY): more informative text that may be present e.g., in a note needs to be provided later.]

- Patent rights declaration(s)

Disney Research Zurich may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).