| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0057 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0040 |

| Title: | MV-HEVC/SHVC HLS: On inter-layer picture selection in RPS and colPic |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

In the current spec, any dependent pictures can be selected in RPS irrelevant to dependency type or motion only dependent picture can be selected.  colPic is selected depend on dependency type but

it can be selected by either the conventional ref_idx or the new layer_id.  Thus the motion only dependent in RPS is inefficient and the colPic indication has redundancy.  This proposal proposes that 1) RPS doesn’t include motion only dependent picture and 2) alternative colPic indication is only used in the case that the colPic is motion only dependent picture (i.e. not included in RPS).

It is also proposed to change the representation of direct_dependency_type for shorter bits signalling.

# Introduction

RPS is used to distinguish reference picture for sample prediction by ref_idx and for motion prediction by col_ref_idx.  But when motion prediction dependency can be indicated by the alternative colPic, it is safe to say that RPS is mainly used for sample prediction. Thus we propose RPS doesn’t include motion only dependent picture to guarantee reference pictures referred by RPS/ref_idx have at least sample information.

Currently colPic indication functionality is redundant, specifically it can be indicated by either ref_idx or layer_id. Thus we propose that alternative colPic is only used in the case that the colPic is motion only dependent picture (i.e. not included in RPS).

Table 1 Relation between reference and direct_dependency_type

|  |  | direct_dependency type |  |  |
| --- | --- | --- | --- | --- |
|  |  | sample + motion | sample only | motion only |
| Current | RPS | Y | Y | Y (needed?) |
|  | alternative colPic | Y (needed?) |  | Y |

# Proposal

We propose

- RPS includes only pictures whose direct_dependency_type shows sample prediction dependency.

- alternative colPic is selected from pictures whose direct_dependency_type shows motion prediction dependency only (exclude sample prediction dependency)

Table 2 Relation between reference and direct_dependency_type

|  |  | direct_dependency type |  |  |
| --- | --- | --- | --- | --- |
|  |  | sample + motion | sample only | motion only |
| Proposal | RPS | Y | Y |  |
|  | alternative colPic |  |  | Y |

#Specificaly we count ActiveSamplePredRefLayerId and ActiveMotionPredRefLayerId as follows and RPS includes only picture whose layer_id is included in ActiveSamplePredRefLayerId.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveSamplePredRefLayers > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |

G.7.4.7.1 General slice segment header semantics

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.  When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to i.

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, NumActiveMotionPredRefLayers, and ActiveMotionPredRefLayerId[ j ] for each value of j in the range of 0 to NumActiveMotionPredRefLayers − 1, inclusive, are derived as follows:

for( i = 0, j = 0,k=0; i < NumActiveRefLayerPics; i++) {

RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

if( SamplePredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] )
		ActiveSamplePredRefLayerId[ k++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

else if( MotionPredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] )
		ActiveMotionPredRefLayerId[ j++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]
}
NumActiveMotionPredRefLayers = j

NumActiveSamplePredRefLayers = k

inter_layer_sample_pred_only_flag equal to 1 specifies that inter prediction is not used in decoding of the current picture. inter_layer_sample_pred_only_flag equal to 0 specifies that inter prediction may be used in decoding of the current picture. When not present, the value of inter_layer_sample_pred_only_flag is inferred to be equal to 0.

The variable InterRefEnabledInRPLFlag is derived as follows:.

–	If NumSamplePredRefLayers[ nuh_layer_id ] is greater than 0 and NumActiveSamplePredRefLayers is greater than 0, InterRefEnabledInRPLFlag is set equal to !inter_layer_sample_pred_only_flag.

–	Otherwise, InterRefEnabledInRPLFlag is set equal to 1

G.8.7.4.7.2 Reference picture list modification semantic

The specifications in subclause Error: Reference source not found apply with following modifications.

–	Equation (7-43) specifying the derivation of NumPocTotalCurr is replaced by:

NumPocTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++)
	if(UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		NumPocTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++)	(G-1)
	if(UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		NumPocTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ] )
		NumPocTotalCurr++
NumPocTotalCurr += NumSampleRefLayerPics

G.8.1.1 Decoding process for inter-layer reference picture set

for( i = 0; i < NumActiveSamplePredRefLayers; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
			nuh_layer_id equal to ActiveSamplePredRefPicLayerId[ i ] ) {
		RefPicSetInterLayer[ i ] = picX
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ] = "no reference picture"
}

G.8.5.1 Derivation process for temporal luma motion vector prediction

# This part is not changed. But counting method of ActiveMotionPredRefLayerId is changed by the proposal.

- If slice_temporal_mvp_enabled_flag is equal to 0, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the following ordered steps apply:

- Depending on the values of alt_collocated_indication_flag, collocated_ref_layer_idx, slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the collocated picture, is derived as follows:

- If alt_collocated_indication_flag is equal to 1, colPic is set equal to the picture in the current access unit with nuh_layer_id equal to ActiveMotionPredRefLayerId[ collocated_ref_layer_idx ].

# Conclusion

A dependency type usage related to inter-layer picture selection in RPS and alternative colPic is proposed in order to provide clear specification and to remove dependency. The change of direct_dependency_type representation is also proposed. It is recommended to adopt these techniques in the next SHVC and MV-HEVC WD and HM / HTM.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).