| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0060 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0043 |

| Title: | MV-HEVC/SHVC HLS: TemporalID alignment and inter-layer prediction restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yoshiya Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

The contribution proposes to introduce a flag inter_layer_tid_alignment_flag in VPS to indicate if TemporalID is aligned across layers.  It is also proposed that if the inter_layer_tid_alignment_flag is 1 and max_tid_il_ref_pics_plus1[0] is less than 7, max_tid_il_ref_pics_plus1[0] is used for common value of max_tid_il_ref_pics_plus1 across layers and inter-layer related signaling in slice segmentation header is not sent if the TemporalID of a layer is larger than the common max value.

Additionally in case the above proposal (Option1) is not agreed, it is proposed as Option2 to include a syntax general max_tid_il_ref_pics_plus1 for common value of max_tid_il_ref_pics_plus1

# Introduction

In the HEVC version1 spec, it is defined that the value of TemporalId shall be the same for all VCL NAL units of an access unit to guarantee all slice segments have the same TemporalID.  However the TemporalID alignment has not extended to apply for across layers.  Although the TemporalID alignment is beneficial to support the same level temporal scalable playback in each layers (SHVC) and to support synchronized temporal scalable playback for several layers (MV-HEVC), it could be achieved if the alignment is not required to allow more pictures for reference. Therefore it is proposed to introduce a flag inter_layer_tid_alignment_flag in VPS to indicate if TemporalID is aligned across layers.

When TemporalID is aligned across layers and the inter-layer prediction reference in specific TemporalID layers of base layer is restricted by max_tid_il_ref_pics_plus1[0], it is appropriate that all layer in the specific TemporalID layers is also restricted. Therefore it is also proposed that if inter_layer_tid_alignment_flag is 1 and max_tid_il_ref_pics_plus1[0] is less than 7, max_tid_il_ref_pics_plus1[0] is used for common max_tid_il_ref_pics_plus1 across layers and inter-layer related signaling in slice segmentation header is not sent if the TemporalID of a layer is larger than the common max value.

Because it is considered that the TemporalID base restriction is the main use-case for the signaling of inter-layer related signaling in slice segmentation header (for exsample pixtures of tid=0 refers 1 inter-layer picture while pictures of tid = 1 doesn’t refer inter-layer pictures), it is beneficial to define common value of max_tid_il_ref_pics_plus1 to remove that case’s useless signaling.

# Proposal

We propose two options

Option1

- introduce a syntax inter_layer_tid_alignment_flag to indicate TemporalID is aligned within an access unit.

- use base layer’s max_tid_il_ref_pics_plus1[ 0 ] for common max_tid_il_ref_pics_plus1 across layers if inter_layer_tid_alignment_flag is 1 and max_tid_il_ref_pics_plus1[ 0 ] is less than 7.

Option2

- TemporalID is always aligned within an access unit (not add inter_layer_tid_alignment_flag)

- introduce a syntax general_max_tid_il_ref_pics_plus1 to indicate max_tid_il_ref_pics_plus1[] in all layers is set equal to a common max value as general_max_tid_il_ref_pics_plus1.

# Option1

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| inter_layer_tid_alignment_flag | u(1) |
| max_tid_il_ref_pics_plus1[ 0 ] | u(3) |
| if (inter_layer_tid_alignment_flag && max_tid_il_ref_pics_plus1[ 0 ] < 7) |  |
| for( i = 1; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] = max_tid_il_ref_pics_plus1[ 0 ] |  |
| else for( i = 1; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| … |  |
| } |  |

inter_layer_tid_alignment_flag equal to 1 specifies that the value of TemporalId of an access unit is the same in all layers. When inter_layer_tid_alignment_flag equal to 1, it is a requirement of bitstream conformance that the value of TemporalId shall be the same for all VCL NAL units of an access unit.

max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ] – 1 are not used as reference for inter-layer prediction. When not present, max_tid_il_ref_pics_plus1[ i ] is unspecified.

When inter_layer_tid_alignment_flag equal to 1 and max_tid_il_ref_pics_plus1[ 0 ] is less than 7 specifies that within the CVS non-IRAP pictures with all nuh_layer_id in an access unit are not used as reference layer prediction.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| general_ max_tid_il_ref_pics_plus1 = (inter_layer_tid_alignment_flag && max_tid_il_ref_pics_plus1[ 0 ] < 7) ? max_tid_il_ref_pics_plus1[ 0 ] : 7 |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 && TemporalID <= general_ max_tid_il_ref_pics_plus1)) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |

# Option2

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| general_max_tid_il_ref_pics_plus1[ 0 ] |  |
| if (general_max_tid_il_ref_pics_plus1[ 0 ] < 7) |  |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] = max_tid_il_ref_pics_plus1[ 0 ] |  |
| else for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| … |  |
| } |  |

general_max_tid_il_ref_pics_plus1 equal to 0 specifies that within the CVS non-IRAP pictures are not used as reference for inter-layer prediction. general_max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that within the CVS pictures with TemporalId greater than general_max_tid_il_ref_pics_plus1[ i ] – 1 are not used as reference for inter-layer prediction.

max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ] – 1 are not used as reference for inter-layer prediction. When not present, max_tid_il_ref_pics_plus1[ i ] is unspecified.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 && TemporalID <= general_ max_tid_il_ref_pics_plus1)) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |

# Conclusion

The inter-layer TemporalID alignment flag and the inter-layer prediction restriction across layers in specific sublayers is proposed.  It is recommended to adopt these scheme in the next SHVC and MV-HEVC WD and HM / HTM.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).