Changes of E0046 are highlighted by Green.

H.8.5.2.2 Decoding process for inter prediction samples

…

  - Depending on ic_flag and nPbW, the arrays predSamplesL ,predSampleCb, and predSampleCr are derived as specified in the following:

    - If ic_flag is equal to 0, the following applies.

      - The array predSampleL of the prediction samples of luma component is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the luma location ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1 and cIdx equal to 0 given as input.

    - Otherwise ( ic_flag is equal to 1), the following applies.

      - The array predSampleL of the prediction samples of luma component is derived by invoking the illumination compensated sample prediction process specified in subclause Error: Reference source not found, with the luma location ( xC, yC ), the size of the current luma coding block nCS, the luma location ( xB, yB ), the width and the height of the current luma prediction block nPbW, nPbH, and the sample arrays predSamplesL0L and predSamplesL1L as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvL0, mvL1 and cIdx equal to 0 given as input.

    - If ic_flag is equal to 0 or nPbW is equal to or less than 8, the following applies.

      - The array predSampleCb of the prediction samples of component Cb is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW/2, nPbHCb set equal to nPbH/2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 1 given as input.

      - The array predSampleCr of the prediction samples of component Cr is derived by invoking the weighted sample prediction process specified in subclause 8.5.2.2.3 with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW/2, nPbHCr set equal to nPbH/2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, and cIdx equal to 2 given as input.

    - Otherwise (ic_flag is equal to 1 and nPbW is more than 8), the following applies.

      - The array predSampleCb of the prediction samples of component Cb is derived by invoking the illumination compensated sample prediction process specified in subclause Error: Reference source not found, with the luma location ( xC, yC ), the size of the current luma coding block nCS, with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCb set equal to nPbW/2, nPbHCb set equal to nPbH/2, and the sample arrays predSamplesL0Cb and predSamplesL1Cb as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 1 given as input.

      - The array predSampleCr of the prediction samples of component Cr is derived by invoking the illumination compensated sample prediction process specified in subclause Error: Reference source not found, with the luma location ( xC, yC ), the size of the current luma coding block nCS, with the chroma location ( xB/2, yB/2 ), the width and the height of the current chroma prediction block nPbWCr set equal to nPbW/2, nPbHCr set equal to nPbH/2, and the sample arrays predSamplesL0Cr and predSamplesL1Cr as well as predFlagL0, predFlagL1, refIdxL0, refIdxL1, mvCL0, mvCL1, and cIdx equal to 2 given as input.

…

H.8.5.2.2.5 Illumination compensated sample prediction process

Inputs to this process are:

- a location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left sample of the current picture,

- the size of current luma coding block nCS,

- a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

- the width and height of this prediction block, nPbW and nPbH,

- two (nPbW)x(nPbH) arrays predSamplesL0 and predSamplesL1,

- prediction list utilization flags, predFlagL0 and predFlagL1,

- reference indices, refIdxL0 and refIdxL1,

- motion vector mvL0 and mvL1

- colour component index, cIdx,

Outputs of this process are:

- the (nPbW)x(nPbH) array predSamples of prediction sample values.

Variables shift1, shift2, offset1 and offset2 are derived as follows.

- The variable shift1 is set equal to 14 − bitDepth and the variable shift2 is set equal to 15 − bitDepth,

- The variable offset1 is derived as follows.

  - If shift1 is greater than 0, offset1 set equal to 1 << ( shift1 − 1 ).

  - Otherwise (shift1 is equal to 0), offset1 is set equal to 0.

- The variable offset2 is set equal to 1 << ( shift2 − 1 ).

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

The derivation process for illumination compensation mode availability and parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of the current luma coding block nCS, prediction list utilization flags, predFlagL0 and predFlagL1, reference indices refIdxL0 and refIdxL1, motion vectors mvL0 and mvL1, the bit depth of samples, bitDepth, a variable cIdx specifying colour component index as the inputs and the outputs are the flags puIcFlagL0 and puIcFlagL1, the variables icWeightL0 and icWeightL1, the variables icOffsetL0 and icOffsetL1.

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x ][ y ] with x = 0..(nPbW)−1 and y = 0..(nPbH)−1 are derived as follows.

- If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

    - clipPredVal= Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL0[ x ][ y ] + offset1 ) >> shift1 )	(Error: Reference source not found-1)

    - predSamples[ x ][ y ] = !puIcFlagL0 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL0 ) >> 5 ) + icOffsetL0 ) 	(Error: Reference source not found-2)

- Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

    - clipPredVal = Clip3( 0, ( 1 << bitDepth ) − 1, ( predSamplesL1[ x ][ y ] + offset1 ) >> shift1 )	(Error: Reference source not found-3)

    - predSamples[ x ][ y ] = !puIcFlagL1 ? clipPredVal : 
( Clip3( 0, ( 1 << bitDepth ) − 1, ( clipPredVal * icWeightL1 ) >> 5 ) + icOffsetL1 ) 	(Error: Reference source not found-4)

- Otherwise,

    - predVal0 = !puIcFlagL0 ? predSamplesL0[ x ][ y ] : 
( ( predSamplesL0[ x ][ y ] * icWeightL0 ) >> 5  ) + ( icOffsetL0 << shift1 ) ) 	(Error: Reference source not found-5)

    - predVal1 = !puIcFlagL1 ? predSamplesL1[ x ][ y ] : 
( ( predSamplesL1[ x ][ y ] * icWeightL1 ) >> 5 ) + ( icOffsetL1 << shift1 ) ) 	(Error: Reference source not found-6)

    - predSamples[ x ][ y ] = 
Clip3( 0, ( 1 << bitDepth ) − 1, ( predVal0 + predVal1 + offset2 ) >> shift2 )	(Error: Reference source not found-7)

- H.8.5.2.2.5.1 Derivation process for illumination compensation mode availability and parameters

Inputs to this process are:

- a luma location ( xCl, yCl ) specifying the top-left sample of the current coding block relative to the top left sample of the current picture,

- the size of the current luma coding block, nCSl,

- prediction list utilization flags, predFlagL0 and predFlagL1,

- reference indices refIdxL0 and refIdxL1,

- motion vectors mvL0 and mvL1

- a bit depth of samples, bitDepth.

- a variable cIdx specifying colour component index.

Outputs of this process are:

- flags puIcFlagL0 and puIcFlagL1 specifying whether illumination compensation is enabled.

- variables icWeightL0 and icWeightL1 specifying weights for illumination compensation

- variables icOffsetL0 and icOffsetL1 specifying offsets for illumination compensation

The variables puIcFlagL0 and PuIcFlagL1 are set equal to 0, the variables icWeightL0 and icWeightL1 are set equal to 1, the variables icOffsetL0 and icOffsetL1 are set equal to 0 and the variables icShiftL0 and icShiftL1 are set equal to 0.

The variables nCS specifying the current luma or chroma coding block size, and the location ( xC, yC ) specifying the top left sample of the current luma or chroma coding block is derived as follows.

  - nCS = ( cIdx = = 0 ) ? nCSl : nCSl / 2 		(Error: Reference source not found-8)

  - ( xC, yC ) =  ( cIdx = = 0 ) ? ( xC, yC ) : ( xCl /2 , yCl / 2 ) 	(Error: Reference source not found-9)

The variable availFlagCurAboveRow specifying the availability of above neighbouring row samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCl, yCl ) and the neighbouring location ( xN, yN ) set equal to ( xCl, yCl−1 ) as the input and the output is assigned to availFlagCurAboveRow.

The variable availFlagCurLeftCol specifying the availability of left neighbouring column samples is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4.1 with the location ( xCurr, yCurr ) set equal to ( xCl, yCl ) and the neighbouring location ( xN, yN ) set equal to ( xCl−1, yCl ) as the input and the output is assigned to availFlagCurLeftCol.

[Ed. (GT) The availability derivation is specified as performed in the software. However, the check of the availability of left and above PU might not be sufficient to guarantee the availability of the whole left column or above row. A check of availability similar to that used for intra prediction might be a better solution. ]

When availFlagCurAboveRow is equal to 0 and availFlagCurLeftCol is equal to 0 the whole derivation process of this subclause terminates.

For X being replaced by 0 and 1, when predFlagLX is equal to 1, the variable puIcFlagLX is derived by the following ordered steps.

  - The variable refPicLX specifying the reference picture from reference picture list X is set equal to RefPicListX[ refIdxLX ] .

  - If the view order index of the picture refPicLX is not equal to ViewIdx, the variable puIvPredFlagLX specifying whether inter-view prediction from List X is utilized is set equal to 1, otherwise (predFlagLX is equal to 0 or the view order index of the picture RefPicListX[ refIdxLX ]  is equal to ViewIdx ), puIvPredFlagLX is set equal to 0.

  - If puIvPredFlagLX is equal to 0, the variable puIcFlagLX is set equal to 0, otherwise (puIvPredFlagLX is equal to 1 ) the following applies.

    - The luma location (xRLX, yRLX) specifying the top-left sample of the reference block in refPicLX is derived as

        - xRLX = xC + ( ( mvLX[ 0 ] + ( cIdx ? 4 : 2 ) ) >> (2 + (cIdx ? 1 : 0)) )	(Error: Reference source not found-10)

        - yRLX = yC + ( ( mvLX[ 1 ] + ( cIdx ? 4 : 2 ) ) >> (2 + (cIdx ? 1 : 0)) ) 	(Error: Reference source not found-11)

    - The variable availFlagAboveRowLX specifying whether the above neighbouring row samples of the current block and the reference block are available is derived as specified in the following.

        - availFlagAboveRowLX = ( yRLX > 0 ) && availFlagCurAboveRow 	 (Error: Reference source not found-12)

    - The variable availFlagLeftColLX specifying whether the left neighbouring column samples of the current block and the reference block are available is derived as specified in the following.

        - availFlagLeftColLX = ( xRLX > 0 ) && availFlagCurLeftCol	(Error: Reference source not found-13)

    - The variable puIcFlagLX is derived as follows.

        - puIcFlagLX = availFlagAboveRowLX | | availFlagLeftColLX 	 (Error: Reference source not found-14)

Depending on the colour component cIdx, the variable curRecSamples specifying the reconstructed picture samples of the current picture is derived as

    - curRecSamples = ( !cidx ) ? RecSamplesL : ( ( icdx == 1 ) ? RecSamplesCb : RecSamplesCr) 	(Error: Reference source not found-15)

- [Ed. (GT). The reconstructed samples before deblocking filter RecSamplesL, RecSamplesCb and RecSamplesCr as used above although not explicitly defined. However, they should be defined in the base spec. ]

- For X being replaced by 0 and 1, when puIcFlagLX is equal to 1, the variables icWeightLX, icOffsetLX are derived by the following ordered steps.

  - Depending on the colour component cIdx, the variable refRecSamples specifying the reconstructed picture samples of the reference picture is derived as specified in the following.

    - If cIdx is equal to 0, refRecSamples is set equal to reconstructed picture sample array SL of picture refPicLX.

    - Otherwise, if cIdx is equal to 1, refRecSamples is set equal to the reconstructed chroma sample array SCb of picture refPicLX.

    - Otherwise (cIdx is equal to 2), refRecSamples is set equal to the reconstructed chroma sample array SCr of picture refPicLX.

  - The lists curNeighSampleListLX and refNeighSampleListLX specifying the neighbouring samples in the current picture and the reference picture are derived as specified in the following.

    - The variable numNeighSamplesLX specifying the number of elements in curNeighSampleListLX and in refNeighSampleLX is set equal to 0.

    - The variable leftNeighOffLX specifying the offset of the left neighbouring samples in curNeighSampleListLX and refNeighSampleLX is derived as

        - leftNeighOffLX = availFlagAboveRowLX ? 0 : nCS 	(Error: Reference source not found-16)

    - For i ranging from 0 to nCS−1, inclusive the following applies.

      - When availFlagAboveRowLX is equal to 1 the following applies.

          - curNeighSampleListLX[ i ] = curRecSamples[ xC + i][ yC − 1 ] 	(Error: Reference source not found-17)

          - refNeighSampleListLX[ i ] = refRecSamples[ xRLX + i ][ yRLX − 1 ]	(Error: Reference source not found-18)

          - numNeighSamplesLX += 1		(Error: Reference source not found-19)

      - When availFlagLeftColLX is equal to 1 the following applies

          - curNeighSampleListLX[ i + leftNeighOffLX ] = curRecSamples[ xC − 1][ yC + i ] 	(Error: Reference source not found-20)

          - refNeighSampleListLX[ i + leftNeighOffLX ] = refRecSamples[ xRLX − 1][ yRLX + i ] 	(Error: Reference source not found-21)

          - numNeighSamplesLX += 1		(Error: Reference source not found-22)

  - The derivation process for illumination compensation parameters as specified in subclause  is invoked, with the list of neighbouring samples in the current picture curNeighSampleList, the list of neighbouring samples in the reference picture refNeighSample list, the number of neighbouring samples numNeighSamlesLX and the size of the current luma coding block nCSl as inputs and the illumination parameters icWeightLX and icOffsetLX as outputs.

- H.8.5.2.2.5.2 Derivation process for illumination compensation parameters

Inputs to this process are:

- a list curSampleList specifying the current samples, ,

- a list refSampleList specifying the reference samples,

- a variable numSamples specifying the number of elements in curSampleList and refSampleList.

- a bit depth of samples, bitDepth.

- the size of the current luma coding block nCSl

Outputs of this process are:

- a variable icWeight specifying a weight for illumination compensation,

- a variable icOffset specifying a offset for illumination compensation,

The variable icWeight is set equal to 1, the variable icOffset is set equal to 0.

- The variable precShift is set equal to Max( 0, bitDepth − 12 )

The variables sumRef, sumCur, sumRefSquare and sumProdRefCur are set equal to 0 and the following applies for i ranging from 0 to numSamples / 2 − 1, inclusive.

  - sumRef += refSampleList[ 2*i ]		(Error: Reference source not found-23)

  - sumCur += curSampleList[ 2*i ]		(Error: Reference source not found-24)

  - sumRefSquare += ( refSampleList[ 2*i ] * refSampleList[ 2*i ] ) >> precShift	(Error: Reference source not found-25)

  - sumProdRefCur += ( refSampleList[ 2*i ] * curSampleList[ 2*i ] ) >> precShift	(Error: Reference source not found-26)

  - -
		-

  - -

  - -

  - -

  - -

The variable avgShift and avgOffset are derived as follows:

  - avgShift = Log2( numSamples / 2 )		(Error: Reference source not found-33)

  - avgOffset = 1 << ( avgShift − 1 )		(Error: Reference source not found-34)

The variables numerDiv and denomDiv are derived as follows:

  - numerDiv= ( (sumProdRefCur + (sumRefSquare>>7)) << avgShift ) – (sumRef * sumCur)>> precShift	(Error: Reference source not found-35)

  - denomDiv= ( (sumRefSquare + (sumRefSquare>>7)) << avgShift ) – (sumRef * sumRef)>> precShift	(Error: Reference source not found-36)

  - numerDiv= Clip3(0, 2* denomDiv, numerDiv)

The variables psShiftNumer and psShiftDenom are derived as follows:

  - psShiftDenom = Max( 0, Floor( Log2( Abs( denomDiv ) ) ) − 5)	(Error: Reference source not found-37)

  - -

  - psShiftNumer = Max( 0, psShiftDenom − 12)	(Error: Reference source not found-39)

The variables psNumerDiv and psDenomDiv are derived as follows:

  - psNumerDiv = numerDiv >> psShiftNumer 		(Error: Reference source not found-40)

  - psDenomDiv = denomDiv >> psShiftDenom 		(Error: Reference source not found-41)

The variable psIcWeight and the variable invPsShift are derived as follows:

    - The variable divCoeff  is derived from Table Error: Reference source not found-1 depending on psDenomDiv.

    - psIcWeight = psNumerDiv * divCoeff 		(Error: Reference source not found-42

    - invPsShift = psShiftDenom – psShiftNumer + 15 – 5    	(Error: Reference source not found-43)

The variable icWeight is derived as follows:

      - -

      - icWeight = psIcWeight >> invPsShift	(Error: Reference source not found-45)

      - -

      - -

      - -

      - -

The variable icOffset is derived as follows:

      - icOffset = ( sumCur – ( ( icWeight*sumRef ) >>  5 ) + avgOffset ) >>  avgShift	(Error: Reference source not found-50)

Table Error: Reference source not found-1 – Specification of divCoeff depending on psDenomDiv

| psDenomDiv | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| divCoeff | 0 | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 | 2979 | 2731 |
| psDenomDiv | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| divCoeff | 2521 | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 | 1560 | 1489 | 1425 | 1365 | 1311 |
| psDenomDiv | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| divCoeff | 1260 | 1214 | 1170 | 1130 | 1092 | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 |
| psDenomDiv | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| divCoeff | 840 | 819 | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 | 643 |
| psDenomDiv | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |  |
| divCoeff | 630 | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 | 537 | 529 | 520 |  |