| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0062 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0048 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: access unit boundary detection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It has been confirmed in the previous JCT-VC meeting that it is desirable to allow access units where the base layer picture is not present for example to enable a base layer @ 30 Hz and a spatial or quality enhancement layer @ 60 Hz.

If there is no NAL unit that starts a new access unit (e.g. an access unit delimiter) present and also if there is no base layer picture present in the access unit, it is asserted that HEVC v1 decoders may consider the following coded enhancement layer pictures as a part of the previous access unit, while SHVC/MV-HEVC decoders are intended to consider them as part of a new access unit. Consequently, as the CPB operates on access unit basis, it is asserted that the HRD parameters may become ambiguous and may be interpreted differently by HEVC v1 decoders and SHVC/MV-HEVC decoders.

It is proposed to require the presence of the access unit delimiter NAL unit when there is no base layer picture present in the access unit.

# Proposed Specification Text

- 7.4.2.4.4	Order of NAL units and coded pictures and their association to access units

This subclause specifies the order of NAL units and coded pictures and their association to access unit for CVSs that conform to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses Error: Reference source not found through Error: Reference source not found.

An access unit consists of one or more coded pictures with different values of nuh_layer_id and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in subclause Error: Reference source not found.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

The first of any of the following NAL units after the last VCL NAL unit of a coded picture specifies the start of a new access unit:

–	access unit delimiter NAL unit (when present),

–	VPS NAL unit (when present),

–	SPS NAL unit (when present),

–	PPS NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 (when present),

–	NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 (when present),

–	first VCL NAL unit of a coded picture with nuh_layer_id equal to 0 (always present when an access unit delimiter NAL unit is not present).

When an access unit contains no VCL NAL unit with nuh_layer_id equal to 0, an access unit delimiter NAL unit shall be present in the access unit.

# Patent rights declaration(s)

The author is not aware of any current or pending patent rights relating to the technology described in this contribution.