| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0063 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0049 |
| --- | --- |

| Title: | REXT/MV-HEVC/SHVC HLS: auxiliary picture layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to specify new scalability dimension in the VPS extension syntax for a layer carrying auxiliary pictures. It is asserted that the proposed approach:

- Enables to indicate CPB HRD parameters separately for the primary pictures (i.e., the base layer) and for the entire bitstream;

- Enables to the bitstream extraction process to extract the bitstream containing the primary pictures only;

- Can be used in systems functionalities, such as session negotiation, as the presence of auxiliary pictures is indicated through the VPS; and

- Does not require any new VCL NAL unit type(s).

# Introduction

The support for auxiliary pictures could be realized using either of the following two approaches:

- Specifying new VCL NAL unit type(s) for carriage of auxiliary pictures. This approach was used in H.264/AVC.

- Specifying new scalability dimension for a layer carrying auxiliary pictures. This approach is proposed in this contribution.

It is asserted in this contribution that specifying auxiliary pictures as layers (approach 2) has the following benefits over approach 1:

- HEVC v1 provides mechanisms to provide CPB HRD parameters separately for the base layer only (containing only “texture” video) and for the entire bitstream (also containing auxiliary pictures). Hence, decoders can choose the CPB HRD parameters that suit best to their use. In other words, decoders can choose either to extract and decode the base-layer sub-bitstream only and use the HRD parameters indicated for the base layer in the decoding process, or to use the HRD parameters indicated for the entire bitstream. If new VCL NAL unit types are used for auxiliary pictures, the CPB HRD parameters always include both texture video and auxiliary pictures.

- The bitstream extraction process can be used to exclude auxiliary picture layers from the bitstream for those decoders, which are not able to make use of them.

- The presence of auxiliary layers can be indicated through VPS and hence be used in session negotiation and other systems functionalities.

- No new VCL NAL unit type(s) are needed.

It is therefore proposed to specify auxiliary pictures as a new scalability dimension as detailed in the next section.

Auxiliary picture layers cannot be used as inter-layer reference for other scalability layers, while can be used for inter-layer prediction of other auxiliary picture layers, which is useful for multiview depth coding.

In the Incheon meeting MPEG Requirements met jointly with JCT-VC and JCT-3V and concluded that the same mechanism is intended to be used for both alpha channel and depth map sequences. It is proposed to specify the auxiliary picture layer as well as the support for alpha channel and depth maps in the MV-HEVC specification.

# Proposal

## VPS Extension Semantics: Scalability dimension for auxiliary picture layer

Table Error: Reference source not found-1 – Mapping of ScalabiltyId to scalability dimensions

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| … |  |  |
| N | auxiliary | AuxId |
| (N+1)-15 | Reserved |  |

…

for ( i = 0; i  <=  vps_max_layers_minus1; i++) {

lId = layer_id_in_nuh[ i ]

for( smIdx= 0, j =0; smIdx < 16; smIdx ++ )

if( scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

ViewId[ lId ] = ScalabilityId[ i ][ 0 ]

AuxId[ lId ] = ScalabilityId[ i ][ N ]

ViewScalExtLayerFlag[ lId ] = ( ViewId[ lId ]  !=  ViewId[ 0 ] )

}

Value N would be the next available scalability_mask index value, currently 2.

...

It is a requirement of bitstream conformance that when AuxId[ RefLayerId[ nuhLayerIdA ][ j ] ] is greater than 0 for any values of nuhLayerIdA and j, AuxId[ nuhLayerIdA ] shall be greater than 0.

## SPS Extension: Indication of the type of auxiliary pictures

| sps_extension( ) { | Descriptor |
| --- | --- |
| aux_layer_idc | ue(v) |
| if( aux_layer_idc = = 1 ) |  |
| sps_alpha_plane_extension( ) |  |
| else if( aux_layer_idc = = 2 ) |  |
| sps_depth_extension( ) |  |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_extension_vui_parameters( ) |  |
| … |  |
| } |  |

aux_layer_idc equal to 0 indicates that this SPS is not an active layer SPS for any auxiliary picture layers. aux_layer_idc equal to 1 indicates that the syntax elements in the sps_alpha_plane_extension( ) syntax structure are valid for the layers with nuh_layer_id equal to nuhLayerIdA for which this SPS is the active layer SPS and AuxId[ nuhLayerIdA ] is greater than 0. aux_layer_idc equal to 2 indicates that the syntax elements in the sps_depth_extension( ) syntax structure are valid for the layers with nuh_layer_id equal to nuhLayerIdA for which this SPS is the active layer SPS and AuxId[ nuhLayerIdA ] is greater than 0.

In bitstreams conforming to this version of this Specification, aux_layer_idc shall be in the range of 0 to 2, inclusive. Although the value of aux_layer_idc shall be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of aux_layer_idc in the range of 0 to 255, inclusive, to appear in the syntax.

## SPS Alpha Plane Extension (example, for information only)

While the exact contents and syntax of the alpha plane and depth related syntax elements are subject to other contributions, an example based on JCTVC-M0175 is provided below as an example.

| sps_alpha_plane_extension( ) { | Descriptor |
| --- | --- |
| alpha_opaque_value | u(v) |
| alpha_transparent_value | u(v) |
| } |  |

A picture with nuh_layer_id equalto nuhLayerIdA is an associated picture for an auxiliary picture with nuh_layer_id equal to nuhLayerIdB, when all of the following apply:

- The pictures reside in the same access unit.

- ScalabilityId[ lIdxA ][ j ] is equal to ScalabilityId[ lIdxB ][ j ] for values of j in the range of 0 to 15, inclusive, that are not equal to N, when lIdxA is set equal to LayerIdxInVps[ nuhLayerIdA ] and lIdxB is set equal to LayerIdxInVps[ nuhLayerIdB ].

alpha_opaque_value specifies the sample value for which the collocated luma and chroma samples of the associated picture for the auxiliary picture are considered opaque for purposes of alpha blending.

alpha_transparent_value specifies the sample value for which the associated luma and chroma samples of the associated picture for the auxiliary picture are considered transparent for purposes of alpha blending.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).