| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0065r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0051r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On IDR picture constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted in this contribution that it would be beneficial to enable activation of layer SPSs at access units where some but not all layers contain an IDR picture for example to:

- Provide the encoder the flexibility to change coding modes controlled by syntax elements in the SPS separately for the enhancement layer than for the base layer, but not require the encoder to code an IDR picture across all layers when new active layer SPS is taken into use.

- Enable changing the spatial resolution of the enhancement layer, for example to reflect the resolution of the source pictures for encoding, without a need to code an IDR picture across all layers.

This contribution proposes to relax the constraint on having IDR pictures present on all layers of an access unit as follows:

- When an IDR picture has nuh_layer_id equal to 0, all other pictures in the same access unit shall be IDR pictures.

- IDR pictures with nuh_layer_id greater than 0 may be present in access unit where the picture with nuh_layer_id equal to 0 is a non-IDR picture.

The proposal is conceptually the same as alternative 1 of JCTVC-M207r1.

Revision 1 additionally proposes a change in POC value derivation. The solution proposed in revision 1 had an assumption that an IDR or a BLA picture in the base layer causes all pictures in enhancement layers to be marked as “unused for reference”. This behavior could be achieved for example by requiring that when an IDR or BLA picture has nuh_layer_id equal to 0, all other pictures in the same access unit shall be IDR or BLA pictures.

Revision 2 proposes a “PicOrderCntLayerDiff” design where:

- When an IDR picture has nuh_layer_id equal to 0, there are no constraints on the other pictures in the same access unit (i.e. the other pictures can be non-IRAP pictures).

- PicOrderCntVal alignment across layers is not required.

- A POC delta value is maintained per layer and used in the bumping process to determine an output order that applies across layers.

Revision 3 added a constraint the sum of POC and POC delta value shall be aligned across layers and proposed to re-count the POC delta when either the base layer or the enhancement layer contains an IRAP picture with noRaslOutputFlag equal to 1.

# PicOrderCntLayerDiff design (proposed in revision 2)

The text in red originates from JCTVC-N0066 (a layer-wise decoding start-up design). The PicOrderCntLayerDiff[ i ] values are initialized when the layer-wise decoding start-up is triggered. Another relation to the layer-wise decoding start-up is the detection of the first picture within a particular layer after the layer-wise start-up process has been started, which in JCTVC-N0066 is done with the FirstPicInLayerDecodedFlag[ layerIdA ] variable. While these two aspects are specified below in relation to JCVC-N0066, the text could be easily modified for any layer-wise start-up specification description. Otherwise this contribution is independent of the layer-wise start-up.

The text in green below is proposed in this contribution.

- 8.1	General decoding process

…

The decoding process is specified such that all decoders will produce numerically identical cropped decoded pictures. Any decoding process that produces identical cropped decoded pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

Variable NoClrasOutputFlag is set as follows.

–	If the current picture is an IRAP picture that is the first picture in the bitstream, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is a BLA picture with nuh_layer_id equal to 0, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable NoClrasOutputFlag equal to a value for an IRAP picture with nuh_layer_id equal to 0, the variable NoClrasOutputFlag is set equal to the value provided by the external means.

–	Otherwise, NoClrasOutputFlag is set equal to 0.

When NoClrasOutputFlag is equal to 1, the variable LayerInitialisedFlag[ i ] is set equal to 0 for all values of i from 0 to 63, inclusive, and the variable FirstPicInLayerDecodedFlag[ i ] is set equal to 0 for all values of i from 1 to 63, inclusive, and PicOrderCntLayerDiff[ i ] is set equal to 0 for all values of i from 0 to 63, inclusive.

- 8.3.1	Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, and for decoder conformance checking (see subclause C.5).

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

- When the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture.

- The variable prevPicOrderCntLsb is set equal to slice_pic_order_cnt_lsb of prevTid0Pic.

- The variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic.

- The variable PicOrderCntMsb of the current picture is derived as follows:

- If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, PicOrderCntMsb is set equal to 0.

- Otherwise, if FirstPicInLayerDecodedFlag[ layerIdA ] is equal to 0 for layerIdA equal to the nuh_layer_id value of the current picture and there is a picture with nuh_layer_id equal to layerIdB in the same access unit such that layerIdB is less than layerIdA, PicOrderCntMsb is set to the PicOrderCntMsb of the picture with nuh_layer_id equal to layerIdB in the same access unit.

- Otherwise, PicOrderCntMsb is derived as follows:

if( ( slice_pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − slice_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-1)
else if( (slice_pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (slice_pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as follows:

PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb	(8-2)

NOTE 1 – All IDR pictures will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one CVS, the PicOrderCntVal values for any two coded pictures having the same value of nuh_layer_id shall not be the same.

When the current picture has nuh_layer_id equal to layerId greater than 0 and either is an IRAP picture with NoRaslOutputFlag equal to 1 or is included in an access unit that contains an IRAP picture with NoRaslOutputFlag equal to 1 and nuh_layer_id equal to 0, PicOrderCntLayerDiff[ layerId ] is set as follows:

- Let blPOC be equal to PicOrderCntVal of the picture with NoRaslOutputFlag equal to 1 and nuh_layer_id equal to 0 in the current access unit.

- PicOrderCntLayerDiff[ layerId ] is set equal to blPOC – PicOrderCntVal.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(8-3)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-4)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

- F.8.1.1	Decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0

Each picture referred to in this subclause is a complete coded picture.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause . This needs to be invoked only for the first slice segment of a picture. It is a requirement of bitstream conformance that for any two pictures picA with nuh_layer_id equal to layerIdA and PicOrderCntVal equal to pocA and picB with nuh_layer_id equal to layerIdB and PicOrderCntVal equal to pocB in the same access unit, pocA + PicOrderCntLayerDiff[ layerIdA ] shall be equal to pocB + PicOrderCntLayerDiff[ layerIdB ]. It is a requirement of bitstream conformance that PicOrderCntVal shall remain unchanged within an access unit.

…

- F.13.5.2.4	"Bumping" process

The "bumping" process consists of the following ordered steps:

- The pictures that are first for output are selected as the ones having the smallest value of the sum PicOrderCntVal + PicOrderCntLayerDiff[ layerId ] (layerId being the nuh_layer_id of each picture) of all pictures in the DPB marked as "needed for output".

- These pictures are cropped, using the conformance cropping window specified in the active SPS for the picture with nuh_layer_id equal to 0 or in the active layer SPS for a non-zero nuh_layer_id value equal to that of the picture, the cropped pictures are output in ascending order of nuh_layer_id, and the pictures are marked as "not needed for output".

- Each picture storage buffer that contains a picture marked as "unused for reference" and that included one of the pictures that was cropped and output is emptied.

# Proposed specification text (revisions 0 and 1)

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| … |  |

- 8.3.1	Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, and for decoder conformance checking (see subclause C.5).

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

- When the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture.

- The variable prevPicOrderCntLsb is set equal to slice_pic_order_cnt_lsb of prevTid0Pic.

- The variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic.

- The variable PicOrderCntMsb of the current picture is derived as follows:

- If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and nuh_layer_id equal to 0, PicOrderCntMsb is set equal to 0.

- Otherwise, if the current picture has nuh_layer_id equal to nuhLayerIdA and there is a picture with nuh_layer_id equal to nuhLayerIdB in the same access unit such that nuhLayerIdB is less than nuhLayerIdA, PicOrderCntMsb is set equal to the PicOrderCntMsb of the picture with nuh_layer_id equal to nuhLayerIdB in the same access unit.

[Ed. (MH): Alternatively the condition could be applied only for the first picture of a CVS within a particular layer as follows:

Otherwise, if the current picture is the first picture within a CVS with nuh_layer_id equal to nuhLayerIdA and there is a picture with nuh_layer_id equal to nuhLayerIdB in the same access unit such that nuhLayerIdB is less than nuhLayerIdA, PicOrderCntMsb is set equal to the PicOrderCntMsb of the picture with nuh_layer_id equal to nuhLayerIdB in the same access unit.

]

- Otherwise, PicOrderCntMsb is derived as follows:

if( ( slice_pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − slice_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-1)
else if( (slice_pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (slice_pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as follows:

PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb	(8-2)

NOTE 3 – All IDR pictures will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one CVS, the PicOrderCntVal values for any two coded pictures shall not be the same.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(8-3)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-4)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive.

NOTE 4 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).