| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0067r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0053r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: on associating ViewId with nuh_layer_id and camera position |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland Li Chen University of Science and Technology of China Hefei, P. R. China | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes:

- To derive ViewNum as a scalability dimension instead of ViewId on the basis of the VPS extension syntax. ViewNum is ascending in the decoding order of views within an access unit.

- The association of ViewNum with ViewId is proposed to be indicated in the picture parameter set (PPS). The ViewId values may indicate a relative view position in a 1D camera setup and can therefore be used in 3D-HEVC for motion vector scaling.

- For non-base views, it is proposed that reference picture lists construction applies to temporal pictures with the same ViewId value as that of the current picture.

It is asserted that:

- The contribution enables the simultaneous use of non-zero ViewId value for the base view and splitting_flag equal to 1, which is reportedly not possible with the current MV-HEVC specification.

- Related to item 3 above, the BD-rate achieved with a presented coding scheme is -3.1% on average when compared to MV-HEVC 2-view common test conditions. This coding gain is achieved by alternating the view for which a CRA picture is coded as well as the inter-view prediction direction.

# Introduction

Indication of non-zero view_id values for the base view, which is needed in 3D-HEVC, was resolved in the JCT3V-D meeting by including dimension_id[ i ][ j ] also for the base layer. However, this was found in the editing period to be incompatible with splitting_flag equal to1. In other words, when a non-zero view_id value for the base view is needed, splitting_flag cannot be set equal to 1, because otherwise the nuh_layer_id value for the base view would be non-zero (which does not conform to HEVC v1). The problematic part of the syntax is reprinted below for convenience.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag && i > 0 ) [Ed. (JB): syntax is not compatible with SHVC, or use of splitting_flag.] |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| … |  |

It is a common practice to use unequal quality between the views in stereoscopic video, which is utilized for example in the JCT-3V common test conditions (JCT3V-C1100). It is preferred to encode the base view with a higher quality, as it serves therefore as a good-quality inter-view reference picture.

Related to asymmetric quality between views, the following has been reported in [W.J. Tam, “Image and depth quality of asymmetrically coded stereoscopic video for 3D-TV”, JVT-W094]:

One issue with using asymmetrically coded stereoscopic images as a display format for 3D-TV is that for an unvarying presentation of reduced-quality images to one eye, depending on the individual, that eye might actually be his or her dominant eye.  In this case, the masking effect will not be optimal and might not even take place.  Another major concern is that the continuous presentation of degraded images to one eye might have an undesirable long-term effect on viewers.  One obvious solution is to present images with the reduced quality alternately to the left and the right eyes so that the image quality is distributed over both eyes in a balanced manner over time.

# Overview of the Proposal

This contribution proposes:

- To derive ViewNum as a scalability dimension instead of ViewId on the basis of the current VPS extension syntax. In 3D-HEVC, if a texture picture and a depth picture have the same ViewNum value in an access unit, they represent the same view. However, ViewNum values do not necessarily indicate a physical camera/view order or a view decoding order. The base view that the HEVC version 1 decoder decodes has ViewNum equal to 0.

- The association of ViewNum to ViewId is proposed to be indicated in the picture parameter set (PPS). The ViewId values may indicate a relative view position in a 1D camera setup and can therefore be used in 3D-HEVC for motion vector scaling. The ViewId value of the layer with nuh_layer_id equal to 0 is not constrained. As the association resides in the PPS, it is possible to switch the base view (with nuh_layer_id equal to 0) to represent a different viewpoint and hence balance the image quality over both eyes over time.

- For non-base views, it is proposed that reference picture lists construction applies to temporal pictures with the same ViewId value as that of the current picture.

Proposal no. 3 enables to code a subset of the views in one access unit as CRA pictures and change the inter-view prediction to happen from those views for which CRA pictures are coded. Consequently, proposal no. 3 enables stepwise random access to the bitstream. The following figure illustrates proposal no. 3.

Figure 1. Alternating the view for which CRA picture is coded as well as the inter-view prediction direction.

A random access picture interval of 24 pictures is assumed and in each random access position the association of ViewNum to ViewId values is changed. The access units are listed in output order from left to right. The top row of pictures represents one camera/view (e.g. ViewId equal to 0) and the bottom row of pictures another camera/view (e.g. ViewId equal to 1). Hierarchical temporal scalability using a dyadic coding hierarchy is assumed, but not all access units are included in the figure for convenience.

In access units with POC 0 to 16, inclusive, the top view has ViewNum equal to 0 and the bottom view is inter-view-predicted from the top view and has ViewNum equal to 1. In access units with POC 17 to 40, inclusive, the bottom view has ViewNum equal to 0 and the top-view is inter-view-predicted from the bottom view and has ViewNum equal to 1.

# Simulation Results

Simulations were performed with HTM-DEV-1.0 using the MV-HEVC common test conditions (JCT3V-D1100) to produce a stereoscopic bitstream. In the tested bitstreams, only one of the views had a CRA picture according to the intra picture period of the common conditions and the ViewNum to ViewId association was switched at each CRA picture position. The following table summarizes the results:

Table 1. Experiment with alternating the view for which a CRA picture is coded as well as the inter-view prediction direction compared to MV-HEVC 2-view common conditions.

# Proposed Specification Text

The specification text changes relative to MV-HEVC (JCT3V-D1004_v4) are presented below in red font.

- F.7.3.2.1.1	Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 10; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag && i > 0 ) [Ed. (JB): syntax is not compatible with SHVC, or use of splitting_flag.] |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |

- F.7.4.3.1.1	Video parameter set extension semantics

…

scalability_mask[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table Error: Reference source not found-2 are present. scalability_mask[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table Error: Reference source not found-2 – Mapping of ScalabiltyId to scalability dimensions

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | multiview | ViewNumViewId |
| 1-15 | Reserved |  |

…

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer and the variable ViewNumViewId[ layer_id_in_nuh[ i ] ] specifying the view numberidentifier of the i-th layer, NumViews specifying the number of unique ViewNum values, ViewNumList[ viewIdx ] specifying the viewIdx-th view number where viewIdx is in the range of 0 to NumViews – 1, inclusive, and the variable ViewScalExtLayerFlag specifying whether the i-th layer is a view scalability extension layer are derived as follows:

NumViews = 0
for ( i = 0; i  <=  vps_max_layers_minus1; i++) {

lId = layer_id_in_nuh[ i ]

for( smIdx= 0, j =0; smIdx < 16; smIdx ++ )

if( ( i  !=  0 )  &&  scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

ViewNumViewId[ lId ] = ScalabilityId[ i ][ 0 ]

ViewScalExtLayerFlag[ lId ] = ( ViewNum[ lId ] > 0 )( ViewId[ lId ]  !=  ViewId[ 0 ] )

newViewFlag = 1

for( k = i – 1; k >= 0; k-- )

if( ScalabilityId[ i ][ 0 ]  = =  ScalabilityId[ k ][ 0 ] )

newViewFlag = 0

if( newViewFlag )
		ViewNumList[ NumViews ] = ViewNum[ lId ]

NumViews += newViewFlag
}

When NumViews is greater than 1, it is a requirement of bitstream conformance that ViewNumList[ viewIdx – 1 ] is smaller than ViewNumList[ viewIdx ] for viewIdx in the range of 1 to NumViews – 1, inclusive.

- F.7.x.x.x	Picture parameter set extension

The picture parameter set extension is proposed to include the following syntax structure:

| view_num_to_view_id( ) { | Descriptor |
| --- | --- |
| num_views | ue(v) |
| for( i = 0; i < num_views; i++ ) { |  |
| view_id[ i ] | ue(v) |
| } |  |
| } |  |

num_views specifies the number of unique ViewNum values. num_views shall be equal to NumViews.

view_id[ i ] specifies the ViewId value for view number equal to ViewNumList[ i ].

It is a requirement of bitstream conformance that num_views and view_id[ i ] are identical in the active PPS and all active layer PPSs for pictures in the same access unit.

- G.8.3.4	Decoding process for reference picture lists construction

… (add at the end of the sub-clause)

When ViewScalExtLayerFlag[ lId ] is equal to 1, RefPicListX[ rIdxLX ] for each value of rIdxLX in the range of 0 to num_ref_idx_lX_active_minus1, inclusive, for X equal to 0 and 1, is replaced by the picture in the same access unit as RefPicListX[ rIdxLX ] having ViewId value equal to that of the current picture.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The authors are not aware of University of Science and Technology of China having any current or pending patent rights relating to the technology described in this contribution.