| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0085v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0057v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On parameter sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Ying Chen Krishna Rapaka 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | -858-651-8345 yekuiw@qti.qualcomm.com cheny@qti.qualcomm.com krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document includes various proposals and discussions related to parameter sets. Firstly, suggestions and discussions for several general topics are presented. Secondly, some specific technical proposals on vps_extension_offset semantics, signaling of scalability dimension identifier and view identifier, signaling of timing and HRD information in VUI, and signaling of bitrate and picture rate for operation points in VPS are proposed. Lastly, pure editorial improvements for the current MV-HEVC specification are provided. The proposed changes are included in the attachment of this document, with changes marked in relative to JCT3V-D1004v4.

In JCTVC-N0085v2/JCT3V-E0057v2, a design of a VUI-like section in the VPS extensoin is provided, with the bit rate and picture rate information included.

# General topics

- Add a restriction "The value of nuh_layer_id of a VPS NAL unit shall be equal to 0."

- (for discussion) Should SPS/PPS IDs with different values of nuh_layer_id share the same value space or have different value spaces? Because different layers may share the same SPS/PPS, it should be better to let them share the same value space.

- (for discussion) VUI includes information such as sample aspect ratio, over scan, source video format (PAL/NTSC etc., sample value range, source color format), field/frame information, bitstream restrictions (including cross-layer bitstream restrictions). Most of such information, including cross-layer bitstream restrictions, is really not layer specific and is the same for all layers. Thus it is just awkward to not have such VUI information signalled in the VPS that naturally applies to all layers. We thus suggest having a general discussion on this, to decide whether something should be done to enable signalling of the above-mentioned VUI information in the VPS.

# Semantics of vps_extension_offset

The semantics of vps_extension_offset is as follows:

vps_extension_offset specifies the byte offset of the next set of fixed-length coded information in the VPS NAL unit, starting from the beginning of the NAL unit.

NOTE –VPS information for non-base layer or view starts from a byte-aligned position of the VPS NAL unit, with fixed-length coded information that is essential for session negotiation and/or capability exchange. The byte offset specified by vps_extension_offset would then help to locate and access those essential information in the VPS NAL unit without the need of entropy decoding, which may not be equipped with some network entities that may desire to access only the information in the VPS that is essential for session negotiation and/or capability exchange.

It is not absolutely clear whether emulation prevention bytes are counted. We propose to clearly specify that emulation prevention bytes are counted. The changed semantics is as follows (the note remains unchanged):

vps_extension_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the next set of fixed-length coded information starting from avc_base_layer_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.

# Signalling of scalability dimension ID and view ID

Currently, the syntax for signalling of scalability dimension ID allows for the values of the base layer, i.e. dimension_id[ 0 ][ j ] (for any j) to be greater than 0. However, as said by some editor's notes, the syntax is not compatible with SHVC or the use of splitting_flag equal to 1. To keep a clean architecture for the common syntax, we should not enable any non-zero scalability dimension ID for the base layer. For MV-HEVC, to enable non-zero view ID for the base layer and the signalling of view ID allocation in the VPS, we should rather make another design that does not mingle and confuse the scalability dimension ID syntax, which is a fundamental part of the common syntax for all HEVC multi-layer extensions.

Therefore, we propose a set of syntax and semantics changes that is summarized as follows:

- For the base layer, dimension_id[ i ][ j ] is not signalled but derived to be equal to 0.

- View IDs can be explicitly signalled in the VPS. When not signalled, they are derived as equal to the scalability dimension ID values of the multiview dimension type (type 0), i.e. ScalabilityId[ i ][ 0 ] with i being the layer index.

- The scalability dimension ID value of the multiview dimension type (type 0) represents view order index instead of view ID.

The exact changes are included in the attachment. For convenience, they are also included below. Note that changes marked as by "Ye-Kui Wang editorial" are purely editorial changes.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| view_id_explicitly_signalled_flag | u(1) |
| if( view_id_explicitly_signalled_flag ) { |  |
| view_id_len_minus1 | u(4) |
| view_id_present_for_all_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) |  |
| if( view_id_present_for_all_layers_flag ¦ ¦ ( i % 2 = = 0 ) ) |  |
| view_id_val[ i ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| ... |  |

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table F-1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present

Table F-1 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Multiview | View order index |
| 1-15 | Reserved |  |

[Ed. (YK): Value 1 for spatial/SNR scalability should also be included in the common base text.]

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

When splitting_flag is equal to 1, the following applies:

–	The variable dimBitOffset[ 0 ] is set equal to 0 and for j in the range of 1 to NumScalabilityTypes − 1, inclusive, dimBitOffset[ j ] is derived as follows:

dimBitOffset[ j ]=dimIdx=0j−1dimension_id_len_minus1[ dimIdx ]+1	(F-1)

[Ed. (YK): The font style in the equation needs to be updated.]

–	The value of dimension_id_len_minus1[ NumScalabilityTypes − 1 ] is inferred to be equal to 5 − dimBitOffset[ NumScalabilityTypes − 1 ].

–	The value of dimBitOffset[ NumScalabilityTypes ] is set equal to 6.

It is a requirement of bitstream conformance that dimBitOffset[ NumScalabilityTypes − 1 ] shall be less than 6.

vps_nuh_layer_id_present_flag equal to 1 specifies that layer_id_in_nuh[ i ] for i from 1 to vps_max_layers_minus1, inclusive, are present. vps_nuh_layer_id_present_flag equal to 0 specifies that layer_id_in_nuh[ i ] for i from 1 to vps_max_layers_minus1, inclusive, are not present.

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. For i in the range of 0 to vps_max_layers_minus1, inclusive, when layer_id_in_nuh[ i ] is not present, the value is inferred to be equal to i.

When i is greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

[Ed. (MH): When splitting_flag is equal to 1, the MSBs of layer_id_in_nuh should be required to be 0 if the total number of bits in segments is less than 6. (YK): Since dimBitOffset[ NumScalabilityTypes ] is equal to 6, it is not possible for the total number of bits in segments to be less than 6. Even possible, why the MSBs (which I guess form the assigned segments) must be 0?]

For i from 0 to vps_max_layers_minus1, inclusive, the variable LayerIdxInVps[ layer_id_in_nuh[ i ] ] is set equal to i.

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

If splitting_flag is equal to 1, for i from 0 to vps_max_layers_minus1, inclusive, and j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ i ][ j ] is inferred to be equal to ( ( layer_id_in_nuh[ i ] & ( (1  <<  dimBitOffset[ j + 1 ] ) − 1) )  >>  dimBitOffset[ j ] ).

Otherwise, for j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ 0 ][ j ] is inferred to be equal to 0.

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer and the variable ViewScalExtLayerFlag specifying whether the i-th layer is a view scalability extension layer are derived as follows:

for ( i = 0; i  <=  vps_max_layers_minus1; i++) {

lId = layer_id_in_nuh[ i ]

for( smIdx= 0, j =0; smIdx < 16; smIdx ++ )

if( scalability_mask_flag[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

ViewScalExtLayerFlag[ lId ] = ( ScalabilityId[ i ][ 0 ] > 0 )

}

view_id_explicitly_signalled_flag equal to 1 specifies that the view identifiers are explicitly assigned for some or all layers specified by the VPS. view_id_explicitly_signalled_flag equal to 0 specifies that the view identifiers are not explicitly assigned for the layers specified by the VPS.

view_id_len_minus1 plus 1 specifies the length, in bits, of the view_id_val[ i ] syntax element.

view_id_present_for_all_layers_flag equal to 1 specifies that the view identifier for each layer specified by the VPS is explicitly signalled. view_id_present_for_all_layers_flag equal to 0 specified that the view identifiers are explicitly signalled for some layers specified by the VPS and derived for other layers specified by the VPS.

view_id_val[ i ] specifies the view identifier of the i-th layer specified by the VPS. The view_id_val[ i ] syntax element is repsented by view_id_len_minus1 + 1 bits.

When view_id_explicitly_signalled_flag is equal to 1, view_id_present_for_all_layers_flag is equal to 0, and i % 2 is equal to 1, the value of view_id_val[ i ] is inferred to be equal to view_id_val[ i − 1 ].

When view_id_explicitly_signalled_flag is equal to 0, the value of view_id_val[ i ] is inferred to be equal to ScalabilityId[ i ][ 0 ].

For each layer with nuh_layer_id equal to nuhLayerId, the variable ViewId is set equal to view_id_val[ LayerIdxInVps[ nuhLayerId ] ]. Each picture in a layer is considered to be associated with the ViewId of the layer.

[Ed. (YK): For SHVC with the value of view_id_info_present_flag can be required to be equal to 0. In a potential extension of MV-HEVC that includes depth, the texture and depth components of one view would be two neighbouring layers and have the same view_id. When texture and depth are always paired, setting view_id_info_present_flag equal to 1 and view_id_present_for_all_layers_flag equal to 0 would be desirable.]

# No timing and HRD information in VUI for SPS with nuh_layer_id > 0

Timing and HRD information is provided in the VPS for any layer set that contains a non-base layer. Therefore, it does not make sense to signal timing and HRD information in the VUI part of any SPS with nuh_layer_id greater than 0.

When an SPS with nuh_layer_id equal to 0 is applied to a layer with nuh_layer_id greater than 0, the timing and HRD information in the SPS is ignored.

The proposed syntax and semantics changes are as follows:

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ... |  |
| if ( nuh_layer_id = = 0 ) |  |
| vui_timing_info_present_flag | u(1) |
| if( vui_timing_info_present_flag ) { |  |
| vui_num_units_in_tick | u(32) |
| vui_time_scale | u(32) |
| vui_poc_proportional_to_timing_flag | u(1) |
| if( vui_poc_proportional_to_timing_flag ) |  |
| vui_num_ticks_poc_diff_one_minus1 | ue(v) |
| vui_hrd_parameters_present_flag | u(1) |
| if( vui_hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| ... |  |

vui_timing_info_present_flag equal to 1 specifies that vui_num_units_in_tick, vui_time_scale, vui_poc_proportional_to_timing_flag, and vui_hrd_parameters_present_flag are present in the vui_parameters( ) syntax structure. vui_timing_info_present_flag equal to 0 specifies that vui_num_units_in_tick, vui_time_scale, vui_poc_proportional_to_timing_flag, and vui_hrd_parameters_present_flag are not present in the vui_parameters( ) syntax structure. When not present, the value of vui_timing_info_present_flag is inferred to be equal to 0.

# Signalling of bit rate and picture rate information for session negotiation

Currently, while the information of profile, tier, and level that is important for session negotiation and content selection is signalled in the VPS, other information that is also important for the same purpose, such as bitrate and picture rate, is not. Signalling of bit rate and picture rate information is supported in SVC and MVC in the scalability information SEI message and the view scalability information SEI message, respectively. In HEVC multi-layer extensions, a lot of those information important for session negotiation conveyed in the scalability information SEI message and the view scalability information SEI message (of the AVC extensions) are now included in the VPS.

We thus propose to signal bitrate and picture rate information in the VPS. The semantics of such information are aligned with their counterparts in systems specifications such as the ISO base media file format and its extension file formats.

The proposed syntax and semantics changes are as follows (and also included in the attachment):

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| bit_rate_present_vps_flag | u(1) |
| pic_rate_present_vps_flag | u(1) |
| if( bit_rate_present_vps_flag ¦ ¦ pic_rate_present_vps_flag ) |  |
| for( i = 0; i <= vps_number_layer_sets_minus1; i++ ) |  |
| bit_rate_pic_rate( ) |  |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| } |  |

bit_rate_present_vps_flag equal to 1 or pic_rate_present_vps_flag equal to 1 specifies that a bit_rate_pic_rate( ) syntax structure is present for each layer set in the VPS. bit_rate_present_vps_flag equal to 0 and pic_rate_present_vps_flag equal to 0 specifies that no bit_rate_pic_rate( ) syntax structure is present in the VPS.

| bit_rate_pic_rate( ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| if( bit_rate_present_vps_flag ) |  |
| bit_rate_present_flag[ i ] | u(1) |
| if( pic_rate_present_vps_flag ) |  |
| pic_rate_present_flag[ i ] | u(1) |
| if( bit_rate_present_flag[ i ] ) { |  |
| avg_bit_rate[ i ] | u(16) |
| max_bit_rate[ i ] | u(16) |
| } |  |
| if( pic_rate_present_flag[ i ] ) { |  |
| constant_pic_rate_idc[ i ] | u(2) |
| avg_pic_rate[ i ] | u(16) |
| } |  |
| } |  |
| } |  |

bit_rate_present_flag[ i ] equal to 1 specifies that the bit rate information for the i-th subset of the layer set is present. bit_rate_present_flag[ i ] equal to 0 specifies that the bit rate information for the i-th subset of the layer set is not present. The i-th subset of a layer set is the output of the sub-bitstream extraction process when it is invoked with the layer set, i, and the layer identifier list associated with the layer set as inputs. When not present, the value of bit_rate_present_flag[ i ] is inferred to be equal to 0.

pic_rate_present_flag[ i ] equal to 1 specifies that picture rate information for the i-th subset of the layer set is present. pic_rate_present_flag[ i ] equal to 0 specifies that picture rate information for the i-th subset of the layer set is not present. When not present, the value of pic_rate_present_flag[ i ] is inferred to be equal to 0.

avg_bit_rate[ i ] indicates the average bit rate of the i-th subset of the layer set, in bits per second. The value is given by BitRateBPS( avg_bit_rate[ i ] ) with the function BitRateBPS( ) being specified by

BitRateBPS( x ) = ( x & ( 214 − 1 ) ) * 10( 2 + ( x  >>  14 ) )	(F-X)

The average bit rate is derived according to the access unit removal time specified in Annex Error: Reference source not found. In the following, bTotal is the number of bits in all NAL units of the i-th set of the layer set, t1 is the removal time (in seconds) of the first access unit to which the VPS applies, and t2 is the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies.

With x specifying the value of avg_bit_rate[ i ], the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

( x & ( 214 − 1 ) )  = =  Round( bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) ) )	(F-X)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

( x & ( 214 − 1 ) )  = =  0		(F-X)

max_bit_rate_layer[ i ] indicates an upper bound for the bit rate of the i-th subset of the layer set in any one-second time window of access unit removal time as specified in Annex Error: Reference source not found. The upper bound for the bit rate in bits per second is given by BitRateBPS( max_bit_rate_layer[ i ] ). The bit rate values are derived according to the access unit removal time specified in Annex Error: Reference source not found. In the following, t1 is any point in time (in seconds), t2 is set equal to t1 + 1 ÷ 100, and bTotal is the number of bits in all NAL units of access units with a removal time greater than or equal to t1 and less than t2. With x specifying the value of max_bit_rate_layer[ i ], the following condition shall be obeyed for all values of t1:

( x & ( 214 − 1 ) )  >=  bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) )	(F-X)

constant_pic_rate_idc[ i ] indicates whether the picture rate of the i-th subset of the layer set is constant. In the following, a temporal segment tSeg is any set of two or more consecutive access units, in decoding order, of the i-th subset of the layer set, fTotal( tSeg ) is the number of access units in the temporal segment tSeg, t1( tSeg ) is the removal time (in seconds) of the first access unit (in decoding order) of the temporal segment tSeg, t2( tSeg ) is the removal time (in seconds) of the last access unit (in decoding order) of the temporal segment tSeg, and avgFR( tSeg ) is the average picture rate in the temporal segment tSeg, which is given by:

avgFR( tSeg )  = =  Round( fTotal( tSeg ) * 256 ÷ ( t2( tSeg ) − t1( tSeg ) ) )	(F-X)

If the i-th subset of the layer set only contains one or two access units or the value of avgFR( tSeg ) is constant over all the temporal segments, the picture rate is constant; otherwise, the picture rate is not constant.

constant_pic_rate_idc[ i ] equal to 0 indicates that the picture rate of the i-th subset of the layer set is not constant. constant_pic_rate_idc[ i ] equal to 1 indicates that the picture rate of the i-th subset of the layer set is constant. constant_pic_rate_idc[ i ] equal to 2 indicates that the picture rate of the i-th subset of the layer set may or may not be constant. The value of constant_pic_rate_idc[ i ] shall be in the range of 0 to 2, inclusive.

avg_pic_rate[ i ] indicates the average picture rate, in units of picture per 256 seconds, of the i-th subset of the layer set. With fTotal being the number of access units in the i-th subset of the layer set, t1 being the removal time (in seconds) of the first access unit to which the VPS applies, and t2 being the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies, the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

avg_pic_rate[ i ]  = =  Round( fTotal * 256 ÷ ( t2 − t1 ) )	(F-X)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

avg_pic_rate[ i ]  = =  0		(F-X)

# Editorial changes

Besides the above technical proposals, some editorial changes are also proposed and included in the attachment of this document, marked as by "Ye-Kui Wang editorial".

# VUI-like VPS section

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag && i > 0 ) [Ed. (JB): syntax is not compatible with SHVC, or use of splitting_flag.] |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) |  |
| vps_vui( ) |  |
| } |  |

vps_vui_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the set of fixed-length coded information starting from bit_rate_present_vps_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.

vps_vui_present_flag equal to 1 specifies that the vps_vui( ) syntax structure is present in the VPS. vps_vui_present_flag equal to 0 specifies that the vps_vui( ) syntax structure is not present in the VPS.

| vps_vui( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_vui_byte_alignment_reserved_one_bit | u(1) |
| bit_rate_present_vps_flag | u(1) |
| pic_rate_present_vps_flag | u(1) |
| if( bit_rate_present_vps_flag ¦ ¦ pic_rate_present_vps_flag ) |  |
| for( i = 0; i <= vps_number_layer_sets_minus1; i++ ) |  |
| bit_rate_pic_rate( ) |  |
| } |  |

vps_vui_byte_alignment_reserved_one_bit shall be equal to 1.

The semantics of bit_rate_present_vps_flag and pic_rate_present_vps_flag, and the syntax and semantics of the syntax struture bit_rate_pic_rate( ) are the same as in Section 5 of JCTVC-N0085v2.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).