| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0091 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0059 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: DPB-related parameters in SPS and VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes to signal the syntax elements sps_max_dec_pic_buffering_minus1 for each output layer set in the VPS, and the syntax elements sps_max_num_reorder_pics and sps_max_latency_increase_plus1 for each layer in the VPS, and not signal these syntax elements for SPS with non-zero nuh_layer_id.

# Introduction and problem statement

In HEVC version 1, the parameters that are related to the decoded picture buffer (DPB) operations – max_dec_pic_buffering_minus1, max_num_reorder_pics, and max_latency_increase_plus1 are signalled in the SPS and in the VPS. The current design allows signaling for each sub-layer - if sub_layer_ordering_info_present_flag is equal to 1, then only one value is signaled for each of the syntax elements and inferred to be the same for all temporal sub-layers; otherwise (sub_layer_ordering_info_present_flag is equal to 0) the values are explicitly signaled for each sub-layer. The signaling of these parameters in the SPS is shown in the table below; the signaling is similar in the VPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| … |  |

In the multi-layer context (e.g. SHVC or MV-HEVC), the number of layer sets and output layers sets are more than one in most cases. Moreover, the DPB size should be signaled for each output layer set. By signaling the DPB size in the SPS, it is unclear how the value of the DPB size for an output layer set would be calculated from DPB sizes signaled in the various active layer SPSs. The DPB size is a cross-layer property and, therefore, should be signaled in the VPS.

There is one another reason to move the signaling of the DPB parameters for SPS with nuh_layer_id greater than 0 into the VPS. The syntax element sps_max_sub_layers_minus1 is only signaled for the SPS with nuh_layer_id equal to 0. For any SPS with non-zero layer ID, the value of sps_max_sub_layers_minus1 is inferred to be equal to vps_max_sub_layers_minus1. Since the signaling of the DPB parameters in the SPS is dependent on the value of sps_max_sub_layers_minus1, there is a parsing dependency for those SPSs whose nuh_layer_id is non-zero.

# Proposal

It is proposed in this document that the DPB-related parameters are signalled in the VPS, and that the DPB-related parameters are not signaled for those SPS that have non-zero nuh_layer_id. Moreover, it is also proposed that for those layers that have non-zero nuh_layer_id which refer to an SPS that has nuh_layer_id equal to 0, the DPB parameters are inferred from the VPS rather than using those signalled in that SPS. The DPB-size related syntax element is signaled for each output layer set, and the reorder and latency-related syntax elements are signalled for each layer. The proposed changes to the syntax are highlighted below in grey. The proposed syntax does not have the issue of parsing dependency. The resulting changes to the Specification are marked in the attached document.

## Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag && i > 0 ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) |  |
| sub_layer_vps_buf_info_present_flag[ i ] | u(1) |
| for( j = ( sub_layer_vps_buf_info_present_flag[ i ] ? 0 : vps_max_sub_layers_minus1; j <= vps_max_sub_layers_minus1; j++ ) |  |
| max_vps_dec_pic_buffering_minus1[ i ][ j ] | ue(v) |
| } |  |
| for( i = 1; i < vps_max_layers_minus1; i++ ) { |  |
| sub_layer_vps_ordering_info_predict_flag[ i ] | u(1) |
| if( sub_layer_vps_ordering_info_predict_flag[ i ] ) |  |
| sub_layer_vps_ordering_info_present_flag[ i ] | u(1) |
| for( j = ( sub_layer_vps_ordering_info_present_flag[ i ] ? 0 : vps_max_sub_layers_minus1; j <= vps_max_sub_layers_minus1; j++ ) { |  |
| max_vps_num_reorder_pics[ i ][ j ] | ue(v) |
| max_vps_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| } |  |

## Video parameter set extension semantics

- sub_layer_vps_buf_info_present_flag[ i ] equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ j ] are present for vps_max_sub_layers_minus1 sub-layers. sub_layer_vps_buf_info_present_flag[ i ] equal to 0 specifies that the values of max_vps_dec_pic_buffering_minus1[ i ][ vps_max_sub_layers_minus1 ] apply to all sub-layers.

- max_vps_dec_pic_buffering_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers for the i-th output layer set signaled and when the HighestTid is equal to j. When max_vps_dec_pic_buffering_minus1[ i ][ j ] is not present for i in the range of 0 to vps_max_sub_layers_minus1 – 1, inclusive, due to sub_layer_vps_buf_info_present_flag[ i ] being equal to 0, it is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i ][ vps_max_sub_layers_minus1 ].

- The value of max_vps_dec_pic_buffering_minus1[ 0 ][ j ] for each value of j is inferred to be equal to vps_max_dec_pic_buffering_minus1[ j ].

- sub_layer_vps_ordering_info_predict_flag[ i ] equal to 1 specifies that the syntax elements sub_layer_vps_ordering_info_present_flag[ i ], max_vps_num_reorder_pics[ i ][ j ], and max_vps_latency_increase_plus1[ i ][ j ] are inferred to be equal to sub_layer_vps_ordering_info_present_flag[ i - 1 ], max_vps_num_reorder_pics[ i - 1 ][ j ], and max_vps_latency_increase_plus1[ i - 1 ][ j ], respectively. sub_layer_vps_ordering_info_predict_flag[ i ]  equal to 0 indicates that the syntax elements sub_layer_vps_ordering_info_present_flag[ i ], max_vps_num_reorder_pics[ i ][ j ], and max_vps_latency_increase_plus1[ i ][ j ] are explicitly signalled . When not present, the value of sub_layer_vps_ordering_info_predict_flag[ i ] is set equal to 0.

- sub_layer_vps_ordering_info_present_flag[ i ] equal to 1 specifies that max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ] are present for max_sub_layers_vps_minus1 + 1 sub-layers. sub_layer_vps_ordering_info_present_flag[ i ] equal to 0 specifies that the values of max_vps_num_reorder_pics[ i ][ vps_max_sub_layers_minus1 ] and max_vps_latency_increase_plus1[ i ][ max_sub_layers_vps_minus1 ] apply to all sub-layers.

- max_vps_num_reorder_pics[ i ][ j ] indicates the maximum allowed number of pictures with nuh_layer_id equal to i that can precede any picture with nuh_layer_id equal to i in the CVS in decoding order and follow that picture with nuh_layer_id equal to i in output order, and when HighestTid is equal to j. When max_vps_num_reorder_pics[ i ][ j ] is not present for i in the range of 0 to vps_max_sub_layers_minus1 – 1, inclusive, due to sub_layer_vps_ordering_info_present_flag[ i ] being equal to 0, it is inferred to be equal to max_vps_num_reorder_pics[ i ][ vps_max_sub_layers_minus1 ].

- max_vps_latency_increase_plus1[ i ][ j ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ i ][ j ], which hspecifies the maximum number of pictures with nuh_layer_id equal to i that can precede any picture with nuh_layer_id equal to i in the CVS in output order and follow that picture with nuh_layer_id equal to i in decoding order when HighestTid is equal to j. When max_vps_latency_increase_plus1[ i ][ j ] is not present for i in the range of 0 to vps_max_sub_layers_minus1 – 1, inclusive, due to sub_layer_vps_ordering_info_present_flag[ i ] being equal to 0, it is inferred to be equal to max_vps_latency_increase_plus1[ i ][ vps_max_sub_layers_minus1 ].

- When max_vps_latency_increase_plus1[ i ][ j ] is not equal to 0, the value of VpsMaxLatencyPictures[ i ][ j ] is specified as follows:

- VpsMaxLatencyPictures[ i ][ j ] = max_vps_num_reorder_pics[ i ][ j ] + 											max_vps_latency_increase_plus1[ i ][ j ] - 1

- When max_vps_latency_increase_plus1[ i ][ j ] is equal to 0, no corresponding limit is expressed. The value of max_vps_latency_increase_plus1[ i ][ j ] shall be in the range of 0 to 232 – 2, inclusive.

## Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).