| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0093r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0061r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On DPB operations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal presents several methods to complete the specification of picture-based removal of decoded pictures from the DPB. A set of target output layers is used to specify the operation point. The DPB is partitioned into sub-DPBs based on spatial resolution, bit depth, and color format. The sub-DPB sizes are signalled in the VPS for the various output layer sets. The management of the DPB is changed to operate on the sub-DPBs.

In revision 1 of this proposal, aspects related to parsing dependency and DPB-related parameters (reorder and latency) as described in N0091/E0059 are included in the attachment document. The revised attachment document also modifies the picture output process in the DPB to be dependent on the reorder and latency parameters of the layer that has the highest layer ID amount the set of target output layers.

# Introduction

The conformance of bitstreams is specified using the behaviour of the hypothetical reference decoder (HRD). The DPB is the component of the HRD that stores decoded pictures. In the SHVC WD2 and MV-HEVC WD4, the DPB operations are not clearly specified yet, but a simple assumption is that the pictures arrive in the DPB such that all pictures with the same output time arrive at the same time instant. The output of pictures is determined based on the nature of decoder conformance as follows. For output order decoder conformance, the output is specified by the number of pictures present for a particular layer; for output timing decoding order conformance, the output is specified by picture output times that can be derived from the HRD parameters signaled in the hrd_parameters( ) structure, buffering period SEI message and the picture timing SEI message (and in some cases, additionally in the sub-picture decoding SEI message). The output of the pictures in the DPB also occurs such that all the pictures with the same output time are outputted at the same instance.

There are two ways of specifying the removal of pictures from the DPB. The removal of pictures from the DPB in a way that pictures in the one AU may be removed at different times is referred to as picture-based DPB removal, and removal of pictures in a way that pictures in one AU are always removed at the same time is referred to as AU-based DPB removal. Again, the DPB operations are not clearly specified yet in MV-HEVC WD4 and SHVC WD2, but a simple assumption is that picture-based DPB removal is supported. In the various discussions at previous JCT-VC/JCT-3V meetings, it was clear that there is considerable support for specifying the picture based DPB removal.

As mentioned earlier, the DPB operations are not clearly specified yet. The DPB size is currently signaled in the SPS. When there are multiple layers in the bitstream, it is not clear whether the DPB size is measured in units of pictures or access units, and so on.

This document proposes several items to complete this functionality, and also proposes to make some improvements in the design. The summary of the proposals are provided below, and a detailed explanation is given in the following section.

- Definition of target output layers and specification of operation point using the set of target output layers

- Partitioning DPB into sub-DPBs based on spatial resolution, bit depth, and color format, and allocating each layer to a sub-DPB

- Signaling of sub-DPB sizes in the VPS

- Modifying the DPB management to operate with sub-DPBs

# Proposal

This section gives details about the proposed items. The changes to the Specification are present in the attachment accompanying this document.

## Output layers and operation points

An SHVC/MV-HEVC bitstream contains two or more layers, and in most cases more than one operating point. A sub-bitstream is generated from an existing bitstream based on the various operation points that are supported. In the current Specification, operation point is specified using a layer set (set of pictures to be decoded) and a highest temporal ID value. This has two shortcomings: it is unclear what pictures should be output. Several candidate output layer sets are signaled in the VPS. However, the decoder does not have a means to decide what pictures to output. Since a layer set may correspond to one or more output layer sets (and hence one or more operating points), there needs to be additional information to help the decoder determine the set of target output layers.

It is proposed that an operating point be associated with a set of target output layers and a temporal ID. A modification in the definition for operating points is proposed, along with a new definition for output layer sets.

- operation point: A bitstream that is created from another bitstream by operation of the sub-bitstream extraction process with the another bitstream, a target highest TemporalId, and a target layer identifier list as inputs, and that is associated with a set of target output layers.

- NOTE 14 – If the target highest TemporalId of an operation point is equal to the greatest value of TemporalId in the layer set associated with the target layer identification list, the operation point is identical to the layer set. Otherwise it is a subset of the layer set.

- output layer set: A layer set that is associated with a set of target output layers.

The set of target output layers corresponds to an output layer set. Hence, it is also proposed that the specification of the operation point to the decoder be done by an output layer set, and a temporal ID value. The decoding process in sub-clause 8.1 is changed to reflect this.

## Partitioning the DPB

The decoded picture buffer contains a set of picture storage buffers. Currently, there is no distinction among the picture storage buffers. This simplified design allows for any decoded picture to be stored in any picture buffer. The downside of this method is that the picture buffers would have to account for the worst-case picture size. In SHVC, where a spatially scalable bitstream can have large differences in the picture resolutions across layers, this design could impose significant penalty on the memory.

In order to maintain the DPB more efficiently, it is proposed that the DPB be split into sub-DPBs based on picture resolution, bit depth and colour format. In other words, any two pictures that have the same resolution, bit depth and colour format would be present in the same sub-DPB, and any two picture with different resolutions or bit depth or colour format would be present in different sub-DPBs. Thus, the DPB is partitioned, and the management of DPBs is done by specifying the size of the sub-DPBs in the VPS for the various output layer sets. The number of sub-DPBs is also signaled in the VPS. The association of each layer with a sub-DPB is defined by a process defined in subclause F.13.1 in the attached document.

## Signaling sub-DPB sizes in the VPS

The DPB sizes are currently signaled in the SPS. In JCTVC-N0091/JCT3V-E0059, it is proposed to signal the DPB-related parameters (including the DPB-sizes) in the VPS for each output layer set. In this document, the sub-DPBs are signaled in the VPS for similar reasons as described in JCTVC-N0091/JCT3V-E0059, for each output layer set. The signaling comprises a set of sub-DPB size arrays that are signaled, followed by signaling the number of sub-DPBs in each layer set. The number of sub-DPBs is only dependent on the layers that are decoded, and not only on the layers that are output; therefore, the number of sub-DPBs are only signaled for each layer set. Finally, the sub-DPB sizes for each sub-DPB in each output layer set that is signaled in the VPS is signaled by indexing the set of sub-DPB size arrays, and also indexing the candidate sub-DPB size. The proposed signaling is presented below.

### Video parameter set extension syntax

| vps_extension( ) { |  |
| --- | --- |
| … |  |
| max_one_active_ref_layer_flag | u(1) |
| max_dpb_sizes_table( ) |  |
| direct_dep_type_len_minus2 | ue(v) |
| … |  |
| } |  |

### DPB sizes table syntax

| max_dpb_sizes_table( ) { |  |
| --- | --- |
| num_sub_dpb_capacities_minus1 | ue(v) |
| for( i = 0; i <= num_sub_dpb_capacities_minus1; i++) { |  |
| num_sub_dpb_size_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_sub_dpb_size_minus1[ i ]; j++) |  |
| delta_sub_dpb_size_minus1[ i ][ j ] | ue(v) |
| } |  |
| for( i = 1, subDpbInfoFlag = 1; i <= vps_number_layer_sets_minus1; i++ ) |  |
| for( k = vps_max_sub_layers_minus1; ( k >= 0 ) && subDpbInfoFlag; k− − ) { |  |
| sub_dpb_info_present_flag[ i ][ k ] | u(1) |
| subDpbInfoFlag = sub_dpb_info_present_flag[ i ][ k ] |  |
| if( num_sub_dpb_capacities_minus1 > 0 ) { |  |
| same_num_sub_dpbs_flag[ i ][ k ] | u(1) |
| if( !same_num_sub_dpbs_flag[ i ][ k ] ) |  |
| delta_num_sub_dpbs_minus1[ i ][ k ] | ue(v) |
| } |  |
| } |  |
| for( i = 1; i < NumOutputLayerSets; i++ ) |  |
| for( k = vps_max_sub_layers_minus1; k >= 0; k++ ) |  |
| if( SubDpbInfoPresentFlag[ output_layer_set_idx_minus1[ i ] ][ k ] ) | u(1) |
| for( j = 0; j < NumSubDPBs[ output_layer_set_idx_minus1[ i ] + 1 ][ k ]; j++) { |  |
| if( num_sub_dpb_capacities_minus1 > 0 ) |  |
| sub_dpb_capacity_idx[ i ][ k ][ j ] | u(v) |
| sub_dpb_size_idx[ i ][ k ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |

### DPB sizes table semantics

num_sub_dpb_capacity_minus1 plus 1 specifies the maximum number of sub-DPBs for all the output layer sets indicated in this VPS. Each sub-DPB has the same capacity in terms of the overall characteristics of each unit of the decoded picture buffer.

Note: For example, all pictures that have the same resolution belong to the same sub-DPB, and different operation points may contain different number of pictures in a sub-DPB due to the difference in the number of layers with the same resolution in different operation points.

num_sub_dpb_size_minus1[ i ] plus 1 specifies the number of sizes signalled for the i-th sub-DPB signalled in the video parameter set.

delta_sub_dpb_size_minus1[ i ][ j ] plus 1 specifies the j-th size of the i-th sub-DPB signalled in the video parameter set.

The j-th size of the i-th sub-DPB, denoted by subDpbSize[ i ][ j ], is derived as follows for j from 0 through num_sub_dpb_size_minus1[ i ].

subDpbSize[ i ][ j ]= delta_sub_dpb_size_minus1[ i ][ j ] + 1 + ( j  >  0 ? subDpbSize[ i ][ j - 1 ] : 0).

sub_dpb_info_present_flag[ i ][ k ] equal to 1 indicates that the information for the k-th temporal subset of the i-th layer set is present, sub_dpb_info_present_flag[ i ][ k ] equal to 0 indicates that the information for the k-th temporal subset of the i-th layer set is not present. When not present, sub_dpb_info_present_flag[ i ][ k ] is inferred to be equal to 0.

same_num_sub_dpbs_flag[ i ][ k ] equal to 1 specifies that the number of sub-DPBs for k-th temporal subset of the i-th output layer set is equal to the number of sub-DPBs for the k-th temporal subset of the ( i – 1 )-th ouput layer set. same_num_sub_dpbs_flag[ i ][ k ] equal to 0 specifies that the number of sub-DPBs for k-th temporal subset of the i-th output layer set is not equal to the number of sub-DPBs for the k-th temporal subset of the ( i – 1 )-th ouput layer set. When not present same_num_sub_dpbs_flag[ i ][ k ] is inferred to be equal to 0.

delta_num_sub_dpbs_minus1[ i ][ k ] specifies the number of the sub-DPBs for the k-th temporal subset of the i-th output layer set.

The number of sub-DPBs for the i-th layer set and the k-th temporal subset, NumSubDpbs[ i ][ k ], is derived as follows:

- If sub_dpb_info_present_flag[ i ][ k ] is equal to 0, NumSubDpbs[ i ][ k ] is set to be equal to 0.

- Otherwise if sub_dpb_info_present_flag[ i ][ k ] is equal to 1 and num_sub_dpb_capacity_minus1 is equal to 0, NumSubDpbs[ i ][ k ] is set to be equal to 1.

- Otherwise (neither sub_dpb_info_present_flag[ i ][ k ] nor num_sub_dpb_capacity_minus1 is equal to 0), NumSubDpbs[ i ][ k ] is derived as follows:

NumSubDpbs[ i ][ k ] = ( !same_num_sub_dpbs_flag[ i ][ k ] ) ? 
		(delta_num_sub_dpbs_minus1[ i ][ k ]+1+ NumSubDPBs[ i - 1 ][ k ]) : 					NumSubDPBs[ i - 1 ][ k ].

The variable NumSubDpbsInOptLayerSet[ i ][ k ] for the i-th output layer set and the k-th temporal subset is derived as follows:

NumSubDpbsInOptLayerSet[ i ][ k ] = NumSubDpbs[ output_layer_set_idx_minus1[ i ] ][ k ]

sub_dpb_capacity_idx[ i ][ k ][ j ] specifies the index to the sub-DPBs. The j-th sub-DPB of the k-th temporal subset of the i-th ouput layer set is the sub_dpb_capacity_idx[ j ]-th sub-DPB signaled with sub_dpb_capacity_idx. sub_dpb_capacity_idx[ i ][ k ][ j ] is in the range of 0 to num_sub_dpb_capacity_minus1, inclusive. When not present, sub_dpb_capacity_idx[ i ][ k ][ j ] is inferred to be equal to 0.

sub_dpb_size_idx[ i ][ k ][ j ] specifies the index to the sizes of the sub_dpb_capacity_idx[ i ][ k ][ j ]-th sub-DPB. sub_dpb_size_idx[ i ][ k ][ j ] is in the range of 0 to num_sub_dpb_size_minus1[ sub_dpb_capacity_idx[ j ] ], inclusive. VpsSubDpbMaxDecPicBuf[ i ][ k ][ j ] is equal to subDpbSize[ sub_dpb_capacity_idx[ i ][ k ][ j ] ][ sub_dpb_size_idx[ i ][ k ][ j ] ].

VpsSubDpbMaxDecPicBuf[ i ][ k ][ j ] specifies the maximum required size of the j-th sub-DPB for the CVS for the i-th output layer set in units of picture storage buffer when HighestTId equal to k. When k is greater than 0, VpsSubDpbMaxDecPicBuf[ i ][ k ][ j ] shall be greater than or equal to VpsSubDpbMaxDecPicBuf [ i ][ k ][ j – 1 ].

## Picture-based DPB removal

By specifying the sizes of each sub-DPB and also associating each layer with a sub-DPB, the picture removal process in both output order conformance and output-time conformance can be specified by constraining the number of picture storage buffers in each sub-DPB to not exceed the size signaled in the VPS. The fullness value is tracked for each sub-DPB (rather than the entire DPB as was earlier) and the picture bumping process is invoked to ensure the fullness value of each sub-DPB does not exceed the size of the respective sub-DPB.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).