| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0172 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0069 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Layer-wise DPB operation and size indications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides specification text for layer-wise DPB operation and size indications.

# Introduction

The operation of the DPB in MV-HEVC/SHVC has been somewhat unclear, especially when it comes to the indication of the size of the DPB for example in terms of the number of picture storage buffers. This contribution attempts to provide clarity in the DPB operation and size signalling.

# Proposal

The specification text provided along the contribution can be summarized as follows:

- max_dec_pic_buffering is provided in VPS for each sub-layer in each layer in each output layer set. Variable MaxDecPicBuffering[ outputLayerSetIdx ][ layerId ][ highestTid ] is derived from VPS. sps_max_dec_pic_buffering_minus1 applies only for the base layer.

Motivation:

- The deallocation of picture storage buffers in the DPB depends on both marking as “used / unused for reference” and “needed / not needed for output”. Hence, max_dec_pic_buffering has to be indicated per output layer set.

- Layer-wise indication of max_dec_pic_buffering was chosen to cause fewer changes in the HRD specification text.

- A target output layer set is chosen for the operation point under test in the HRD.

- Each layer has a different DPB conceptually with the size indicated by the MaxDecPicBuffering[ outputLayerSetIdx ][ layerId ][ highestTid ] variable:

- The constraint on the maximum number of pictures in the DPB is specified layer-wise.

- The bumping process is invoked when the number of pictures in the DPB and within a layer layerId meets the limit specified by MaxDecPicBuffering.

- For the Stereo Main profile, MaxDecPicBuffering for each layer is limited the same way as sps_max_dec_pic_buffering_minus1 is for the HEVC version 1 bitstreams.

# Discussion

The following things should be noted:

- The proposal is sub-optimal in the following kinds of cases. Assume that layer 0 requires N picture storage buffers for time period 0 to T and M picture storage buffers for time period T + 1 onwards (where N > M). Layer 1 in the same bitstream (and with the same decoded picture size as for layer 0) requires M picture storage buffers for time period 0 to T and N picture storage buffers for time period T + 1 onwards. In the proposal, the indicated max_dec_pic_buffering for both layers is equal to N and hence the total number of picture storage buffers is equal to 2*N. In an optimal case, both layers were conceptually stored in the same DPB with the number of picture storage buffers equal to M+N.

- If the number of layers in a bitstream is not fixed by the specification of a profile, the level-specific limits for MaxDecPicBuffering should be specified for the memory used across all layers rather than the number of picture storage buffers per layer (as proposed for the Stereo Main profile).

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).