| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0158 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0070 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Bitstream restrictions on tiles and wavefronts across layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Ye-Kui Wang Adarsh K. Ramasubramonian Jianle Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | krapaka@qti.qualcomm.com yekuiw@qti.qualcomm.com aramasub@qti.qualcomm.com cjianle@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes various bitstream restrictions on use of tiles and wavefront parallel processing (WPP) across layers.

# On tiles and WPP configuration across layers

It is asserted that supporting all possible configurations of tiles and WPP across layers burdens the pipelining design and implementation of the codec with no noticeable benefits. The pipeline design (allocation of resources across layers) need to change for different configurations of tiles and WPP, as a result, the memory, cycles and the delay requirements are different and a codec needs to consider all possible cases to design its worst case. Hence, it would be advantageous if non-typical configurations across layers are normatively restricted, thereby reduce design and implementation complexity.

A few example configurations of tiles and WPP across layers are shown in figure 1 and figure 2. In figure 1 the configuration (a) has more tiles in the base layer (BL) compared to the enhancement layer (EL) and the configuration (b) has more tiles in the EL compared to the BL. It is also noted that the spatial resolution of the EL may be higher than that of the BL.

Figure 1: Example tile configurations in multi-layer contexts

For a multi-core system design, it is asserted that the tile configuration (a) does not provide any benefit over (b) and it has the following problems: 1) unevenly distributed processing load across layers 2) unfriendly for synchronized inter-layer parallelism. We propose to disallow such configurations to reduce design and implementation complexity.

A conforming bitstream shall obey the following constraints:

a) The value of num_tile_columns_minus1 of a layer with a particular value of nuh_layer_id greater than 0 shall be greater than or equal to the value of num_tile_columns_minus1 of any of the layer's direct reference layers.

b) The value of num_tile_rows_minus1 of a layer with a particular value of nuh_layer_id greater than 0 shall be greater than or equal to the value of num_tile_rows_minus1 of any of the layer's direct reference layers.

A few example configurations for co-existence of tiles and WPP across layers are shown in figure 2. The configuration (a) has only WPP in the BL and only tiles in the EL and configuration (b) has only tiles in the BL and only WPP in the EL. For the same reason discussed above, it is asserted that the configuration (a) and (b) does not provide any benefit and thus we propose to disallow such configurations, among others, to reduce design and implementation complexity.

Figure 2: Example tile and WPP configurations across layers

All of the following restrictions apply:

a) All PPSs that may be activated within a CVS shall have the same value of entropy_coding_sync_enabled_flag. [Note: This disallows the use of WPP in one layer but not in another layer.]

b) Within a CVS, when tiles_enabled_flag is equal to 0 for an active PPS for a particular layer, tiles_enabled_flag shall also be equal to 0 for an active PPS for any particular layer directly or indirectly depends on as specified by the VPS. [Note: This disallows the use of tiles in a lower layer but not in a higher layer that depends on the lower layer, but it would allow for the use of tiles in a spatial enhancement layer but not in the base layer.]

c) Within a CVS, when tiles_enabled_flag is equal to 1 for an active PPS for a particular layer, entropy_coding_sync_enabled_flag shall be equal to 0 for all PPSs that may be activated. [Note: This disallows the use of WPP in any layer of a CVS when one layer in the CVS uses tiles.]

d) Within a CVS, when entropy_coding_sync_enabled_flag is equal to 1 for an active PPS for a particular layer, tiles_enabled_flag shall be equal to 0 for all PPSs that may be activated. [Note: This disallows the use of tiles in any layer of a CVS when one layer in the CVS uses WPP.]

# On cross layer alignment of loop filtering across tiles

When tiles are used across layers for parallelism, loop filters can be applied in two options: cross tile boundaries or not. If loop filter may cross the tile boundary it affects the parallelism across tiles and introduces a delay whereas it may also be argued that loop filter across the tile boundaries helps improve video quality. We think that if loop filtering across tile boundaries should be either allowed for all layers or not allowed for all layers, as mixing it for different layers defeats the purpose of the allowance for some of the layers. We thus propose the following restriction:

All PPSs that may be activated within a CVS shall have the same value of loop_filter_across_tiles_enabled_flag.

# On alignment of CTB sizes across layer for SNR scalability

HEVC supports three CTB sizes 16x16, 32x32 and 64x64. These sizes represent a trade-off between coding efficiency (larger CTB) and encoder/decoder latency (smaller CTB). However, the small CTB size comes with a cost to pipelined decoders. Specifically, every pipeline stage has a certain amount of overhead per CTB for initialization and setup. In JCTVC-J0334, it is asserted that the overhead time for a 16x16 CTB versus a 32x32 CTB may represent up to 4 times and the cumulative overhead for the 16x16 CTB can represent up to a 10% increase in overall worst-case decode time as compared to a 32x32 CTB.

Figure 3: Example processing time and overhead of 16x16 CTB’s vs 32x32 CTB’s

Figure 4: Example configuration with EL 32x32 CTB’s and BL16x16 CTB’s

For a multi-layer system (esp. when resolution of different layers are same) that employs CTB synchronous encode/decode as shown in Figure 4, following problems are asserted when CTB sizes across layers are different : a) The layer with smaller CTB size is bottleneck for every CTB and determines the worst case. For example in figure 4, one CTB at EL can complete faster than one CTB of BL, but still needs to wait for BL CTB b) The mapping of collocate LCU become complicated.

A conforming bitstream shall obey the following constraints:

In other words the CTB size of current layer shall be equal to the CTB size of its active reference layers for SNR scalability.

The spec text could be as follows:

Let the variables PicWidthInSamplesL and PicHeightInSamplesL be set equal to width and height of current layer picture in units of luma samples, respectively and let the variables RefLayerPicWidthInSamplesL and RefLayerPicHeightInSamplesL be set equal to the width and height of the decoded active reference layer picture in units of luma samples, respectively. The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftOffset = scaled_ref_layer_left_offset[ dRlIdx ] << 1
ScaledRefLayerTopOffset = scaled_ref_layer_top_offset[ dRlIdx]  << 1
ScaledRefLayerRightOffset = scaled_ref_layer_right_offset[ dRlIdx ]  << 1
ScaledRefLayerBottomOffset = scaled_ref_layer_bottom_offset[ dRlIdx ]  << 1

Let the variable RefLayerCtbLog2SizeY be set equal to the CtbLog2SizeY of the decoded active reference layer picture.

When PicWidthInSamplesL of current layer is equal to RefLayerPicWidthInSamplesL and PicHeightInSamplesL of current layer is equal to RefLayerPicHeightInSamplesL and the values of ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are all equal to 0, then CtbLog2SizeY of current layer shall be equal to the RefLayerCtbLog2SizeY.

# On number of tile constraint across layers

HEVC version 1 specification restricts the maximum number of tiles per picture as specified in Table A-1. We propose to adapt the restriction to multi-layer context as follows:

For a particular layer with nuh_layer_id equal to currLayerId, and currLayerId greater than 0, the value of num_tile_columns_minus1[ currLayerId ] shall be less than MaxTileCols and the value of num_tile_rows_minus1[ currLayerId ] shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Error: Reference source not found for the level that is the lowest level in Table A-1 that allows for the spatial resolution of that particular layer with nuh_layer_id equal to currLayerId.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).