| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AU, 29 July – 2 August 2013 | Document: JCT3V-E0073 |
| --- | --- |

| Title: | 3D-HEVC: Crosscheck on MediaTek Proposal JCT3V-E0184 |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Dong Tian (MERL) 201 Broadway, 8th floor Cambridge, MA 02139 | Tel: Email: | +1-617-621-7556 tian@merl.com |
| Source: | Mitsubishi Electric Research Labs (MERL) |  |  |

_____________________________

# Abstract

This document reports the outcome of cross-verification of JCT3V-E0184, "3D-CE3.h related: Sub-PU level inter-view motion prediction", performed by Mitsubishi Electric Research Labs (MERL). The coding/synthesis results obtained for this cross-verification match those provided by the proponents. Crosscheckers confirmed that the software and the text provided by the proponent are consistent.

# Introduction

In contribution JCT3V-E0184 [1], it is proposed to extend the inter-view motion prediction to a sub-PU level. Upon request, MERL performed the crosschecking on three test cases presented in the proposal.

# Performance of proposed method

Source code of the proposal was provided and the crosscheckers confirm that the software is consistent with the proposal. In the current design of inter-view motion prediction, a single motion vector is inherited for a PU block. With the proposal, a sub-PU block may have an individual motion vector predicted from an inter-view reference picture. The sub-PU block sizes that are tested in the proposal are 4x4, 8x8 and 16x16, which are cross-verified in this document.

With the proponents' software, a new run-time parameter is added to indicate the size of sub-PU blocks: SubPUSizeIdxMultiviewMvPred.

Test case 1 (Proposal w/ 4x4 sub-PU block vs. Anchor, CTC)

With this test, the HTM 7.0r1 is used to create the anchor bitstreams under CTC. For the proponent software, SubPUSizeIdxMultiviewMvPred is set equal to 4 which indicate the sub-PU block size is 4x4. The results obtained by proponents and crosscheckers are shown in Table 1.a and 1.b; which are matched.

Test case 2 (Proposal w/ 8x8 sub-PU block vs. Anchor, CTC)

With this test, the HTM 7.0r1 is used to create the anchor bitstreams under CTC. For the proponent software, SubPUSizeIdxMultiviewMvPred is set equal to 3 which indicate the sub-PU block size is 8x8. The results obtained by proponents and crosscheckers are shown in Table 2.a and 2.b; which are matched.

Test case 1 (Proposal w/ 16x16 sub-PU block vs. Anchor, CTC)

With this test, the HTM 7.0r1 is used to create the anchor bitstreams under CTC. For the proponent software, SubPUSizeIdxMultiviewMvPred is set equal to 2 which indicate the sub-PU block size is 16x16. The results obtained by proponents and crosscheckers are shown in Table 2.a and 2.b; which are matched.

Table 1.a Proponent’s results (Proposal w/ 4x4 sub-PU block vs. Anchor, CTC)

Table 1.b. Crosschecker’s results (Proposal w/ 4x4 sub-PU block vs. Anchor, CTC)

Table 2.a Proponent’s results (Proposal w/ 8x8 sub-PU block vs. Anchor, CTC)

Table 2.b. Crosschecker’s results (Proposal w/ 8x8 sub-PU block vs. Anchor, CTC)

Table 3.a Proponent’s results (Proposal w/ 16x16 sub-PU block vs. Anchor, CTC)

Table 3.b. Crosschecker’s results (Proposal w/ 16x16 sub-PU block vs. Anchor, CTC)

# References

- J. An, K. Zhang, J.-L. Lin, S. Lei (MediaTek), "3D-CE3.h related: Sub-PU level inter-view motion prediction", JCT3V-E0184, Vienna, AU, July 2013

- D. Rusanovskyy, K. Mueller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-D1100, Incheon, KR, April 2013