| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0154r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0074 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On signalling of inter-layer RPS in slice segment header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung Won Kang Hahyun Lee Jinho Lee Jin Soo Choi | Tel: Email: | +82-42-860-5137 jungwon@etri.re.kr hanilee@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

In slice segment header of the current SHVC as well as MV-HEVC draft text, there are syntax elements to signal direct reference layers, which are a subset of inter-layer RPS signalled in VPS extension, used by the current slice for inter-layer prediction.

This contribution asserts that it is not necessary to signal direct reference layers used by the current slice for inter-layer prediction in the slice segment header when those are identical to the inter-layer RPS for a picture having nuh_layer_id equal to layer_id of the current picture signalled in VPS extension. Two alternative syntax changes are proposed to solve the problem.

# Problem Statement

In slice segment header of the current SHVC as well as MV-HEVC working draft, there are syntax elements to signal direct reference layers, which are a subset of inter-layer RPS signalled in VPS extension, used by the current slice for inter-layer prediction.

These syntax elements are always signalled even if direct reference layers used by the current slice are identical to the inter-layer RPS for a picture having nuh_layer_id equal to layer_id of the current picture signalled in VPS extension. However, in this case, direct reference layers for the current slice can be inferred to inter-layer RPS signalled in VPS extension without signalling in the slice segment header. Therefore, this contribution proposes to signal direct reference layers used by the current slice in the slice segment header only when direct reference layers used by the current slice are not identical to the inter-layer RPS for a picture having nuh_layer_id equal to layer_id of the current picture signalled in VPS extension.

This contribution proposes two alternatives to solve the problem.

# Proposal

## Alternative 1

Syntax elements to indicate direct reference layers used by the current slice are signalled only when the number of direct reference layers signalled in VPS extension, NumDirectRefLayers[nuh_layer_id], is not equal to the number of direct reference layers used by the current slice, NumActiveRefLayerPics.

Additions are yellow highlighted.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if(NumDirectRefLayers[ nuh_layer_id ] != NumActiveRefLayerPics) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| … |  |
| } |  |

## Alternative 2

A flag, layer_dependency_vps_flag, is introduced to specify if direct reference layers used by the current slice are identical to the inter-layer RPS for a picture having nuh_layer_id equal to layer_id of the current picture signalled in VPS extension.

Additions are yellow highlighted.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| layer_dependency_vps_flag | u(1) |
| if( !layer_dependency_vps_flag ){ |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| … |  |
| } |  |

layer_dependency_vps_flag equal to 1 specifies that num_inter_layer_ref_pics_minus1 +1 is inferred to be equal to NumDirectRefLayers[nuh_layer_id] and inter_layer_pred_layer_idc [i] is inferred to i. layer_dependency_vps_flag equal to 0 specifies that inter_layer_pred_layer_idc[i] is explicitly signalled. When not present, the value of layer_dependency_vps_flag is inferred to be equal to 0. When max_one_active_ref_layer_flag is equal to 1, layer_dependency_vps_flag shall be 0.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).