| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0197 WG11 Number: m29828 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0080 |
| --- | --- |

| Title: | On Signaling DPB Parameters in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes signaling the maximum required size of the decoded picture buffer for the highest layer in an output layer set for sub-layers in the layer for number of output layer sets in VPS extension for SHVC and MV-HEVC. A variant syntax proposes signaling the maximum required size of the decoded picture buffer for all layers in output layers sets and sub-layers in the layer.

This document also proposes signaling other DPB related parameters - max_vps_num_reorder_pics  and max_vps_latency_increase_plus1 for layers in VPS extension for SHVC and MV-HEVC.

It is asserted that signaling above parameters in VPS is useful for session negotiation.

# Introduction

Currently in SHVC Draft Text 2 [1] and MV-HEVC Dratf Text 4 [2] the parameters sps_max_num_reorder_pics[ ], sps_max_latency_increase_plus1[ ] and sps_max_dec_pic_buffering[ ] are only signaled in SPS of each layer.

This document proposes the following:

- This document proposes signaling max_vps_dec_pic_buffering_minus1 parameters for output layer sets in VPS extension for SHVC and MV-HEVC.

- It also proposes signaling max_vps_num_reorder_pics  and max_vps_latency_increase_plus1 for layers in VPS.

It is asserted that these syntax elements in VPS extension can help in session negotiation. For example in the proposed variant 1 max_vps_dec_pic_buffering_minus1 [ i ][  j ] signals the maximum required size of the decoded picture buffer for the CVS for layer with nuh_layer_id equal to highestLayerId in the output layer set associated with index i in units of picture storage buffers when HighestTid is equal to j. This information is useful for the decoder in terms of session negotiation based on its available DPB size in determining if it can select to operate at a certain operation point and the associated output layer set.

It is also asserted that signalling these parameters in VPS allows defining values for these  parameters to allow bumping of pictures depending upon the particular set of layers included in the output layer set and operation point under test. To this end a separate proposal proposing operation for DPB using the VPS signalled DPB parameters is proposed in our companion document JCTVC-N0198.

# Proposal

We propose following two syntax variants and semantics. The proposed changes are highlighted below compared to SHVC Draft Text 2 [1] and are also applicable to MV-HEVC Draft Text 4 [2].

## Variant 1:

In variant 1 syntax the maximum required size of the decoded picture buffer (max_vps_dec_pic_buffering_minus1 parameters) for the highest layer in an output layer set for sub-layers in the layer are signaled in VPS extension. Also other DPB related parameters - max_vps_num_reorder_pics  and max_vps_latency_increase_plus1 for layers are signaled in VPS extension.

## Syntax and Semantics:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| … |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| num_dpb_info_parameters | ue(v) |
| for( i = 0; i < num_dpb_info_parameters; i++ ) { |  |
| output_point_layer_set_idx[ i ] | ue(v) |
| oop_dpb_maxbuffering_parameters( i ) |  |
| } |  |
| layer_dpb_info_parameters_presence_flag |  |
| if(layer_dpb_info_parameters_presence_flag) { |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| layer_dpb_info_parameters(i) |  |
| } |  |
| } |  |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| single_layer_for_non_irap_flag | u(1) |
| } |  |

| oop_dpb_maxbuffering_parameters ( i ) { |  |
| --- | --- |
| sub_layers_vps_max_minus1[ i ] | u(3) |
| sub_layer_vps_maxdecbuffering_info_present_flag [ i ] | u(1) |
| for( j = (sub_layer_vps_maxdecbuffering_info_present_flag [ i ] ? 0 : sub_layers_vps_max_minus1[ i ] ); j <= sub_layers_vps_max_minus1[ i ]; j++ ){ |  |
| max_vps_dec_pic_buffering_minus1[ i ][ j ] | ue(v) |
| } |  |

| layer_dpb_info_parameters( i ) { |  |
| --- | --- |
| max_sub_layers_vps_minus1[ i ] | u(3) |
| sub_layer_vps_ordering_info_present_flag[ i ] | u(1) |
| for( j = (sub_layer_vps_ordering_info_present_flag [ i ] ? 0 : max_sub_layers_vps_minus1[ i ]); j <= max_sub_layers_vps_minus1[ i ]; j++ ) { |  |
| max_vps_num_reorder_pics[ i ][ j ] | ue(v) |
| max_vps_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |

- num_dpb_info_parameters specifies the number of oop_dpb_maxbuffering_parameters( ) syntax structures present in the VPS extension RBSP. num_dpb_info_parameters decoders shall be in the range of 0 to numOutputLayerSets-1, inclusive.

- output_point_layer_set_idx[ i ] specifies the index, into the list of output layer sets to which the i-th oop_dpb_maxbuffering_parameters( i )  syntax structure in the VPS extension applies.

- The value of output_point_layer_set_idx[ i ] should be in the range of 0 to numOutputLayerSets-1, inclusive. It is requirement of bitstream conformance that

- output_point_layer_set_idx [ i ] shall not be equal to output_point_layer_set_idx [ j ] for any j not equal to i.

sub_layers_vps_max_minus1[ i ]  plus 1 specifies the maximum number of temporal sub-layers that may be present in the CVS for layer with nuh_layer_id equal to highestLayerId in the output layer set associated with index i. The value of sub_layers_vps_max_minus1[ i ] shall be in the range of 0 to 6, inclusive.

sub_layer_vps_maxdecbuffering_info_present_flag[ i ] equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ j ] are present for layer with nuh_layer_id equal to highestLayerId in the output layer set associated with index i for sub_layers_vps_max_minus1[ i ] + 1 sub-layers. sub_layer_vps_maxdecbuffering_info_present_flag[ i ] equal to 0 specifies that the values of max_vps_dec_pic_buffering_minus1[ i ][ sub_layers_vps_max_minus1[ j ] ] apply to all sub-layers for layer with nuh_layer_id equal to highestLayerId in the output layer set associated with index i.

max_vps_dec_pic_buffering_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS for layer with nuh_layer_id equal to highestLayerId in the output layer set associated with index i in units of picture storage buffers when HighestTid is equal to j. The value of max_vps_dec_pic_buffering_minus1[ i ][ j ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause Error: Reference source not found), inclusive. When j is greater than 0, max_vps_dec_pic_buffering_minus1[ i ][ j ] shall be greater than or equal to max_vps_dec_pic_buffering_minus1[ i ][ j - 1 ]. When max_vps_dec_pic_buffering_minus1[ i ][ j ] is not present for j in the range of 0 to sub_layers_vps_max_minus1[ i ] − 1, inclusive, due to sub_layer_vps_maxdecbuffering_info_present_flag[ i ]  being equal to 0, it is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i ][ sub_layers_vps_max_minus1[ i ] ].

- layer_dpb_info_parameters_presence_flag[ i ] equal to 1 specifies that the syntax elements layer_dpb_info_parameters(i) are present for vps_max_layers_minus1 layers. layer_dpb_info_parameters_presence_flag[ i ] equal to 0 specifies that the syntax elements layer_dpb_info_parameters(i) are not present for vps_max_layers_minus1 layers.

max_sub_layers_vps_minus1[ i ] plus 1 specifies the maximum number of temporal sub-layers that may be present in the CVS for layer with index i. The value of max_vps_sub_layers_vps_minus1[ i ] shall be in the range of 0 to 6, inclusive.

sub_layer_vps_ordering_info_present_flag[ i ] equal to 1 specifies that max_vps_num_reorder_pics[ i ][ j ] and max_vps_latency_increase_plus1[ i ][ j ]are present for layer with index i for max_sub_layers_vps_minus1[ i ] + 1 sub-layers. sub_layer_vps_ordering_info_present_flag[ i ] equal to 0 specifies that the values of max_vps_num_reorder_pics[ i ][ max_sub_layers_vps_minus1[ j ] ] and max_vps_latency_increase_plus1[ i ][ max_sub_layers_vps_minus1[ j ] ]  apply to all sub-layers for layer with index i.

max_vps_num_reorder_pics[ i ][ j ] indicates the maximum allowed number of pictures that can precede any picture in the CVS for layer with index i in decoding order and follow that picture in output order when HighestTid is equal to j. When j is greater than 0, max_vps_num_reorder_pics[ i ][ j ] shall be greater than or equal to max_vps_num_reorder_pics [ i ][ j ]. When max_vps_num_reorder_pics[ i ][ j ] is not present for j in the range of 0 to max_sub_layers_vps_minus1[ i ]  − 1, inclusive, due to sub_layer_vps_ordering_info_present_flag[ i ] being equal to 0, it is inferred to be equal to max_vps_num_reorder_pics[ i ][ max_sub_layers_vps_minus1 [ i ] ].

max_vps_latency_increase_plus1[ i ][ j ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ i ][ j ], which specifies the maximum number of pictures that can precede any picture in the CVS for layer with index i in output order and follow that picture in decoding order when HighestTid is equal to j.

When max_vps_latency_increase_plus1[ i ][ j ] is not equal to 0, the value of VpsMaxLatencyPictures[ i ][ j ] is specified as follows:

VpsMaxLatencyPictures[ i ][ j ] = max_vps_num_reorder_pics[ i ][ j ] + 	vps_max_latency_increase_plus1[ i ][ j ] − 1

When max_vps_latency_increase_plus1[ i ][ j ] is equal to 0, no corresponding limit is expressed.

The value of max_vps_latency_increase_plus1[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive. When max_vps_latency_increase_plus1[ i ][ j ] is not present for i in the range of 0 to max_sub_layers_vps_minus1 [ i ]− 1, inclusive, due to sub_layer_vps_ordering_info_present_flag[ i ] being equal to 0, it is inferred to be equal to max_vps_latency_increase_plus1[ i ][ max_sub_layers_vps_minus1[ i ] ].

## Variant 2:

The difference between variant 2 syntax compared to the variant 1 syntax is that the maximum required size of the decoded picture buffer (max_vps_dec_pic_buffering_minus1 parameters) is signaled for all layers (i.e. not only for the highest layer) in output layers sets and sub-layers in the layer.

The syntax difference between variant 1 and variant 2 is highlighted with color.

## Syntax and Semantics:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| … |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| num_dpb_info_parameters | ue(v) |
| for( i = 0; i < num_dpb_info_parameters; i++ ) { |  |
| olsIdx=output_point_layer_set_idx[ i ] | ue(v) |
| for( k = 0 ; k < NumLayersInIdList[ olsIdx ] − 1; k++) |  |
| oop_layer_dpb_maxbuffering_parameters( i, k ) |  |
| } |  |
| layer_dpb_info_parameters_presence_flag |  |
| if(layer_dpb_info_parameters_presence_flag) { |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| layer_dpb_info_parameters(i) |  |
| } |  |
| } |  |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| single_layer_for_non_irap_flag | u(1) |
| } |  |

| oop_layer_dpb_maxbuffering_parameters ( i , k) { |  |
| --- | --- |
| sub_layers_vps_max_minus1[ i ][ k ] | u(3) |
| sub_layer_vps_maxdecbuffering_info_present_flag [ i ][ k ] | u(1) |
| for( j = (sub_layer_vps_maxdecbuffering_info_present_flag [ i ][ k ] ? 0 : sub_layers_vps_max_minus1[ i ][ k ] ); j <= sub_layers_vps_max_minus1[ i ][ k ]; j++ ){ |  |
| max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] | ue(v) |
| } |  |

sub_layers_vps_max_minus1[ i ][ k ]  plus 1 specifies the maximum number of temporal sub-layers that may be present in the CVS for layer with nuh_layer_id equal to layer_id_in_nuh[ k ] in the output layer set associated with index i. The value of sub_layers_vps_max_minus1[ i ] shall be in the range of 0 to 6, inclusive.

sub_layer_vps_maxdecbuffering_info_present_flag[ i ][ k ]  equal to 1 specifies that max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] are present for layer with nuh_layer_id equal to layer_id_in_nuh[ k ] in the output layer set associated with index i for sub_layers_vps_max_minus1[ i ] + 1 sub-layers. sub_layer_vps_maxdecbuffering_info_present_flag[ i ][ k ]  equal to 0 specifies that the values of max_vps_dec_pic_buffering_minus1[ i ][ k ][ sub_layers_vps_max_minus1[ i ][ k ]] apply to all sub-layers for layer with nuh_layer_id equal to layer_id_in_nuh[ k ] in the output layer set associated with index i.

max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS for layer with nuh_layer_id equal to layer_id_in_nuh[ k ] in the output layer set associated with index i in units of picture storage buffers when HighestTid is equal to j. The value of max_vps_dec_pic_buffering_minus1[ i ][ k ][  j ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause Error: Reference source not found), inclusive. When j is greater than 0, max_vps_dec_pic_buffering_minus1[ i ][ k ][  j ] shall be greater than or equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ j - 1 ]. When max_vps_dec_pic_buffering_minus1[ i ][ k ][ j ] is not present for j in the range of 0 to sub_layers_vps_max_minus1[ i ][ k ] − 1, inclusive, due to sub_layer_vps_maxdecbuffering_info_present_flag[ i ][ k ]   being equal to 0, it is inferred to be equal to max_vps_dec_pic_buffering_minus1[ i ][ k ][ sub_layers_vps_max_minus1[ i ][ k ] ].

# Conclusion

This document proposes two syntax variants for signaling max_vps_dec_pic_buffering_minus1 parameters for output layer sets  and for signaling max_vps_num_reorder_pics  and max_vps_latency_increase_plus1 for layers in VPS extension for SHVC and MV-HEVC. It is asserted that these parameters are useful for session negotiation.

We propose to adopt signaling from one of the two variants in SHVC and MV-HEVC draft texts.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 1,” JCTVC-M1008, Incheon, May 2013

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 4 (ISO/IEC 23008-2:201x/PDAM2),” JCTVC-D1004, Incheon, May 2013.