| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0198 WG11 Number: m29829 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0081 |
| --- | --- |

| Title: | On DPB Operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document includes two proposals for DPB operation for HEVC Extensions.

1. In JCTVC-N0197 signaling of DPB related parameters in VPS is proposed. This document proposes the operation and bumping process for DPB of SHVC and MV-HEVC to use the DPB parameters signaled in VPS extension if they are signaled for the selected output layer set. Otherwise the SPS level parameters are used.

2. Other changes to the text for DPB operation are proposed with operation on picture basis and some steps applied on AU basis.

# Introduction

SHVC Draft Text 2 [1] and MV-HEVC Draft Text 4 [2] defines operation of output order DPB. DPB operation consists of several separate steps including picture removal, picture output (bumping), picture decoding, picture storage, picture marking, additional picture output (additional bumping).

This document includes two proposals for DPB operation for HEVC Extensions.

- Proposal 1: In JCTVC-N0197 signaling of DPB related parameters in VPS is proposed.

It is asserted that DPB parameters signalled in VPS allows defining values for these parameters to allow bumping of pictures depending upon the particular set of layers included in the output layer set and operation point under test.

This document proposes the operation and bumping process for output order DPB of SHVC and MV-HEVC to use the DPB parameters signaled in VPS extension if they are signaled for the selected output layer set. Otherwise the SPS level parameters are used.

The changes corresponding to proposal 1 are marked with color.

This proposal is an updated version of our proposal JCTVC-M0210.

- Proposal 2: Other changes to the text for DPB operation are proposed with operation on picture basis and some steps applied on AU basis.

The proposed changes related to proposal 2 fall in the following categories:

  - Some changes are related to clarifying the text. These includes changes such as clarifying nuh_layer_id value when an active SPS or a picture is referred in the text.

  - Some changes are editorial and define variables (e.g. DPBFullness[currLayerId])

  - Some changes are related to applying an operation on an AU basis.

The changes corresponding to proposal 1 are marked with color.

# Proposal

The proposed changes are highlighted below compared to SHVC Draft Text 2 [1] and are also applicable to MV-HEVC Draft Text 4 [2].

- Highlight Color is used for proposal 1 related changes. Proposal 1 proposes the operation and bumping process for output order DPB of SHVC and MV-HEVC to use the DPB parameters signaled in VPS extension in JCTVC-N0197 if they are signaled for the selected output layer set. Otherwise the SPS level parameters are used.

- Highlight Color is used for proposal 2 related text changes. Proposal 2 proposes changes to the text for DPB operation with operation on picture basis and some steps applied on AU basis.

F.13  Hypothetical reference decoder

F.13.1 General

The specifications in subclause C.1 apply with following changes:

- An operation point under test, denoted as TargetOp, is selected.An output layer set for the operation point under test, with its index denoted as TargetOpLs is selected. The layer identifier list OpLayerIdList of TargetOp consists consists of the list of nuh_layer_id values, in increasing order of nuh_layer_id values, present in the bitstream subset associated with TargetOp and TargetOpLs, which is a subset of the nuh_layer_id values present in the bitstream under test. The OpTid of TargetOp is equal to the highest TemporalId present in the bitstream subset associated with TargetOp.

…

F.13.2 Operation of coded picture buffer (CPB)

The specifications in subclause C.2 apply.

F.13.3 Operation of the decoded picture buffer (DPB)

The specifications in subclause C.3 apply separately for each set of decoded pictures with a particular value of nuh_layer_id.

PicOutputFlag for pictures that are not included in a target output layer is set equal to 0.

Decoded pictures with the same DPB output time and with PicOutputFlag equal to 1 are output in ascending order of nuh_layer_id values of these decoded pictures.

F.13.5 Decoder conformance

F.13.5.1 General

The specifications in subclause C.5.1 apply.

      - F.13.5.2 Operation of the output order DPB

F.13.5.2.1 General

The decoded picture buffer contains picture storage buffers. Each layer consists of its own set of picture storage buffers. Thus picture storage buffers of each layer are associated with the nuh_layer_id value of the layer. The number of picture storage buffers for nuh_layer_id equal to 0 is derived from the active SPS for that non-zero nuh_layer_id value. The number of picture storage buffers for each non-zero nuh_layer_id value is derived from the active layer SPS for that non-zero nuh_layer_id value. Each of the picture storage buffers contains a decoded picture that is marked as "used for reference" or is held for future output. The process for output and removal of pictures from the DPB as specified in subclause Error: Reference source not found is invoked, followed by the invocation of the process for picture decoding, marking, additional bumping, and storage as specified in subclause Error: Reference source not found. The "bumping" process is specified in subclause Error: Reference source not found and is invoked as specified in subclauses Error: Reference source not found and Error: Reference source not found.

F.13.5.2.2 Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the access unit containing the current picture is removed from the CPB and proceeds as follows.

The decoding process for RPS as specified in subclause 8.3.2 is invoked.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS corresponding to the nuh_layer_id value of the current picture is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture with the nuh_layer_id value equal to the nuh_layer_id value of the current picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers corresponding to all the nuh_layer_id values  in the DPB are emptied without output of the pictures they contain, and the DPB fullness DPBFullness[nuh_layer_id] for all nuh_layer_id values in the DPB is set equal to 0.

- Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers corresponding to all the nuh_layer_id values containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB corresponding to all the nuh_layer_id values in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause Error: Reference source not found, and the DPB fullness DPBFullness[nuh_layer_id] for all nuh_layer_id values in the DPB is set equal to 0.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 or with nuh_layer_id not equal to 0), the variable currLayerId is set equal to nuh_layer_id of the current decoded picture. All picture storage buffers corresponding to the nuh_layer_id value of the current picture containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness DPBfullness[currLayerId] corresponding to the nuh_layer_id value of the current decoded picture is decremented by one. The variable currLayerId is set equal to nuh_layer_id of the current decoded picture.

The variables MaxNumReorderPics[ currLayerId ][ HighestTid ], MaxLatencyIncreasePlus1[ currLayerId ][ HighestTid ], MaxLatencyPictures[ currLayerId ][ HighestTid ], MaxDecPicBufferingMinus1[ currLayerId ][ HighestTid ] are derived as follows:

–	If layer DPB information parameters layer_dpb_info_parameters() are present in VPS, MaxNumReorderPics[ currLayerId ][HighestTid ] is set to vps_max_num_reorder_pics[ HighestTid ] when currLayerId is equal to 0 or is set to max_vps_num_reorder_pics[ CurrLayerId ][  HighestTid ] for the currLayerId when currLayerId is greater than 0. Otherwise if layer DPB information parameters layer_dpb_info_parameters() are not present MaxNumReorderPics[ currLayerId ][  HighestTid ] is set to sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	If layer DPB information parameters layer_dpb_info_parameters() are present in VPS, MaxLatencyIncreasePlus1[ currLayerId ][  HighestTid ] is set to vps_max_latency_increase_plus1[ HighestTid ] when currLayerId is equal to 0 or is set to max_vps_latency_increase_plus1[ CurrLayerId ][  HighestTid ] for the currLayerId when currLayerId is greater than 0. If layer DPB information parameters layer_dpb_info_parameters() are present in VPS, MaxLatencyPictures[ currLayerId ][ HighestTid ] is set to VpsMaxLatencyPictures [ HighestTid ] when currLayerId is equal to 0 or is set to VpsMaxLatencyPictures [ CurrLayerId ][  HighestTid ] for the currLayerId when currLayerId is greater than 0. Otherwise if layer DPB information parameters layer_dpb_info_parameters() are not present for the for the operation point under test,  MaxLatencyIncreasePlus1[ currLayerId ][  HighestTid ] is set to sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId and MaxLatencyPictures[ currLayerId ][ HighestTid ] is set to SpsMaxLatencyPictures[ currLayerId ][ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	If operation point DPB information parameters oop_dpb_maxbuffering_parameters () are present for the selected output layer set, MaxDecPicBufferingMinus1[ currLayerId ][HighestTid ] is set to vps_max_dec_pic_buffering_minus1[ HighestTid ] when currLayerId is equal to 0 or is set to max_vps_dec_pic_buffering_minus1[ TargetOpLs ][ CurrLayerId ][  HighestTid ] for the currLayerId for the operation point under test when currLayerId is greater than 0. Otherwise if operation point DPB information parameters oop_dpb_maxbuffering_parameters () are not present for the operation point under test,  MaxDecPicBufferingMinus1[ currLayerId ][HighestTid ] is set to sps_max_dec_pic_buffering_minus1[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

When one or more of the following conditions are true, the "bumping" process specified in subclause Error: Reference source not found is invoked repeatedly while further decrementing the DPB fullness DPBfullness[currLayerId] by one for each additional picture storage buffer corresponding to the nuh_layer_id value of the current decoded picture that is emptied, until none of the following conditions are true:

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than MaxNumReorderPics[CurrLayerId][HighestTid] sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	If MaxLatencyIncreasePlus1[CurrLayerId][HighestTid] sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] is greater than or equal to MaxLatencyPictures[CurrLayerId][HighestTid] SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB DPBfullness[currLayerId] is greater than or equal to MaxDecPicBuffering[CurrLayerId][HighestTid] sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

F.13.5.2.3 Picture decoding, marking, additional bumping, and storage

The processes specified in this subclause happen instantaneously when the last decoding unit of access unit n containing the current picture is removed from the CPB.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture.

For each picture in the DPB that is marked as "needed for output" and that has a nuh_layer_id value equal to currLayerId, the associated variable PicLatencyCount[ currLayerId ] is set equal to PicLatencyCount[ currLayerId ] + 1.

The current picture is considered as decoded after the last decoding unit of the picture is decoded. The current decoded picture is stored in an empty picture storage buffer corresponding to currLayerId (the nuh_layer_id value of the current picture) in the DPB, and the following applies:

–	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output" and its associated variable PicLatencyCount[ currLayerId ] is set equal to 0.

–	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

The current decoded picture is marked as "used for short-term reference".

When one or more of the following conditions are true, the "bumping" process specified in subclause Error: Reference source not found is invoked repeatedly until none of the following conditions are true.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than MaxNumReorderPics[CurrLayerId][HighestTid] sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	MaxLatencyIncreasePlus1[CurrLayerId][HighestTid] sps_max_latency_increase_plus1[ HighestTid ] is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] that is greater than or equal to MaxLatencyPictures[CurrLayerId][HighestTid] SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

F.13.5.2.4 "Bumping" process

The "bumping" process consists of the following ordered steps:

- The pictures that are first for output are selected as the ones having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- These pictures are cropped, using the conformance cropping window specified in the active SPS for the picture with nuh_layer_id equal to 0 or in the active layer SPS for a nuh_layer_id value equal to that of the picture, the cropped pictures are output in ascending order of nuh_layer_id, and the pictures are marked as "not needed for output".

- Each picture storage buffer that contains a picture marked as "unused for reference" and that included one of the pictures that was cropped and output is emptied.

# Conclusion

This document includes two proposals for DPB operation for HEVC Extensions.

1. This document proposes the operation and bumping process for output order DPB of SHVC and MV-HEVC to use the DPB parameters signaled in VPS extension in JCTVC-N0197 if they are signaled for the selected output layer set. Otherwise the SPS level parameters are used.

2. Other changes to the text for DPB operation are proposed with operation on picture basis and some steps applied on AU basis.

We propose to adopt the proposals in SHVC and MV-HEVC draft texts.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 1,” JCTVC-M1008, Incheon, May 2013

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 4 (ISO/IEC 23008-2:201x/PDAM2),” JCTVC-D1004, Incheon, May 2013.