| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0147 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0085 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On restriction and indication of cross-layer IRAP picture distribution |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Ye-Kui Wang Krishna Rapaka Adarsh K. Ramasubramonian Hendry 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8208 cjianle@qti.qualcomm.com yekuiw@qti.qualcomm.com krapaka@qti.qualcomm.com aramasub@qti.qualcomm.com fhendry@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposed to have restriction of cross-layer IRAP picture distribution and sequence level indication of cross-layer IRAP pictures alignment.

# Problem statement

In the current MV-HEVC and SHVC working draft, CRA pictures are not required to be cross-layer aligned, while IDR pictures and BLA pictures are required to be cross-layer aligned, i.e.

- When one picture in an AU is an IDR picture, all pictures in the AU shall be IDR pictures.

- When one picture in an AU is a BLA picture, all pictures in the AU shall be BLA pictures.

Not requiring CRA pictures to be cross-layer aligned allows for the following two advantageous usage scenarios:

- This allows for more frequent random access points (RAPs) at lower layers than at higher layers, e.g. twice as many RAPs at the base layer than at the enhancement layer in a two-layer bitstream. This would enable tuning-in to a broadcast or multicast session with low delay (compared with both layers having the same frequency of RAPs as that of the enhancement layer) and high efficiency (compared with both layers having the same frequency of RAPs as that of the base layer), as the receiver may tune-in to receive only the base layer first at an AU for which the base layer is a CRA picture while the enhancement layer is not and then add the enhancement layer at a position where both layers in an access unit are CRA pictures.

- This also allows for more frequent random access points (RAPs) at higher layers than at lower layers, e.g. twice as much RAPs at the enhancement layer than at base in a two-layer setup. This would enable faster layer up-switching with high efficiency. Note that with multi-loop decoding, which is the current assumption for both 3DV and scalable extensions, layer (set) down-switching is possible at any AU.

In spite of the advantages listed above for allowing for non-cross-layer-aligned picture types such as CRA pictures and IDR pictures, however, if a bitstream is encoded with one or more or all picture types cross-layer aligned, it would be beneficial for system entities, such as servers and media-aware network elements (MANEs), also referred to as intelligent media gateways, to know and make use of this information. Therefore, it would be desirable to provide mechanisms for indications of cross-layer alignment of picture types in a way that the indication is easily accessible by different entities in media transmission and application systems.

As mentioned above, there are mainly two purposes to allow cross-layer non-alignment of IRAP picture types: 1) allows for more frequent random access points (RAPs) at lower layers than at higher layers, 2) allows for more frequent random access points (RAPs) at higher layers than at lower layers. However, the current non-alignment scheme might be overly flexible than what is necessary to attain the above functionality. For example, the current HEVC multi-layer extensions allow that two layers have the same number of IRAP pictures in a CVS while the IRAP pictures are not cross-layer synchronized, or that layer A has more IRAP pictures than layer B in a CVS but for an IRAP picture in layer B in the CVS the picture in layer A in the same access unit is not an IRAP. Such flexibilities potentially increase implementation cost, e.g. decoder conformance testing, as well as complexity for application systems to find random access and layer switching points while they do not provide any benefits, e.g. additional useful functionality or coding efficiency improvement.

# Proposal

The contribution proposes to have a restriction of cross-layer IRAP pictures distribution structure; only one of the following IRAP pictures distribution patterns is allowed in any CVS to avoid the useless cases mentioned in section 1.

- IRAP pictures are cross-layer aligned, that is, when a picture of one layer in an AU is an IRAP picture, all other pictures in the same AU are IRAP pictures.

- Lower layers have more IRAP pictures such that when a picture of a current layer in an AU is an IRAP picture, all pictures in the same AU of the layers, which are specified as direct dependent layers of the current layer in VPS shall also be IRAP pictures.

- Higher layers have more IRAP pictures such that when a picture of a current layer in an AU is an IRAP picture, all pictures in the same AU of the layers, for which the current layer is specified as its direct dependent layer in VPS shall also be an IRAP picture.

Furthermore, this contribution proposes to signal an indication of IRAP distribution pattern in the VPS extension since it’s beneficial for system entities to know the information.

# Specification changes

The proposed changes are as follows. Note that through requiring the value of vps_cross_layer_irap_align_idc to be in range of 0 to 2, inclusive, the proposed restriction that allows for only one of the three IRAP pictures distribution patterns is also imposed.

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| vps_cross_layer_pic_type_align_flag | u(1) |
| if( vps_cross_layer_pic_type_align_flag ) |  |
| vps_cross_layer_irap_align_idc | u(2) |
| ... |  |
| } |  |

vps_cross_layer_pic_type_align_flag equal to 1 specifies that within a CVS that refers to the VPS, all VCL NAL units that belong to an access unit have the same value of nal_unit_type. vps_cross_layer_pic_type_align_flag equal to 0 specifies that within a CVS that refers to the VPS, all VCL NAL units in each access unit may or may not have the same value of nal_unit_type.

vps_cross_layer_irap_align_idc equal to 0 specifies that IRAP pictures in each CVS that refer to the VPS are cross-layer aligned, i.e. for any layer index i in the range of 0 to vps_max_layers_minus1, inclusive, and any layer index j (j!=i) in the range of 0 to vps_max_layers_minus1, inclusive, when a picture with layer index i in an AU is an IRAP picture and direct_dependency_flag[ i ][ j ] or direct_dependency_flag[ j ][ i ] is equal to 1, the picture with layer index j in the same AU, when present, is an IRAP picture. vps_cross_layer_irap_align_idc equal to 1 specifies that within each CVS that refers to the VPS, for any layer index i in the range of 1 to vps_max_layers_minus1, inclusive, and any layer index j in the range of 0 to i − 1, inclusive, when a picture with layer index i in an AU is an IRAP picture and direct_dependency_flag[ i ][ j ] is equal to 1, the picture with layer index j in the same AU, when present, is an IRAP picture. vps_cross_layer_irap_align_idc equal to 2 specifies that within each CVS that refers to the VPS, for any layer index i in the range of 1 to vps_max_layers_minus1, inclusive, and any layer index j in the range of 0 to i − 1, inclusive, when a picture with layer index j in an AU is an IRAP picture and direct_dependency_flag[ i ][ j ] is equal to 1, the picture with layer index i in the same AU, when present, is an IRAP picture. The value of vps_cross_layer_irap_align_idc shall be in range of 0 to 2, inclusive. When not present, the value of vps_cross_layer_irap_align_idc is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when single_layer_for_non_irap_flag is equal to 0, for any layer index i in the range of 1 to vps_max_layers_minus1, inclusive, a picture with layer index i in an AU shall not be an IRAP picture, unless, for any layer index j in the range of 0 to i − 1, inclusive, the picture with layer index j in the same AU is present in the bitstream when direct_dependency_flag[ i ][ j ] is equal to 1.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).