| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0165 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0086 |
| --- | --- |

| Title: | On VPS extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | {yongjin9.cho, b.d.choi, m.w.park, jinyoung79.lee, hc.wey, and dionism} @samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes restructuring of the current design of the video parameter set extension. Also, a semantics change of default_one_target_output_layer_flag is proposed.

# Introduction

Syntax elements in the current vps_extension( ) can be categorized based on their applicable target scalability type/dimension and number of scalability types in a bitstream. Tabe 1 shows an example of such categorization, where ‘common’ refers that corresponding syntax elements are supposed be common for all (or at least, those defined in the current draft) scalability types, ‘mixed’ refers to syntax elements that are necessary when the number of scalability types is greater than 1.

Table 1. Example of categorization of syntax elements in vps_extension( )

| Syntax Element | Category |
| --- | --- |
| vps_extension_byte_alignment_reserved_one_bit | Common |
| avc_base_layer_flag | Common |
| splitting_flag | Common/Mixed |
| scalability_mask | Common |
| dimension_id_len_minus1 | Mixed |
| vps_nuh_layer_id_present_flag | Multiview |
| layer_id_in_nuh | Multiview |
| dimension_id | Mixed |
| direct_dependency_flag | Common |
| max_tid_il_ref_pics_plus1 | Common |
| vps_num_layer_sets_minus1 | Multiview |
| vps_num_profile_tier_level_minus1 | Common |
| vps_profile_present_flag | Common |
| profile_ref_minus1 | Common |
| more_output_layer_sets_than_default_flag | Multiview |
| num_add_output_layer_sets_minus1 | Multiview |
| default_one_target_output_layer_flag | Mixed |
| output_layer_set_idx_minus1 | Multiview |
| output_layer_flag | Multiview |
| profile_level_tier_idx | Multiview |
| max_one_active_ref_layer_flag | Common |
| direct_dep_type_len_minus2 | Common |
| direct_dependency_type | Common |
| single_layer_for_non_irap_flag | Spatial/SNR |

Based on the observation in Table 1, we proposed to re-structure the current vps_extension syntax as the following:

- Move common syntax elements to front

- Separate non-common syntax elements based on scalability_mask[i] (based on whether a bitstream contains multiview scalability dimensions or not) and number of scalability types.

In previous meetings, similar contributions have been proposed [1],[2].

# Proposal

Proposal 1. Change of semantics of default_one_target_output_layer_flag semantics (changed parts are highlighted in yellow)

Current semantics:

default_one_target_output_layer_flag equal to 1 specifies that only the highest layer in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers.

Proposed semantics:

default_one_target_output_layer_flag equal to 1 specifies that only the layer with the highest DependencyId in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all multiview layers with the highest DependencyId in each of the default output layer sets are target output layers. When NumScalabilityTypes is 1, the value of default_one_target_output_layer_flag is inferred to be 0 for multiview scalability, and 1 for spatial/SNR scalability.

Proposal 2. Arrangement of syntax elements of vps_extension () based on the categorization in Table 1.

In Table 1, we have the following claims:

- vps_nuh_layer_id_present_flag, layer_id_in_nuh, vps_num_layer_sets_minus1, more_output_layer_sets_than_default_flag, num_add_output_layer_sets_minus1, output_layer_set_idx_minus1, output_layer_flag, and profile_level_tier_idx are not required for an SHVC profile. That is, they correspond only to multiview scalability based on the current working draft.

- single_layer_for_non_irap_flag is not required for multiview scalability. That is, it corresponds only to an SHVC profile based on the current working draft.

- default_one_target_output_layer_flag is not required when the number of scalability types is equal to 1

Because direct_dep_type_len_minus2 syntax element is moved to the front, we propose to change its descriptor from ue(v) to u(5) (highlighted in green).

- Proposed Syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| for (i=1; i<=vps_num_layers_minus1; i++) |  |
| for (j=0; j<i; j++) |  |
| direct_dependency_flag[i][j] | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | u(5) |
| for (i=1; i<=vps_max_layers_minus1; i++) |  |
| for (j=0; j<i; j++) |  |
| if (direct_dependency_flag[i][j]) |  |
| direct_dependency_type[i][j] | u(v) |
| for (i=0, NumScalabilityTypes=0; i<16; i++) { |  |
| scalability_mask[i] | u(1) |
| NumScalabilityTypes += scalability_mask[i] |  |
| } |  |
| if (NumScalabilityTypes > 1) { |  |
| splitting_flag | u(1) |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| for (i=0; i<=vps_max_layers_minus1; i++) |  |
| if (!splitting_flag) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| default_one_target_output_layer_flag | u(1) |
| } |  |
| else { |  |
| if (scalability_mask[0]==1) { // multiview scalability |  |
| vps_nuh_layer_id_present_flag | u(1) |
| if (vps_nuh_layer_id_present_flag) |  |
| for (i=1; i<=vps_max_layers_minus1; i++) |  |
| layer_id_in_nuh[i] | u(6) |
| vps_number_layer_sets_minus1 | u(10) |
| numOutputLayerSets = vps_number_layer_sets_minus1+1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if (more_output_layer_sets_than_default_flag) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1+1 |  |
| } |  |
| for (i=1; i<numOutputLayerSets; i++) { |  |
| if ( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[i] | u(v) |
| lsIdx=output_layer_set_idx_minus1[i]+1 |  |
| for (j=0; j<NumLayersInIdList[lsIdx]-1; j++) |  |
| output_layer_flag[i][j] | u(1) |
| } |  |
| profile_level_tier_idx[i] | u(v) |
| } |  |
| } |  |
| else if (scalability_mask[1]==1) { // spatial/SNR scalability | u(1) |
| single_layer_for_non_irap_flag |  |
| } |  |
| else if (scalability_mask[i]==1) { |  |
| // to be defined for other scalability types |  |
| } |  |
| } |  |

- Proposal 3. In this proposal, more syntax elements than those in Proposal 2 are considered to be not required for spatial/snr scalability. They are direct_dependency_flag and max_one_active_ref_layer_flag with some semantics change.

Proposed syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| for (i=0, NumScalabilityTypes=0; i<16; i++) { |  |
| scalability_mask[i] | u(1) |
| NumScalabilityTypes += scalability_mask[i] |  |
| } |  |
| if (NumScalabilityTypes > 1) { |  |
| splitting_flag | u(1) |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| for (i=0; i<=vps_max_layers_minus1; i++) |  |
| if (!splitting_flag) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| default_one_target_output_layer_flag | u(1) |
| } |  |
| else { |  |
| if (scalability_mask[0]==1) { |  |
| vps_nuh_layer_id_present_flag | u(1) |
| if (vps_nuh_layer_id_present_flag) |  |
| for (i=1; i<=vps_max_layers_minus1; i++) |  |
| layer_id_in_nuh[i] | u(6) |
| for (i=1; i<=vps_num_layers_minus1; i++) |  |
| for (j=0; j<i; j++) |  |
| direct_dependency_flag[i][j] | u(1) |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for (i=1; i<=vps_max_layers_minus1; i++) |  |
| for (j=0; j<i; j++) |  |
| if (direct_dependency_flag[i][j]) |  |
| direct_dependency_type[i][j] | u(v) |
| vps_number_layer_sets_minus1 | u(10) |
| numOutputLayerSets = vps_number_layer_sets_minus1+1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if (more_output_layer_sets_than_default_flag) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1+1 |  |
| } |  |
| for (i=1; i<numOutputLayerSets; i++) { |  |
| if ( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[i] | u(v) |
| lsIdx=output_layer_set_idx_minus1[i]+1 |  |
| for (j=0; j<NumLayersInIdList[lsIdx]-1; j++) |  |
| output_layer_flag[i][j] | u(1) |
| } |  |
| profile_level_tier_idx[i] | u(v) |
| } |  |
| } |  |
| if (scalability_mask[1]==1) { // spatial/SNR scalability |  |
| single_layer_for_non_irap_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for (i=1; i<=vps_max_layers_minus1; i++) |  |
| direct_dependency_type[i][i-1] | u(v) |
| } |  |
| if (scalability_mask[i]==1) { |  |
| // to be defined for other scalability types |  |
| } |  |
| } |  |
| } |  |

# References

- J. Boyce et. al., “High-level syntax modifications for HEVC extensions,” Document JCTVC-M0045, the 13th JCTVC meeting, Incheon, KR, Apr. 2013

- Hendry et. al., “On layer id and dimension id mapping in VPS extension,” Document JCTVC-L0390, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).