| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0267 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0087 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On changing of the highest layer ID across AUs and multi-mode bitstream extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Ying Chen 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document discusses allowing changing of the highest value of nuh_layer_id across AUs within a CVS, and proposes a multi-mode bitstream extraction process.

# On changing of highest layer ID across AUs within a CVS

In both SVC and MVC, the extensions of AVC, the highest value of nuh_layer_id is not allowed to change across AUs within a CVS. In the current WD of MV-HEVC and SHVC, this is allowed. In particular, the VPS flag single_layer_for_non_irap_flag relies on this for support of adaptive resolution change.

The semantics of single_layer_for_non_irap_flag is as follows:

single_layer_for_non_irap_flag equal to 1 indicates either that all the VCL NAL units of an access unit have the same nuh_layer_id value or that two nuh_layer_id values are used by the VCL NAL units of an access unit and the picture with the greater nuh_layer_id value is an IRAP picture. single_layer_for_non_irap_flag equal to 0 indicates that nuh_layer_id values may or may not be constrained beyond constraints specified in other parts of this Recommendation | International Standard.

This (single_layer_for_non_irap_flag equal to 1) seems to allow a lot of flexibilities. To better understand the design intent, we raise the following questions:

- When there are only two or more layers, is it allowed that all the layers have the same spatial resolution, color format, and bit depth? Or in a more generic form, is it allowed to have two layers that have the same spatial resolution, color format, and bit depth?

- How does the decoder differentiate between 1) there was no picture in the bitstream at the position of the dashed box, and 2) there was a picture but lost during transmission?

- If there are only two layers and in each AU there are two pictures, if the standard allows the highest value of nuh_layer_id change across AUs within a CVS, then any number of enhancement layers pictures starting from the last one in a CVS can be freely dropped. Then there can be a huge number of possible sub-bitstreams as a result of such dropping. Are these sub-bitstream conforming bitstreams? If not, how to know whether such a bitstream is a conforming bitstream or not? If yes, how many set of HRD parameters need to be specified for these conforming bitstreams?

A lot more such questions on the allowed behaviors could be asked. For example, in JCTVC-N0110 some interesting DPB management behaviors are discussed.

We think that two much flexibility is allowed, and sort out of control as the result of allowing the highest value of nuh_layer_id to change across AUs within a CVS. Thus we would like to suggest that, either issues such as the above are resolved with concrete solutions, or, if not, questions to suggest this to be disallowed, changing of the highest value of nuh_layer_id across AUs within a CVS is disallowed.

# On multi-mode bitstream extraction

In SVC, MVC, MVC+D, and HEVC version 1, the sub-bitstream extraction process is specified. Each extractable sub-bitstream is required to be a conforming bitstream.

One issue that has been considered in the design of the bitstream extraction process, e.g. in MVC development, is whether to target the best sub-bitstream self-completeness (such that each sub-bitstream further extracted from a sub-stream, recursively, can be a conforming bitstream) or the optimal size of a sub-bitstream (such that some or all pictures that are not needed for correct decoding of the target output layers do not have to be included in the extracted sub-bitstream).

Targeting best self-completeness makes the bitstream extraction process easier to specify. A self-complete sub-bitstream is still "fully extractable" and is more useful if the bitstream extraction is performed by a streaming server or an intermediate network element and each subset of the bitstream may be further requested by some end users. However, on the other hand, if the extracted sub-bitstream is directly sent to end users and is not supposed to be further extracted, better transmission bandwidth efficiency can be achieved by targeting the optimal size of the bitstream.

To enable either advantage to be applied depending on applications, the sub-bitstream extraction process can be designed to adaptively choose the best extraction operation.

Therefore, we propose to support multi-mode bitstream extractions. The text changes are provided below (with additions being highlighted in green, and removals strikethrough).

- Sub-bitstream extraction processInputs to this process are a bitstream, a target highest TemporalId value tIdTarget, and a target layer identifier list layerIdListTarget, a target output layer identifier list TargetOptLayerIdList, and an extraction mode subExtModeIdc, which is specified by an external means when present, or set equal to 0 when not specified by an external means.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream of the process specified in this subclause with tIdTarget equal to any value in the range of 0 to 6, inclusive, and layerIdListTarget equal to the layer identifier list associated with a layer set specified in the active video parameter set shall be a conforming bitstream when subExtModeIdc is equal to 0.

NOTE 1 – A conforming bitstream contains one or more coded slice segment NAL units with nuh_layer_id equal to 0 and TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	When one or more of the following two conditions are true, remove all SEI NAL units that have nuh_layer_id equal to 0 and that contain a non-nested buffering period SEI message, a non-nested picture timing SEI message, or a non-nested decoding unit information SEI message:

–	layerIdListTarget does not include all the values of nuh_layer_id in all NAL units in the bitstream.

–	tIdTarget is less than the greatest TemporalId in all NAL units in the bitstream.

NOTE 2 – A "smart" bitstream extractor may include appropriate non-nested buffering picture SEI messages, non-nested picture timing SEI messages, and non-nested decoding unit information SEI messages in the extracted sub-bitstream, provided that the SEI messages applicable to the sub-bitstream were present as nested SEI messages in the original bitstream.

–	Remove all NAL units with TemporalId greater than tIdTarget or nuh_layer_id not among the values included in layerIdListTarget.

–	When subExtModeIdc is greater than 0, further remove NAL units with nuh_layer_id not among the values included in TargetOptLayerIdList and with a TemporalId greater than max_tid_il_ref_pics_plus1[ LayerIdxInVps[ nuh_layer_id ] ] – 1.

–	When subExtModeIdc is greater than 1, further remove NAL units with nuh_layer_id not among the values included in TargetOptLayerIdList and discardable_flag equal to 1.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).