| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0119r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0090r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On collocated picture indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee Jin Soo Choi Visual Media Research Section, ETRI 218 Gajeong-ro, Yuseong-gu Daejeon, Rep. of Korea | Tel: Email: | +82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

In SHVC/MV-HEVC draft text, reference layer picture can be used as a collocated picture for temporal motion vector prediction. For this, there are two syntax elements ‘alt_collocated_indication_flag’ and ‘collocated_ref_layer_idx’ in the slice segment header. However, when ‘alt_collocated_indication_flag’ is equal to 1 and current slice is B slice, since current draft text does not specify the syntax element ‘collocated_from_l0_flag’, it is not possbile to know whether a collocated picture is derived from reference picture list 0 or reference picture list 1. Also the motion vector of collocated prediction block can not be derived without low-level changes. This contribution proposes two alternatives to solve these problems.

# Problem Statement

In SHVC/MV-HEVC draft text[1][2], a reference layer picture can be used as a collocated picture for temporal motion vector prediction(TMVP). For this, there are two syntax elements ‘alt_collocated_indication_flag’ and ‘collocated_ref_layer_idx’ in the slice segment header as shown in table below. For inter-layer motion prediction, when ‘alt_collocated_indication_flag’ is equal to 1, a reference layer picture indicated by ‘collocated_ref_layer_idx’ is used as a collocated picture for TMVP. However, since draft text does not specify the syntax element ‘collocated_from_l0_flag’ when reference layer picture is used as a collocated picture for TMVP, there are following problems.

| slice_segment_header() { |  |
| --- | --- |
| … |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

## Problem 1

In the process of the reference picture list construction, when current slice is B slice, inter-layer reference pictures are inserted into both reference picture list 0(RefPicList0) and reference picture list 1(RefPicList1). This means that there is the inter-layer reference picture indicated by ‘collocated_ref_layer_idx’ in the both RefPicList0 and RefPicList1. When ‘alt_collocated_indication_flag’ is equal to 1 and current slice is B sice, draft text only specifies which one is used as a collocated picture among inter-layer reference pictures but it does not specify direction of the reference picture list. Therefore, it is necessary to give information such as ‘collocated_from_l0_flag’ whether a inter-layer reference picture in the RefPicList0 or RefPicList1 is selected as a collocated picture for TMVP.

## Problem 2

In the 8.5.3.2.8 derivation process for collocated motion vectors of HEVC[3], if collocated prediction block has motion information in the both LIST0 and LIST1 and the following low delay condition is not true, mvCol, refIdxCol, and listCol of collocated prediction block depend on the value of ‘collocated_from_l0_flag’.

- low delay condition : DiffPicOrderCnt(aPic, currePic) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice.

As previous mentioned, since current draft text does not specify the syntax element ‘collocated_from_l0_flag’ when ‘alt_collocated_indication_flag’ is equal to 1 and current slice is B slice, motion information (mvCol, refIdxCol and listCol) of collocated prediction block can not be derived without low-level change.

Therefore, this contribution proposes two alternatives to solve the problems.

# Proposed solutions

## Alternative 1

The position of syntax element ‘collocated_from_l0_flag’ is changed as shown in the following table. In addition to this, decoding process of “G.8.5.3.2.7/G.8.5.1” is changed as below.

| slice_segement_header() { |  |
| --- | --- |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( nuh_layer_id > 0 && alt_collocated_indication_flag ) { |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| } |  |

G.8.5.3.2.7 / G.8.5.1 Derivation process for temporal luma motion vector prediction (SHVC/MV-HEVC)

- Depending on the values of alt_collocated_indication_flag, collocated_ref_layer_idx, slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the collocated picture, is derived as follows:

- If alt_collocated_indication_flag is equal to 1, colPic is set equal to the picture in the current access unit with nuh_layer_id equal to ActiveMotionPredRefLayerId[ collocated_ref_layer_idx ].

- If alt_collocated_indication_flag is equal to 1 and slice_type is equal to B and collocated_from_l0_flag is equal to 0, colPic is set equal to the picture in the RefPicList1 with nuh_layer_id equal to ActiveMotionPredRefLayerId[collocated_ref_layer_idx].

- otherwise (alt_collocated_indication_flag is equal to 1 and slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), colPic is set equal to the picture in the RefPicList0 with nuh_layer_id equal to ActiveMotionPredRefLayerId[collocated_ref_layer_idx].

- Otherwise, if slice_type is equal to B and collocated_from_l0_flag is equal to 0, colPic is set equal to RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), colPic is set equal to RefPicList0[ collocated_ref_idx ].

## Alternative 2

Compare to draft text, only semantic of syntax element ‘alt_collocated_indication_flag’ is changed. In this, the syntax element ‘collocated_from_l0_flag’ can be inferred from the syntax element ‘alt_collocated_indifcation_flag’. In addtion to this, decoding process of G.8.5.3.2.7/G8.5.1 is chaged as belows.

alt_collocated_indication_flag equal to 0 specifies that a collocated picture for temporal motion vector prediction is indicated by collocated_from_l0_flag, when present, and collocated_ref_idx. alt_collocated_indication_flag equal to 1 specifies that a collocated picture for temporal motion vector prediction is indicated by collocated_ref_layer_idx. When alt_collocated_indication_flag is not present, it is inferred to be equal to 0. When alt_collocated_indication_flag is equal to 1, collocated_from_l0_flag is inferred to be equal to 1.

G.8.5.3.2.7 / G.8.5.1 Derivation process for temporal luma motion vector prediction (SHVC/MV-HEVC)

- Depending on the values of alt_collocated_indication_flag, collocated_ref_layer_idx, slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the collocated picture, is derived as follows:

- If alt_collocated_indication_flag is equal to 1, colPic is set equal to the picture in the current access unit with nuh_layer_id equal to ActiveMotionPredRefLayerId[ collocated_ref_layer_idx ].

- If alt_collocated_indication_flag is equal to 1, colPic is set equal to the picture in the RefPicList0 with nuh_layer_id equal to ActiveMotionPredRefLayerId [collocated_ref_layer_idx].

- Otherwise, if slice_type is equal to B and collocated_from_l0_flag is equal to 0, colPic is set equal to RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), colPic is set equal to RefPicList0[ collocated_ref_idx ].

# References

[1] “SHVC Working Draft 2”, JCTVC-M1008, 13th JCT-VC Meeting, Incheon, Korea, April 2013.

[2] “MV-HEVC Draft Text 4”, JCT3V-D1004, 4th JCT-3V Meeting, Incheon, Korea, April 2013

[3] “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” JCTVC-L1003, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).