| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0290 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0098 |
| --- | --- |

| Title: | Ultra-low delay with SHVC, MV-HEVC and 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Karsten Sühring Yago Sanchez Thomas Schierl | Email: | robert.skupin@hhi.fraunhofer.de karsten.suehring@hhi.fraunhofer.de yago.sanchez@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The current bitstream order in SHVC, MV-HEVC and 3D-HEVC prohibits ultra-low delay applications. This contribution proposes an ultra-low delay enabling bitstream order and discusses respective coded picture buffer operation as well as fall back solutions for devices that are incapable of ultra-low delay operation. The same document is submitted to JCT-VC as JCTVC-N0290.

# Introduction

The HEVC base specification contains the necessary tools to allow decoding of bitstreams with ultra-low delay, i.e. through CPB operation on DU level and CPB timing information with DU granularity as opposed to CPB operation on AU level as in H.264/AVC. Thus, a device can operate on sub-portions of pictures in order to reduce occurring processing delays.

For similar ultra-low delay operations in the multi-layer SHVC, MV-HEVC and 3D-HEVC extensions of HEVC, e.g. for an encoder to send enhancement layer slices before all base layer slices are encoded, two requirements have to be fulfilled:

- A flexible bitstream order that does not prohibit layer-wise interleaving of DUs within an AU.

- Low impact handling of interleaved bitstreams for devices that are not ultra-low delay capable.

At the last JCT-VC meeting, a bitstream order of DUs was proposed in JCTVC-M0204 [1] that optionally allows interleaving of DUs within an AU based on the spatial segmentation offset constraints determined by the predictive coding and spatial segmentation setup. While signalling of the respective offset constrains (JCTVC-M0463) and an AU definition that does not prohibit interleaving were adopted, the proposed bitstream order was put for further study. This document proposes an updated flag-controlled interleaving-enabled bitstream order with constraints based on the decoding units of an access unit to satisfy requirement 1 as mentioned above.

In order to address requirement 2, i.e. provide a low impact mechanism for processing of interleaved bitstreams on devices that are only capable of processing non-interleaved bitstreams, this document proposes the use of the CPB removal process. A flag based indication that DU CPB removal timings for interleaved processing are signalled within the bitstream is proposed. Two CPB solutions are described in this document, either using a single CPB for all layers or individual CPBs for each layer. As in the base spec, devices that are not capable of (interleaved) DU processing can fall back to an AU based CPB operation mode in which all DUs of an AU are removed with a layer ordering constraint to derive an AU with non-interleaved DUs.

# Proposal

This document proposes to add a flag to the VPS extension syntax to indicate interleaving of DUs across layers within an AU as follows.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| du_interleaving_enabled_flag | u(1) |
| ... |  |

du_interleaving_enabled_flag equal to 1, specifies that decoding units of different coded picture are interleaved across layers within an access unit. When du_interleaving_enabled_flag is equal to 0, for each view/layer picture component, all decoding units follow each other directly in the bitstream (i.e. no interleaving is used.)

## Bitstream order

Section F.7.4.2.3 of JCTVC-M1008_v3 and JCT3V-D1004_v4 adds the following constraint regarding the bitstream order of coded pictures.

A coded picture with nuh_layer_id equal to nuhLayerIdA shall precede in decoding order all coded pictures with nuh_layer_id greater than nuhLayerIdA in the same access unit.

Section 7.4.2.4.5 Order of VCL NAL units and association to coded pictures of the HEVC base specification applies in the current extension drafts and this document proposes the following changes highlighted in yellow.

Order of VCL NAL units and association to coded pictures

This subclause specifies the order of VCL NAL units and association to coded pictures.

Each VCL NAL unit is part of a coded picture.

The order of the VCL NAL units within a coded picture is constrained as follows:

–	The first VCL NAL unit of the coded picture shall have first_slice_segment_in_pic_flag equal to 1.

–	Let sliceSegAddrA and sliceSegAddrB be the slice_segment_address values of any two coded slice segment NAL units A and B within the same coded picture. When either of the following conditions is true, coded slice segment NAL unit A shall precede the coded slice segment NAL unit B:

–	TileId[ CtbAddrRsToTs[ sliceSegAddrA ] ] is less than TileId[ CtbAddrRsToTs[ sliceSegAddrB ] ].

–	TileId[ CtbAddrRsToTs[ sliceSegAddrA ] ] is equal to TileId[ CtbAddrRsToTs[ sliceSegAddrB ] ] and CtbAddrRsToTs[ sliceSegAddrA ] is less than CtbAddrRsToTs[ sliceSegAddrB ].

If an access unit consists of more than one coded picture the order of the associated VCL NAL units is constrained as follows:

– 	If du_interleaving_enabled_flag is equal to 0, all VCL NAL units of a A coded picture with nuh_layer_id equal to nuhLayerIdA shall precede in decoding order the VCL NAL units of all coded pictures with nuh_layer_id greater than nuhLayerIdA in the same access unit.

–	Otherwise (du_interleaving_enabled_flag is equal to 1), all VCL NAL units of decoding unit m with nuh_layer_id equal to nuhLayerIdA shall precede in decoding order all VCL NAL units of decoding unit n with nuh_layer_id greater than nuhLayerIdA in the same access unit for which the decoding process depends on syntax elements or decoded sample values that belong to the decoding unit m.

## CPB operation

This document proposes to use the decoding unit information SEI message to carry CPB timings for the case that DUs are interleaved across layers within an access unit. CPB timing with DU granularity for ultra-low delay applications can be provided as given in the following.

| decoding_unit_info( payloadSize ) { | Descriptor |
| --- | --- |
| decoding_unit_idx | ue(v) |
| if( !sub_pic_cpb_params_in_pic_timing_sei_flag ) |  |
| du_spt_cpb_removal_delay_increment | u(v) |
| dpb_output_du_delay_present_flag | u(1) |
| if( dpb_output_du_delay_present_flag ) |  |
| pic_spt_dpb_output_du_delay | u(v) |
| } |  |

The indication that the HRD parameters for sub-picture based processing describe a bitstream with DUs interleaved across layers is the du_interleaving_enabled_flag and CPB timings respect the order of NAL units within the bitstream.

The impact of interleaved DUs across layers within an AU for the processing delay is illustrated in the following. In Figure 1 the arrival of decoding units (1) of an access unit n is shown for a layer-wise ordered bitstream. The numbers in the box refers to the value of nuh_layer_id of the NAL units of the DU. As shown, first all DUs of layer 0 arrive before DUs of layer 1 and so on.

Figure 1: Bitstream order of DUs of three layers over time (consecutive DUs).

In Figure 2 the arrival of decoding units (1) of an access unit n is shown for an interleaved bitstream according to the proposed interleaved bitstream order. As shown, DUs of different layers can be mixed within the access unit.

Figure 2: Bitstream order of DUs of three layers over time (interleaved DUs).

A CPB removal time is associated with each decoding unit. The CPB removal time cannot be lower than the final arrival time of a decoding unit, exemplarily shown as (2) for the first decoding unit. The final arrival time of the first decoding unit of the second layer, which is labelled with (3), can be lowered by using an interleaved bitstream order as shown in Figure 2 compared to the bitstream order shown in Figure 1, which allows earlier decoding of enhancement layers decoding units.

A device may base its decision to operate the CPB on access unit or sub-picture level on the du_interleaving_enabled_flag and fall back to access unit based operation when it is not capable of decoding interleaved DUs. The necessary recombining or rearranging of the interleaved DUs to an access unit with non-interleaved DU order can be achieved using the value of nuh_layer_id and the order of NAL units as an ordering constraint within the access unit and also depends on the CPB structure. Either a single CPB is used to accommodate the data of all layers or multiple CPB are provided to each accommodate the data of a single layer as described in the following subsections.

### Single CPB solution

When using a single CPB for the DUs of all layers, the memory fragmentation can be avoided by constraining the signalled timing in the following way: for any DU n that precedes the DU m in bitstream order, the CPB removal time for DU n shall be lower or equal to the CPB removal time of DU m. When arriving packets are stored at consecutive memory addresses in the CPB (typically in a ring buffer), this constraint avoids fragmentation of the free memory in the CPB.

On an AU scale, data is removed in the same order as received. The same is valid for DU based sub-picture CPB operation when the above constraint is obeyed. A decoder can be implemented that only keeps the start and the end address of used memory blocks instead of a list of used and free memory blocks. This also ensures that newly arriving DUs do not need to be split into several memory locations because used and free memory are continuous blocks.

However, the fall back to AU based CPB operation with the single CPB solution may requires non-continuous read operations to recombine an AU with non-interleaved DUs, i.e. all DUs from layer m precede DUs from layer m+1 when being sent to the decoder, so that the reference layer DUs are always removed before the referencing layer DUs but are not necessarily stored in consecutive memory blocks.

### Multi CPB solution

Alternatively multiple CPBs can be used for the DUs of each layer, i.e. depending on the value of nuh_layer_id, a NAL unit is written to the respective CPB, as shown in Figure 3.

Figure 3: Bitstream order, timing and memory usage of multi layer DUs in multiple CPBs.

The left side of Figure 3 shows the assignment of DUs to different CPBs. For each layer (number in the box), a separate CPB is operated and DUs are stored into different memory locations for each CPB. Exemplarily the arrival timing of an interleaved bitstream is shown. The assignment works in the same way for non-interleaved bitstreams based on the layer identifier. The right side of Figure 3 shows the memory usages with separate CPBs. DUs that belong to the same layer are stored in consecutive memory locations.

A multi-layer decoder can take advantage of such a memory layout because the DUs belonging to the same layer can be accessed at consecutive memory addresses. DUs arrive in decoding order for each layer. The used memory block always covers a continuous block in each CPB. The multi CPB solution also has advantages for bitstreams that are split layer-wise at the transport layer. If individual layers are transmitted using different channels the multiplexing of DUs into a single bitstream can be avoided. Thus the multi-layer video decoder does not have to implement this extra step and implementation cost can be reduced.

In case a decoder is not capable of CPB operation on sub-picture level for ultra-low delay mode and wants to use the AU based CPB operation, the recombining of DUs into an AU from multiple CPBs has to follow the same constraints as the single CPB solution, i.e. using value of nuh_layer_id (or CPB layer identifier) and the order of NAL units as an ordering constraint within the access unit. However, as all NAL units of a given layer are stored in continuous memory blocks, recombining the AU with non-interleaved DUs does not require a comparable recombining of AUs with non-continuous read operations as in the single CPB solution.

# References

- K. Suehring, R. Skupin, Y. Sanchez, T. Schierl, ”Ultra-low delay multi-layer decoding by interleaving of decoding units in SHVC and 3D/MV-HEVC”, JCTVC-M0204, Incheon, KR, April 2013.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).