| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0101 |
| --- | --- |

| Title: | MV-HEVC: RD performance of combined scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye Yan Univ. of Science and Technology of China Hefei, P. R. China Payman Aflaki Tampere Univ. of Technology Tampere, Finland Miska M. Hannuksela Nokia Research Center Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

JCT-3V has not decided whether the Stereo Main profile of MV-HEVC includes the support of AVC-coded base view. To provide input for the decision-making, this contribution presents results on 2-view coding with AVC base view and MV-HEVC non-base view compared against MVC. In the presented results, the Bjontegaard delta bitrate was -18.5% on average.

# Simulations

## Coding configurations

The combined scalability bitstreams were coded using the following steps:

- The JM software was used to code the base view. The parameter settings corresponded to the 3D-AVC CTC configuration (JCT3V-D1100).

- The reconstructed base view pictures were given as an input to SHM_AVC. The non-base view was coded as an enhancement layer with SHM_AVC using reference index based framework. In order to exploit inter-view prediction, the macro “REF_IDX_ME_ZEROMV” was turned off to allow motion vector searching between the base layer and the enhancement layer.

To follow the 3D-AVC CTC configuration as much as possible, the conventional SHM configuration was modified as follows:

- The hierarchical GOP structure of SHM was modified to be consistent with the 3D-AVC CTC (including cascaded QP setting and reference list construction).

- The lambda setting was modified based on the 3D-AVC setting.

- The other main changes and important parameters are specified in Table 1.

Table 1. Coding configuration of SHM for multi-view video coding

| Coding Parameters | Settings |
| --- | --- |
| Multi-view scenario | Three views (C2) |
| Coding Order | Right-Left |
| Multi-view resolution ratio | 1.0-1.0 |
| Inter prediction structure | HierarchicalB, GOP8 |
| Intra period | 24 |
| QP settings for base view | 41, 36, 31, 26 |
| SearchRange | 96 |
| Encoder settings | RDOQ OFF, RODQTS OFF |
| Test sequences and coded views | Texture views specified in CTC |

## Results

MVC bitstreams were used as the anchor, against which the Bjontegaard delta values were derived. The anchor was derived with the 3DV-ATM reference software, using the MVC+D (HP) configuration and using only the texture coding statistics (without including depth maps bitrates). In other words, the anchor bitstreams were formed by extracting the texture views from the HP anchor of the CTC (JCT3V-D1100).

Table 2 shows simulation results produced with the SHM_AVC (with AVC base view and HEVC enhancement view) compared against the two-view MVC anchor. It can be seen from Table 2, compared with the anchor, SHM_AVC can reduce the bitrates by 18.46% BD-rate for two views.

Table 2. Simulation results produced with SHM_AVC 
(with AVC base view and MV-HEVC non-base view), compared against MVC

|  | Non_base view |  | Two views |  |
| --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR, dB | dBR, % | dPSNR, dB |
| Balloons | -50.27 | 3.32 | -21.31 | 1.27 |
| Kendo | -48.27 | 3.08 | -20.78 | 1.18 |
| Newspaper | -34.53 | 1.68 | -13.70 | 0.65 |
| GT_Fly | -48.91 | 2.24 | -16.70 | 0.67 |
| Pozan_Hall2 | -63.00 | 3.35 | -29.35 | 1.11 |
| Poznan_Street | -31.30 | 0.86 | -10.29 | 0.33 |
| Dancer | -44.80 | 1.97 | -17.10 | 0.64 |
| 1024x768 | -44.36 | 2.69 | -18.60 | 1.03 |
| 1920x1088 | -47.00 | 2.11 | -18.36 | 0.69 |
| Average | -45.87 | 2.36 | -18.46 | 0.84 |