| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0102 |
| --- | --- |

| Title: | MV-HEVC HLS: depth representation information for auxiliary pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It was agreed in a joint MPEG Requirements / JCT-VC / JCT-3V meeting in April 2013 to specify support of depth coding with an auxiliary picture mechanism. This contribution proposes a syntax structure to indicate some properties associated with depth/disparity auxiliary pictures. The proposed indications are asserted to be useful in processing decoded texture and depth pictures prior to rendering on a 3D display, such as in view synthesis. The proposed indications follow those of the depth representation information SEI message of MVC+D with a few changes.

# Introduction

JCTVC-N0077 proposes auxiliary pictures to be specifies new VCL NAL unit type(s) and their characteristics indicated in the SPS extension. JCTVC-N0063r1/JCT3V-E0049r1 proposes auxiliary pictures to reside in auxiliary picture layers and includes two options to indicate their characteristics, either in the SPS extension or as SEI messages (a different SEI message per each auxiliary picture type).

This contribution proposes the syntax structure to indicate the characteristics of the depth/disparity auxiliary pictures. The proposed syntax structure can be included in either proposed container structure, i.e. in the SPS extension or as an SEI message. Moreover, the proposed syntax structure can be applied regardless of whether auxiliary pictures are specified with new VCL NAL unit type(s) (as proposed in JCTVC-N0077) or with auxiliary picture layers (as proposed in JCTVC-N0063/JCT3V-E0049).

The motivation to use the depth representation information SEI message of MVC+D as a basis for the proposal was that the SEI message is used for a similar purpose in MVC+D and hence apparently contains all the necessary information. Furthermore, the syntax of the SEI message was followed as much as possible.

The main differences compared to the proposed syntax and the depth representation information SEI message of MVC+D can be summarized as follows:

- The SEI message documents the properties of all depth views in the bitstream. The proposed syntax structure is specified in a manner that it documents the properties of a single view/picture.

This design choice is believed to be aligned with the spirit of HEVC SPS, which is agnostic of the layer structure of the bitstream (beyond temporal sub-layers). In case the auxiliary picture layer design is used with the proposed SEI message approach, the scalable nesting SEI message can include the proposed SEI message and indicate which auxiliary layers it applies to.

- The Z-axis reference view was not included in the proposed syntax and the units for the Z values are not specified, because these values are meaningful only relative to extrinsic camera parameters and at the moment there is no mechanism to indicate the extrinsic camera parameters in MV-HEVC.

# Proposed specification text

## Common part regardless of the containing syntax structure

| depth_representation_info_struct( ) { | Descriptor |
| --- | --- |
| z_near_flag | u(1) |
| z_far_flag | u(1) |
| d_min_flag | u(1) |
| d_max_flag | u(1) |
| depth_representation_type | ue(v) |
| if( d_min_flag ¦ ¦ d_max_flag ) |  |
| disparity_ref_view_id | ue(v) |
| if( z_near_flag ) |  |
| depth_ext_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| if( z_far_flag ) |  |
| depth_ext_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| if( d_min_flag ) |  |
| depth_ext_element( DMinSign, DMinExp, DMinMantissa, DMinManLen ) |  |
| if( d_max_flag ) |  |
| depth_ext_element( DMaxSign, DMaxExp, DMaxMantissa, DMaxManLen ) |  |
| if( depth_representation_type = = 3 ) { |  |
| depth_nonlinear_representation_num_minus1 | ue(v) |
| for( i = 1; i <= depth_nonlinear_representation_num_minus1 + 1; i++ ) |  |
| depth_nonlinear_representation_model[ i ] |  |
| } |  |
| } |  |

| depth_ext_element( OutSign, OutExp, OutMantissa, OutManLen ) { | Descriptor |
| --- | --- |
| da_sign_flag | u(1) |
| da_exponent | u(7) |
| da_mantissa_len_minus1 | u(5) |
| da_mantissa | u(v) |
| } |  |

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of auxiliary pictures as specified in Table XXX. In Table XXX, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera.

- Table XXX – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an auxiliary picture represents an inverse of Z value that is uniformly quantized into the range of 0 to 255, inclusive. |
| 1 | Each decoded luma sample value of an auxiliary picture represents disparity that is uniformly quantized into the range of 0 to 255, inclusive. |
| 2 | Each decoded luma sample value of an auxiliary picture represents a Z value uniformly quantized into the range of 0 to 255, inclusive. |
| 3 | Each decoded luma sample value of an auxiliary picture represents a nonlinearly mapped disparity, normalized in range from 0 to 255, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[ i ]. |
| Other values | Reserved for future use |

disparity_ref_view_id specifies the ViewId value against which the disparity values are derived.

The variables in the x column of Table YYY are derived as follows from the respective variables in the s, e, n, and v columns of Table YYY as follows.

–	If 0 < e < 127, x = ( −1 )s * 2e−31 * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x = ( −1 )s * 2−( 30+v ) * n.

NOTE – The above specification is similar to that found in IEC 60559.

- Table YYY – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the coded picture with ViewId equal to ViewId of the auxiliary picture.

The units for the ZNear and ZFar values, if present, are identical but unspecified.

depth_nonlinear_representation_num_minus1 + 2 specifies the number of piecewise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity.

depth_nonlinear_representation_model[ i ] specifies the piecewise linear segments for mapping of decoded luma sample values of an auxiliary picture to a scale that is uniformly quantized in terms of disparity.

NOTE – When depth_representation_type is equal to 3, an auxiliary picture contains nonlinearly transformed depth samples. Variable DepthLUT[ i ], as specified below, is used to transform coded depth sample values from nonlinear representation to the linear representation – disparity normalized in range from 0 to 255. The shape of this transform is defined by means of line-segment-approximation in two-dimensional linear-disparity-to-nonlinear-disparity space. The first (0, 0) and the last (255, 255) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to 255, inclusive, with spacing depending on the value of nonlinear_depth_representation_num_minus1.

Variable DepthLUT[ i ] for i in the range of 0 to 255, inclusive, is specified as follows.

depth_nonlinear_representation_model[ 0 ] = 0
depth_nonlinear_representation_model[depth_nonlinear_representation_num_minus1 + 2 ] = 0
for( k=0; k<= depth_nonlinear_representation_num_minus1 + 1; ++k ) {
	pos1 = ( 255 * k ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev1 = depth_nonlinear_representation_model[ k ]
	pos2 = ( 255 * ( k+1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 ) )
	dev2 = depth_nonlinear_representation_model[ k+1 ]

	x1 = pos1 − dev1
	y1 = pos1 + dev1
	x2 = pos2 − dev2
	y2 = pos2 + dev2

	for ( x = max( x1, 0 ); x <= min( x2, 255 ); ++x )
			DepthLUT[ x ] = Clip3( 0, 255, Round( ( ( x - x1 ) * ( y2 - y1 ) ) ÷ ( x2 - x1 ) + y1 ) )
}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an auxiliary picture in the range of 0 to 255, inclusive, represents disparity that is uniformly quantized into the range of 0 to 255, inclusive.

## Containing structures

### SPS extension (example for option 1 of JCTVC-N0063/JCT3V-E0049)

| sps_extension( ) { | Descriptor |
| --- | --- |
| aux_layer_idc | ue(v) |
| if( aux_layer_idc = = 1 ) |  |
| sps_alpha_plane_extension( ) |  |
| else if( aux_layer_idc = = 2 ) |  |
| depth_representation_info_struct( ) |  |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_extension_vui_parameters( ) |  |
| … |  |
| } |  |

### SEI message

| depth_representation_info_sei( payloadSize ) { | Descriptor |
| --- | --- |
| depth_representation_info_struct( ) // the structure would be written out here |  |
| } |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).