| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0105r1 M29991r1 |
| --- | --- |

| Title: | AHG8: Revision of Depth View Information SEI Message in 3D-AVC DT6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto Masayuki Tanimoto | Tel: Email: | +81-42-327-7262 {senoh, k.wakunami, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp +81-52-781-1883 tanimototentative3@yahoo.co.jp |
| Source: | National Institute of Information and Communications Technology (NICT), Nagoya Industrial Science Research Institute (NISRI) |  |  |

_____________________________

# Abstract

Responding to the editor’s comments on the semantics description of depth view information SEI message in 3D-AVC DT6, a revised text is proposed.

# Proposed revision

      - Depth view information SEI message semantics

The depth view information SEI message specifies an alternative 3D format [Ed. (MH): Unclear what is a “3D format”. At the moment there is only one depth_type specified, so the SEI message specifies only one type of a “3D format” as far as I can see]. The depth view information SEI message, when present, shall be associated with an IDR access unit. The information signaled in the SEI message applies to all the access units in the coded video sequence.

depth_type specifies the type of the depth view information. depth_type equals to 0 specifies the alternative 3D format is Global View and Depth (GVD). The values of depth_type larger than 0 are reserved for future use[Ed. (MH): I’d think values greater than 0 should rather be reserved for future use].

num_residual_texture_views_minus1 plus1 specifies the number of sub-residual texture views packed in the residual texture view. num_residual_texture_views_minus1 is in the range of 0 to 3, inclusive.

residual_depth_flag equal to 1 specifies that the number of packed depth views in a depth view component of non-base view is equal to the number of packed texture views in a texture view component. residual_depth_flag equal to 0 specifies there is only one depth view.

NOTE: When depth_type is equal to 0, num_anchor_refs_l0[ i ], num_anchor_refs_l1[ i ], num_non_anchor_refs_l0[ i ] and num_non_anchor_refs_l1[ i ] shall be 0 in order to disable tools for inter-view prediction and avc_3d_extension_flag shall be 0 in order to disable depth and texture interaction coding . [Ed. (MH): The note should rather refer to how to set syntax element values in the SPS or which profile should be used.]

The texture and depth view components of the non-base view contain up to four GVD’s sub-residual views [Ed. (MH) Term “residual view” should be defined], each represents a difference picture of non-base view and base view[Ed. (MH) difference between what?] that is horizontally and vertically decimated in half leaving even samples [Ed. (MH): Sub-sampling process should be explicitly specified. For example, is there filtering involved in the sub-sampling or just pure decimation? From which positions of the sampling grid is sub-sampling derived?] relative to the corresponding texture or depth base view components.

depth_type equal to 0 indicates that N = num_residual_texture_views_minus1 + 2 input texture views and associated depth views (N=2, 3, 4 and up to 5) are converted into Global texture view (one base texture view and one residual texture view) and Global depth view (one base depth view or one base depth view plus one residual depth view).

Base texture view (B) is the input from central camera (camera ID=3) as shown in Figure J-1. In the case N=2, camera ID=2 and 3 are used. In the case N=3, camera ID=2, 3 and 4 are used. In the case N=4, camera ID=1, 2, 3 and 4 are used. In the case N=5, camera ID=1, 2, 3, 4 and 5 are used. [Ed. (MH) phrase “the first right view” is unclear.]. The other N-1 input texture views except for the base view are converted to N-1 half-size sub-residual texture views (R-x) and packed in the residual texture view from top-left (R-2), to bottom-left (R-4), top-right (R-1) and bottom-right (R-5) order. Their top-left co-ordinates in the residual view of width=W and height=H is shown in Table J-X. The residual texture view represents occluded area or out-of-frame area of the base texture view when it is projected to the N-1 input texture views by GVD process [Ed. (MH): It is unclear which process “is projected” exactly refers to. Are there normative impacts of this projection into semantics of the SEI message?] The number of sub-residual texture views is signalled by num_residual_texture_views_minus1.

1     2     3     4     5

R-1   R-2    B    R-4   R-5

camera ID

GVD view

- Figure J-1 – Relation between camera ID and GVD texture views

- Table J-X – Top-left corner co-ordinates of GVD residual views packed in a Residual view of width=W and height=H.

| R-1 | (W/2, 0) |
| --- | --- |
| R-2 | (0, 0) |
| R-4 | (0, H/2) |
| R-5 | (W/2, H/2) |

Base depth view (B) is a sample-wise median between corresponding pixels [Ed. (MH): “median” should be more clearly specified. Is it a sample-wise median between collocated samples?] of all input depth views and is associated with the base texture view (camera ID=3) as shown in Figure J-2. Residual depth view may exist or may not exist. residual_depth_flag equal to 1 specifies the existence of a residual depth view. The other N-1 input depth views are converted to N-1 half-size sub-residual depth views (R-x) by GVD process. The generation of sub-residual depth view is described in the following paragraph. [Ed. (MH): the generation of “sub-residual depth view” should be specified] They are packed in the residual depth view in the same positions as the residual texture view shown in Table 1. Each sub-residual depth view associates with its sub-residual texture view. The generation of sub-residual texture view by GVD process is described in the following paragraph [Ed. (MH) the generation of “sub-residual texture view” should be specified].

When residual_depth_flag = 0 and the number of input views is even (2 or 4), base depth view (B) is the center of input views (camera ID=2.5).

Since the input depth views are originally sub-sampled to the half-size of the input texture views, the sub-residual depth views are sub-sampled to the quarter-size of input texture views. The sub-residual depth view represents occluded area or out-of-frame area of the base depth view when it is projected to the N-1 input depth views.

1     2     3     4     5

(R-1)  (R-2)  B   (R-4)  (R-5)

camera ID

GVD depth

1       2       3       4

2.5

B

camera ID

(even)

GVD depth

wo/res.

- Figure J-2 – Relation between camera ID and GVD depth views

Base depth view is generated by GVD process projecting N-input depth views to the base depth view (camera ID=3) and taking the pixel-wise median of them. [Ed. (MH): which process is used in the projection?]

Residual depth view is generated by GVD process encoding and decoding the base depth view and then projecting it to the other input depth views (camera ID=1,2, 4 and 5), and then extracting the un-covered area by the projected base depth view to sub-residual depth views and then packing them in a residual depth view.

In the case of residual_depth_flag = 0, residual texture view is generated by GVD process projecting the base texture view to the other input texture views (camera ID=1, 2, 4 and 5) by the decoded base depth view, and then extracting the un-covered area in the other input texture views (camera ID=1, 2, 4 and5) by the projected base texture view into sub-residual texture views and then packing them in a residual texture view.

In the case of residual_depth_flag = 1, residual texture view is generated by GVD process encoding and decoding the residual depth view, and then projecting the base texture view to the other input texture views (camera ID=1, 2, 4 and 5) by the decoded base depth view and the decoded residual depth views, and then extracting the un-covered area in the other input texture views (camera ID=1, 2, 4 and5) by the projected base texture view into sub-residual texture views and then packing them in a residual texture view.

# Patent rights declaration(s)

NICT (National Institute of Information and Communications Technology) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI (Nagoya Industrial Science Research Institute) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).