| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0106 M29993 |
| --- | --- |

| Title: | AHG8: Revision of Global View and Depth Format Description for Depth View SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Takanori Senoh, Koki Wakunami, Yasuyuki Ichihashi, Hisayuki Sasaki, Kenji Yamamoto, Masayuki Tanimoto | Tel: Email: | +81-42-327-7262 {senoh, k.wakunami, y-ichihashi, sasaki, k.yamamoto}@nict.go.jp +81-52-781-1883 tanimototentative3@yahoo.co.jp |
| Source: | National Institute of Information and Communications Technology (NICT), Nagoya Industry Science Research Institute (NISRI) |  |  |

_____________________________

# Abstract

According to the following mandate of AHG8, this contribution revises Global View and Depth (GVD) format description for Depth View SEI message.

•	Improve the description of global view and depth format description in 3D-AVC Test Model.

# Introduction

GVD is an Alternative 3D Format that removes inter-view redundancy by global view and depth. GVD makes encoding and decoding simple and fast by executing view synthesis prediction in frame layer. Decision whether some area is redundant or not is made by depth analysis.

In GVD format, input N texture views and associated N depth maps (N=2, 3, 4 and up to 5) are converted into Global texture View (one base texture view and one residual texture view) and Global depth map (one base depth map or one base depth map plus one residual depth map). They are encoded/ decoded and used for synthesis of target multi-views as illustrated in Figure 1.

Figure 1 Global view and depth (GVD) format

Base texture view (B) is the input from central camera (camera ID=3) as shown in Figure 2. In 2-view case, input views are from camera ID=2 and 3. In 3-view case, input views are from camera ID=2, 3 and 4. In 4-view case, input views are from camera ID=1, 2, 3 and 4. In 5-view case, input views are from camera ID=1, 2, 3, 4 and 5. The other N-1 input texture views except for the base texture view are converted to N-1 half-size sub-residual texture views (R-x) and packed in the residual texture view from top-left (R-2), to bottom-left (R-4), top-right (R-1) and bottom-right (R-5) order. Their top-left co-ordinates in the residual view of width=W and height=H is shown in Table 1. The residual texture view represents occluded area or out-of-frame area of the base texture view when it is projected to the N-1 input texture views by GVD process. The number of sub-residual texture views is signalled by num_residual_texture_views_minus1. How to get the residual view is described in the following section.

Figure 2 Frame packing structure of global texture view

Table 1 Top-left corner co-ordinates of GVD residual views packed in a Residual view of width=W and height=H.

| R-1 | (W/2, 0) |
| --- | --- |
| R-2 | (0, 0) |
| R-4 | (0, H/2) |
| R-5 | (W/2, H/2) |

Base depth view is a sample-wise median between corresponding pixels of all input depth views. Input depth views are associated with the input texture views. The relation between input depth views (camera ID= 1 to 5) and base depth view (B) is given in Figure3.

Δxi=fBi255(1Znear−1Zfar)di(xi,y)+fBiZfar (i=1, 2, 3, 4, uptp 5)dgi(xi+Δxi,y)=di(xi,y) dg(x,y)=mediandg1(x,y)+dg2(x,y)+dg3(x,y)+dg4(x,y)+dg5(x,y)

Residual depth view may exist or may not exist. The existence of residual depth view is signalled by residual_depth_flag. When residual_depth_flag = 1, residual depth view exists and the base depth view is the center view (camera ID=3). The other N-1 input depth views are converted to N-1 sub-sampled sub-residual depth views (R-x) and packed in the residual depth view in the same positions as the sub-residual texture views are. Each sub-residual depth view associates with its sub-residual texture view. The size of residual depth view is half of the base depth view. The residual depth view represents occluded area or out-of-frame area of the base depth view when it is projected to the N-1 input depth views. How to get residual depth view is described in the following section.

When residual_depth_flag = 0, residual depth view does not exist. In this case, if the number of input views is even (2 or 4), base depth view (B) is the center of input views (camera ID=2.5).

Figure 3 Frame packing structure of global depth

The features of Global View and Depth format are,

- Simple and fast since inter-view prediction is done by depth map in frame-base. No block-base inter-view prediction is required.

- Compact and light since the residual views for textures or depth maps are packed in a residual texture stream or a residual depth map stream. The resultant texture or depth map streams are compatible with the normal 3DV texture stream or depth stream such as 3D-AVC or 3D-HEVC.

Signaling of Global View and Depth format for view synthesis process is done by Depth View Information SEI message in 3D-AVC specifications or Alternative Depth Information SEI message in 3D-HEVC specifications.

Global View and Depth format has two types according to the absence/existence of residual depth views, namely Type-1 and Type-2. Type-1 uses no residual depth views. Type-2 uses residual depth views.

The following sections describe the encoding, decoding and view synthesis process for Type-1 and Type-2 of Global View and Depth format.

# Type-1 Global View and Depth format

## Data Format and System Description for type-1

Input Data are multiple texture views and associated depth views as illustrated in Figure 4. The Global View and Depth codec consists of Global depth generator, Residual view generator, Core codecs, and View synthesizer. Global view is a logical set of the Base texture view, which is one of the input texture view, and Residual texture view, which is generated by Residual view generator. Core codec is 3D-AVC or 3D-HEVC.

Global depth generator generates, encodes and decodes a Base depth view from the input depth views. In the type-1 codec, residual depth view is not generated. Since Residual view generator has no interaction mechanism with Core codec, Base depth view is encoded and decoded once in Global depth generator. Then, Residual view generator generates a Residual texture view from the input texture views based on the decoded Global depth view given by Global depth generator. After that, Core codec encodes and decodes Base texture view, Residual texture view and Base depth view together. View synthesizer synthesizes multiple views from the decoded Base texture view, Base depth view and Residual texture view. Base texture view is forward-compatible with the conventional 2D codec such as MPEG-4 AVC or HEVC.

Figure 4: Overview of system structure and data format for Type-1 Global View and Depth format

## Coding Algorithm

### Generation of Global Depth View

For the case of even number of views (i=2, 3 or i=1, 2, 3, 4), Base depth view is a median of input depth views after projecting them to the center of the input views (i=2.5) as shown in Figure 5(a). The depth value di(xi, y) in the input depth view is projected to the base depth values dbi(xbi, y) at Base depth view position according to the following equation. Where f is the camera focal length. Bi is the baseline length measured from Base depth view (i=2.5) to the input depth view (i=1, 2, 3, 4). The holes in each projected depth map are inpainted with the smaller depth value in left or right of the hole. Then projected global depth maps are filtered by a median filter and yield a global depth view db(x,y).

Δxi=fBi255(1Znear−1Zfar)di(xi,y)+fBiZfar (i=1, 2, 3, 4, uptp 5)dbi(xi+Δxi,y)=di(xi,y) db(x,y)=mediandb1(x,y)+db2(x,y)+db3(x,y)+db4(x,y)+db5(x,y)	(1)

For the case of odd number of views (i=2, 3, 4 or i=1, 2, 3, 4, 5), the base depth view is a sample-wise median of corresponding pixels in input depth views after projecting them to the center view (i=3) by equation (1). Where Bi is the baseline length measured from Base depth view (i=3) to the input depth view (i=1, 2, 3, 4, 5).  Since the input depth views may include some depth errors, generated Base depth view increases the reliability.

(a) for even view case                      (b) for odd view case

Figure 5 Global depth (Base depth view) generation from the input depth views

Generated Base depth view is decimated to half horizontally and vertically and then it is encoded and decoded by a codec in Global depth generator, which function is same as Core codec. If Core codec doesn’t support asymmetrical size between texture views and depth views, Global depth view is not decimated. The decoded base depth view is used by Residual view generator.

### Generation of Residual Texture View

Since Base texture view covers most of the area of the other input views when it is projected to the other texture views based on the base depth view, Residual texture view keeps only the uncovered area. This uncovered area (Residual texture view) is derived as illustrated in Figure 6 (two view case) and 7 (three view case). In order to avoid mismatch between the encoder and decoder, locally decoded Base depth view is used to detect the residual area. First, decoded Base depth view is up-sampled to the original size if it is sub-sampled, and then projected to the non-base views and their in-between sample views. In case of two views in Figure 6, Left view is for i= 2 and Base view is for i=3. During the up-sampling, simple 2-tap (0.5, 0.5) filter generates in-between pixels if contiguous two pixel value difference is smaller than a threshold (=5, encoder dependent). If not, larger pixel value is copied (depth dilation). After that, the depth map is smoothed by 5x5 median filter. Median filter smoothens slanted depth edges. Dis-occluded area and uncovered area in the projected depth map is inpainted with the smaller depth value on the left or right of the hole. Figure 6 shows the residual view generation for R2 of 2-view case.

Figure 7 shows the residual view generation for R2 and R4 of 3-view case. For the other residual view generation of 4-view or 5-view case, the same operation is applied to view 1 (R1) or view 5 (R5) by changing the view IDs accordingly. For residual view 1 (R1) generation, (X2, X2.25, X2.5, X2.75) (X=D, M, V) are replaced with (X1, X1.25, X1.5, X1.75). Fore residual view 5 (R5) generation, (X3.25, X3.5, X3.75, X4) are replaced with (X4.25, X4.5, X4.75, X5).

Figure 6: Residual view generation for R2 of 2-view case

Figure 7: Residual view generation for R2 and R4 of 3-view case

Then, occlusion holes are detected by checking the depth map as follows. If the base view is on the right hand side of the currently checking depth map as shown in Figure 8, an occlusion hole exists where a foreground object (large depth value) exists on the right of the currently checked pixel. The pixel is in the occlusion hole or not is determined by the following steps.

- Start from the maximum disparity distance δ= (fB (1/Znear-1/Zfar), where f=focal length and B is the baseline length between the current depth view and the base texture view, compare two depth values of dx (x, y) and dx+δ(x+δ, y). If (dx+δ- dx) >= th, pixel (x, y) may be in an occlusion hole. th is given in the following equation..

if (dx+δ−dx)≥th, x is in occlusion hole.th=255fB(1Znear−1Zfar)δ, (0≤δ≤fBZnear−fBZfar)	(2)

- If not, reduceδby 1/8 and repeat step (1). This repetition detects skinny foreground objects like poles.

- When the pixel is possibly in a occlusion hole, further check the depth value of d x+ε(x+ε, y). εis a disparity value of (dx+δ- dx) and given in the following equation.

ε=fB255(1Znear−1Zfar)(dx+δ−dx)	(3)

- If (dx+ε- dx) >= k*th, the pixel (x, y) is registered to the hole mask. Where k is 0.5 which saves the decay of depth edge.

- Uncovered area on the left edge in the depth map is also registered to the hole mask. This area is detected by checking the corresponding address of (x-Δ)≥0 in the base view, whereΔ is given in equation (1).

- If the base view is in the left side of the currently checked depth map, the above steps are repeated by changing the (x+δ) to (x–δ), (x+ε) to (x–ε), and (x –Δ) ≥0 to (x+Δ)<width of picture.

Figure 8: Occlusion hole detection for 3-view case

This occlusion hole detection is repeated at four sample view points (D2.0, D2.25, D2.5, D2.75) for left residual view of two view case and at another four sample view points (D3.25, D3.5, D3.75, D4.0) for right residual view of 3-view case. In the case of the other number of views, the occlusion hole check for points (D1.0, D1.25, D1.5, D1.75) and (D4.25, D4.5, D4.75, D5.0) are added accordingly. These occlusion hole check ensures that the residual view covers all holes including pseudo common holes caused by the wrong depth values. Generated hole masks are projected to the nearest left view (V1 or V2) or the nearest right view (v4 or V5) and merged there. After that, they are dilated by 8 to increase the margin of residual views. With the merged hole masks (M1, M2, M4 or M5), left views (V1 or V2) and right views (V4 or V5) are cut out to the left residual (R1 or R2) and the right residual (R4 or R5). These residuals are down-sampled to half vertically and horizontally after filtered with a three-tap (1/4, 1/2, 1/4) low pass filter and then packed into a frame of Residual texture view from top-left (R2), to bottom-left (R4), top-right (R1) and bottom-right (R5) order. The no-pixel area is paved with the average color value of the input views. This paving eases the artifacts of no-pixel area in the target view at the decoder. Three pixels at the edge of no-pixel area are linearly smoothed to increase the coding efficiency.

### Coding of Base View, Global depth and Residual view

Base texture view (V3), Residual view and non-coded Base depth view are encoded and decoded by Core codec. They are independently encoded and keeps forward compatibility of Base texture view with the conventional 2D codecs such as MPEG-4/AVC(ITU-T H.264) or HEVC.

## View synthesis

After decoding bitstreams for Base texture view, Residual texture view and Base depth view by Core codec, View synthesizer synthesizes target views including left view (V1 or V2) and right view (V4 or V5) from them. Figure 9 illustrates view synthesis from decoded base texture view (V3), residual texture view (R2) and base depth view in 3-view case. Where target view is between view 2 and view 3. First, decoded base depth view is up-sampled (D3’) and projected to the target view (Dt) as the same manner as in the residual view generation process. Next, decoded base texture view (V3’) is projected to the target view (Vt) by the target depth map (Dt). During the projection, occlusion holes are detected and registered to the base hole mask (Mb) as same manner as in the residual coding process. Then, decoded residual texture view is separated to left residual texture view and right residual texture view. They are up-sampled to the original size (R2’, R4’) with a two-tap (1/2, 1/2) filter by smoothing the no-pixel edge. The up-sampled residual texture view (R2’) is projected to the target view (Rt). The projection of base texture view (V3’) and residual texture view (R2’) is done by up-sampling chroma components to the original luma component size for a simplicity. View projection with sub-pixel accuracy is realized by the interpolation from two contiguous pixels according to the mantissa of disparity value calculated from the target depth value.

Then, the base hole mask is dilated by two pixels to cover the mismatch between the depth map and texture. Dilated base hole mask limits the residual projection only inside of the base hole mask. This limitation increases the view synthesis speed. During the residual projection, occlusion holes are detected and registered to the residual hole mask (Mr) as well. Projected residual texture view (Rt) is over-written on the hole area in the target view (Vt) according to the base hole mask (Mb). After that, common hole mask is created from the base hole mask and the residual hole mask. Common holes are inpainted according to the common hole mask. Then, each pixel on the both sides of the base hole mask edge are filtered by three-tap (1/4, 1/2, 1/4) filter to reduce the hole edge contour if their difference is smaller than 20. If the difference is equal or larger than 20, they are left as the object edge. Finally the target view is out-put as a synthesized view (St). The other target views are synthesized in the same manner by selecting the residual texture view appropriately.

Figure 9: Target view synthesis between residual texture view (R2) and base texture view (V3)

# Type-2 of Global View and Depth format

## Data Format and System Description for Type-2

Input data format is same as those of 3D video coding. Figures 10 shows Type-2 Global View and Depth data converted from input MVD data for two or three viewpoints, where the parameters W and H show the width of input full-size texture view and the height of input full-size texture view, respectively. Type-2 GVD data also consists of global view and global depth. Furthermore, the Type-2 global view consists of a base texture view and residual texture view as same as that for Type-1 global view. On the other hand, Type-1 global depth consists of only base depth view, while the Type-2 global depth consists of base depth view and residual depth view.

Figure 10: Type-2 GVD data format

Figure 11 shows an overview of system structure for Global View and Depth data format of 3-view case.

In this example, three views are named as V1, V2 and V3. The system consists of Codec, Base view generator, Base depth generator, Residual view generator, Residual depth generator and View synthesizer. The codec is 3DV-AVC or 3D-HEVC.

The function of base depth generator is completely same as that of the global depth generator for the Type-1 GVD data format described in Section 2.1. Residual depth generator generates, encodes and decodes a residual depth view from the input depth views D1 and D3 and the decoded base depth D2 that was output from the base depth generator. The residual depth view is encoded and decoded once in the residual depth generator. Then, residual view generator generates a residual view from the input multi-views based on the decoded base depth view and residual depth view given by the base depth generator and residual depth generator. View synthesizer synthesizes multiple views from the decoded base view, base depth view, residual view and residual depth view. Base view is forward-compatible with the conventional 2D codec such MPEG-4 AVC or HEVC.

Figure 11: Overview of system structure for Type-2 Global View and Depth format of 3-view case

## Coding Algorithm

### Generation of Base Depth

The method for generating base depth of the Type-2 global depth is same as that for Type-1 global depth. The base depth view is a median of all input depth views by projecting them to the base depth point, or may be the original depth map at the base viewpoint. The obtained base depth view is sub-sampled to half size of original depth view.

### Generation of Residual Depth

Since base view can cover most of the area of the other views when it is projected to the other views based on the base depth view, residual view and residual depth view keep only the uncovered area. Depth information, which is used for filling this uncovered area, is derived as illustrated in Figures 12 and 13. The depth information is residual depth view. It should be noted that in order to avoid mismatch between hole mask for residual depth generation at the encoder side and that at the decoder side, locally decoded base depth view is used to detect the residual area in non-base depth views. The detail of residual depth generation process is as follows.

- Decoded base depth is up-sampled to that of the original size. During the up-sample, simple 2-tap filter generates in-between pixels if contiguous two pixel difference is smaller than a threshold (=5). If not, larger pixel is copied (depth dilation).

- Shift the up-sampled base depth to Left or Right viewpoint.

- The shifted depth map is smoothed by 3x3 median filter. Median filter smoothens slant depth edges.

- Dis-occluded areas or uncovered areas are detected from the smoothed depth map and are registered to a hole mask for left or right residual depth, which are dilated by 4 pixels to add a margin to the residual depth.

- By using the left or right hole mask that was obtained in (4), left depth or right depth is cut out to the left or right residual depth, respectively. The no-pixel area is paved with the average depth value of base depth.

- The left or right residual depth that was obtained in (5) is down sampled to 1/4 vertically and 1/4 horizontally after filtered with a three-tap low pass filter and stacked to a frame. Finally, the residual depth are packed into a frame of quarter of the full frame size in the vertical raster scan order.

Here, it should be noted that hole masks for residual depth are not transmitted to a receiver side but are reconstructed at the receiver side.

Figure 12: Residual depth generation for 2 view case

Figure 13: Residual depth generation for 3 view case

### Generation of Residual View

The method for generating Type-2 residual views is similar to that for Type-1 residual views. The area, which is uncovered by the base view, is derived as illustrated in Figures 14 and 15. In order to avoid mismatch between hole mask for residual view generation at the encoder side and that at the decoder side, decoded base depth and decoded residual depth are used to detect uncovered area. The detail of residual view generation process is as follows.

- Decoded base depth and decoded residual depth are up-sampled to the original size as same as the residual depth generation process.

- The up-sampled base depth is shifted to Left or Right view and hole masks for Left or Right residual depth are generated as same as the residual depth generation process.

- The up-sampled base depth is also shifted to intermediate target viewpoints. Hole pixels in each shifted depth map are detected and are registered to a temporal hole mask, which is smoothed by 3x3 median filter and is dilated by 2 pixels. It should be noted that each depth map in intermediate viewpoint is not smoothed in this step yet.

- Only the residual pixels of Residual Left or Right depth is shifted to each target viewpoint by using their residual hole masks. Only the pixels of depth map in intermediate viewpoint, which are considered hole pixels in the smoothed hole mask in (3), are filled with the shifted residual pixels.

- The depth map that was obtained in (4) are smoothed by 3x3 median filter. Remaining hole pixels of the depth map are filled by inpainting with the smaller depth value between existing left and right pixels around the hole.

- For each intermediate viewpoint, the pixels that were not filled with base depth but filled with residual depth or inpainting are backwardly warped to the left or right view. For each intermediate viewpoint, corresponding pixels in the left or right view are detected and are registered to a temporal hole mask for residual view. The temporal hole mask is smoothed by 3x3 median filter.

- The hole masks that were obtained in (2) and (6) are merged and are dilated by 4 pixels to add a margin to the residual depth.

- By using the hole mask that was obtained in (7), left or right view is cut out to left or right residual view, respectively. The no-pixel area is paved with the average value of base view.

- The left or right residual view that was obtained in (8) is down sampled to half vertically and half horizontally after filtered with a three-tap low pass filter and stacked to a frame. Finally, the residual view are packed into a frame of the original frame size in the vertical raster scan order as shown in Fig. 16.

Here, it should be noted that hole masks for residual views are not transmitted to a receiver side but are reconstructed at the receiver side.

Figure 14: Residual view generation for 2 view case

Figure 15: Residual view generation for 3 view case

## View Synthesis

At the decoder side, view synthesizer synthesizes target views including left and right views from decoded base view, base depth, residual view and residual depth. Figure 16 illustrates a case where target view is on the left of base view. The detail of view synthesis is described in below.

- Decoded base depth is up-sampled and shifted to the target view as same manner as in the residual depth and residual view generating process. Then, hole masks for target views are generated

- Decoded residual depth is separated to those for non-base viewpoints. The residual depth in each non-base viewpoint is up-sampled and shifted to target views by using hole masks generated in (1) as same manner as in the residual view generating process.

- Remaining hole pixels of target views are filled by inpainting.

- Decoded base view is backwardly projected to target views by using the target depth and the hole mask that were generated in (1).

- Decoded residual view is separated to those for non-base viewpoints, which are up-sampled to the original size with simple two-tap filter. The up-sampled residual is backwardly projected to the target view by the target depth map generated in (2) and the hole mask generated in (1).

- Common hole mask is created from the base hole mask and the residual hole mask. Common holes are inpainted according to the common hole mask. Then, two pixels on the both sides of base hole mask edge are filtered by three-tap filter to reduce the hole edge contour. Finally the target view is out-put as a synthesized view. The right target view is synthesized in the same manner.

Figure 16. View Synthesis

# Software

## Software

The source code for the software will be available in the MPEG SVN repository. A preliminary version of the software is available upon request by e-mail.

tanimototentative3@yahoo.co.jp for Type-2 global view and depth format

senoh@nict.go.jp for Type-1 global view and depth map format.

## Build system

The software can be built under Windows using Microsoft Visual Studio.Net. The software is using Open CV2.2. For UNIX and Linux, makefiles are provided.

# Reference

[1] 	ISO/IEC JTC1/SC29/WG11/M26014[JCT2-A0072]: " Proposal on Simple Test Model for 3DV Coding based on CE7," (July 2012)

[2] 	ISO/IEC JTC1/SC29/WG11/M27793[JCT2-C0058]: "AHG08: Technical Description of residual data generation and target view synthesis in GVD (Global View and Depth) 3D Format," (Jan 2013)

NICT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NISRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).