| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0130 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0112 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On temporal sub-layer management |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In HEVC layered-extension, in order to support different frame rates for different layers, several constraints are proposed. In addition, if frame rates for non-base layers greater than a base layer are allowed, the total number of sub-layers may be increased. Then, signalling the increased number of sub-layers, profile-tier-level information of the additional sub-layers, and sub-layer ordering information for additional sub-layers is necessary in video parameter set extension.

# Definitions

To explain the proposals, basic definitions are given as follows:

step-wise temporal sub-layer access (STSA) access unit: An access unit in which all the coded pictures are STSA pictures.

step-wise temporal sub-layer access (STSA) picture: A coded picture for which each VCL NAL unit has nal_unit_type equal to STSA_R or STSA_N.

temporal sub-layer access (TSA) access unit: An access unit in which all the coded pictures are TSA pictures.

temporal sub-layer access (TSA) picture: A coded picture for which each VCL NAL unit has nal_unit_type equal to TSA_R or TSA_N.

sub-access unit (Sub-AU): An access unit in which nuh_layer_ids of all VCL NAL units are not equal to 0.

# Proposed constrains for different frame rates

In HEVC layered-extension, in order to support different frame rates for different layers, the following constraints are proposed.

- All pictures in an access unit shall have the same TemporalId.

- When the temporal rate of an enhancement layer is greater than the temporal rate of a base layer, an access unit may not have a picture with nuh_layer_id equal to 0.

- If an access unit does not have a VCL NAL unit with nuh_layer_id equal to 0, namely sub-access unit, the TemporalId of the pictures belonging to that access unit shall have a TemporalId greater than the TemporalId of the picture belonging to the previous access unit containing a VCL NAL unit with nuh_layer_id equal to 0.

- From the different of TemporalId between access units containing the a VCL NAL unit with nuh_layer_id equal to 0 and access units not containing the a VCL NAL unit with nuh_layer_id equal to 0, access unit boundary can be detected when allowing different frame rate for different layers.

- All pictures belonging to sub-access unit (Sub-AU) cannot be referenced by a picture belonging to the access unit in which a picture with nuh_layer_id equal to 0 is present. It means that a picture in Sub-AU unit cannot be used for inter-prediction for non-Sub-AU.

- If a VCL NAL unit with nuh_layer_id equal to 0 is not present in access unit, that access unit is STSA access unit or TSA access unit.

- If a VCL NAL unit with nuh_layer_id equal to 0 is not present in access unit, the pictures in that access unit shall have nal_unit_type equal to TSA_R, TSA_N, STSA_R, and STSA_N.

# Information of extended sub-layer in VPS extension

If frame rates for non-base layers greater than a base layer are allowed, the total number of sub-layers may be increased. Then, signalling the increased number of sub-layers, profile-tier-level information of the additional sub-layers, and sub-layer ordering information for additional sub-layers is necessary in video parameter set extension.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_max_ext_sub_layers | u(3) |
| if(vps_max_ext_sub_layers>0) { |  |
| ext_profile_tier_level( 1, vps_max_ext_sub_layers-1) |  |
| vps_ext_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_ext_sub_layer_ordering_info_present_flag ? 0 : vps_ext_max_ext_sub_layers ); i < vps_max_ext_sub_layers; i++ ) { |  |
| vps_max_ext_dec_pic_buffering_minus1[ i ] | ue(v) |
| vps_max_ext_num_reorder_pics[ i ] | ue(v) |
| vps_max_ext_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

| ext_profile_tier_level( extNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| for( i = 0; i < extNumSubLayersMinus1 ; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| } |  |
| if(extNumSubLayersMinus1 > 0 ) |  |
| for( i = extNumSubLayersMinus1 ; i < 8; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |
| for( i = 0; i < extNumSubLayersMinus1 ; i++ ) { |  |
| if( sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_progressive_source_flag[ i ] | u(1) |
| sub_layer_interlaced_source_flag[ i ] | u(1) |
| sub_layer_non_packed_constraint_flag[ i ] | u(1) |
| sub_layer_frame_only_constraint_flag[ i ] | u(1) |
| sub_layer_reserved_zero_44bits[ i ] | u(44) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

vps_max_ext_sub_layers plus 1 specifies the extended number of temporal sub-layers that are present in non-base layers, however not present in a base layer. The value of vps_max_ext_sub_layers shall be in the range of 0 to 6, inclusive.

- vps_ext_sub_layer_ordering_info_present_flag equal to 1 specifies that vps_ext_max_dec_pic_buffering_minus1[ i ], vps_ext_max_num_reorder_pics[ i ], and vps_ ext_max_latency_increase_plus1[ i ] are present for vps_max_ext_sub_layers sub-layers. vps_ext_sub_layer_ordering_info_present_flag equal to 0 specifies that the values of vps_max_dec_pic_buffering_minus1[ vps_max_sub_layers_minus1 ], vps_max_num_reorder_pics[ vps_max_sub_layers_minus1 ], and vps_max_latency_increase_plus1[ vps_max_sub_layers_minus1 ] apply to all extended sub-layers.

- vps_max_ext_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i + vps_max_ext_sub_layers. The value of vps_max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When i is greater than 0, vps_max_ext_dec_pic_buffering_minus1[ i ] shall be greater than or equal to vps_max_ext_dec_pic_buffering_minus1[ i − 1 ]. When vps_max_ext_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to vps_max_ext_sub_layers, inclusive, due to vps_ext_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_dec_pic_buffering_minus1[ vps_max_sub_layers_minus1 ].

- vps_max_ext_num_reorder_pics[ i ] indicates the maximum allowed number of pictures that can precede any picture in the CVS in decoding order and follow that picture in output order when HighestTid is equal to i + vps_max_ext_sub_layers. The value of vps_max_ext_num_reorder_pics[ i ] shall be in the range of 0 to vps_max_ext_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, vps_max_ext_num_reorder_pics[ i ] shall be greater than or equal to vps_max_ext_num_reorder_pics[ i − 1 ]. When vps_max_ext_num_reorder_pics[ i ] is not present for i in the range of 0 to vps_max_ext_sub_layers, inclusive, due to vps_ext_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_num_reorder_pics[ vps_max_sub_layers_minus1 ].

- vps_max_ext_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ i + vps_max_ext_sub_layers ], which specifies the maximum number of pictures that can precede any picture in the CVS in output order and follow that picture in decoding order when HighestTid is equal to i + vps_max_ext_sub_layers.

- When vps_max_ext_latency_increase_plus1[ i ] is not equal to 0, the value of VpsMaxLatencyPictures[ i ] is specified as follows:

VpsMaxLatencyPictures[ i + vps_max_ext_sub_layers] = vps_max_ext_num_reorder_pics[ i ] + vps_max_ext_latency_increase_plus1[ i ] − 1

- When vps_max_ext_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed.

The value of vps_max_ext_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_ext_latency_increase_plus1[ i ] is not present for i in the range of 0 to vps_max_ext_sub_layers, inclusive, due to vps_ext_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_latency_increase_plus1[ vps_max_sub_layers_minus1 ].

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).