| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0131r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0113r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On interlayer reference picture set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The improvements of the inter-layer reference picture set (IL-RPS) are proposed.

- Signaling num_max_active_ref_layer instead of max_one_active_ref_layer_flag.

- Moving the IL-RPS related syntax from slice header to PPS extension.

- Replacing inter_layer_pred_layer_idc with inter_layer_pred_layer_idc_delta

- Forcing RefLayerId[i] to be included in RefPicLayerId[ i ] in a descending order.

# Introduction

This contribution proposes the improvement of the inter-layer reference picture set (IL-RPS) for HEVC multi-layered extension. The following aspects are proposed.

- Signaling num_max_active_ref_layer instead of max_one_active_ref_layer_flag.	(highlighted in green)

- Each picture cannot have the number of inter-layer reference pictures greater than num_max_ active_ref_layer.

- num_max_active_ref_layer can be used for estimating the decoded picture buffer (DPB) size for inter-layer prediction.

- Signalling max_one_active_ref_layer_flag can be removed, because num_max_active_ref_layer is a super-set of max_one_active_ref_layer_flag,.

- Moving the IL-RPS related syntax from slice header to PPS extension. (highlighted in yellow)

- Unlike the temporal prediction structure depending on GOP structures, the inter-layer dependency is not frequently changed.

- Replacing inter_layer_pred_layer_idc with inter_layer_pred_layer_idc_delta. (highlighted in sky-blue)

- Signalling the delta is more efficient than signalling the index, because most inter_layer_pred_layer_idc_delta values are expected to be equal to 1. Moreover, signaling delta is beneficial for forcing the order of inter-layer reference picture to be included in a list in a descending or ascending order.

- Forcing RefLayerId[i] to be included in RefPicLayerId[ i ] in a descending order. (highlighted in grey)

- This constraint enables that the inter-layer reference pictures are included in an inter-layer RPS from closer to father layers to the current layer.

## Video parameter set extension syntax and semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| num_max_active_ref_layer | ue(v) |
| max_one_active_ref_layer_flag | u(1) |
| } |  |

num_max_active_ref_layer indicates the maximum number of reference pictures that are used for inter-layer prediction for each picture in the CVS.

## Parameter set extension syntax and semantics

The IL-RPS related syntax can be signalled in PPS or slice header.

| pps_extension( ) { | Descriptor |
| --- | --- |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag) { |  |
| if(num_max_active_ref_layer > 0) { |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1 |  |
| } else |  |
| NumActiveRefLayerPics = 1 |  |
| inter_layer_pred_layer_idc_present_flag | u(1) |
| if(inter_layer_pred_layer_idc_present_flag) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc_delta[ i ] | ue(v) |
| } |  |
| } |  |

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture. When not present, the value of inter_layer_pred_enabled_flag is inferred to be equal to 0.

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictrues that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( num_max_active_ref_layer] ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to num_max_active_ref_layer − 1, inclusive.

All slices of a picture shall have the same value of NumActiveRefLayerPics.

inter_layer_pred_layer_idc_present_flag equal to 0 indicates that inter_layer_pred_layer_idc_delta [ i ] or inter_layer_pred_layer_idc [ i ] is not signaled, and the first or last N RefLayerId[nuh_layer_id ][i] (0=< i < NumActiveRefLayerPics) are mapped to RefPicLayerId[ i ].

inter_layer_pred_layer_idc_delta [ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. When not present, inter_layer_pred_layer_idc_delta [ i ] is inferred to be equal to 1.

The variable RefPicLayerId[ i ] is derived as follows:

InterLayerPredLayerIdc[ 0 ] = inter_layer_pred_layer_idc_delta[0 ] -1

RefPicLayerId[ 0 ] = RefLayerId[ nuh_layer_id ][ InterLayerPredLayerIdc [ 0 ] ]

for( i = 1, ; i < NumActiveRefLayerPics; i++) {
	InterLayerPredLayerIdc [ i ] = InterLayerPredLayerIdc [ i -1] + inter_layer_pred_layer_idc_delta[ i ]

RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ InterLayerPredLayerIdc [ i ] ]

}

All slices of a picture shall have the same value of inter_layer_pred_layer_idc_delta[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

- Changes to force the inter-layer reference pictures to be included in a descending order.

The variable RefPicLayerId[ i ] is derived as follows:

InterLayerPredLayerIdc[ 0 ] = inter_layer_pred_layer_idc_delta[0 ]

RefPicLayerId[ 0 ] = RefLayerId[ nuh_layer_id ][ InterLayerPredLayerIdc [ 0 ] ]

for( i = 1, ; i < NumActiveRefLayerPics; i++) {
	InterLayerPredLayerIdc [ i ] = InterLayerPredLayerIdc [ i -1] + inter_layer_pred_layer_idc_delta[ i ]

RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ NumActiveRefLayerPics - InterLayerPredLayerIdc [ i ]  ]

}

All slices of a picture shall have the same value of inter_layer_pred_layer_idc_delta[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

## Slice segment header syntax and semantics

The IL-RPS related syntax can be signalled in PPS or slice header.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if(num_max_active_ref_layer > 0) { |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1 |  |
| } else |  |
| NumActiveRefLayerPics = 1 |  |
| inter_layer_pred_layer_idc_present_flag | u(1) |
| if(inter_layer_pred_layer_idc_present_flag) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc_delta[ i ] | ue(v) |
| } |  |
| } else |  |
| NumActiveRefLayerPics = 0 |  |
| } |  |
| } |  |

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture. When not present, the value of inter_layer_pred_enabled_flag is inferred to be equal to 0.

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictrues that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( num_max_active_ref_layer] ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to num_max_active_ref_layer − 1, inclusive.

All slices of a picture shall have the same value of NumActiveRefLayerPics.

inter_layer_pred_layer_idc_present_flag equal to 0 indicates that inter_layer_pred_layer_idc_delta [ i ] or inter_layer_pred_layer_idc [ i ] is not signaled, and the first or last N RefLayerId[nuh_layer_id ][i] (0=< i < NumActiveRefLayerPics) are mapped to RefPicLayerId[ i ].

inter_layer_pred_layer_idc_delta [ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. When not present, inter_layer_pred_layer_idc_delta [ i ] is inferred to be equal to 1.

The variable RefPicLayerId[ i ] is derived as follows:

InterLayerPredLayerIdc[ 0 ] = inter_layer_pred_layer_idc_delta[0 ] -1

RefPicLayerId[ 0 ] = RefLayerId[ nuh_layer_id ][ InterLayerPredLayerIdc [ 0 ] ]

for( i = 1, ; i < NumActiveRefLayerPics; i++) {
	InterLayerPredLayerIdc [ i ] = InterLayerPredLayerIdc [ i -1] + inter_layer_pred_layer_idc_delta[ i ]

RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ InterLayerPredLayerIdc [ i ] ]

}

All slices of a picture shall have the same value of inter_layer_pred_layer_idc_delta[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

- Changes to force the inter-layer reference pictures to be included in descending ordered

The variable RefPicLayerId[ i ] is derived as follows:

InterLayerPredLayerIdc[ 0 ] = inter_layer_pred_layer_idc_delta[0 ]

RefPicLayerId[ 0 ] = RefLayerId[ nuh_layer_id ][ InterLayerPredLayerIdc [ 0 ] ]

for( i = 1, ; i < NumActiveRefLayerPics; i++) {
	InterLayerPredLayerIdc [ i ] = InterLayerPredLayerIdc [ i -1] + inter_layer_pred_layer_idc_delta[ i ]

RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ NumActiveRefLayerPics - InterLayerPredLayerIdc [ i ]  ]

}

All slices of a picture shall have the same value of inter_layer_pred_layer_idc_delta[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).