| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0132 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0114 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On interlayer prediction type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In order to enable an independent configuration of each inter-layer dependency type, the video parameter set extension syntax is improved by using a mask signalling similar to scalability_mask[ i ]. In addition, the proposed flag motion_only_decoding_flag indicates that only the motion vector related data are needed for inter-layer prediction without the full decoding of pixel date. It removes unnecessary decoding process of unused decoded pixel data.

.

# Proposal on inter-layer dependency type

In order to enable an independent configuration of each inter-layer dependency type, the video parameter set extension syntax is improved by using a mask signalling similar to scalability_mask[ i ].

## Video parameter set extension syntax and semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0, NumPredictionTypes = 0; i < 16; i++ ) { |  |
| prediction_mask[ i ] | u(1) |
| NumPredictionTypes += prediction_mask[ i ] |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) { |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| if( direct_dependency_flag[ i ][ j ] ) { |  |
| for( k = 0; k <NumPredictionTypes; k++ ) |  |
| direct_prediction_type_flag[ i ][ j ][ k ] | u(1) |
| } |  |
| } |  |
| … |  |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| … |  |
| } |  |

prediction_mask[ i ] equal to 1 indicates that direct_prediction_type_flag syntax elements corresponding to the i-th prediction type in Table Error: Reference source not found-2 are present. direct_prediction_type_flag syntax elements corresponding to the i-th prediction type are not present.

Table Error: Reference source not found-2 – Mapping of ScalabiltyId to scalability dimensions

| prediction_mask index | Prediction type | PredEnabledFlag mapping |
| --- | --- | --- |
| 0 | Decoded sample | SamplePredEnabledFlag |
| 1 | Motion vector | MotionPredEnabledFlag |
| 2-15 | Reserved |  |

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

direct_prediction_type_flag[ i ][ j ][ k ] is used to derive the variables NumSamplePredRefLayers[ i ], NumMotionPredRefLayers[ i ], SamplePredEnabledFlag[ i ][ j ], and MotionPredEnabledFlag[ i ][ j ]. k shall be in the range of 0 to 1, inclusive, in bitstreams conforming to this version of this Specification.

The variables NumSamplePredRefLayers[ i ], NumMotionPredRefLayers[ i ], SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ], NumDirectRefLayers[ i ], DirectRefLayerIdx[ i ][ j ],  RefLayerId[ i ][ j ], MotionPredRefLayerId[ i ][ j ], and SamplePredRefLayerId[ i ][ j ] are derived as follows:

for( i = 0; i < 64; i++ ) {
	NumSamplePredRefLayers[ i ] = 0
	NumMotionPredRefLayers[ i ] = 0
	NumDirectRefLayers[ i ] = 0
	for( j = 0; j < 64; j++ ) {
		SamplePredEnabledFlag[ i ][ j ] = 0
		MotionPredEnabledFlag[ i ][ j ] = 0
		RefLayerId[ i ][ j ] = 0
		SamplePredRefLayerId[ i ][ j ] = 0
		MotionPredRefLayerId[ i ][ j ] = 0
	}
}

for( i = 1; i  <=  vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	for( j = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ] ) {
			DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDirectRefLayers[ iNuhLId ]

RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]

k = 0;

if( prediction_mask[ 0 ] ) {

SamplePredEnabledFlag[ iNuhLId ][ j ] = direct_prediction_type_flag[ i ][ j ][ k++ ]
					NumSamplePredRefLayers[ iNuhLId ] += SamplePredEnabledFlag[ iNuhLId ][ j ]

}

if( prediction_mask[ 1 ] ) {

MotionPredEnabledFlag [ iNuhLId ][ j ] = direct_prediction_type_flag[ i ][ j ][ k++ ]
					NumMotionPredRefLayers[ iNuhLId ] += MotionPredEnabledFlag[ iNuhLId ][ j ]

}
		}

}

# Indication of dependency type in slice header

In slice headers, signalling the inter-layer dependency type is beneficial for avoiding unnecessary decoding. The proposed flag motion_only_decoding_flag indicates that only the motion vector related data are needed for inter-layer prediction. Then, only the motion vectors are decoded and stored without the full decoding of pixel data, when the slices are decoded.

## Slice header syntax and semantics

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| if( num_extra_slice_header_bits > 0 ) |  |
| discardable_flag | u(1) |
| if( num_extra_slice_header_bits > 1 ) |  |
| motion_only_decoding_flag | u(1) |
| for( i = 2; i < num_extra_slice_header_bits ; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |

motion_only_decoding_flag equal to 1 specifies that the motion vector of the coded picture is used for inter-layer prediction, while the decoded pixel data of the coded picture is not used as a reference picture for inter prediction and is not used for an inter-layer prediction in the decoding process of subsequent pictures in decoding order. motion_only_decoding_flag equal to 0 specifies that the coded picture may be used as a reference picture for inter prediction and may be used as an inter-layer reference picture in the decoding process of subsequent pictures in decoding order. When not present, the value of motion_only_decoding_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).