| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 02 Aug. 2013 | Document: JCT3V-E0117 |
| --- | --- |

| Title: | CE6: Simplified DC calculation for SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyunho Jo, Woong Lim, Jonghoon Yoo, Donggyu Sim 507 Bima-building, Kwangwoon Univ. Kwangwoon-Ro 20, Nowon-Gu, Seoul, Korea | Tel: Email: | +82-10-8388-7536 limwoong@kw.ac.kr mmyjh@kw.ac.kr dgsim@kw.ac.kr |
| Source: | Kwangwoon University (KWU) |  |  |

_____________________________

# Abstract

This contribution proposes a simplified DC calculation for simplified depth coding (SDC) to reduce the computational complexity. For the computational complexity reduction, the current reference software (HTM 7.0r1) employs reference samples sub-sampling for SDC [1]. However, accumulation with varying number of samples, branch, and division operations are still utilized and these operations increase computational complexity in encoder and decoder. The proposed method calculates DC value using the partial samples of the predicted block based on the characteristics of three types of prediction modes (DC, Planar, and DMM Mode-1) of SDC coded block. For DC prediction mode, the right-bottom sample in a predicted block is used as the DC value. For DMM Mode-1 prediction mode, the left-top, right-top, left-bottom, and right-bottom samples in a predicted block are used to find the DC values of segment-1 and segment-2, respectively. For the Planar prediction mode, the left-top, right-top, left-bottom, and right-bottom samples in a predicted block are averaged to get the DC value. The experimental results show that the proposed method achieves 0.1% coding gain for synthesized views in common test condition (CTC) with the significantly reduced number of the computing operations.

# Introduction

SDC coding mode consists of three prediction modes (DC, Planar, and DMM Mode-1). A coded block will be represented with one segment in the case of DC or Planar prediction modes and with two segments in the case of DMM Mode-1 prediction mode. At the encoder side, the SDC module calculates the mean of the original depth value (dorig) and the predicting depth value (dpred) that is made by intra prediction with mode DC, Planar, or DMM Mode-1. A depth lookup table (DLT) is utilized to map the dorig and dpred to index values. The residual index to be transmitted to the decoder is given by

iresi=I(dorig)−I(dpred)

where I(·) denotes the DLT. The current reference software (HTM 7.0r1) employs partial samples in a predicted block by sub-sampling method to decrease the computational complexity in DC value calculation. However, the process still uses accumulation operation with varying number of samples, branch, and division operations. These operations have high computation complexity and it is not appropriate to implement them with hardware logics.

# Simplified DC calculation for SDC

This contribution introduces a simplified DC calculation method for the SDC coded block to get a DC value. The proposed method calculates DC value using the partial samples of the predicted block based on the characteristics of three types of prediction modes (DC, Planar, and DMM Mode-1) of SDC coded block. For DC prediction mode, the right-bottom sample in a predicted block is used as the DC value, as shown in Fig. 1. Note that inner boundary filtering is utilized for the 8×8 and 16×16 for the DC prediction mode.

Figure 1: Modified selection of a sample for simplified DC calculation for SDC (DC mode)

For DMM Mode-1 prediction mode, the left-top, right-top, left-bottom, and right-bottom samples in a predicted block are used to find the DC values of segment-1 and segment-2, as show in Fig. 2.

(a) Case 1 			  	      (b) Case 2

(c) Case 3 			  	      (d) Case 4

Figure 2: Modified selection of samples for simplified DC calculation for SDC (DMM Mode-1)

For Planar prediction mode, the left-top, right-top, left-bottom, and right-bottom samples in a predicted block are averaged to get the DC value as show in Fig. 3. The average value to get the DC value is calculated by follows:

DC=(pleft−top+pright−top+pleft−bottom+pright−bottom+2)>>2

Figure 3: Modified selection of samples for simplified DC calculation for SDC (Planar mode)

The number of pixels involved for the proposed method and HTM 7.0r1 are as follows:

Table 1: Comparison of number of pixels involved in HTM 7.0r1 and proposed method

| Prediction Mode | Block Size | Number of pixels involved |  | Ratio |
| --- | --- | --- | --- | --- |
|  |  | HTM 7.0r1 | Proposed |  |
| DC | 8x8 | 64 | 1 | 1.56% |
|  | 16x16 | 256 | 1 | 0.39% |
|  | 32x32 | 256 | 1 | 0.39% |
|  | 64x64 | 1024 | 1 | 0.10% |
| Planar | 8x8 | 64 | 4 | 6.25% |
|  | 16x16 | 256 | 4 | 1.56% |
|  | 32x32 | 256 | 4 | 1.56% |
|  | 64x64 | 1024 | 4 | 0.39% |
| DMM Mode-1 | 8x8 | 64 | 4 | 6.25% |
|  | 16x16 | 256 | 4 | 1.56% |
|  | 32x32 | 256 | 4 | 1.56% |
|  | 64x64 | n/a | n/a | n/a |

The types of operations and number of those operations for the proposed method and HTM 7.0r1 are as follows:

Table 2: Comparison of involved operation types and number of that operations in HTM 7.0r1 and proposed method

| Prediction Mode | Block Size | Operation types and number of that operations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HTM 7.0r1 |  |  | Proposed |  |  |
|  |  | Addition | Division | Branch | Addition | Shift | Branch |
| DC | 8x8 | 126 | 1 | 64 | - | - | - |
|  | 16x16 | 510 | 1 | 256 | - | - | - |
|  | 32x32 | 510 | 1 | 256 | - | - | - |
|  | 64x64 | 2046 | 1 | 1024 | - | - | - |
| Planar | 8x8 | 126 | 1 | 64 | 4 | 1 | - |
|  | 16x16 | 510 | 1 | 256 | 4 | 1 | - |
|  | 32x32 | 510 | 1 | 256 | 4 | 1 | - |
|  | 64x64 | 2046 | 1 | 1024 | 4 | 1 | - |
| DMM Mode-1 | 8x8 | 126 | 2 | 64 | - | - | - |
|  | 16x16 | 510 | 2 | 256 | - | - | - |
|  | 32x32 | 510 | 2 | 256 | - | - | - |
|  | 64x64 | n/a | n/a | n/a | n/a | n/a | n/a |

# Modifications of Draft Specification Text

H.8.4.4.3 Depth value reconstruction process

…

- 3. The variable log2SubSample is set equal to ( nT < 32 ) ? 0 : 1.

- 4.  For p in the range of 0 to 1, inclusive, the variable dcPred[ p ] is derived as specified in the following:

sumPred = 0
		numPred = 0. 
		for( x = 0; x < ( nT >> log2SubSample ); x++ ) { 
			xS  = x << log2SubSample  
			for ( y = 0; y < ( nT >> log2SubSample ) ;y++ ) { 
				yS = y << log2SubSample .
				if ( p = = wedgePattern[ xS ][ yS ] ) {
					sumPred += predSamples[ xS ][ yS ]
					numPred += 1
				}
			}
		dcPred[ p ] = ( numPred > 0 ) ? ( sumPred / numPred ) : 0

If intraPredMode is equal to Intra_DC, the following applies.

dcPred[ 0 ] = predSamples[ nT-1 ][nT-1 ]

If intraPredMode is equal to Intra_Planar, the following applies.

dcPred[ 0 ] = ( predSamples[ 0 ][ 0 ] + predSamples[ 0 ][ nT–1 ] + predSamples[ nT–1 ][ 0 ] 							+ predSamples[ nT–1 ][ nT–1 ]+2 ) >> 2

If intraPredMode is equal to Intra_DepthPartition( 35 ), the following applies.

dcPred[ wedgePattern[ 0 ][ 0 ]] = predSamples[ 0 ][ 0 ]

dcPred[ wedgePattern[ nT-1 ][ 0 ]] = predSamples[ nT-1 ][ 0 ]

dcPred[ wedgePattern[ 0 ][ nT-1 ]] = predSamples[ 0 ][ nT-1 ]

dcPred[ wedgePattern[ nT-1 ][ nT-1 ]] = predSamples[ nT-1 ][ nT-1 ]

# Simulation Results

The proposed method is implemented on top of HTM-7.0r1 and compared with HTM 7.0r1 using CTC [2] and “Intra-only” configurations.

Table 3: BD rate results for 3-view case under CTC

| Sequences | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | 99.6% | 100.8% |
| Kendo | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 99.6% | 100.4% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | 100.2% | 101.9% |
| GT_Fly | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 101.0% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 100.8% | 101.0% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 101.2% |
| Undo_Dancer | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% | 100.0% | 100.9% |
| 1024x768 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 99.8% | 101.0% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 101.0% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 100.1% | 101.0% |

Table 3: BD rate results for 3-view case under Intra-only

| Sequences | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.9% | 100.0% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 98.5% | 99.5% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 100.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 102.8% | 100.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 99.6% | 98.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 102.7% | 99.7% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.9% | 99.0% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 99.7% | 99.9% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 101.3% | 99.5% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.6% | 99.6% |

# Conclusion

In this contribution a simplified DC calculation method for SDC is proposed to reduce the computational complexity. The proposed method reduced the number of samples to be used and the number of addition operation. Furthermore, the proposed removed the accumulation, branch, and division operations by using a simple weighted mean equation or a just selection of particular sample. The experimental results show that the proposed method achieves 0.1% coding gain for synthesized views in CTC. At the same time, the proposed modification clearly achieves computational complexity reduction.

# References

- X. Zheng, Y. Lin, and J. Zheng, “Reference samples sub-sampling for SDC and DMM,” JCT3V-C0154, Geneva, CH, 17–23 Jan. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Geneva, CH, 17–23 Jan. 2013.

# Patent rights declaration

Kwangwoon University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).