| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0120 |
| --- | --- |

| Title: | CE5.H: Results on DMM and RBC depth intra modes performance (Test1) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philipp Merkle Karsten Müller | Email: | philipp.merkle@hhi.fraunhofer.de, karsten.mueller@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the results of the tool test in CE5.H on the performance of DMM and RBC depth intra modes.

# Introduction

According to the description in section 3.1 of the CE5 description [1], the purpose of Test1 is investigating the performance of the five bi-partition modes, referred to as DMM1~4 and RBC. For this purpose the RD and runtime performance is evaluated for disabling individual modes or complete tool sets as specified in Table 1. Here, the modes are only disabled in the encoding process, such that signaling and decoding is still supported, but not used.

Table 1. Test1 configuration

| Test | DMM1 | DMM2 | DMM3 | DMM4 | RBC | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1-A1 | off | on | on | on | on | disable mode via macro |
| 1-A2 | on | off | on | on | on | disable mode via macro |
| 1-A3 | on | on | off | on | on | disable mode via macro |
| 1-A4 | on | on | on | off | on | disable mode via macro |
| 1-B1 | on | on | on | on | off | disable mode via macro |
| 1-B2 | off | off | off | off | on | disable DMM via config parameter |
| 1-C | off | off | off | off | off | disable DMM via config parameter and RBC via macro |

# Test1 results

The following tables summarize the experimental results for Test1 under CTC [2] and all-intra configuration with HTM-7.0r1. Additionally, the percentage distribution of intra coding modes for depth blocks is analyzed for Test1. Furthermore, the potential of individual modes is tested by only enabling one mode at a time, using test 1-C with all five modes disabled as the reference.

Table 2. Test1 results for 3-view scenario under CTC: 
BD-rates and runtimes

| Test | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| 1-A1 | 0.0% | -0.1% | 0.4% | 97.1% | 100.4% | 99.7% |
| 1-A2 | 0.0% | 0.0% | 0.1% | 99.6% | 102.1% | 100.5% |
| 1-A3 | 0.0% | 0.0% | 0.1% | 99.1% | 102.2% | 100.2% |
| 1-A4 | 0.0% | 0.0% | 0.1% | 99.6% | 101.7% | 99.9% |
| 1-B1 | 0.0% | 0.0% | 0.2% | 99.8% | 100.4% | 100.1% |
| 1-B2 | 0.1% | -0.1% | 1.6% | 94.5% | 100.3% | 99.6% |
| 1-C | 0.1% | -0.2% | 2.7% | 94.4% | 98.0% | 99.9% |

Figure 1. Test1 results for 3-view scenario under CTC: 
percentages of depth intra blocks

Table 3. Inverse results for 3-view scenario under CTC: 
BD-rates and runtimes with reference 1-C

| Test | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| DMM1 only | -0.1% | 0.2% | -2.4% | 103.0% | 100.6% | 99.4% |
| DMM2 only | 0.0% | 0.0% | -0.4% | 100.4% | 101.0% | 99.6% |
| DMM3 only | -0.1% | 0.1% | -1.9% | 100.7% | 101.2% | 99.8% |
| DMM4 only | 0.0% | 0.0% | -0.5% | 100.5% | 101.5% | 99.9% |
| RBC only | 0.0% | 0.1% | -1.1% | 100.1% | 102.4% | 99.7% |

Table 4. Test1 results for 3-view scenario under all-intra configuration: 
BD-rates and runtimes

| Test | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| 1-A1 | 0.0% | -0.1% | 1.6% | 63.7% | 101.6% | 99.1% |
| 1-A2 | 0.0% | 0.0% | 0.0% | 94.7% | 100.2% | 99.2% |
| 1-A3 | 0.0% | 0.0% | 0.1% | 89.6% | 100.0% | 99.2% |
| 1-A4 | 0.0% | 0.0% | 0.1% | 94.5% | 99.3% | 99.0% |
| 1-B1 | 0.0% | 0.0% | 0.1% | 99.2% | 98.7% | 99.0% |
| 1-B2 | 0.0% | -0.1% | 4.9% | 41.4% | 100.6% | 99.6% |
| 1-C | 0.0% | -0.2% | 7.3% | 40.6% | 100.0% | 99.3% |

Figure 2. Test1 results for 3-view scenario under all-intra configuration: 
percentages of depth intra blocks

Table 5. Inverse results for 3-view scenario under all-intra configuration: 
BD-rates and runtimes with reference 1-C

| Test | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| DMM1 only | 0.0% | 0.3% | -6.4% | 189.5% | 99.6% | 100.0% |
| DMM2 only | 0.0% | 0.1% | -1.0% | 113.4% | 100.2% | 100.7% |
| DMM3 only | 0.0% | 0.2% | -4.5% | 125.8% | 103.6% | 102.7% |
| DMM4 only | 0.0% | 0.0% | -1.7% | 114.0% | 100.3% | 101.2% |
| RBC only | 0.0% | 0.1% | -2.3% | 102.1% | 100.6% | 100.3% |

# Cross check

The cross check of Test1 was done by Qualcomm. They did not find any issues with the source code and the simulation results perfectly matched.

# Conclusion

The test results show that the five bi-partition modes lead to significant bit rate reductions of 2.7% for random access and 7.3% for all-intra configuration. Given these gains, the set of modes does not lead to an increase in decoder runtime. The encoder runtime is increased, however it should be noted that fast mode decision techniques for the set of bi-partition modes have not been adopted yet, even though they are already in use for the regular intra modes. Except for DMM1, disabling only one out of the five modes does not have a substantial impact on the coding performance, as the losses are partially compensated by the other four modes. This observation is confirmed by the additional analysis of depth intra modes distributions for all Test1 configurations. Moreover, the full potential of individual modes is analyzed by the “inverse” experiments, i.e. enabling only one of the five bi-partition modes and taking test 1-C with all modes disabled as the reference. These results indicate that DMM3 is partially overlapping with DMM1, which has already been identified in contributions to the last meeting.

# References

- P. Merkle, "Description of Core Experiment 5 (CE5) on Depth Bi-partition Modes", JCT3V-D1105, Incheon, Korea, April 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-D1100, Incheon, Korea, April 2013.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 4,” JCT3V-D1005, Incheon, Korea, April 2013.

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).