| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0121 |
| --- | --- |

| Title: | CE5.H related: Results on DMM wedgelet prediction unification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle Karsten Müller | Email: | philipp.merkle@hhi.fraunhofer.de, karsten.mueller@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a unification of the wedgelet prediction in DMM2 and DMM3. In particular, the current DMM2 is removed and the redundant intra prediction part of DMM3 is replaced (as it partially overlapped with DMM1). The unified wedgelet prediction always derives an intra angular mode from the co-located texture or left/above neighbouring blocks. For this angle the encoder search and signaling method based on angle-dependent wedgelet lists as part of current DMM3 is applied.

# Introduction

The performance evaluation and proposed tools under test for DMM in CE5.H indicate that the two wedgelet prediction modes referred to as DMM2 and DMM3 can be further simplified and unified.

DMM2 realizes intra wedgelet prediction by continuing the direction of left/above neighbor blocks as a wedgelet in the current block. Furthermore, a refinement of the wedgelet direction in the form of an offset can be estimated at the encoder and signaled in the bit stream. Relative to the gains achieved by DMM2, the specification of this mode is rather complex.

DMM3 realizes inter-component wedgelet prediction. In case the co-located texture block is of type intra and angular mode, the best matching wedgelet from an angle-dependent wedgelet list (a small subset of the full wedgelet list used in DMM1) is estimated at the encoder and the list index is signaled in the bit stream. In case the co-located texture block not of type intra or not angular mode, the best matching wedgelet from a coarse subset of the full wedgelet list is estimated at the encoder and the list index is signaled in the bit stream. This second case is more or less redundant, as the same coarse wedgelet list is also part of the wedgelet search in DMM1.

In order to unify and simplify wedgelet prediction, we propose to remove DMM2 and replace the redundant part of DMM3 by wedgelet intra prediction applying the same approach as the inter-component prediction part of DMM3. This has the advantage that the overlapping and lower performing parts of DMM wedgelet prediction are replaced by a unified and generic method that builds upon the intra angular mode of the HEVC base standard.

# Proposed method

The proposed method removes DMM2 and replaces DMM3 by a unified wedgelet intra prediction method.

The prediction process always results in an intra direction and works as follows: In case the co-located texture block is of type intra and angular mode, it is selected like in current DMM3. Otherwise, the intra angle is predicted from the left and above neighboring depth blocks. Depending on whether these blocks are of type intra or not and whether they are a wedgelet mode or an intra angular mode or not, the intra direction is derived according to Table 1.

Table 1. Derivation of intra direction

| left ref. mode | above ref. mode |  |
| --- | --- | --- |
| wedgelet | wedgelet | maxSlope( Dir(WL), Dir(WA) ) |
| wedgelet | angular/other | Dir(WL) |
| angular/other | wedgelet | Dir(WA) |
| angular | angular | maxSlope( DirL, DirA ) |
| angular | other | DirL |
| other | angular | DirA |
| other | other | maxSlope( HOR, VER ) |

In Table 1 function “Dir(W)” maps the wedgelet direction to the nearest intra direction (same function is used for generating the angle-dependent wedgelet lists); function “maxSlope( Dir1, Dir2 )” analyzes the left and above adjacent samples and if the maximal slope between two samples is found on the left, it returns “Dir1”, otherwise it returns “Dir2”.

Given the predicted intra direction, the processing of DMM3 works as follows: At the encoder the angle-dependent wedgelet list corresponding to the predicted intra direction is searched for the minimum distortion wedgelet, now using WVSD like all the other depth intra modes. The list index of the best wedgelet is signaled in the bitstream like in current DMM3. The number of bins is reduced, now determined by the size of the angle-dependent wedgelet list instead of the coarse subset. At the decoder the depth block is reconstructed with the wedgelet indicated by the transmitted index in the angle-dependent wedgelet list corresponding to the predicted intra direction.

Besides removing several pages of specification for DMM2 and the redundant part of DMM3, the proposed modification of DMM3 is motivated by having one generic wedgelet prediction method that builds upon the intra angular mode of the HEVC base standard. In contrast to the current DMM3, the proposed wedgelet prediction supports flexible coding order (FCO): In case depth is coded before texture, the first case (co-located texture block) can be skipped and the intra direction is always predicted from the left and above neighbor block.

# Experimental results

According to the CE5 description in JCT3V-D1106 [1] and the common test conditions in JCT3V-D1100 [2], the proposed method is implemented in HTM-7.0r1 and evaluated for random access (CTC) and all-intra configuration.

Table 2. Result for 3-view scenario under CTC

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.03% | 100.0% | 99.0% | 100.7% |
| Kendo | 0.0% | 0.0% | 0.03% | 99.5% | 98.9% | 100.4% |
| Newspapercc | 0.0% | 0.0% | -0.24% | 99.4% | 97.2% | 99.6% |
| GhostTownFly | 0.0% | 0.0% | -0.03% | 99.7% | 104.0% | 100.4% |
| PoznanHall2 | 0.0% | 0.0% | 0.00% | 99.4% | 95.8% | 98.3% |
| PoznanStreet | 0.0% | 0.0% | 0.00% | 100.3% | 99.3% | 100.4% |
| UndoDancer | 0.0% | 0.0% | -0.05% | 99.7% | 100.7% | 98.2% |
| 1024x768 | 0.0% | 0.0% | -0.06% | 99.6% | 98.4% | 100.2% |
| 1920x1088 | 0.0% | 0.0% | -0.02% | 99.8% | 99.9% | 99.3% |
| average | 0.0% | 0.0% | -0.04% | 99.7% | 99.3% | 99.7% |

Table 3. Result for 3-view scenario under all-intra configuration

|  | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.00% | 98.6% | 100.5% | 100.1% |
| Kendo | 0.0% | 0.0% | 0.01% | 98.6% | 98.8% | 99.8% |
| Newspapercc | 0.0% | 0.0% | 0.02% | 97.8% | 101.2% | 100.4% |
| GhostTownFly | 0.0% | 0.0% | 0.00% | 99.6% | 101.9% | 101.6% |
| PoznanHall2 | 0.0% | 0.0% | 0.00% | 100.0% | 103.6% | 101.3% |
| PoznanStreet | 0.0% | 0.0% | 0.00% | 98.8% | 97.6% | 100.5% |
| UndoDancer | 0.0% | 0.0% | 0.00% | 99.3% | 99.2% | 100.0% |
| 1024x768 | 0.0% | 0.0% | 0.01% | 98.3% | 100.2% | 100.1% |
| 1920x1088 | 0.0% | 0.0% | 0.00% | 99.4% | 100.5% | 100.8% |
| average | 0.0% | 0.0% | 0.00% | 98.9% | 100.4% | 100.5% |

# Draft specification

The proposed method requires changing the specification in Annex H of JCT3V-D1005 [3]. The specification text can be found in the attached document “JCT3V-E0121_spec.doc”.

# Cross check

The cross check of the proposed method was done by Santa Clara University. They did not find any issues with the source code and the simulation results perfectly matched.

# Conclusion

The results show that the unified wedgelet prediction method has the same performance as the current method, with a slight decrease in encoder complexity. In conclusion, our proposed method dramatically simplifies the specification by removing DMM2 and replaces the redundant part of DMM3 by a unified wedgelet prediction scheme without a loss in coding performance or functionality and should therefore be considered for adoption to 3DV-HTM.

# References

- P. Merkle, "Description of Core Experiment 5 (CE5) on Depth Bi-partition Modes", JCT3V-D1105, Incheon, Korea, April 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-D1100, Incheon, Korea, April 2013.

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 4,” JCT3V-D1005, Incheon, Korea, April 2013.

# Patent rights declaration

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).